3GPP TSG RAN WG2 #132	                                                                                                  R2-2508061 
Dallas, U.S., Nov 17th – 21st, 2025

Agenda item:	10.3.2.3

Title:					Discussion on 6G Radio Initial and System Access

Source:			Fraunhofer IIS, Fraunhofer HHI

Document for:	Discussion

# Introduction

- Single technology framework based on a stand-alone architecture (Note1) …, including: .

  - Energy efficiency and energy saving: both for network and device.

- Physical Layer structure for 6GR,

- Initial access [RAN1, RAN2, RAN4]

    - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4]

    - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4]

- 6GR spectrum utilization and aggregation.  [RAN1, RAN2, RAN4]

The functionalities of 6G Radio system information, paging, initial synchronization and spectrum aggregation have been initially discussed in RAN2#131-bis [1]. Such aspects also form a fundamental area of innovation and discussion for 6G, as they have a direct effect on the energy efficiency goals for 6G. The 6G SID [2] states to study:

These aspects of 6G study are discussed in this contribution, focusing on a RAN2 perspective.

# Energy efficiency of common signals and Initial Access

What happens at 3 a.m. in the least used cells of 5G networks? There is still a lot going on: Synchronization Signals and MIB are repeated, in multiple directions, even when no UE would be listening. And the SIB-1 is broadcast in multiple directions, even when no UE would be listening. The gNB attempts to receive RACH from multiple directions, even when no UE would be sending RACH. Note that in a multi-carrier deployment (as usual) this overhead and extra network energy consumption happens for each carrier.  Notwithstanding, the design of common signals and Initial Access has been identified as the ultimate bottleneck for Network Energy Savings (NES) at low and zero loads.

In fact, a review of the NES techniques captured in the NES study [3] and subsequent standardization would show that many NES techniques are about minimizing the resource footprint of common signals and making them sparser in some domain:

- Time domain: SSB on-demand, standalone SIB-1 on-demand, common signals periodicity adaptation

- Frequency domain (time domain of specific carriers/cells in CA): SSB-less SCells, SIB-1-less SCells, SIB-1 on-demand with anchor carrier, SSB on-demand for SCells

- Spatial domain (coupled with time domain for analog beams): Paging from specific SSB beams, partial beam sweeping (SSB/SIB-1/paging)

However, many of these discussed or standardized NES techniques had significant backward compatibility constraints. Now, in 6G Radio design it is a unique opportunity to focus on energy efficiency from the beginning and make an optimal design without backward compatibility constraints.

From the network perspective, to save more energy, the following principles should apply to the design of 6G Radio:

- SS (synchronization signals), MI (master information), SI (system information), Paging and RACH are sparse by default in time domain, clustered and time aligned to the on-duration a Cell DTX procedure

- Favor frequency-domain multiplexing of SS, MI, SI, Paging instead of time-domain alone

- On multi-carrier co-located deployments SS, MI, SI, Paging and RACH should be present only on coverage layers and omitted whenever possible from capacity layers

The rationale is the following. Sparse common signals in time domain allow the base station (BS) implementation to achieve deeper sleep states than frequent SS/MI/SI/Paging/RACH. The base station also can sleep only if transmission and reception are off, irrespective of common signals or UE specific signals. Thus, alignment to Cell DTX on-duration and smooth integration of Cell DTX to Idle/inactive is a must, as further discussed in our accompanying contribution on EE [4].

A finding based on the energy consumption model of TR 38.864 [4] is that reducing the number of RBs of the transmission does not save much energy compared to a full band transmission. For example, under Cat 1 BS with a 20 RB SSB transmission you still get a relative power consumption value of 155 compared to maximum value 280 with a full band (273 RBs) transmission. So, with 7.3% of resources, you are still consuming 55.3% of the energy. The time occupation instead is vital for energy saving because of micro-sleep and sleep states. Thus, the design of SS, MI, SI, Paging should favor frequency-domain multiplexing over time-domain.

Finally, multi-carrier considerations are probably the most important and most low-hanging fruit to optimize energy efficiency without further service impact. Due to site acquisition and rental costs, co-located multi-carrier deployment is the de-facto standard of deploying more capacity with minimal TCO. Thus, following the 20/80 rule, the 6G design should be optimized for this common deployment. There is no need to send SS/MIB/SIB/Paging on up to 64 beams in FR2 with meager coverage when at the same site there is an 800 MHz carrier which can do the broadcast job with a single beam and full coverage.

Proposal 1: The following common signal design principles are adopted to maximize 6GR (network) EE:

- SS, MI, SI, Paging and RACH are sparse by default on time domain, clustered and time aligned to the on-duration a Cell DTX procedure

- Favor frequency-domain multiplexing of SS, MI, SI, Paging instead of time-domain one

- On multi-carrier co-located deployments SS, MI, SI, Paging and RACH should be present only on coverage layers and omitted whenever possible from capacity layers

At the same time the design of SS, MI, SI, Paging and RACH had profound implications on UE power saving and, also on Initial Access performance, especially in terms of Control Plane latency. Therefore, the design for network EE needs to be carefully balanced between UE design and CP latency.

As in TR 38.840 [5] regarding UE power saving, the I-DRX cycle is crucial for the UE power consumption. The UE needs to wake-up well before the paging occasion in order to re-synchronize. A UE operating under poor radio conditions must wake up several cycles earlier to obtain sufficient synchronization through multiple SS receptions.  In addition, under poor radio conditions, the UE needs to combine multiple PBCH occasions for successful decoding and multiple SIB-1 (multiple redundancy versions) for successful decoding of SIB1. These repetitions and scalability of signals can serve both the network and the UE EE if the principle of Proposal 1 is adopted: preference for frequency-domain multiplexing of SS, MI, SI and Paging instead of time-domain alone.

Figure 1 shows the 5G NR design, where the SSBs and SIB-1s occupy few RBs and need to be transmitted every 20 ms due to initial access constraints. If we consider instead a design where these common signals are sent every 80 ms but all repetitions are done on frequency domain, it results in longer sleep periods for the base station. Also, the UEs can benefit by having more synchronization signals to re-synchronize quickly and to decode MI and SI in one shot. This is shown in Figure 2.

Figure 1 – 5G NR SS/MI/SI design with a common signal periodicity of 20 ms resulting in short sleep periods.

Figure 2 – Potential 6G SS/MI/SI design using FDM for repetitions. For completeness alignment with RACH and Cell DTX are included.

A comparison of the designs in Figure 1 (5G) and Figure 2 (6G) shows that the same number of signals are transmitted:

- The extended sleep time and reduced time utilization allow for increased network energy savings

- More UE power can be saved as the UE does not need to remain awake in poor radio conditions demanding multiple signals for repetitions. The same decoding performance can be expected as the same number of repetitions are available

As a result UEs can achieve the same performance in poor radio conditions – with improved EE.

One aspect that still may be affected by a design as shown in Figure 2 is the initial cell search. As synchronization signals are sparser, also the time spent on each GSCN trying to detect the PSS (primary SS) would increase. This can be compensated in a cell search, e.g. with a high number of GSCNs that are designed to meet the smallest bandwidth possible with appropriate spacing. However, most deployments may have much larger bandwidths, making the search for each individual GSCN during the initial cell search unnecessarily costly..

Observation 1: The Initial Cell Search procedure needs to be improved to compensate for sparse PSS in time domain.

The Initial Cell Search procedure can be improved considering the following two stages:

- In the first stage the UE searches only for the presence of a system. A System Presence Indicator (SPI) is associated to a number of GSCNs.

- Only if the UE detects the SPI then it searches for the corresponding GSCNs, otherwise it can skip the whole block of GSCNs

The SPI can be sent on a sparser grid than the synchronization grid reducing the amount of frequencies that need to be scanned.

Proposal 2: Study enhancements to the Initial Cell Search procedure considering a two-stage approach: first identify whether a 6G system is available and only if a system is present the exact PSS frequency is scanned.

Figure 3: Concept of compensating Initial Cell search performance for sparser PSS in time: in two stages, first find a System Presence Indicator (SPI), then find the PSS frequency.

# Reducing SI overhead – SI layers and SI on-demand

In RAN2#131-bis the following agreements were made regarding System Information [1]:

Agreements on System Information

-	System information design should consider energy efficiency and low complexity for both network and UE.

-	System information design should ensure that the public warning requirements are met.

-	Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs)

-	Study how to improve flexibility and extensibility of scheduling SI design

-	Study mechanisms to improve SI update mechanism

-	Study aspects related to SIB1 size.  Understand the issues and desired content/size from RAN2 point of view.

-	Study areas specific SIB design.  Understand 5G pain points and what improvements can be considered.

NOTE: this is a starting list

In 5G NR, the multi-carrier nature of the deployment is most pervasive in RRC_CONNECTED mode. In RRC_IDLE/RRC_INACTIVE the UE may perform cell re-selection among frequency layers but it does not really know whether a particular cell belongs to a set of cells of the same site/sector. This leads to suboptimal overhead where System Information and Paging have to be broadcast in all layers and PRACH resources have also to be configured in all layers. Such approach is suboptimal both in terms of overhead as well as Network Energy consumption.

Observation 2: 5G NR has suboptimal overhead on co-located multi-carrier deployments.

Instead, 6G can be designed such that high frequency layers avoid the typical broadcast common channels (system information, paging) and instead point to lower frequencies where such information can be obtained. On-demand SIB1 (Rel-19) was a first step in that direction, but with a clean slate design it can be done much more efficiently and have wide adoption.

# Paging

In RAN2#131-bis the following agreement was made regarding paging [1]:

Agreements

Paging design should consider energy efficiency and simplicity for both UE and Network.

Another opportunity to reduce overhead and power consumption in multiple carriers is the paging design. In 5G NR a paging message needs to be sent in any frequency layer where the UE may be camping, even if just temporarily until cell re-selection is performed. This is a significant waste of resources as it is well known that coverage area on different carriers is widely different. Instead, it would be much more efficient to only page on the carriers which have a good coverage and good outdoor-to-indoor penetration properties.  In order to do that, 6G system information allows the UE to distinguish paging layers from non-paging layers.

Proposal 3: 6GR system information allows the UE to distinguish paging layers from non-paging layers

As possible paging and access design strategies in 5G one can favor coverage by configuring the highest priority to a low-frequency carrier (e.g. below 1 GHz) or favor capacity by configuring the highest priority to the carrier with largest bandwidth (e.g C-band). The UE will camp and access via a certain band. In general, however, paging is more efficient when done at frequencies with better coverage whereas the access itself needs to take into account the load on different carriers (PRACH load and traffic channel load)

Observation 3: In 5G NR one has to favor coverage or capacity when configuring layer priorities. However, paging is more efficient when done at frequencies with better coverage whereas the access itself needs to take into account the load on different carriers (PRACH load and traffic channel load)

This trade-off can be greatly improved by decoupling paging layers from access layers, i.e., a UE should be allowed to camp on a layer where paging is more efficient and be re-directed to perform random access on another layer as requested by the network.

Proposal 4: 6GR adopts decoupling paging layers from access layers, i.e., a UE should be allowed to camp on a layer where paging is more efficient and be re-directed to perform random access on another layer as requested by the network.

# Spectrum Aggregation

The following agreements on Spectrum aggregation were made on RAN2#131-bis [1]:

Agreements on Spectrum aggregation

1	Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed.

2	Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc).  Understand the deployment scenarios and problems to address

Long-established operators typically have a large number of fragmented spectrum assets and because of that the spectrum aggregation framework is essential to provide the best possible service to end users. However, the existing spectrum aggregation framework for 5G NR has grown to be overly complex and yet inflexible in a certain sense.

The basis of existing framework comes from a time where bands had similar bandwidths (10 – 20 MHz) and not so different propagation characteristics (e.g. 2.1 vs 2.6 GHz). Nowadays, 5G NR bandwidths differ 3 orders of magnitude (3 – 400 MHz) and propagation characteristics are completely different (e.g. FDD bands vs C-band vs FR2). This needs to be considered on a better aggregation framework for 6GR.

One issue with the existing carrier and carrier aggregation framework is that, in essence, every carrier is designed to be able to support a fully standalone system with that carrier only. Thus, carriers provide the whole support of functionalities and full overhead: SSBs, SIBs, paging, PRACH, RS, etc. Also, these overheads are multiplied by the number of analog beams used by the deployment (up to 8 in FR1, up to 64 in FR2). And yet in most cases carriers are used on a multi-carrier deployment. The sheer number of newly standardized features to turn off or remove functionality from carriers is a great indication that they are not always needed: SCell dormancy (Rel-16), Inter band SSB-less operation (Rel-18), SSB on-demand for SCells (Rel-19), SIB1 on-demand for multi-carrier scenarios (Rel-19). Always using fully-fledged carriers in all bands is unnecessary, reduces capacity and is detrimental to UE and network energy consumption.

Observation 4: Always using fully-fledged carriers in all bands is unnecessary, reduces capacity and is detrimental to UE and network energy consumption.

Another issue with the existing spectrum aggregation mechanism is that UEs tend to be anchored to a layer with more capacity (e.g. on C-band). Unfortunately, these are also typically layers with uplink coverage issues. The introduction of a supplementary uplink band is sought to help solve this issue, but it is not the most robust nor the simplest way to do it. In fact, as shown on the Figure  below, while 5G operates on the philosophy: “capacity first, all bands have duplexing, all bands have common signals”, 6GR could move to the philosophy: “coverage first, duplexing is flexible (multi-carrier), signals on-demand”. This would create a simpler, neater and more robust system.

Figure 4 – 6GR carrier aggregation can be made simpler, reduce overhead, increase capacity and reduce power consumption both at UE and network

While most mobile Internet traffic is downlink (92%),  FDD bands have equal share of downlink and uplink resources, while TDD bands in practice have an asymmetric but still static and unmatching share of resources: 76% DL / 24% UL . So far dynamic TDD did not become practical – full alignment is needed even among operators and DDDSU with S: 10D:2G:2U is most widely used. A wiser strategy to handle current traffic would be to have all (8%) uplink traffic on FDD bands – which anyway have more uplink resources (50%) and much better uplink coverage and use TDD bands for 100% downlink traffic. Given the asymmetry of traffic, the 24% UL time on TDD bands is not gained uplink capacity – it is lost downlink capacity.  Duplexing could be instead achieved with multi-carrier operation.

Observation 5: TDD deployment is practically restricted to use 24% UL time. If this time was used for DL it could increase 30% of peak data rate and capacity. Duplexing could be instead achieved with flexible multi-carrier operation.

Given all these considerations we propose that 6GR spectrum aggregation framework shall consider the following characteristics:

- 6GR is optimized to be deployed with control plane at low frequencies, especially FDD

- Duplexing in 6GR is designed more flexibly with flexible pairing a set of DL and UL bands

- 6GR adopts two different types of carriers: fully-fledged carriers (with SS, SIB, Paging, PRACH, mobility) and simplified carrier (capacity boosting only).

- 6GR Simplified carriers (capacity boosting) can operate in a single duplexing direction (downlink only or uplink only)

Proposal 5: Duplexing in 6GR is designed more flexibly with flexible pairing of DL and UL bands

Proposal 6: 6GR adopts two different types of carriers: fully-fledged carriers (with SS, SIB, Paging, PRACH, mobility) and simplified carrier (capacity boosting only).

Proposal 7: 6GR Simplified carriers (capacity boosting) can operate in a single duplexing direction (downlink only or uplink only)

# Conclusions

We have made the following observations and proposals in this contribution:

Observation 1: The Initial Cell Search procedure needs to be improved to compensate for sparse PSS in time domain.

Observation 2: 5G NR has suboptimal overhead on co-located multi-carrier deployments.

Observation 3: In 5G NR one has to favor coverage or capacity when configuring layer priorities. However, paging is more efficient when done at frequencies with better coverage whereas the access itself needs to take into account the load on different carriers (PRACH load and traffic channel load)

Observation 4: Always using fully-fledged carriers in all bands is unnecessary, reduces capacity and is detrimental to UE and network energy consumption.

Observation 5: TDD deployment is practically restricted to use 24% UL time. If this time was used for DL it could increase 30% of peak data rate and capacity. Duplexing could be instead achieved with flexible multi-carrier operation.

Proposal 1: The following common signal design principles are adopted to maximize 6GR (network) EE:

- SS, MI, SI, Paging and RACH are sparse by default on time domain, clustered and time aligned to the on-duration a Cell DTX procedure

- Favor frequency-domain multiplexing of SS, MI, SI, Paging instead of time-domain one

- On multi-carrier co-located deployments SS, MI, SI, Paging and RACH should be present only on coverage layers and omitted whenever possible from capacity layers

Proposal 2: Study enhancements to the Initial Cell Search procedure considering a two-stage approach: first identify whether a 6G system is available and only if a system is present the exact PSS frequency is scanned.

Proposal 3: 6GR system information allows the UE to distinguish paging layers from non-paging layers

Proposal 4: 6GR adopts decoupling paging layers from access layers, i.e., a UE should be allowed to camp on a layer where paging is more efficient and be re-directed to perform random access on another layer as requested by the network.

Proposal 5: Duplexing in 6GR is designed more flexibly with flexible pairing of DL and UL bands

Proposal 6: 6GR adopts two different types of carriers: fully-fledged carriers (with SS, SIB, Paging, PRACH, mobility) and simplified carrier (capacity boosting only).

Proposal 7: 6GR Simplified carriers (capacity boosting) can operate in a single duplexing direction (downlink only or uplink only)

# References

- “RAN2 Chair Notes”, InterDigital (Chair), 3GPP RAN2 Meeting #131-bis, Prague, Czech Republic, October 13-17, 2025

- RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO (Moderator), 3GPP TSG RAN Meeting #109 Beijing, China, September 15-18, 2025.

- TR 38.864, “Study on network energy savings for NR (Release 18)”, V18.1.0, March 2023.

- R2-2508064, “6G Radio: Energy efficiency by default”, 3GPP RAN2 Meeting #132, Dallas, USA, November 17-21, 2025

- TR 38.840, “Study on User Equipment (UE) power saving in NR (Release 16)”, V16.0.0, June 2019.