3GPP TSG RAN WG2 #132         	                                                                   R2-2508068

Dallas, USA, Nov 17th – 21st, 2025

Source:	CATT

Title:	Considerations on Spectrum Aggregation, Initial and System Access

Agenda Item:	10.3.2.3

Document for:	Discussion and Decision

- Introduction

In RAN2#131bis, spectrum aggregation, initial considerations on system information and paging were discussed. In this contribution, we would like to give further analysis on spectrum aggregation, initial and system access in 6GR.

- Discussion

  - Spectrum aggregation

In RAN2#131bis meeting, the following agreements were reached regarding to deployment scenarios and spectrum aggregation:

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

Agreements on Spectrum aggregation

1	Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed.

2	Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc).  Understand the deployment scenarios and problems to address

Hence, in this section, cell model and spectrum aggregation will be discussed based on standalone deployment. In LTE/5G, in order to improve the peak data rate, CA is well deployed and justified. Hence, in 6G spectrum aggregation, CA can be reused as baseline.

Observation 1: In LTE and 5G NR, CA is well deployed and justified.

In 6G, there may be multiple fragmented carriers within one band. One example of fragmented spectrums in FR1 in Canada is shown in [2]:

Figure-1  Example of fragmented spectrum in one band

Hence, based on 5G CA framework, it can further discuss whether these fragmented spectrums should be modeled as different carriers which belonging to different cells or only one virtual carrier belonging to only one cell (e.g., name it as virtual carrier, and one virtual carrier is composed of multiple fragmented spectrums).

From RAN2 perspective, if each fragmented spectrum is modeled as an different carriers which belonging to different cells, there will be some issues, listed below:

- Each fragmented DL spectrum needs to transmit its own SSB/SIB1 for IDLE UE camping. This is not benefit for network power saving. While in TR38.914[1], energy efficiency is a key performance requirement.

- If each fragmented spectrum is modeled as different carriers belonging to different cell, SCell addition should be used in order to add more resources to CONNECTED UE. SCell addition involves RRC procedure, which brings more latency. This is not benefit for spectrum utilization efficiency. While in TR38.914[1], it clearly stated that 6GR aims to support improved spectrum utilization and operations over one or more carriers/band, compared to 5G NR.

- If each fragmented spectrum is modeled as different carriers belonging to different cell, when UE moves within the coverage range of these different cells, cell change or handover shall be performed. This will introduce signaling overhead compared with aggregating them into one cell.

Observation 2: If each fragmented spectrum carrier is modeled as different carriers belonging to different cell, there will be some issues on energy efficiency, spectrum utilization efficiency and signaling overhead during mobility.

In order to solve the above issues, it had better aggregate fragmented spectrums, e.g., at least fragmented spectrums within the same band, as a virtual carrier. This makes the cell model is compatible with the legacy CA, e.g., one cell includes UL/DL carrier or only DL carrier, and the only difference is that the carrier is virtual carrier. An example is shown in Figure-2. In Figure-2, UE aggregates three cells, e.g., Cell 1, Cell 2 and Cell 3. The virtual carrier belonging to Cell 1 and Cell 2 is composed of only one continuous spectrum similar as in legacy 5G; while the virtual carrier belonging to Cell 3 is composed of two fragmented spectrums.

Figure-2  Example of 6G CA model

Proposal 1: In 6G, CA is supported for both intra and inter-band. On top of 5G CA framework, RAN2 further study extending the definition of a component carrier to be a virtual carrier aggregating fragmented spectrums, i.e. each component carrier may include continuous spectrum or multiple fragmented spectrums.

Aggregating multiple fragmented spectrums into one virtual carrier will also impact RAN1/4, e.g.,:

- Whether there is any restriction in RAN4 on which fragmented spectrums can be aggregated into one virtual carrier?

- Whether there are any technical issues for aggregating multiple spectrum carriers into a single virtual carrier in RAN1 and RAN4?

Therefore, it had better send LS to RAN1/4 to check whether Proposal 1 is feasible from their point of view.

Proposal 2: If P1 is agreeable, send LS to RAN1/4 to inform RAN2 conclusions. Ask them if it is feasible from their point of view.

  - Initial and System Access

SSB transmission

In Rel-15 NR, time-domain positions of transmitted SSBs within a half frame are semi-statically configured. In order to reduce network energy consumption for SSB transmission, on-demand SSB and SSB adaptation in time domain were introduced in NR Rel-19. However, in order to avoid impact on legacy UEs, these mechanisms are constrained to SCell.

Observation 3: In NR, on-demand SSB and SSB adaptation in time domain were introduced in NR Rel-19 to reduce network energy consumption. However, to avoid impact on legacy UEs, these mechanisms are constrained to SCell.

In NR, when a UE performs initial access, the UE assumes that the periodicity of the SSB burst is 20ms. To meet the requirements of initial access, the gNB has to transmit the SSB burst with a periodicity no larger than 20ms for initial access. As a result, even if there are no UE in the cell or the traffic load is very low, the gNB still needs to transmit the SSB burst at least once every 20ms. This prevents the gNB from entering a deep sleep state and increases network energy consumption. In order to achieve more network energy saving, extending the default periodicity of the SSB at least for initial access in 6GR has already been agreed to be studied and evaluated in RAN1 [3].

Observation 4: For the purpose of network energy saving, extending the default periodicity of the SSB (at least for initial access) in 6GR has already been agreed to be studied and evaluated in RAN1.

On-demand SSB for RRC non-connected UEs

With extension of the default periodicity of SSB for initial access, the access delay for non-connected UEs may be increased as well. In order to reduce the access delay, on-demand SSB may need to be considered. For example: Longer periodicity of SSB may impact UE to obtain MIB timely. When a UE reselects to a cell, in order to obtain MIB as quickly as possible, the UE may need to trigger on-demand SSB request for additional SSB transmission.

On-demand SSB for RRC connected UEs

6.1.1.2 NR FR1 - NR FR1 Handover //skip 6.1.1.2.2 Interruption time The interruption time is the time between the end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay. When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Where: //skip T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW. T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW. Tprocessing is time for UE processing. Tprocessing can be up to 20 ms. Tmargin is time for SSB post-processing. Tmargin can be up to 2 ms. TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in table 8.1-1 of TS 38.213 [3]. Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cellin the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the measObjectNRs having the same SSB frequency and subcarrier spacing configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms. If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

Above is an example of interruption time for traditional NR FR1 –NR FR1 handover in TS 38.133 [4]. According to the requirement on interruption time for mobility in connected in NR, the periodicity of SSB has impact on the interruption time. With sparse SSB transmission, the interruption time of mobility in connected may be increased. In order to reduce the interruption time of mobility, on-demand SSB for connected UEs may need to be provided. For most mobility in connected, network-triggered on-demand SSB transmission may be enough. But for conditional handover, UE-triggered on-demand SSB transmission can be considered.

Proposal 3: RAN2 study on-demand SSB transmission for both RRC non-connected UEs and connected UEs.

SIB1

In NR, only one SIB, i.e., SIB1, is specified for Remaining Minimum SI (RMSI). This can reduce the latency of initial access as all information needed for initial access is combined in SIB1. However, SIB1 contains quite a lot of information, e.g. information for camping, information required for initial access, access control parameters, the scheduling of other system information blocks and so on. And during the NR spec evolution, more and more parameters are extended in SIB1. The SIB1 becomes bloated.

In order to reduce network energy consumption, On-demand SIB1 was introduced in NR Rel-19. However, it is not energy efficient enough with bloated SIB1. For example:

- When the network updates the info of SIBx, although the UE only cares about the scheduling info of SIBx in SIB1, the network has to provide the whole SIB1 and the UE has to acquire the complete SIB1.

- When a UE requests other SI, the UE only cares about whether the concern SI is broadcasting or not broadcasting. However, the UE has to acquire the complete SIB1 to obtain the broadcast status of the concern SI.

In order to further reduce network and UE energy consumption, it is beneficial to restructure RMSI. Considering the latency of initial access, RMSI in NR (SIB1) can be restructured into sub-RMSI1 and sub-RMSI2. Sub-RMSI1 contains some necessary information for camping, e.g., cell selection related information, cell access related information, SI scheduling information and so on. The size of sub-RMSI1 is small. While sub-RMSI2 contains other remaining information for initial access, e.g., access control parameters, common downlink parameters of the serving cell, common uplink parameters of the serving cell, and so on. If sub-RMSI1 is small, the network can broadcast with a sparse periodicity and sub-RMSI2 can be transmitted with on-demand. The details can be further studied.

Observation 5: RMSI (i.e., SIB1) in NR has expanded significantly with network evolution, leading to information overload. The current structure of RMSI results in suboptimal energy efficiency.

Proposal 4: In order to further reduce energy consumption and the complexity for both network and UE, RAN2 study to restructure RMSI in 6GR, e.g., RMSI in NR (SIB1) is restructured into sub-RMSI1 (e.g, information for camping) and sub-RMSI2 (e.g, information for access).

SI change notification

In NR, SI change is notified via Short Message. The Short Message includes one bit to indicate the presence of a PWS notification, and another bit indicates SI update other than SIBs for PWS and posSIBs. When a UE receives Short Message that indicates SI update other than SIBs for PWS and posSIBs, the UE has to acquire SIB1 to see whether the concerned SIB(s)(other than SIBs for PWS and posSIBs) are updated or not. Indeed, it is inefficient to use 5G NR SI change notification when the concerned SIB(s) remain unchanged.

In order to improve the efficiency of SI change notification, the following possible options can be considered:

- Option 1: The network indicates SI change notification via LP-WUS.

- Option 2: The network indicates finer granularity of SI change notification via DCI. For example, the network indicates UEs which SIB or SI or SIB/SI groups is changed via DCI.

However, when SI change notification mechanism is studied, the overhead of SI change notification needs to be considered together, which needs to wait for RAN1’s conclusion.

Proposal 5: In order to improve the efficiency of SI change notification, the following potential options can be studied in 6GR. However, the overhead of SI change notification needs to be evaluated together, which needs to wait for RAN1’s conclusion.

- Option 1: The network indicates SI change notification via LP-WUS.

- Option 2: The network indicates finer granularity of SI change notification via DCI.

Paging

Adaptation of paging in time domain was introduced in NR Rel-19. The value range for parameter N, which is the number of paging frames in one paging cycle, is extended to make it possible to have increased interval between PFs. With this, “sleep time” for the gNB can be increased which can reduce network energy consumption as much as possible.

In NR Rel-17, a PDCCH-based paging early indication (PEI) was introduced to reduce unnecessary PDCCH monitoring in POs. With PEI, the group of UEs monitoring the same PO can be further divided into multiple subgroups. If the subgroup to which the UE belongs is paged as indicated via associated PEI, the UE monitors PDCCH in its PO. Since the false paging probability is reduced with subgrouping, the UE can be avoided to decode undesired PDCCH/PDSCH. UE power saving can be achieved accordingly.

In NR Rel-19, low-power wake-up signal and receiver (LP-WUS/WUR) was introduced to further reduce the UE power consumption on paging reception in RRC_IDLE/INACTIVE state. LP-WUS carries subgroup indication and up to 31 subgroups can be configured for LP-WUS. If the UE detects LP-WUS and the LP-WUS is associated with the UE, the UE monitors PDCCH in its PO or PEI. Compared with PEI, LP-WUS in RRC_IDLE/INACTIVE in NR can achieve more UE power saving. On the other hand, it is noted that only the UE power saving gain was achieved with LP-WUS in RRC_IDLE/INACTIVE. The power consumption of network is increased as the network has to transmit the additional LP-WUS/LP-SS besides paging message via Main Receiver (MR). It is not consistent with intention of energy saving in 6GR. Therefore, it is beneficial to consider joint network and UE power saving in 6GR. For example: With LP-WUS, some common signaling/channel transmission via MR can be reduced. Further network energy saving can be achieved.

Observation 6: Adaptation of paging in time domain in NR can achieve network energy efficiency.

Observation 7: Subgrouping mechanism is introduced in both PEI and LP-WUS to reduce the false alarm rate in NR.

Observation 8: Compared to PEI in NR, LP-WUS in RRC_IDLE/INACTIVE in NR can achieve greater UE power saving. However, this gain is achieved at the cost of increased network energy consumption.

Proposal 6: Adaptation of paging in time domain, subgrouping mechanism, and LP-WUS in RRC_IDLE/INACTIVE in NR can all be considered as starting point of paging in 6GR.

- Conclusion

In this contribution, we discuss spectrum aggregation, initial and system access in 6GR. The following observations and proposals are provided:

Spectrum Aggregation

Observation 1: In LTE and 5G NR, CA is well deployed and justified.

Observation 2: If each fragmented spectrum carrier is modeled as different carriers belonging to different cell, there will be some issues on energy efficiency, spectrum utilization efficiency and signaling overhead during mobility.

Proposal 1: In 6G, CA is supported for both intra and inter-band. On top of 5G CA framework, RAN2 further study extending the definition of a component carrier to be a virtual carrier aggregating fragmented spectrums, i.e. each component carrier may include continuous spectrum or multiple fragmented spectrums.

Proposal 2: If P1 is agreeable, send LS to RAN1/4 to inform RAN2 conclusions. Ask them if it is feasible from their point of view.

SSB transmission

Observation 3: In NR, on-demand SSB and SSB adaptation in time domain were introduced in NR Rel-19 to reduce network energy consumption. However, to avoid impact on legacy UEs, these mechanisms are constrained to SCell.

Observation 4: For the purpose of network energy saving, extending the default periodicity of the SSB (at least for initial access) in 6GR has already been agreed to be studied and evaluated in RAN1.

Proposal 3: RAN2 study on-demand SSB transmission for both RRC non-connected UEs and connected UEs.

SIB1

Observation 5: RMSI (i.e., SIB1) in NR has expanded significantly with network evolution, leading to information overload. The current structure of RMSI results in suboptimal energy efficiency.

Proposal 4: In order to further reduce energy consumption and the complexity for both network and UE, RAN2 study to restructure RMSI in 6GR, e.g., RMSI in NR (SIB1) is restructured into sub-RMSI1 (e.g, information for camping) and sub-RMSI2 (e.g, information for access).

SI change notification

Proposal 5: In order to improve the efficiency of SI change notification, the following potential options can be studied in 6GR. However, the overhead of SI change notification needs to be evaluated together, which needs to wait for RAN1’s conclusion.

- Option 1: The network indicates SI change notification via LP-WUS.

- Option 2: The network indicates finer granularity of SI change notification via DCI.

Paging

Observation 6: Adaptation of paging in time domain in NR can achieve network energy efficiency.

Observation 7: Subgrouping mechanism is introduced in both PEI and LP-WUS to reduce the false alarm rate in NR.

Observation 8: Compared to PEI in NR, LP-WUS in RRC_IDLE/INACTIVE in NR can achieve greater UE power saving. However, this gain is achieved at the cost of increased network energy consumption.

Proposal 6: Adaptation of paging in time domain, subgrouping mechanism, and LP-WUS in RRC_IDLE/INACTIVE in NR can all be considered as starting point of paging in 6GR.

- Reference

- TR38.914  Study on 6G Scenarios and Requirement (Rel-20)

- TR38.755  Study on NR FR1 DL fragmented carriers (Rel-19)

- R1-2508186 , Summary #7 of 6GR Energy Efficiency Study, Moderators (Ericsson, MediaTek), RAN1#122bis

- TS 38.133, "NR; Requirements for support of radio resource management"