3GPP TSG RAN WG2 Meeting #132	R2-2508081

Dallas, USA, Nov. 17th - 21st 2025

Agenda Item:	10.3.2.3

Source:	Xiaomi

Title:	On system information, paging, initial access and spectrum aggregation related aspects

Document for:	Discussion and Decision

Introduction

This contribution discusses spectrum aggregation, system information, paging and initial access aspects based on the progress in the last meeting [1].

Discussion

The discussion in this section is handled in the way that the Spectrum aggregation specific aspects are gathered in subclause 2.1, whereas discussions in 2.2/2.3/2.4 correspond to the SI, Paging and initial access related aspects not dependent with spectrum aggregation.

Spectrum aggregation related aspects

In the last meeting, it was concluded to further study the spectrum aggregation related RAN2 impacts with the following design principles: [1]

Agreements on Spectrum aggregation

1	Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed.

2	Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc).  Understand the deployment scenarios and problems to address

Below the impacts to RRC_IDLE and RRC_CONNECTED specific to Spectrum aggregation are discussed respective.

Impacts to RRC_IDLE functionalities

Applying “single cell multiple carrier” (SCMC) modelling in IDLE mode operations

During the last meeting, there were the discussions on whether to support the SCMC modelling for related IDLE mode operations, e.g. random access, paging, SI acquisition, etc. in the multi-carrier deployment. The main motivation is to improve the initial access related operations with below motivations:

- Motivation 1: To improve the energy/signaling efficiency for the “always-on” signaling in 6G, compared with 5G NR.

Lesson from 5G NR is that the SSB needs to be always signaled on the DL frequency by each cell that is intended to provide PCell access for RRC_IDLE/INACTIVE UEs, in order for the UE to first detect the serving cell from the cell's own SSB, camping on it to acquire SIB1 and then perform random access, paging and SI acquisition thereon. Although 5G NR supports the SSB-less operation, it is only supported for SCell in CA and thus is not applicable to save the network energy consumption and signaling overhead for the IDLE mode operations in multiple-carrier deployment. There were also SUL operations which associate both SUL and NUL carriers to the same DL carrier. However, SUL aims only at coverage extension of the NUL carriers (e.g. on FR2), and cannot support a number of DL/UL carriers (e.g. in the capacity layer) associated with a common reference carrier (e.g. in the coverage layer) sharing the same initial access related configurations, so that these carriers still need to broadcast their own “always-on” signaling on their own DL.

Above typically means that a cell in 5G NR cannot provide initial access related resource configurations (e.g. CSS for paging/SI/Random access) on other carriers than its own carrier. Since each serving cell has to broadcast its own SSB and its own SIs (at least SIB1) for IDLE/INACTIVE operation, such a design in 5G NR is not efficient enough from signalling overhead and network energy efficiency perspective and leads to potential resource under-utilization, with each serving cell has to spare its own resource for its own SSB/SIB1/SI transmission (especially when there is no UE actually camping on it).

- Motivation 2: To shorten the UE initial access delay in the multi-carrier deployment.

In order to get access to the carrier(s) in capacity layers (e.g. FR2), the UE in 5G NR typically needs to first reselects to the serving cells on these carriers, acquires system information and perform initial access with related initial access configurations. This will also cause more cell (re)selections, and is not friend to UE power consumption Or, it needs to first get access into the PCell on a carrier in the coverage layer which then adds the SCells in the capacity layer to support the high-throughput requirement. Both of the above manners will lead to extra delay for the UE to get access to the carrier in the capacity layer.

- Motivation 3: To effectively aggregate the fragmented spectrums available for 6G deployment.

It is envisioned that a new Frequency range, i.e. FR3, is to be introduced as one of the main spectrums supporting the deployment of 6G, but it is impossible for the whole frequency spectrum to be available to 6G only, due to some portions of the spectrums in this FR already assigned for other purposes. Also, there is the possibility that some fragmented spectrums re-farmed (e.g. from 2G/3G) with small bandwidth in low bands can be applied for 6G deployment. Since it is possible that such fragmented spectrums may with comparatively small bandwidth, if the broadcast “always-on” signaling thereon leads to resources under-utilization, thus unable to realize high-throughput transmission with the aggregation of these fragmented spectrums.

Observation 1 [Lessons learnt from 5G]: In 5G NR, each serving cell for IDLE/INACTIVE mode has to signal “always-on” signaling (i.e. SSB/SIB1) of its own. This leads to inefficient “always-on” signaling design from energy consumption and system overhead perspective, results in additional delay for UE’s camping and access to the intended carrier and cannot effectively support the aggregation of fragmented spectrums. 6G initial access design should consider improvements w.r.t. above aspects in multi-carrier deployment scenario.

By the SCMC model, the carriers that can share the same synchronization reference (e.g. intra-frequencies/adjacent inter-frequencies, frequencies with collocated gNB deployment, etc.) can be modelled into a single cell: the “always-on” signaling (e.g. SSB and SIB1) can be broadcast on the specific carrier which functions as the reference frequency for synchronization and MIB/SIB1 acquisition, provides the SSB for synchronization and resources configurations for initial access (e.g. PRACH configuration, common search space configuration, etc.) applicable on the other frequencies. With such an SCMC modeling, the limitation in 5G NR as shown above could be addressed:

- The “always-on” signaling does not need to be broadcast on every carrier in the cell but only on the reference frequency(ies), thus saving the signaling overhead and also network energy consumption.

- UE is allowed to directly perform initial access on the other carriers (e.g. capability layer) per the configurations provided by the reference frequency (e.g. coverage layer), without extra access delay.

- Resource occupation by the “always-on” signaling can be avoided on the fragmented spectrums with limited bandwidth, thus enabling the spectrums fully utilized for data transmission.

As the cell is defined by the PCI and need be detected by the IDLE UE itself by decoding the SSB, the feasibility of applying the SCMC model for IDLE mode functionality in the multi-carrier deployment depends on whether it is feasible/allowed for the cell-defined SSB to be broadcast only by a specific reference carrier across all the carriers sharing the same PCI. This issue has tight relationship with the deployment scenarios, w.r.t. collocated vs. non-collocated deployment, and inter-band vs. inter-band carriers, which are further related to the potential impacts on synchronization, measurements and PCI planning.

- Collocated multi-carrier deployment

Figure 1 shows examples for the collocated Multi-carrier deployment scenario. It can include the case of collocated inter-band frequencies (e.g. carriers in the capacity layer collocated with the carrier in coverage layer) and the case of collocated intra-band/adjacent inter-band carriers (e.g. several carriers with similar coverage collocated with each other). In the collocated multi-carrier deployment, multiple carriers can share the same timing and common measurement which are acquired/maintained by the UE based on the SSB signaled by the reference carrier and are applicable to the other carriers. Then, based on the measurement, the UE can determine whether it is within the coverage of each carrier, get synchronized with the timing information, and perform access on the corresponding carrier with the associated initial access configuration provided by the reference carrier (e.g. in SIB1). This typically means that the “always-on” signaling only needs to be broadcast on the reference carrier, and thus justifies the motivation to enable SCMC modelling for IDLE mode operations.

Figure 1 Collocated Multi-carrier deployment

- Non-collocated MC deployment

By contrast, situation becomes more complicated for the non-collocated multi-carrier deployment, with an example shown in Figure 2 below. Typically, different carriers deployed by different Sites/TRPs can hardly (if possible at all) share a common synchronization reference, nor a common measurement result. Take UE1 as an example. Because of different relative positions w.r.t. the sites/TRPs deploying carrier F1 and F2, UE1 may need to:

- acquire/maintain the timing for carrier F2 and another two different timing for carrier F1 (w.r.t. the closest Sites/TRP for carrier F2, and two closest sites/TRPs for carrier F1 in the figure), respectively;

- perform measurements for carrier F1 and F2 separately, in order to determine whether it is within the coverage and thus able to perform the access on the corresponding frequency.

Due to above synchronization and measurements difference across carriers, it is typically the case that for at least synchronization and measurement purposes, the carriers with non-collocated deployment may still need to broadcast their own SSBs, which are hardly able to be incorporated into one common SSB signaled by one reference carrier as in above collocated deployment.

Figure 2 Non-collocated Multi-carrier deployment

Considering the motivation and benefit of applying SCMC modeling for IDLE operation in co-located deployment looks much clearer, RAN2 may study SCMC modeling for IDLE operation in co-located multi-carrier deployment as a starting point, taking into account the cases that the SCMC comprises both capacity layer and coverage layer carriers (e.g. inter-band), and SCMC comprises carriers with similar coverage (e.g. intra-band or adjacent inter-band). At the same time, RAN2 can consider informing RAN1 of the benefit of SCMC for IDLE from RAN2 perspective, and checking with them on the feasibility from RAN1 perspective (w.r.t. e.g. PCI planning, SSB design, sync/measurement etc.)

For non-collocated deployment, since even the motivation of applying SCMC is unclear, companies may need to further justify the motivation before bringing it into the RAN2 study scope.

Support of RACH offloading

In the last meeting, there were contributions proposing to support the RACH offloading in the multi-carrier deployment in 6G, where a UE can acquire from a DL carrier (e.g. reference carrier) the RACH configurations across multiple UL carriers, and chooses in which carrier to perform random access. The motivation is to support improved RACH offloading mechanism, compared with 5G NR.

In 5G NR, RACH offloading was mainly handled by cell reselection which was based on the radio quality and priority handling in cell reselection. The problem is that UE may always choose to camp on a certain frequency (e.g. higher priority frequencies) and may not be able to be offloaded to other carriers, if the measurements of the current serving cell/frequency keeps above a certain level. As the UE is not allowed to switch its access attempt to another UL carrier in 5G NR, it has to perform RACH, if triggered, on the carrier not with best link quality. This leads to impact on the RACH performance. Moreover, if RACH is already overloaded on the current serving cell/carrier, the UE may still need to make its access attempts on the serving cell/carrier, so that it not only suffers from access congestion but also aggravates the RACH overload. This also leads to potential RACH resource under-utilization, if there is another carrier which is much under-loaded.

Therefore, in 6G, RACH load on each carrier needs to be sufficiently considered in the multi-carrier deployment.

Observation 2 [Lessons learnt from 5G]: In 5G NR, the purpose of RACH offloading across multiple carriers is mainly supported by cell reselection which would lead to impacts on UEs’ RACH performance, and impacts the resource efficiency of RACH resources from system-level perspective.

Companies mentioned in the last meeting that the RACH offloading mechanism was already supported in NB-IoT. NB-IoT RACH offloading is to deal with the limited bandwidth on each carrier which further leads to rather limited RACH capacity per carrier; it was designed based on a random carrier selection mechanism that can evenly distribute the RACH loads across carriers within a given coverage level.

Although in 6G there is not similar bandwidth limitation as in NB-IoT, there are more factors that need to be taken when designing the RACH offloading in 6G account. For example, the device type of the UE may be taken into account, so that the UE of a given device type can perform the random access on the intended carrier. As shown above, when the random access is triggered due to service with high-throughput requirement, it is better for the UE to directly perform random access on the carriers in the capacity layer (if the UE is under the coverage), in order to shorten access delay. In this case, the UE’s service requirement and the coverage of the carriers also need to be taken into account. In addition, for the MT services, the carriers where the UE needs to perform paging can also be indicated by the NW, which takes into account the actual service that triggers the UE’s access attempt. Above factors certainly cannot be well handled by NB-IoT RACH-offloading.

Observation 3: In 6G, the RACH offloading design may need to consider factors like e.g., service requirement, device type, coverage, etc. which cannot be directly handled by the RACH offloading mechanism in NB-IoT.

Therefore, it is proposed that in the multi-carrier deployment RAN2 study related mechanism to support RACH offloading across multiple carriers, where a UE can acquire from a DL carrier (e.g. reference carrier) the RACH configurations across multiple UL carriers, and chooses in which carrier to perform random access. RAN2 further study which factors to take into account for RACH offloading design, e.g. coverage, load, service requirements, device type, etc.

Paging offloading

There were also discussions on whether to support paging offloading in the multi-carrier deployment in 6G. Basically, the intention of paging offloading is to avoid repeating the same paging message transmitted cross multiple carriers, thus improving the paging capacity.

In NB-IoT, paging offloading is also supported, with the motivation also to deal with the limited paging capacity due to the narrow bandwidth on each anchor/non-anchor carrier. Specifically, the paging offloading in NB-IoT is performed based on a weight-based carrier selection mechanism, where UEs are offloaded based on the UE ID and the weight associated with each carrier.

In 6G, there may not be the similar problem related paging capacity caused by too narrow channel bandwidth. However, considering that IoT UE is likely to be one of the device type in 6G, it is possible that there would be requirement to support Paging for a large number of IoT UEs. Therefore, the NW may need to offload IoT UEs onto different carriers. Considering also the co-existence of different UE types within a given TA/RNA (if supported), there is further the need for the NW to offload different device types onto different paging carriers. Such a balance across different device types may not be handled directly by the NB-IoT paging offloading mechanism, and needs to be taken into account in 6G paging offloading design. It is also noted that from deployment point of views paging offloading can be supported only across the carriers that can provide full coverage for a given area respectively (e.g. deployment as in Figure 2) and/or among the carriers with same/similar coverage (e.g. in Figure 1, b)). As a result, the coverage the carriers in the multi-carrier deployment scenarios should also be taken into account for the paging offloading discussion.

Therefore, paging offloading should be studied by 6G taking into account the factors like, e.g. device type, per carrier coverage, deployment scenario, etc. which cannot be covered directly by NB-IoT mechanism.

Proposal on Spectrum aggregation in IDLE mode

Based on the above discussions, it is proposed that RAN2 study the support of SCMC modelling for IDLE operations, RACH offloading and paging offloading in the multi-carrier deployment in 6G.

Proposal 1: RAN2 study the following enhancements for IDLE operations in multi-carrier deployment:

- Support of SCMC modelling for IDLE operation in the collocated multi-carrier deployment scenario, where a cell can comprise multiple carriers among which a reference carrier provides “always-on” signaling (e.g. SSB/SIB1);

- Support of RACH offloading in multi-carrier deployment, where UE acquires RACH configurations across multiple UL carriers from a DL carrier and selects a carrier to perform RACH. FFS factors to be considered for carrier selection (e.g. coverage, RACH load, service requirements, device type, etc.);

- Support of paging offloading where the UE can select the carrier to monitor paging with the alignment with the NW. FFS factors to be taken into account (e.g., device type, per carrier coverage, deployment scenarios, etc.).

Impacts to RRC_CONNECTED functionalities

In the last meeting, whether to apply the SCMC modeling to support the multi-carrier operation for an RRC_CONNECTED UE was also discussed. Different from legacy CA framework in 4G/5G, the SCMC modeling enables multiple carriers aggregated within one cell, instead of having to be managed as multiple Cells as in legacy CA, so that within the SCMC UE can only focus on the related operations across carriers without further signaling indicating  which cell is activated/deactivated as in legacy CA at cell level.

During last meeting discussion, main argument to replace legacy CA framework with SCMC modelling from companies was the complicated cell management mechanism and excessive signaling overhead caused, when the UE is configured with a number of carriers in legacy CA. Specifically, the in 5G NR, the signaling for CA configurations involves the signaling levels in "CG configuration >> Serving Cell configurations >> BWP configuration", and there are additional signaling for SCell (de)activation per CG (L2) and for BWP (de)activation per serving Cell (L1 or RRC).

One mechanism to support SCMC modelling was to formulate the multiple carriers within one cell as different BWPs, which then transform “per BWP per serving cell operation” in CA into “multi-BWP handling within the same cell”. Based on this modelling, it seems most of the L2 or RRC operations can imitate/reuse the related operations for SpCell/SCell in legacy CA from RAN2 perspective. To see this, in the below table, we analyze the potential RAN2 impacts to support the multi-BWP handling in a cell based on SCMC framework, showing also the counterpart functionalities in NR CA that might be reused/imitated.

Table 1. Potential RAN2 impacts to support multi-BWP operations and related counterpart operations in NR CA

| Impacted Functionalities |  | Potential impacts/operations to be discussed | Counterpart functionality in NR CA |
| --- | --- | --- | --- |
| Cell/BWP configuration |  | Whether multiple SCMC can be configured (e.g. inter-band SCMCs); If yes, whether configuration signaling simplification/reduction can still be reached. | SCell list configuration BWP list configuration |
| BWP (de)activation | Multiple active BWP | Whether multiple active BWP within the same Cell is supported; Whether signaling based and/or UE autonomous BWP switching are supported. | Multiple activated SCell (to support multiple numerologies in parallel). Signaling based and UE autonomous BWP switching (SpCell only) |
|  | (De)activation signalling | Whether to use L1 signaling, L2 signaling (e.g. MAC CE) and/or RRC signaling for BWP activation/deactivation | BWP activation with DCI and RRC SCell activation/deactivation with MAC CE |
| PUCCH/SR |  | On which BWP(s) PUCCH/SR resources can be configured and used by each BWP | PUCCH Scell and secondary PUCCH group |
| RACH |  | Which BWP(s) can be configured with RACH configurations. Which BWP(s) can be used to perform CBRA and/or CFRA. | CBRA on SpCell only; CFRA only on SCell. |
| PDCP duplication |  | Whether to support PDCP duplication over multiple BWPs within the same cell. | PDCP duplication on different SCells/Carriers |
| RLF/RLM |  | Based on which BWP(s) the RLM/RLF is conducted | RLM/RLF on SpCell only |
| Measurement /Mobility | Meas on Serving Cell | On which BWP(s) the measurements resources are configured and measurement are performed | Per Cell Meas config. and measurement result derivation. Per frequency per Cell Meas config. and measurement result derivation |
|  | Meas on neighbor cell/Freq | Whether per BWP measurement configuration is supported for neigbor cell measurements. |  |

From the Table, there seems no big problem to reuse the corresponding NR CA operations to support the majority of operations of multi-BWP handling or with only slight adaptive adaptation. The main RAN2 impacts to support “SCMC with multi-BWPs” may be located on the signaling design for the configuration and activation of SCMC and associated Multi-BWPs.

Observation 4: Compared with legacy CA framework, main RAN2 impacts to support the Multi-BWP operation in SCMC modeling are located in the configuration and activation aspects for SCMC and associated BWPs.

By contrast, applying the SCMC modeling with carriers handled as multiple BWP may result in or depend on more RAN1/4 impacts than RAN2. For example:

- There may be potential impacts in RAN1 on how to design the cross carrier/multi-carrier scheduling in L1. There are a number of contributions in RAN1 ([2] - [10]) proposing aggregating multiple carriers into the same cells. One of the intentions is to facilitate the multi-/cross-carriers scheduling in a more efficient way. Specifically, the physical resources located in multiple carriers are treated as the resources across different BWPs that belong to the same cell. In this way, the resources can be uniformly numbered in L1, so that the DCI can jointly schedule one or more resources across different carriers as if intra-cell scheduling is performed, w/o need of differentiating carriers anymore.

- There may be potential RAN4 impacts related to the potential requirements in RF/RRM to support SCMC which further impacts the feasibility w.r.t. on which specific bands/carriers can really be aggregated in the same cell. This further impacts the applicability of the SCMC modelling. It is intuitive that the intra-band carriers or carriers cross adjacent band share similar RF characteristics and possibly use the same RF chain to cover, and so may be able to be handled uniformly as in one cell. However, for the inter-band carriers, it may not be feasible to aggregate them into the same cell, due to their different RF characteristics and even the need to of RF chains to cope with them. If inter-band carriers cannot be formulated as one SCMC, there may even be the need to support aggregation of multiple SCMCs each of which aggregates a number of adjacent carriers. If this is the case, the multi-Cell configuration is inevitable (similar to the Scell list in NR), making it questionable on whether SCMC can really achieve the configuration signalling saving gain compared with legacy CA (as in the 1st row of Table 1).

Observation 5: Supporting multi-carrier operation with multi-BWP handling in SCMC may lead to/depend on related RAN1 impacts (e.g. on scheduling), as well as potential RAN4 impacts (w.r.t. e.g., bands/frequencies can be really aggregated into one cell, related RF/RRM requirements, etc.) which impact the feasibility and applicability of SCMC framework.

To this end, considering that RAN2 impact may mainly concern the configuration part which need depend on RAN1/4 design, RAN2 should wait for more RAN1/4 progress on based on which RAN2 may further study the support of SCMC framework from configuration/activation perspective (for e.g. SCMC and multi-BWP configuration/activation), e.g. how to simply the signalling design on top of existing CA framework.

Proposal 2: RAN2 wait for RAN1 and RAN4 progress on whether to support SCMC modelling in RRC_CONNECTED. If it is supported, RAN2 study can focus on the SCMC/Multi-BWP configuration/activation aspects (e.g. how to simply the signalling design on top of existing CA framework).

System information

Discussion on SIB1 content

Agreements on System Information

-	Study aspects related to SIB1 size.  Understand the issues and desired content/size from RAN2 point of view.

In last RAN2 meeting, RAN2 discussed the SIB 1 size. In 5G NR, SIB1 has been upgraded release by release, and comprises an amount of information, leading to a large message size of SIB1. A UE needs to read and decode SIB1 every time when it needs to acquire (updated) system information, and the large size of SIB1 leads to power waste for the UE. Meanwhile, since SIB1 is broadcast more frequently than other SIBs, including unnecessary information in SIB1 would also waste resources.

The SIB1 size optimization related to the SIB1 splitting in the last meeting. With respect to the SIB1 splitting, two motivations were observed:

- to include only truly essential information, ensuring it fits within the size limit;

- to separate cell-specific configurations from area-specific ones, thereby supporting area-specific SIB1 designs for power saving.

For the first motivation above, 6G SIB1 can minimize its size while accommodating the continuous expansion of content, aligned with the core requirement of SIB1 as RMSI (Remaining Minimum System Information). To reduce payload size, 6G SIB1 should retain only the most essential information, e.g., at least provide camping-related information, and the necessary information that determine validity of the SIB, scheduling/resource details for other SI acquisition (if on-demand SI is supported), and scheduling/resource details for paging monitoring. Based on the above principles that includes the first motivation for splitting and the core role of SIB1 as RMSI, the below Table 2 analyzes the usage of each piece of information in SIB1 to determine what must be included:

Table 2: Disassemble NR SIB1 information

| SIB1 information | Essential or not? | Usage | Detailed IEs in NR SIB1 |
| --- | --- | --- | --- |
| PLMN/TA info | Yes | For cell camping: The UE needs to know which operator's network this is in order to decide whether to access it. | plmn-IdentityInfoList, |
| Cell barred info | Yes | For cell camping: The UE must immediately know if the cell is allowed to be accessed. | cellBarredNTN |
| Cell Reservation Info | Yes | For cell camping: Same as above; it is part of the access control and cell camping. | cellReservedForOperatorUse/OtherUse/FutureUse |
| Cell selection Info | Yes | For cell camping: The basic criteria for the UE to judge if the signal is good enough for camping | q-RxLevMin, q-QualMin |
| SI Scheduling info | Yes | For SI scheduling: Without this information, UE cannot know how and when to receive other SIBs, which affect UE’s normal behavior, e.g. cell reselction | si-SchedulingInfo { sib-MappingInfo, si-WindowLength…} |
| DL related configs | Yes | For DL related configuration: The parts that define the DL configurations, e.g. initial BWP and the CORESET#0. Without this, the UE cannot correctly receive downlink data (including paging). | DL related configuration included in ServingCellConfigCommonSIB e.g. frequencyInfoDL, initialDownlinkBWP, bcch-config, pcch-config, ssb-PositionsInBurst,tdd-UL-DL-ConfigurationCommon |
| UL RACH info | FFS | For RACH: Used for UE to initiate access and establishment connection to the network, or request SI (if OD-SI supported). | frequencyInfoUL, initialUplinkBWP |

From the above analysis, to achieve the goal of leveraging the reduced size of SIB1 in 6G and ensuring UEs’ experience of fast network access, discussions on 6G SIB1 can be based on Table 1 above, and the essential information can be retained as the baseline for 6G SIB1 design. Thus, the essential information must include at least information used by the UE for cell camping, and common scheduling/resource configuration information for other SI acquisition and paging monitoring.

Regarding to the UL RACH related info, RAN2 can be further discuss and evaluate whether it still needs to be kept in SIB1. As RACH configuration at least needs to be used for the request of other SIs, then at least the RACH configuration used for SI request needs to be kept in SIB1, in case on-demand SI is agreed for 6G. In 5G NR, the RACH configuration used for SI request, is Similar to 5G NR (e.g. rach-ConfigSI), RAN2 can still assume that it is included in the common scheduling info for other SIs. For the RACH configuration used by UE to perform access upon service initiation, RAN2 may further consider whether it is possible to move them into other SIs (e.g. SIB2) to reduce SIB1 size, considering such RACH configurations are actually used by the UE in an on-demand way, i.e. pending whether the UE really has service initiated.

Therefore, it is proposed that at least the content in the SIB1:

Proposal 3: 6G SIB1 should include at least the following essential common information:

- Information used by the UE for cell camping (e.g. PLMN/TA info, Cell barring info, cell selection info, etc. as in 5G NR);

- Common scheduling/resource configuration information for other SI acquisition and paging monitoring (i.e. SI schedule info, broadcast channel, paging channel related info, validity of SIB, etc. as in 5G NR).

- RACH configuration for SI request (which may be included in SI scheduling info), if OD-SI is supported.

FFS whether RACH configurations used for other purposes can be placed in other SIs.

Area specific SIB and on-demand SIB

In RAN2#131bis, it was agreed to further study the on-demand SI and Area specific SI in 6G. [1]

Agreements on System Information

-	System information design should consider energy efficiency and low complexity for both network and UE.

-	Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs)

-	Study areas specific SIB design.  Understand 5G pain points and what improvements can be considered.

The discussion carried out in the last meeting concerning the support of OD-SI and Area specific SI to both SIB1 and other SIs. Actually, the area-specific SIB design is proposed based on lessons learned from NR: On-demand SI mechanisms introduce in 5G NR (i.e. OD-SI/OD-SIB1) intends for NES purpose, but are unfriendly to UE power saving and/or difficult for operators to deploy. In 5G NR early release, other SI which is not essential SI can be provided on demand. In NES WI, SIB1 can also be provided on demand. Clearly, on-demand SI and on-demand SIB1 are beneficial for network energy saving, but it also causes more UE power consumption and acquisition latency due to having to trigger RACH procedure. There should be a trade-off in between, for example: when some SI is not common for multiple cells and essential for UE to have quick cell access, on-demand request and response may not be the best way and in this case network energy saving may give ways to providing shorter communication latency. One of the drawbacks of OD-SIB1 in 5G NR is that, due to the existence of legacy UEs, when OD-SIB1 is not provided, network has to bar legacy UEs, and this makes it difficult to deploy OD-SIB1 cells in coverage layers.

In a word, from our perspective, in 6G the areas specific SI and OD-SI should be supported together, in order to balance the NW energy saving and UE power saving. Specifically:

- For the SIs other than SIB1, the area specific SI and OD-SI mechanism can be studied for 6G with the mechanisms in 5G NR as the starting point.

- For SIB1, whether it is feasible to support both mechanisms may depend on whether the information included in SIB1 can be area specific, e.g. based on the SIB1 split as discussed in 2.2.1, and the potential latency that is caused to the UE for the camping and access, in case SIB1 is made on-demand.

It should be noted that for OD-SIB1, it is currently unclear whether SIB1 can really be made area-specific, considering that some information may have to be cell specific at least in 5G NR (e.g. CellIdentity). From this perspective, it is proposed that RAN2 first support the area specific SI and OD-SI mechanisms in 6G for the system information other than SIB1, taking 5G NR as baseline, and only study OD-SIB1 until Area-specific SIB1 is firstly justified (based on specific SIB1 content).

Proposal 4: For SI other than SIB1, RAN2 support the area specific SI and OD-SI mechanism in 6G, taking the 5G NR mechanisms as the starting point. OD-SIB1 is not considered until the feasibility of area specific SIB1 is first justified (pending the specific SIB1 content).

Paging

As discussed in [12], in NR, I-DRX is the fundamental mechanism for UE to monitor paging message. We believe this should be inherited in 6G, as continuous paging monitoring clearly would waste more UE power than duty cycle-based monitoring. 5G NR also further optimized UE power consumption on paging monitoring, and various wake-up signals before PO monitoring have been introduced in different releases, e.g. PEI which is based on PDCCH signal and LP-WUS which can be detected using low power receiver. On the one hand, supporting these different wake-up signals obviously increases UE’s complexity; on the other hand, it also makes it difficult for operators to deploy, due to the reason that in the worst case network may need to upgrade and support all these functions as they do not know for each function how many idle state UEs in their network have supported. In 6G, we need to be careful to not introduce too many pre-alert signals.

Observation 6: Two kinds of wake-up signal designs for paging power saving lead to complicated UE implementation and difficulty for operators to deploy in NR.

For paging related network energy saving, in early 5G NR, time-domain evenly distributed paging occasions were specified for network to distribute the paging load. In later NES WI, paging adaptation was introduced to have clustered paging occasions for better network energy saving. However, due to the existence of legacy UEs, network has to support both kinds of paging mechanism and this actually would cause even more network energy consumption. In 6G, we do not have legacy UEs anymore, and a cleaner paging adaptation scheme should be the design target.

Observation 7: Due to legacy UEs, 5G NR has to introduce addition paging adaptation for further network energy saving. Two sets of paging mechanism (evenly distributed and clustered resources) actually do not save much network energy. 6G study should aim at a cleaner paging adaptation scheme.

Proposal 5: In 6G, for paging related UE power saving and network energy saving, RAN2 should try to reduce options among existing 5G solutions, and study solutions based on:

- Paging cycle based DRX monitoring, with single wake-up signal optimization.

- Unified/single paging adaptation scheme (should be considered in 6G day 1 as basic feature.).

In 5G NR, paging among different UEs are distributed onto the band/carrires that are really supported, and this is achieved by the exchange of “UERadioPagingInformation” between CN and gNB or between gNBs. From a system-level perspective, this avoids the NW blindly repeating paging a UE on every carrier in the TAwhich casues paging resource wastage on the carriers/bands the UE does not really support. This needs to be inherited in 6G paging design. However, as discussed in [13], the lesson learnt from 5G is that there may be the problem if the paging capability is generated by the gNB based on UE radio capability and forwarded between network nodes since legacy gNB could not understand the new capability. The issue shall be studied under UE capability framework discussion.

Proposal 6: UE radio paging capability (e.g. supported paging band) exchange between CN and gNB is supported in 6G. How to generate UE radio paging capability can be studied under UE capability framework agenda. FFS on inter-gNB exchange of UE radio paging capability (pending whether to support RAN paging in 6G or not).

Initial access, CP related aspects

The RRC signalling design during initial access is discussed in this subclause, w.r.t. the supported size for the intial access related RRC message in 6G. At the early stage of 5G NR, it was concluded [11][10] that there are two Msg3 message sizes supported during initial access, i.e. 56 bits and 72 bits, which result in two UL logical channels defined for Msg3 transmission, i.e. CCCH and CCCH1, respectively. The 72-bit Msg3 message size was defined specifically to include the RRCResumeRequest1, which includes the full I-RNTI that results in a Msg3 size more than 56 bits (corresponding to RRCResumeRequest/RRCSetupRequest/RRCReestablishmentRequest).

Dating back to the discussions in [11][10], the main reason for such a differential design on CCCH and CCCH1 is the concern on the coverage of Msg3 transmission during initial access phase. Some companies at that time were arguing that the coverage of 5G NR should at least not be worse than LTE, so 5G NR should take the 56-bit Msg3 size, same as LTE, as the baseline. Finally, it was agreed that the supported size of the RRC resume Request message will be indicated in the SIB1 [11], and so it is actually left to the network on whether/how to support the bigger Msg3 size, i.e. 72 bits, with the coverage of Msg3 transmission not impacted.

From the UE perspective, such differentiated Msg3 size and CCCH design is not favourable, as this increases complexity for UE implementation. Alternatively, to support a unified Msg3 size (applicable to all kinds of UL RRC signalling during initial access phase) with coverage also guaranteed is much more friendly to UE implementation. Actually, in the later Releases of 5G NR (i.e. Rel-17), some coverage enhancement mechanisms (e.g. Msg.3 repetition) were specified to enhance Msg3 coverage, and they can function as the candidate solutions that enable a bigger Msg3 size without impacting the Msg3 coverage. Then, with a bigger Msg3 size enabled from coverage perspective, a unified Msg.3 size that can accommodate all kinds of Msg.3 payload can be expected, which further enables the unified UL logical channel design used for initial access. Although these coverage enhancement designs related to initial access are not likely to be deployed in 5G NR due to the introduction at a late stage (w.r.t. the concern on the coverage of legacy UEs), they are technically feasible candidates for the support of a unified Msg3 size and UL logical channel design for the initial access, if they are taken into account from Day1 in 6G.

Therefore, it is proposed that the study on initial access in RAN2 should aim at a uniform design on the message size and logical channel for the RRC signalling during initial access phase to ease UE implementation. This needs coordination with RAN1 on how to address the coverage issue during initial access.

Proposal 7: Study a uniform design on the message size and logical channel for the signalling transfer (e.g. for Msg3) during initial access to ease UE implementation (considering RAN1 progress on the coverage during initial access).

Conclusion

Spectrum aggregation, system information, paging and initial access related aspects are discussed with proposals below:

Spectrum aggregation related aspects

Observation 1 [Lessons learnt from 5G]: In 5G NR, each serving cell for IDLE/INACTIVE mode has to signal “always-on” signaling (i.e. SSB/SIB1) of its own. This leads to inefficient “always-on” signaling design from energy consumption and system overhead perspective, results in additional delay for UE’s camping and access to the intended carrier and cannot effectively support the aggregation of fragmented spectrums. 6G initial access design should consider improvements w.r.t. above aspects in multi-carrier deployment scenario.

Observation 2 [Lessons learnt from 5G]: In 5G NR, the purpose of RACH offloading across multiple carriers is mainly supported by cell reselection which would lead to impacts on UEs’ RACH performance, and impacts the resource efficiency of RACH resources from system-level perspective.

Observation 3: In 6G, the RACH offloading design may need to consider factors like e.g., service requirement, device type, coverage, etc. which cannot be directly handled by the RACH offloading mechanism in NB-IoT.

Observation 4: Compared with legacy CA framework, main RAN2 impacts to support the Multi-BWP operation in SCMC modeling are located in the configuration and activation aspects for SCMC and associated BWPs.

Observation 5: Supporting multi-carrier operation with multi-BWP handling in SCMC may lead to/depend on related RAN1 impacts (e.g. on scheduling), as well as potential RAN4 impacts (w.r.t. e.g., bands/frequencies can be really aggregated into one cell, related RF/RRM requirements, etc.) which impact the feasibility and applicability of SCMC framework.

Proposal 1: RAN2 study the following enhancements for IDLE operations in multi-carrier deployment:

- Support of SCMC modelling for IDLE operation in the collocated multi-carrier deployment scenario, where a cell can comprise multiple carriers among which a reference carrier provides “always-on” signaling (e.g. SSB/SIB1);

- Support of RACH offloading in multi-carrier deployment, where UE acquires RACH configurations across multiple UL carriers from a DL carrier and selects a carrier to perform RACH. FFS factors to be considered for carrier selection (e.g. coverage, RACH load, service requirements, device type, etc.);

- Support of paging offloading where the UE can select the carrier to monitor paging with the alignment with the NW. FFS factors to be taken into account (e.g., device type, per carrier coverage, deployment scenarios, etc.).

Proposal 2: RAN2 wait for RAN1 and RAN4 progress on whether to support SCMC modelling in RRC_CONNECTED. If it is supported, RAN2 study can focus on the SCMC/Multi-BWP configuration/activation aspects (e.g. how to simply the signalling design on top of existing CA framework).

System information related aspects

Proposal 3: 6G SIB1 should include at least the following essential common information:

- Information used by the UE for cell camping (e.g. PLMN/TA info, Cell barring info, cell selection info, etc. as in 5G NR);

- Common scheduling/resource configuration information for other SI acquisition and paging monitoring (i.e. SI schedule info, broadcast channel, paging channel related info, validity of SIB, etc. as in 5G NR).

- RACH configuration for SI request (which may be included in SI scheduling info), if OD-SI is supported.

FFS whether RACH configurations used for other purposes can be placed in other SIs.

Proposal 4: For SI other than SIB1, RAN2 support the area specific SI and OD-SI mechanism in 6G, taking the 5G NR mechanisms as the starting point. OD-SIB1 is not considered until the feasibility of area specific SIB1 is first justified (pending the specific SIB1 content).

Paging related aspects

Observation 6: Two kinds of wake-up signal designs for paging power saving lead to complicated UE implementation and difficulty for operators to deploy in NR.

Observation 7: Due to legacy UEs, 5G NR has to introduce addition paging adaptation for further network energy saving. Two sets of paging mechanism (evenly distributed and clustered resources) actually do not save much network energy. 6G study should aim at a cleaner paging adaptation scheme.

Proposal 5: In 6G, for paging related UE power saving and network energy saving, RAN2 should try to reduce options among existing 5G solutions, and study solutions based on:

- Paging cycle based DRX monitoring, with single wake-up signal optimization.

- Unified/single paging adaptation scheme (should be considered in 6G day 1 as basic feature.).

Proposal 6: UE radio paging capability (e.g. supported paging band) exchange between CN and gNB is supported in 6G. How to generate UE radio paging capability can be studied under UE capability framework agenda. FFS on inter-gNB exchange of UE radio paging capability (pending whether to support RAN paging in 6G or not).

Initial access CP related aspects

Proposal 7: Study a uniform design on the message size and logical channel for the signalling transfer (e.g. for Msg3) during initial access to ease UE implementation (considering RAN1 progress on the coverage during initial access).

Reference

[1]	RAN2#131bis Meeting Minutes

[2]	R1-2506897	Overview of 6GR air interface

[3]	R1-2506918	High-level views on 6GR

[4]	R1-2507013	Overview of 6GR air interface

[5]	R1-2507057	Overview of 6GR air interface

[6]	R1-2507104	Outline and highlight of 6GR air interface

[7]	R1-2507175	Overview of 6GR air interface

[8]	R1-2507334	Overview of 6GR air interface

[9]	R1-2507606	Overview of 6GR air interface

[10]	R2-1811001 Report of 3GPP TSG RAN2#102 meeting, Busan, Korea

[11]	R2-1811002 Report of 3GPP TSG RAN2 NR AdHoc 1807 meeting, Montreal, Canada

[12]	R2-2508084 Discussion on 6G energy efficiency, Xiaomi

[13]	R2-2508076 UE Capability pain points and considerations, Xiaomi