3GPP TSG-RAN WG2 Meeting #132	R2-2508084

Dallas, USA, Nov. 17th -21st, 2025

Agenda Item:	10.3.3.2

Source:	Xiaomi

Title:	Discussion on 6GR energy efficiency

Document for:	Discussion and Decision

# Introduction

In RAN#108 meeting, RAN WG study item on “Study on 6G Radio” [1] was approved. In the SID, following energy efficiency objective is mentioned.

The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

In the last RAN2 meeting, 6G energy efficiency was briefly discussed and following guidance was captured in the chairman notes.

Design goal 1. Strive to develop solutions that consider network energy efficiency and UE power saving For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose. Focus on solutions that have RAN2 impacts.

In this contribution, we would go through NR UE/NW power saving features and try to identify which features can be considered in 6G study.

# Discussion

## 2.1 UE power saving features

In NR, UE power saving has been enhanced from Rel-15 to Rel-19 with following summarized features.

| Release | UE power saving features | Remark |
| --- | --- | --- |
| Rel-15 | i-DRX | Duplicated functions: Related to paging power saving for RRC_IDLE and RRC_INACTIVE Network energy: May be improved |
|  | C-DRX | Duplicated functions: Related to control channel monitoring in RRC_CONNECTED Network energy: May be improved |
|  | BWP switch | Duplicated functions: Related to control channel monitoring in RRC_CONNECTED Network energy: Small impact (e.g. PDCCH-based switch) |
|  | UE overheating assistance information report | Network energy: No impact |
| Rel-16 | DCP: WUS signal for C-DRX adaptation | Duplicated functions: Related to control channel monitoring in RRC_CONNECTED Network energy: Impacted (e.g. extra WUS transmission) |
|  | Cross slot scheduling | Duplicated functions: Related to control channel monitoring in RRC_CONNECTED Network energy: No impact |
|  | UE power saving UAI report | Network energy: No impact |
|  | Idle/Inactive state RRM measurement relaxation | Network energy: No impact |
| Rel-17 | PEI | Duplicated functions: Related to paging power saving for RRC_IDLE and RRC_INACTIVE Network energy: Impacted (e.g. extra PEI transmission) |
|  | Provision of Connected state TRS configurations to Idle/Inactive-state UEs | Duplicated functions: Related to paging power saving for RRC_IDLE and RRC_INACTIVE Network energy: Impacted |
|  | RLM/BFD relaxation | Network energy: No impact |
|  | Connected state RRM measurement relaxation | Network energy: No impact |
|  | SSSG switching | Duplicated functions: Related to control channel monitoring in RRC_CONNECTED Network energy: No impact |
|  | PDCCH skipping | Duplicated functions: Related to control channel monitoring in RRC_CONNECTED Network energy: No impact |
| Rel-18/19 | LP-WUS for idle mode and connected mode operation | Duplicated functions: Related to paging power saving for RRC_IDLE and RRC_INACTIVE Duplicated functions: Related to control channel monitoring in RRC_CONNECTED Network energy: Impacted (e.g. extra LP-WUS transmission) |

In what follows, we will categorize these features for different purposes and try to analyse their impact to the network side.

(1) Power saving for RRC_IDLE and RRC_INACTIVE

In NR, paging monitoring in RRC_IDLE and RRC_INACTIVE states is based on DRX cycle, and paging related UE power saving enhancements include PEI, LP-WUS, and TRS configuration in Idle/Inactive states. Both PEI and LP-WUS are wake-up signals, and the difference is that they are designed for different UE receives, i.e. MR and LR. RAN2’s main specification impact is to define UE’s behavior after detecting the WUS signal. The power saving gain for the UE comes in the case when no paging message is sent for the UE’s subgroup and UE can skip monitoring paging PDCCH and paging messages after detecting WUS message. The network impact however is that network is supposed to always send a WUS signal before the PO that carries paging messages, which actually increases network’s energy consumption. In 6G, to study paging power saving using wake-up signal, 3GPP should aim at a single WUS solution instead of having two for the same purpose and meanwhile try to not increase too much network energy consumption. The detailed wake-up signal design can be left to RAN1.

Another paging power saving feature is providing TRS configuration in Idle and Inactive states, and it can reduce UE’s power consumption on SSB synchronization before paging monitoring. RAN2’s work is mainly to define the SIB broadcasting signaling, and the impact to the network is that network needs to broadcast and indicate the availability of TRS configuration, which somehow increases network’s energy consumption especially when TRS configuration is large considering so many connected state UEs. We think that study of this feature can have lower priority than wake-up signal.

Lesson learnt from 5G (1): Duplicated power saving mechanism for RRC_IDLE and RRC_INACTIVE, e.g., DRX, PEI, LP-WUS, TRS

Proposal 1: In 6G, regarding UE power saving for RRC_IDLE and RRC_INACTIVE, RAN2 should try to reduce options among existing 5G solutions, and study solutions based on

- duty cycle-based paging monitoring

- single wake-up signal meanwhile taking network’s energy consumption into account

- providing connected state TRS configuration for Idle/(Inactive) state UEs (with lower priority)

(2) RRM/RLM/BFD relaxation

RRM/RLM/BFD relaxation enables UE’s power saving when the certain criteria are fulfilled, while still meeting the requirements defined in RAN4 spec. The main network impact is to configure some relaxation criteria (e.g. not-cell-edge, low-mobility, stationary, etc.) for UE to evaluate, which does not increase too much network energy consumption. These features are for different purposes and should be inherited to 6G.

Proposal 2: RRM/RLM/BFD relaxation should be considered and studied in 6G.

(3) Connected state control channel monitoring

In NR, connected state UE can monitor PDCCH using C-DRX to save UE power. For further UE power saving, DCP and LP-WUS are introduced in later releases. Note that LP-WUS operation with option 1-1 works exactly the same way as DCP in terms of starting the onDurationTimer, so they are literally the parallel features introduced for the same purpose. While WUS operation benefits UE’s power saving, it actually increases network’s energy consumption as network needs to send the extra wake-up signal. In 6G, study of WUS operation for connected state PDCCH monitoring should aim to have a single WUS solution and should also address the concern from extra network energy consumption.

Other control channel monitoring solutions include PDCCH skipping and search space group switching. These are purely RAN1 features that have very limited RAN2 impact. We can leave this to RAN1 study.

Lesson learnt from 5G (2): Duplicated power saving mechanism for RRC_CONNECTED, e.g., C-DRX, LP-WUS, DCP, PDCCH skipping, SSSG switching, Cross slot scheduling, BWP switching

Proposal 3: In 6G, regarding UE power saving for RRC_CONNECTEDE, RAN2 should try to reduce options among existing 5G solutions, and study solutions based on

- C-DRX

- single wake-up signal meanwhile taking network’s energy consumption into account

(4) Indication of UE power saving preference

As discussed in [3], In 5G, UAI message has been enhanced to indicate the overheating issue and UE’s power saving preference on various radio configurations (e.g. DRX configuration, maximum bandwidth/CC/MIMO layers, release preference, etc.). However, there’s no clear network’s action defined for the reported UE preference on power saving, and it is possible that network can ignore UE’s indication, which makes UAI not that useful. Note that some configuration’s preference (e.g. maximum bandwidth/CC/MIMO layers) may be accompanied with the corresponding capability update, in which case, it would be more desirable that network can respect UE’s indication, otherwise it would cause configuration failure. In 6G, we think that indication of UE power saving preference should be studied with UAI message as baseline and it can be further discussed how to indicate the power saving preference related to capability update.

Lesson learnt from 5G (3): UE preference on power saving reporting in UAI without clear network behaviour specified, which raises a risk for those information being ignored by the network

Proposal 4: UE power saving preference indication should be studied in 6G, with a high-level specified network behaviour that the network should respect/follow the received indication for network configuration.

## 2.2 Network energy saving features

In NR, network energy saving has been specified in Rel-18 and Rel-19, and includes the following features:

| Release | NES features | Remark |
| --- | --- | --- |
| Rel-18 | Cell DTX/DRX | UE power saving: Improved |
|  | SSB-less SCell | Duplicated functions: SSB reduction UE power saving: No impact |
| Rel-19 | OD-SIB1 | UE power saving: Impacted (e.g. sending OD-SIB1 request) See [2] |
|  | OD-SSB for SCell | Duplicated functions: SSB reduction UE power saving: No impact |
|  | SSB adaptation | Duplicated functions: SSB reduction UE power saving: No impact |
|  | Paging adaptation | UE power saving: Small impact (e.g. receiving separate configuration in SIB) |
|  | RACH adaptation | UE power saving: Small impact (e.g. receiving separate configuration in SIB) |

We will analyze these NR NES features for their UE impact and see whether they can be inherited in 6G. Note that SIB-related part will be discussed in our separate contribution [2].

(1) cell DTX/DRX

In NR, cell DTX/DRX is introduced only for connected state UEs and it enables network energy saving by not transmitting PDCCH and SPS in cell DTX’s inactive period and not receiving CG and SR in cell DRX’s inactive period. RAN2’s main impact is to provide cell DTX/DRX configurations (e.g. cell DTX/DRX pattern, activation/deactivation state, etc.) to the connected state UEs and to configure NES-specific CHO. Regarding the UE impact, cell DTX/DRX actually can work pretty well with UE DRX as long as their patterns overlap to some extent. This means that UE DRX and cell DTX/DRX can be considered as a joint UE/NW power saving feature, matching exactly with the 6G’s SID energy efficiency objective. In 6G, we can even study the feasibility of extending cell DTX/DRX to Idle/(Inactive) state, if appropriate.

Proposal 5: NR Cell DTX/DRX should be considered and studied in 6G, with potential extension and improvement to Idle/(Inactive) state operation.

(2) SSB-less SCell/on-demand SSB/SSB adaptation

In NR, all these SSB-related NES operations are applicable only to SCell. Among these, SSB-less SCell works for intra-band or inter-band CA where network can configure a reference cell for the UE to obtain timing reference and AGC source for the SSB-less SCell. For on-demand SSB, the main RAN2’s impact is to L2/L3 signal and determine the activation/deactivation state of OD-SSB transmission. For the SSB adaptation, RAN2’s impact is the SMTC selection based on the L1 SSB adaptation indication. These features are more or less similar and the impact on UE’s power consumption is rather limited. If CA framework is reused in 6G, we think SSB adaptation for SCell can be studied, but should have a unified solution instead of having them all. In addition, in 6G, to maximum network energy saving, we can also study the feasibility of SSB adaptation for PCell, or even for Idle/(Inactive) state, if appropriate.

Lesson learnt from 5G (4): Duplicated network energy saving mechanism on SSB reduction, e.g., SSB-less SCell/on-demand SSB/SSB adaptation

Proposal 6: In 6G, regarding network energy saving on SSB reduction, a single SSB adaptation solution for SCell (aligning SSB-less SCell/on-demand SSB/SSB adaptation) should be considered and studied in 6G (if 6G reuses the CA framework), with potential extension and improvement to PCell and/or Idle/(Inactive) state operation.

(3) PRACH adaptation

In NR, PRACH adaptation enables base station’s longer sleep and thus network energy saving by having more clustered RACH occasions. This feature has marginal impact on UE’s performance and the RAN2’s spec impact is just to configure the additional RACH resources, due to the fact that legacy UEs will still use the legacy RACH resources. In 6G, we don’t have the legacy UE issue and we can have a cleaner PRACH adaptation solution.

Lesson learnt from 5G (5): in 5G, PRACH adaptation does not work well since it is introduced in the late stage, and network has to consider legacy UEs.

(4) Paging adaptation

In early 5G NR, time-domain evenly distributed paging occasions are specified for network to distribute the paging load. In later NES WI, paging adaptation has been introduced to have clustered paging occasions for better network energy saving. However, due to the existence of legacy UEs, network has to support both kinds of paging mechanism and this actually leads to even more network energy consumption. In 6GR, we do not have legacy UEs anymore, and a cleaner paging adaptation scheme should be the design target.

Lesson learnt from 5G (6): in 5G, paging adaptation does not work well since it is introduced in the late stage, and network has to consider legacy UEs.

Proposal 7: PRACH adaptation and paging adaptation should be considered and studied in 6G day 1 as basic feature.

# Conclusion

Based on the discussion, we have following proposals:

UE power saving features:

Lesson learnt from 5G (1): Duplicated power saving mechanism for RRC_IDLE and RRC_INACTIVE, e.g., DRX, PEI, LP-WUS, TRS

Proposal 1: In 6G, regarding UE power saving for RRC_IDLE and RRC_INACTIVE, RAN2 should try to reduce options among existing 5G solutions, and study solutions based on

- duty cycle-based paging monitoring

- single wake-up signal meanwhile taking network’s energy consumption into account

- providing connected state TRS configuration for Idle/(Inactive) state UEs (with lower priority)

Proposal 2: RRM/RLM/BFD relaxation should be considered and studied in 6G.

Lesson learnt from 5G (2): Duplicated power saving mechanism for RRC_CONNECTED, e.g., C-DRX, LP-WUS, DCP, PDCCH skipping, SSSG switching, Cross slot scheduling, BWP switching

Proposal 3: In 6G, regarding UE power saving for RRC_CONNECTEDE, RAN2 should try to reduce options among existing 5G solutions, and study solutions based on

- C-DRX

- single wake-up signal meanwhile taking network’s energy consumption into account

Lesson learnt from 5G (3): UE preference on power saving reporting in UAI without clear network behaviour specified, which raises a risk for those information being ignored by the network

Proposal 4: UE power saving preference indication should be studied in 6G, with a high-level specified network behaviour that the network should respect/follow the received indication for network configuration.

Network energy saving features:

Proposal 5: NR Cell DTX/DRX should be considered and studied in 6G, with potential extension and improvement to Idle/(Inactive) state operation.

Lesson learnt from 5G (4): Duplicated network energy saving mechanism on SSB reduction, e.g., SSB-less SCell/on-demand SSB/SSB adaptation

Proposal 6: In 6G, regarding network energy saving on SSB reduction, a single SSB adaptation solution for SCell (aligning SSB-less SCell/on-demand SSB/SSB adaptation) should be considered and studied in 6G (if 6G reuses the CA framework), with potential extension and improvement to PCell and/or Idle/(Inactive) state operation.

Lesson learnt from 5G (5): in 5G, PRACH adaptation does not work well since it is introduced in the late stage, and network has to consider legacy UEs.

Lesson learnt from 5G (6): in 5G, paging adaptation does not work well since it is introduced in the late stage, and network has to consider legacy UEs.

Proposal 7: PRACH adaptation and paging adaptation should be considered and studied in 6G day 1 as basic feature.

# Reference

[1] RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator)

[2] R2-2508081, On system information, paging, initial access and spectrum aggregation related aspects, Xiaomi

[3] R2-2508076, UE Capability pain points and considerations, Xiaomi