3GPP TSG-RAN WG2 Meeting #132                                                           R2-2508087

Dallas, USA, Nov. 17th - 21st

Source:	CATT

Title:	Discussion on 6GR mobility

Agenda Item:	10.4

Document for:	Discussion and Decision

- Introduction

For 6G radio, the mobility aspects should be discussed, according to 6G_WG_SID [1].

- Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

In last meeting, RAN2 has discussion on 6GR mobility and some agreements were reached on the requirements. This contribution will further elaborate on the further detailed design of 6GR mobility.

The contribution is organized as the following. In section 2, we discuss the following aspects,

- Mobility Framework and Targets

- Measurement Framework

- Intra-RAT Connected Mobility

- Cell Selection and Reselection

- Inter-RAT mobility

Our observations and proposals are summarized in Section 3.

- Discussion

  - Mobility Framework and Targets

- Mobility Framework

According to SID [1], the following objective is included:

- Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

To prevent excessive configurations, a unified framework represents a viable solution. Within such a framework, a single configuration could be utilized by the network to provide different indications, enabling the UE to perform various functions. For instance, currently in 5G mobility it often duplicates the same configuration across separate functional settings—such as candidate cell configurations being specified independently in both CHO and LTM configurations. This duplication increases configuration redundancy and elevates signaling overhead. Furthermore, the solutions to improve HO performance are limited in specific handover types, e.g. early TA acquisition is only used for LTM. Even though some solutions could be reused for other handover type, e.g. early CSI acquisition which is introduced for LTM is also introduced for L3 handover, but currently the same solution is with separate configuration for the 2 handover types.

Observation 1: Key lessons learned from 5G mobility design: Redundant design across 3GPP releases (for different handover types), and Solutions to improve HO performance are often limited to specific handover types.

In 6GR, different types of handover are still needed to support diverse deployment scenarios, different service requirements, different device types, different network implementation for resource reservation, and different implementation complexity etc. Leveraging the lessons learned from 5G mobility design, 6GR should design a unified mobility framework which would enable the sharing of configuration structure, core functions, and standard procedures across all supported handover types.

Proposal 1: RAN2 design a unified 6GR mobility framework, aiming at the following for all supported handover types,

- Same signaling structure for configuration

- Maximizing the reuse of same set of building blocks, i.e., functions/procedure steps

- Target performances

In lasting meeting, the following requirement was agreed for 6G mobility

1 Study mobility with the following requirements in 6G mobility design: - Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions. - Robustness of mobility procedures - Energy efficiency for both UE and NW

In the remainder of this section, we elaborate on the detailed solutions to meet the agreed 6GR mobility requirements.

In order to reduce the interruption time in NR, DAPS is supported, but it is not commercial due to implementation complexity. Then in R18 LTM, LTM supports functionalities including pre-configuration, early DL synchronization, early TA (Timing Advance) acquisition, and UE configuration pre-processing to reduce interruption time. Additionally, RACH-less L3 based handover is also supported with limited application scenarios e.g. intra-gNB with TA equal to 0ms or source TA. In order to reduce throughput degradation during handover, early CSI (Channel State Information) acquisition is supported for LTM and L3 handover in NR. And in Release 20, dynamic SCell activation will be supported to further improve the throughput.

However, due to the limited time, and the design is based on backward compatibility, the performance of 5G NR mobility doesn’t reach the ideal performance. The performance degradation is still suffered such as the latency and data loss led in by unnecessary L2 handling at least for intra-DU scenario, and throughput degradation due to the feature deactivation (e.g. MTRP) during handover. Besides, there are emerging new services which have more stringent requirements on latency and service continuity (i.e. minimize throughput degradation during mobility).

The above functionalities supported in NR mobility could effectively improve the handover performance (i.e., reduce interruption time and reduce throughput degradation during mobility) while maintaining reasonable implementation complexity. Thus, these can be adopted as the starting point for 6GR mobility design. We can further explore additional mechanisms to enhance handover performance—ensuring alignment with 6GR mobility requirements—while maintaining reasonable implementation complexity.

To be more specifically, we can consider enhancement on L2 handling (e.g. HARQ buffer continuation) to further reduce the interruption time. And to minimize throughput degradation, from 6G day1 we can consider to support more advanced features during handover (e.g.,mTRP activation during handover)..

Proposal 2: To meet the performance requirements of 6GR mobility, adopt LTM-based solutions as the baseline and support additional enhancements, including

- Solutions to minimize interruption time: early DL synchronization, early TA acquisition, UE configuration pre-processing, and avoiding unnecessary L2 resets such as not flushing the HARQ buffer.

- Solutions to minimize throughput degradation: early CSI acquisition, dynamic SCell activation during handover, and mTRP activation during handover.

  - Measurement Framework

5G NR includes three measurement types: RRM measurements, LTM L1 measurements, and L1 measurements for beam management/channel state information (CSI) reporting.

For 6G NR measurements, first clarify the required types—all three above are recommended to be retained, with justifications as follows:

- RRM measurements: Provide filtered results to avoid network misjudgements from occasional measurement fluctuations. While network-side filtering is proposed, it would add high complexity due to large neighbour cell counts; they also support legacy handovers and cell reselection (preferably via L3 measurements) to prevent ping-pong effects.

- LTM L1 measurements: Help select optimal neighbour beams when serving cell beam quality drops. Combined with L2 commands, they reduce LTM latency and avoid delayed handovers, with minimal data interruption during switching (making ping-pong effects less critical). The network can configure TTT to filter anomalies, and filtering via periodic reports (for limited candidate cells) can be left to network implementation.

- L1 measurements (beam management/CSI reporting): Enable optimal UE scheduling, boosting throughput and data transmission efficiency.

In 5G NR, these measurements use separate frameworks. To clarify these existing frameworks, we compare them across key aspects: Purpose, RS Configuration, Report Type, Configuration Signaling, and Report Signaling. The comparison is shown in the table below.

Table 1 Comparison of different measurement type

|  | Purpose | RS configuration | Report type | Configuration Signaling | Report signaling |
| --- | --- | --- | --- | --- | --- |
| RRM measurement | - For RRC connected/idle/inactive sate cell level mobility - For CA configuration | - SSB, - CSI-RS(Cell level configuration) | - event trigger, - periodic | MeasConfig in RRC message RRCReconfiguration | RRC |
| L1 measurement for beam management | - Beam management, - CSI acquisition - tracking | - SSB, - CSI-RS(BWP level configuration) | - event trigger, - periodic, - semi-Persistent, - aperiodic | CSI-MeasConfig in RRC message RRCReconfiguration | UCI |
| LTM L1 measurement | - For cell level mobility (LTM), - CSI acquisition (LTM) | - SSB, - CSI-RS(Cell level configuration) | - event trigger, - periodic, - semi-Persistent, | RS configuration In LTM-Config-r18 RRC message RRCReconfiguration Report configuration in CSI-MeasConfig | - UCI - MAC CE |

As the table shows, in 5G NR the UE needs separate measurement configurations, independent measurements (on different or same RS), and separate signaling to report results to the network—creating notable redundancy between measurement types. Thus, a unified measurement framework for 6G NR is critical to boost UE power efficiency, to cut implementation complexity, and to improve signaling efficiency.

In this framework, standardize configurations, triggering mechanisms, and reporting signaling where possible. At minimum, it should cover RRM and LTM L1 measurements (both for mobility, within RAN2’s scope).

L1 measurements for beam management fall mainly to RAN1 (discussions decided by RAN1). Integrating them into the framework may be challenging, as they differ greatly from the other two types in RS configuration, report types, and reporting signaling (per the table).

Observation 2: The feasibility of integrating L1 measurements for beam management into the unified measurement framework shall be determined by RAN1.

RRM measurements and LTM L1 measurements can share at minimum RS configurations and certain report type configurations. RAN2 may consider designing unified RS and report type configurations for these two measurement types. Regarding reporting signaling:

- Direct reporting to the CU should be supported, given the CU-DU split architecture.

- Security is critical: Since RRM measurements include cell-level data with PCI information. They must be reported with security protection to prevent location exposure—making RRC signalling-based reporting necessary.

- For neighbouring cell L1 measurements, MAC CE support is essential to ensure scheduling flexibility, especially for event-triggered reporting.

- UCI-based reporting is unnecessary for neighbouring cell L1 measurements, as MAC CE already meets latency requirements.

Therefore, it is proposed that,

Proposal 3: RAN2 design a unified measurement framework that is applicable to RRM measurements and LTM L1 measurements. This unified measurement shall encompass the following aspects:

- Unified configuration of measurement resources (e.g., SSB, CSI-RS)

- Unified triggering mechanism for measurement reporting (e.g., event-triggered, periodic reports)

- Support of measurement report via either MAC CE or RRC signalling

  - Intra-RAT Connected Mobility

- Scenarios

With the evolution from 5G to 6G, mobile communication networks are required to support more diverse application scenarios and user demands—unlike 5G, which primarily focused on balancing coverage, capacity, and basic low-latency needs, 6GR aims to achieve "ubiquitous, intelligent, and high-reliability" mobility management. However, existing 5G mobility solutions have shown limitations in adapting to scenario-specific characteristics: for example, dense user gatherings often lead to signaling congestion and handover delays, high-speed predictable movement (such as rail transit) struggles with frequent cell switching and latency fluctuations, fixed-area mobility lacks optimized resource allocation strategies, and delay-sensitive services (e.g., industrial control, remote surgery) have increasingly stringent requirements for handover stability.​

Against this backdrop, 6GR mobility design cannot rely on a "one-size-fits-all" framework. Instead, it must fully consider the differentiated needs of typical user scenarios and service types. These scenarios not only represent high-frequency real-world usage scenarios but also cover core challenges that 6GR needs to address—including dense user access, high-speed mobility, fixed-area resource optimization, and extreme low-latency services. Therefore, identifying and analyzing these key scenarios is a prerequisite for formulating targeted 6GR mobility solutions, ensuring that the network can provide stable, efficient, and tailored mobility support across different usage contexts.​

Figure 1 key scenarios

Observation 3: The following scenarios shall be considered as key use cases for 6GR mobility design,

- Case 1: High-UE-density public venues

- Case 2: High-speed scenarios with predictable trajectories

- Case 3: Fixed mobility range scenarios

- Case 4: Delay-sensitive service scenarios

- Handover types

In 5G NR, due to the multiple beams, both beam level and cell level mobility are supported. And for the beam level mobility which is related with TCI activation, it is belong to RAN1 studied scope. RAN2 focus on cell level mobility. The same work division should be followed for 6GR.

In 5G NR for cell level mobility, a diverse set of handover mechanisms has been introduced to address the complex mobility requirements of emerging use cases. These handover types are designed to optimize connectivity across varying network conditions, device speeds, and service demands.

To provide a comprehensive understanding of their practical implications, we analyze and compare the advantages and limitations of each handover type from multiple critical perspectives—including latency performance, signaling overhead, network resource consumption, robustness against mobility speed, and implementation complexity. The following table presents this comparative analysis:

Table 2 Comparison of NR handover type

|  | Resource efficiency | Mobility interruption time | HO Robustness | Implementation Complexity |
| --- | --- | --- | --- | --- |
| L3 handover | No pre-handover resource reservation | Long interruption time |  | Low Complexity |
| CHO | Need pre-handover resource reservation | Long interruption time | Improved Robustness | Low Complexity |
| DAPS | No pre-handover resource reservation | 0ms interruption time |  | High Complexity |
| LTM | Need pre-handover resource reservation | near 0ms interruption time |  | Medium Complexity |
| CLTM | Need pre-handover resource reservation | near 0ms interruption time | Improved Robustness | Medium Complexity |
| RACH-Less L3 HO | No pre-handover resource reservation | interruption time reduction |  | Low Complexity with limited deployment |

In 6GR, selecting appropriate handover mechanisms tailored to diverse scenarios is critical to meeting both network and UE requirements. Below is an analysis of optimal handover strategies for each key scenario:

Table 3 analysis of optimal handover strategies for each key scenario

|  | Key handover performance requirement | Applicable Solution |
| --- | --- | --- |
| Case 1: High-UE-Density Environments | Dense UE deployments bring constrained radio resources; handover schemes needing RAN pre-reserve resources for individual UEs worsen inefficiency. | NW controlled handover without resource reservation (e.g., L3 HO-like) |
| Case 2: High-Speed Mobility with Predictable Trajectories | Ensuring ultra-high-speed UE (up to 1000km/h) HO robustness is key and fixed trajectories need no subsequent HOs. | UE controlled handover with resource reservation for once handover per candidate cell (e.g., CHO-like) |
| Case 3: Fixed Mobility Range Scenarios | UEs with fixed mobility (e.g., commute, workplace) have predictable patterns; | Once configuration for subsequent handover (e.g., (C)LTM-like) |
| Case 4: Delay-Sensitive Services | Delay-sensitive services need near-zero data interruption; (C)LTM (intra-DU ~0ms) is better than DAPS for 6GR. | With resource reservation and RACH-less access (e.g., (C)LTM-like) |

Based on the above analysis, the following conclusions can be drawn:

Proposal 4: 6GR mobility supports both beam level mobility and cell level mobility, with RAN2 focusing on the study of cell level mobility. The following handover types should be supported for cell level mobility:

- Handover with prepared configuration or immediate in-use configuration

- NW triggered or UE triggered handover procedure

- One-shot handover or subsequent handover execution

- Handover with RACH-less or RACH-based procedure

- RLM/RLF

RLM mechanism is utilized to monitor the serving link quality. In 5G NR, RLM encompasses the detection of physical layer issues, RACH problems, and RLC problem. Upon detecting an RLF, if AS security is activated and the UE is configured with at least one DRB, the UE shall trigger RRC reestablishment. Otherwise, the UE transits to the idle state.

For 6GR, as the protocol specifications will undergo re-discussion, determinations regarding which problem detections should be included in RLM may be postponed. However the RLM/RLF should be supported in 6GR to allow UE perform RRC connection recovery. In 5G NR, fast recovery is supported e.g. recovery via candidate cell of CHO or (C)LTM. However the interruption couldn’t be avoided even by fast recovery mechanism. Thus, 6GR should explore mechanisms to reduce the RLF rate. As complete RLF avoidance is not feasible, fast recovery for RLF should be studied too.

Proposal 5: RAN2 study mechanisms to reduce RLF rate and perform fast recovery when RLF occurs.

  - Cell Selection and Reselection

In both 4G and 5G, cell reselection relies on Radio Resource Management (RRM) measurements, which directly reflect channel quality—an objective and real-time indicator of link suitability. Consequently, the RRM-based mobility framework (e.g., S criteria, R criteria, frequency based priority) for cell (re)selection, as established in 5G, should be retained as the baseline for 6GR. This principle applies uniformly regardless of whether the network is terrestrial (TN) or non-terrestrial (NTN).

Proposal 6: The 5G cell (re)selection mechanism (e.g., S criteria, R criteria, and frequency based priority) is the baseline for 6GR cell selection and reselection.

  - Inter-RAT mobility

According to the SID the mobility between 6GR and 5G NR should be supported.

- Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4] RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

In order to support the mobility between 5G NR and 6GR, the legacy 5G NR to EUTRA mobility mechanisms (e.g., inter-RAT handover, cell (re)selection, and redirection) could take as baseline. In order to support these mobility mechanism, the related inter-RAT measurement should be supported for idle/inactive UE and connected UE.

As for the state of inactive or sub-state, which is discussed for 6GR, no matter which modeling it is, when UE in this state, It allows UE in this state to perform UE based mobility e.g. cell reselection, when UE selects one 5G NR cell to camp on, the UE should go to idle state in order to avoid complexity of network implementation, which is align with 5G principle, i.e. the inactive UE will go to idle when it reselect to 4G cell.

Proposal 7: 6GR mobility supports mobility between 6GR and 5G NR, taking the legacy mobility mechanisms between 5G NR and E-UTRA (e.g., inter-RAT handover, cell (re)selection, and redirection) as the baseline.

- Conclusion

Based on the analysis in Section 2, our observations and proposals are summarized as follows:

Observation 1: Key lessons learned from 5G mobility design: Redundant design across 3GPP releases (for different handover types), and Solutions to improve HO performance are often limited to specific handover types.

Observation 2: The feasibility of integrating L1 measurements for beam management into the unified measurement framework shall be determined by RAN1.

Observation 3: The following scenarios shall be considered as key use cases for 6GR mobility design,

- Case 1: High-UE-density public venues

- Case 2: High-speed scenarios with predictable trajectories

- Case 3: Fixed mobility range scenarios

- Case 4: Delay-sensitive service scenarios

Mobility Framework and Targets

Proposal 1: RAN2 design a unified 6GR mobility framework, aiming at the following for all supported handover types,

- Same signaling structure for configuration

- Maximizing the reuse of same set of building blocks, i.e., functions/procedure steps

Proposal 2: To meet the performance requirements of 6GR mobility, adopt LTM-based solutions as the baseline and support additional enhancements, including

- Solutions to minimize interruption time: early DL synchronization, early TA acquisition, UE configuration pre-processing, and avoiding unnecessary L2 resets such as not flushing the HARQ buffer.

- Solutions to minimize throughput degradation: early CSI acquisition, dynamic SCell activation during handover, and mTRP activation during handover.

Measurement Framework

Proposal 3: RAN2 design a unified measurement framework that is applicable to RRM measurements and LTM L1 measurements. This unified measurement shall encompass the following aspects:

- Unified configuration of measurement resources (e.g., SSB, CSI-RS)

- Unified triggering mechanism for measurement reporting (e.g., event-triggered, periodic reports)

- Support of measurement report via either MAC CE or RRC signalling

Intra-RAT Connected Mobility

Proposal 4: 6GR mobility supports both beam level mobility and cell level mobility, with RAN2 focusing on the study of cell level mobility. The following handover types should be supported for cell level mobility:

- Handover with prepared configuration or immediate in-use configuration

- NW triggered or UE triggered handover procedure

- One-shot handover or subsequent handover execution

- Handover with RACH-less or RACH-based procedure

Proposal 5: RAN2 study mechanisms to reduce RLF rate and perform fast recovery when RLF occurs.

Cell selection and reselection

Proposal 6: The 5G cell (re)selection mechanism (e.g., S criteria, R criteria, and frequency based priority) is the baseline for 6GR cell selection and reselection.

Inter-RAT

Proposal 7: 6GR mobility supports mobility between 6GR and 5G NR, taking the legacy mobility mechanisms between 5G NR and E-UTRA (e.g., inter-RAT handover, cell (re)selection, and redirection) as the baseline.

- References

- RP- 252912 New SID: Study on 6G Radio, RAN#108

- Draft_RAN2_131bis_MeetingReport_v1,https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_131bis/Report