3GPP TSG-RAN WG2 Meeting #132	R2-2508108

Dallas, USA, Nov. 17th - 21th, 2025

Agenda Item:	10.3.3.3

Source:	OPPO

Title:	Discussion on the RAN2-related 6G security aspects

Document for:	Discussion, Decision

# Introduction

According to the discussion in the RAN2#131bis, RAN2 made the following agreements [18] and sent an LSout [19] to assist the MAC CE security discussion in SA3:

- Send LS to SA3 to indicate the existing 5G MAC CE information and that some of these control information may be carried over in 6G L2. Ask them what information would require security. Explain RAN2 concerns of overhead (size and mobility security context exchange) and processing. Please identify only critical information that needs to be secure and what type of security (i.e. integrity, ciphering). - Indicate that if there are information that critical to be protected RAN2 and SA3 should work jointly to develop a solution.

In this contribution, we provide our understandings on the required coordination between RAN and SA3 for the 6G security requirements, and the corresponding RAN2 standard works.

# Discussion

## RAN2 6G security functions/requirements inherited from 5G

### 5G AS security functions reused for 6G

3GPP TS 38.331 [8]: ### 5.3.6 Counter check #### 5.3.6.1 General Figure 5.3.6.1-1: Counter check procedure The counter check procedure is used by the network to request the UE to verify the amount of data sent/ received on each DRB. More specifically, the UE is requested to check if, for each DRB, the most significant bits of the COUNT match with the values indicated by the network. NOTE: The procedure enables the network to detect packet insertion by an intruder (a 'man in the middle').

3GPP TS 38.323 [9]: ## 5.8 Ciphering and deciphering … ## 5.9 Integrity protection and verification …

According to the 5G AS procedures, the AS-level security functions include:

- RRC counter check procedure [8]

- Integrity protection and cyphering for SRBs (i.e. dedicated RRC messages) at PDCP layer [9]

- Integrity protection and cyphering for DRBs at PDCP layer [9]

From our understanding, the above AS-level security functions of 5G can be reused for 6G, so that the Rel-20 security study in RAN2 can focus on the extra 6G enhancements of security. For the integrity protection and cyphering of SRBs and DRBS, keeping the security function at the PDCP layer can reduce the number of key changes, as when the PDCP anchor is not changed, the security key refresh is not always performed during handover. By implementing the count check procedure via RRC signalling, the DRB does not have to be configured with the integrity protection, so that the extra signalling overheads (i.e. MAC-I) and the processing load over the DRB SDUs can be reduced. On the other hand, as the 5G UE only re-establishes the RRC connection when the SRB fails the integrity verification, and drops the PDCP PDU of DRB which fails the integrity verification, the gNB by using the count check procedure can detect the maliciously inserted packets of the DRB.

Observation 1: In 5G, the integrity protection and cyphering for SRB and DRB at the PDCP layer reduces the number of key changes during handover. The count check procedure helps the gNB to detect maliciously inserted packets of the DRB.

Proposal 1: RAN2 confirms the following 5G security functions reused in 6G AS: 1) Count check procedure via RRC; 2) Integrity protection and cyphering for SRB and DRB at PDCP layer

### 5G RAN security requirements reused for 6G

SA3 6G SID in [2]:

According to the endorsed SA3 6G SID as highlighted above, the security studies in SA3 need to take into account the requirements from other working groups, including “the architecture related requirements from RAN”.

Observation x.1: The SA3 6G security study needs to consider the architecture related requirements from RAN.

3GPP TR 38.913 [3] provided by RAN WGs: ## 10.12 Security and Privacy related requirement relevant for Radio Access The RAN design for the Next Generation Radio Access Technologies shall ensure support for integrity and confidentiality protection of radio signalling messages, including messages between RAN and Core network nodes. The RAN design for the Next Generation Radio Access Technologies shall ensure the ability to support integrity and confidentiality protection of user plane messages, including messages between RAN and Core network nodes, with the use of such security to be configurable during security set-up. The RAN design for the Next Generation Radio Access Technologies shall ensure support for the allocation and use of identities to provide user privacy, e.g. reduce the need for sending any permanent identities in the clear. The RAN design for the Next Generation Radio Access Technologies shall ensure the efficient establishment of RAN security mechanisms. The RAN design for the Next Generation Radio Access Technologies shall ensure resilience against jamming. NOTE1: Security and Privacy-related system requirements are reflected in 3GPP TR 33.899 [19]. This TR includes security areas on "RAN security" and "Privacy security", which is a possible source of security and privacy related requirements for the Radio Access.

According to the 3GPP TR 38.913 [3] provided by RAN WGs, RAN provided the security-related requirements from the radio access point of view, which is based on the study outcomes from both RAN and SA3 (e.g. 3GPP TR 33.899 [4]).

From our understanding, the 5G security requirements as quoted above for the radio access should also be applicable for 6G. The security requirements inherited from 5G include:

- Requirement 1: The integrity protection and cyphering for SRB, which is used to carry both dedicated RRC message and NAS message.

- Requirement 2: The integrity protection and cyphering for DRB, which is used to carry both user plane data and the NAS message (over UP).

- Requirement 3: Avoid using permanent UE ID, by using temporary UE ID (e.g. C-RNTI or I-RNTI) instead.

- Requirement 4: Efficient establishment of RAN security context, e.g. avoid frequent key change during mobility, efficient temporary UE ID allocation and efficient detection of integrity protection.

- Requirement 5: Resilience against jamming, e.g. detecting the jamming signal via measurement and reducing the interference impact caused by the jamming signal.

According to the above requirements, the requirements of 1+2 require the SA3 to design the proper solutions for the integrity protection and cyphering for both SRB and DRB, and the requirements of 3+4+5 can be realized by the RAN solutions (e.g. temporary UE ID allocation and radio signal measurement). The details on which message/data via which SRB/DRB needs integrity protection and/or cyphering can be discussed later-on based on the solutions provided by both SA3 and RAN, so that the system is able to “ensure the efficient establishment of RAN security mechanisms”. For example, the 5G integrity protection applied for each PDCP SDU causes lots of signalling overheads and UE processing, especially for the traffic with a large number of small packets. In 6G, we can consider to concatenate multiple PDCP SDUs to a single PDCP PDU, which is used as a single input to create one MAC-I.

Observation x.2: The “Security and Privacy related requirement relevant for Radio Access” provided by RAN is based on the study outcomes from both RAN and SA3.

### Mobility impacts on security

3GPP TS 38.300: # 13 Security ## 13.1 Overview and Principles … Figure 13.1-1: 5G Key Derivation … ## 13.2 Security Termination Points … ## 13.3 State Transitions and Mobility …

According to the 5G TS 38.300, the RAN2-led stage-2 specification needs to clarify the AS-level security functions, including key derivation, security termination points and key generation for state transitions and mobility. The security section in the RAN2 stage-2 specification is based on the inputs from SA3 and the solutions (e.g. how to handle PDCP COUNT wrap around and how to handle the failed integrity check) from RAN.

Figure 1: Relation between L2 reset and key change

Since SA3 are planning to discuss some enhanced solutions for 6G key derivation (e.g. more keys for different RAN nodes including the key differentiation amongst CU-CP/CU-UP, multiple CU-UP(s) and DU), so as to provide more robust 6G security for the UE. However, as the security key refresh impacts the mobility procedure (e.g. L2 reset) as illustrated above, we think that RAN2 should inform SA3 that any key refresh at the UE will cause the UE L2 reset, which leads to the packet loss buffered at the UE L2. This is the reason why the UE in 5G performs key refresh for KgNB (i.e. no partial key refresh for KUPenc, KUPint, KRRCenc and KRRCint) and resets L2 whenever the CU-CP or CU-UP serving the UE is changed. Here, we have some detailed descriptions on how the security key refresh impacts the UE L2 buffer. As explained in the above figure, the relation between L2 reset and key change can be described as follows:

- Step 1: When the CU-UP key from PDCP#1 and PDCP#2 (or the CU-CP key from PDCP#5) changes, the UE needs to re-establish PDCP#1 and PDCP#2 (or PDCP#5) to update the key configuration of PDCP.

- Step 2: As the key configuration of PDCP is changed, the UE needs to stop the reception and transmission of the old packets using the old key. Otherwise, the transmission resource will be wasted, as the old packets encrypted by the old key is not able to be decrypted.

  - To stop the reception and transmission of the old packets using the old key, the UE needs to reset PDCP, RLC and MAC, which flushes all HARQ buffer (i.e. dropping the MAC PDU) in MAC. The packets from all RBs in the same MAC entity are dropped.

Figure 2: Potential issue of 6G key derivation

In 5G, if the CU is not changed, when one DRB is offloaded from one CU-UP to another CU-UP, the UE does not need to refresh its KgNB and does not result in L2 reset. In 6G, if SA3 introduces multiple CU-UP keys, the UE needs to change the key of a DRB#x when the DRB#x is offloaded from one CU-UP entity to another CU-UP entity. However, the partial key change should be avoided due to the UE L2 reset, and the UE should refresh KgNB, instead of refresh one set of (KUPenc, KUPint) of CU-UP. SA3 can then take into account the RAN2 concerns while designing the 6G security solutions.

3GPP TR 38.804 [5]: # 11 Security Security key refresh is not performed at every mobility procedure (i.e. handover), at least for the case of mobility where the PDCP anchor point is not changed. NOTE: It is to be confirmed by SA3 considering whether it has any implication on the inputs for key derivation, e.g. PCI. For DC between LTE and NR where the master RAT is LTE, S-KeNB is derived from KeNB of the master node.

3GPP TS 33.501 [6]: ##### 6.9.2.3.4 UE handling The UE behaviour is the same regardless if the handover is intra-gNB-CU, intra ng-eNB, Xn, or N2 with the exception that during intra-gNB-CU handover, the UE may retain the same key based on an indication from the gNB. The UE behaviour is also same in case of conditional handover, as specified in TS 38.300 [52], i.e., the UE shall use the parameters of the selected target cell in KNG-RAN* derivations.

According to the 3GPP 5G TR 38.804 drafted by RAN2 as quoted above, RAN2 decided that security key refresh is not performed at every mobility procedure when the PDCP anchor point is not change, so as to provide more efficient RAN handover procedure by reducing the number of security key refresh and the L2 reset caused by the security key refresh. According to the RAN requirements for the security key refresh, SA3 specified that “during intra-gNB-CU handover, the UE may retain the same key based on an indication from the gNB”, so that the security key refresh is not always performed when the PDCP anchor is not changed. We consider that the same 5G requirement regarding the security key refresh should be reused for 6G, to reduce the impacts (e.g. the packet loss caused by the security key refresh) on the UE mobility.

Observation 2.1: The RAN2 stage-2 specification needs to clarify the security functions, including key derivation, security termination points and key generation for state transitions and mobility.

Observation 2.2: The UE refreshes KgNB and resets L2 whenever a security key (i.e. KUPenc, KUPint, KRRCenc and KRRCint) changes, i.e. no partial key change in 5G. According to 3GPP TR 38.804 of 5G, RAN decided that security key refresh is not always performed during handover when the PDCP anchor point is not changed, and SA3 followed the RAN2 requirement for the security key refresh.

Proposal 2: RAN2 informs SA3 the following 5G mobility requirements reused for 6G security: 1) The security key change during mobility should be minimized to avoid the packet loss due to L2 reset; 2) Security key change is not always performed at handover when the PDCP anchor point is not changed.

## RAN2 6G new security functions/requirements different from 5G

A summary of SA3 6G security proposals impacting RAN2, according to the tdocs submitted in Section “6.3 New 6G SIDs/WIDs” in SA3#123 [10]: - WT#2.1 (RAN related generic security): Study security and privacy aspects of 6G RAN architecture. - WT#2.2 (Lower layer protection): Study security and privacy aspects of radio lower layers and define a security mechanism for vulnerable lower layer signalling including protection of MAC-CE. - WT#2.3 (SIB protection): Study security for SIBs to avoid FBS (False Base Station) attacks - WT#2.4 (Msg 3/5 protection): Study security for messages (e.g. Msg3/Msg5) exchanged prior to AS Security Mode Command - WT#2.5 (RAN UP security enhancements): Study enhanced user plane security for 6G to enable finer granularity control for the UP security and support UP key isolation for all the CU-UPs for a 6G RAN node in case of CU-CP/CU-UP RAN split architecture.

According to the 6G SID discussion in SA3#123 meeting, the work tasks related to the AS security as quoted above were discussed. Compared with the 5G security schemes, we consider that the extra RAN2 impacts of the 6G security solutions include MAC CE security, SIB security, Msg3/Msg5 security and multiple-sets of UP keys (given that 5G UE has only-one set of UE UP keys (KUPenc, KUPint)). Once SA3 decides to support the extra 6G security direction(s), RAN2 can start to design the corresponding RAN2 solutions, including signalling format (e.g. input variables like COUNT and (MAC) PDU format), configuration procedure, verification handling and so on).

Observation x.1: Compared with 5G, the potential extra 6G RAN-related security impacts as currently being discussed in SA3 include: 1) MAC CE security; 2) SIB security; 3) Msg3/Msg5 security; 4) Finer granularity of the UP security, i.e. support UP key isolation for different CU-UP(s).

For the above 6G enhancements of security, RAN2 needs to consider how the security related parameters (e.g. COUNT and/or MAC-I) is added in MAC layer and RRC layer (e.g. CCCH RRC message), and how the verification procedure is performed by the UE. As the 6G enhancements of security are still ongoing in SA3 and the 6G CP/UP study is also ongoing in RAN2, we think that the 6G CU/UP design should have sufficient flexibility to add the extra 6G RAN-related security enhancements.

Observation x.2: The 6G CU/UP design should have sufficient flexibility to add the 6G RAN-related security enhancements.

Since RAN2 has already sent an LS to assist the SA3 discussion on the MAC CE security, we think that RAN2 can focus on other 6G security enhancements, and restart the discussion related to the MAC CE security after SA3 made some progresses.

### MAC-I processing issue in PDCP

3GPP TS 24.501: #### 9.11.4.7 Integrity protection maximum data rate .. 8; 7; 6; 5; 4; 3; 2; 1. Integrity protection maximum data rate IEI; octet 1. Maximum data rate per UE for user-plane integrity protection for uplink; octet 2. Maximum data rate per UE for user-plane integrity protection for downlink; octet 3. Figure 9.11.4.7.1: Integrity protection maximum data rate information element

According to the 3GPP TS 38.323 [9], when DRB integrity protection is configured, each PDCP SDU with a separate SN creates a separate MAC-I, which causes heavy processing load at the UE and lots of signalling overhead. In 5G, according to the 3GPP TS 24.501 [16], due to the UE processing limitation, an extra UE capability indication (i.e. “Integrity protection maximum data rate”) is introduced to differentiate the UE processing capability for DRB integrity protection at different data rate (i.e. one for 64 kbps and one for full data rate). The capability as quoted above means that if the DRB integrity protection is configured, some low-cost UE implementation may only be able to support up-to 64 kbps data rate. In 6G, when multiple PDCP SDUs are concatenated to a single PDCP PDU with single SN and single MAC-I, the UE processing load can be reduced for calculating MAC-I, and the signalling overhead of separate SN and MAC-I for each PDCP SDU is also reduced.

Observation 3: In 5G, the MAC-I generated per PDCP SDU causes extra signalling overheads and UE processing (i.e. impacting the UE’s transmission bit rate).

Proposal 3: More than one PDCP SDUs can be used to generate a single MAC-I.

### SIB security

Figure 4: Security threat of fake SIBx

In 5G, the system information works without security protection, and the AS security is not activated before the establishment of SRB1. According to the SA3 study on the FBS (False Base Station) as illustrated above, without the security protection for the system information, it may happen that the UEs camping on a real cell receives rogue SI message or previous SI message from an attacker, which induces the UE reselecting to incorrect neighbouring cells or responding to inexistent PWS message. The detailed descriptions of use cases for SIB security thread can be found in 3GPP TR 33.809 [11]. Apparently, attacks mounted by a false base station is a major security flaw in the existing 5G system. Additionally, the SA1 6G study also proposed to increase the level of security between the RAN entities and the UE during the initial network selection procedure against FBS, as described in 3GPP TR 22.870 [12]. In 6G, we think that it is a good opportunity to address this concern, without considering the backward compatibility (e.g. how to introduce the SIB security without impacting the legacy UE/network).

3GPP TS 38.331 [8]: NOTE 2: The physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits.

3GPP TR 33.809 [11]: ## 5.2 Key Issue #2: Security protection of system information … ## 6.0 Mapping between key issues and solutions Solutions; Key Issues. 1; 2; 3; 4; 5; 6; 7. Solution #1: Protection for the UE Capability Transfer; x. Solution #2: Protection of RRCReject message in RRC_INACTIVE state; x. Solution #3: Protection of uplink UECapabilityInformation RRC message; x. Solution #4: Enriched measurement reports; x. Solution #5: Mitigation against the authentication relay attack; x. Solution #6: Avoiding UE connecting to false base station during HO; x. Solution #7: Verification of authenticity of the cell; x. Solution #8: Network detection of nearby false base stations from call statistics and measurements; x. Solution #9: Using symmetric algorithm with assistance of USIM and home network; x; x. Solution #10: Protection on the unicast message based on ECDH; x. Solution #11: Certificate based solution against false base station; x; x. Solution #12: ID based solution against false base station; x; x. Solution #13: Protecting RRCResumeRequest against MiTM; x. Solution #14: Shared key based MIB/SIBs protection; x. Solution #15: Mitigation against the authentication relay attack with different PLMNs; x. Solution #16: Protection of RRC Reject Message; x. Solution 17: Integrity protection of the whole RRCResumeRequest message; x. Solution #18: Avoiding UE connecting to False Base Station during Conditional Handover; x. Solution #19: AS security based MIB/SIBs integrity information provided by gNB; x. Solution #20: Digital Signing Network Function (DSnF); x. Solution #21: Certificate based solution against false base station for Non-Public Networks; x; x. Solution #22: Detecting false base stations based on UE positioning measurements; x. Solution #23: Cryptographic CRC to avoid MitM relay nodes; x; x; x. Solution #24: UE&Network-assisted UE avoidance and Network detection of FBS; x. Solution #25: Detection of Man-in-the-Middle false base stations; x. Solution #26: KI#2 with PKC-based and without tight time synchronization; x. Solution #27: Short-lived asymmetric key-based solution for protecting system information; x. One candidate solution of SIB security: Figure 6.7.2.1-1: System Information verification using Digital Signatures

On the other hand, as the security parameters (e.g. verification fields for integrity protection) could range from tens of bits to thousands of bits (which may/may not impact the 6G cell coverage) depending on the security solution(s) [11] selected by SA3, the SA3 also needs the RAN inputs regarding whether it is feasible to include the extra bits of security parameters in SIB. Given that the maximum number of bits in SIB1 or SI in 5G is 2976 bits, and the transmission of SIB1 or SI does not support RLC segmentation (i.e. only TM mode RLC is supported for SIB1 and SI [8].), RAN2 could also send some guidance/requirements to SA3, e.g. to minimize the number of security bits included in SIB1 or SI.

Observation 4.1: Without SI security protection, the UE may reselect to incorrect neighbouring cells or respond to inexistent PWS message due to attacks from a false base station, as captured in 3GPP 5G TR 33.809 and 3GPP 6G TR 22.870.

Observation 4.2: As the addition of the security parameters ranging from tens of bits to thousands of bits may/may not impact the coverage of the 6G cell, SA3 needs to know whether the extra bits for SIB security can be supported by 6G RAN, and RAN can inform SA3 the transmission requirements of SIB.

Proposal 4: RAN2 studies whether it is feasible to support the SIB security in 6G, in alignment with SA3.

# Conclusion

For the 6G security, we consider that with the increasing use of more advanced tools (e.g. AI and quantum computer) by the attackers, the 6G cellular network can be more vulnerable to various attacks. In order to make the 6G security more robust and efficient, we think that a tight coordination between RAN and SA is needed, so that the RAN can use the SA3 inputs for the initial design of 6G RAN, and SA3 can take the RAN inputs for the security design to avoid the security impacts on the RAN procedures (e.g. mobility).

5G AS security functions reused for 6G

Observation 1: In 5G, the integrity protection and cyphering for SRB and DRB at the PDCP layer reduces the number of key changes during handover. The count check procedure helps the gNB to detect maliciously inserted packets of the DRB.

Proposal 1: RAN2 confirms the following 5G security functions reused in 6G AS: 1) Count check procedure via RRC; 2) Integrity protection and cyphering for SRB and DRB at PDCP layer

Mobility impacts on security

Observation 2.1: The RAN2 stage-2 specification needs to clarify the security functions, including key derivation, security termination points and key generation for state transitions and mobility.

Observation 2.2: The UE refreshes KgNB and resets L2 whenever a security key (i.e. KUPenc, KUPint, KRRCenc and KRRCint) changes, i.e. no partial key change in 5G. According to 3GPP TR 38.804 of 5G, RAN decided that security key refresh is not always performed during handover when the PDCP anchor point is not changed, and SA3 followed the RAN2 requirement for the security key refresh.

Proposal 2: RAN2 informs SA3 the following 5G mobility requirements reused for 6G security: 1) The security key change during mobility should be minimized to avoid the packet loss due to L2 reset; 2) Security key change is not always performed at handover when the PDCP anchor point is not changed.

MAC-I processing issue in PDCP

Observation 3: In 5G, the MAC-I generated per PDCP SDU causes extra signalling overheads and UE processing (i.e. impacting the UE’s transmission bit rate).

Proposal 3: More than one PDCP SDUs can be used to generate a single MAC-I.

SIB security

Observation 4.1: Without SI security protection, the UE may reselect to incorrect neighbouring cells or respond to inexistent PWS message due to attacks from a false base station, as captured in 3GPP 5G TR 33.809 and 3GPP 6G TR 22.870.

Observation 4.2: As the addition of the security parameters ranging from tens of bits to thousands of bits may/may not impact the coverage of the 6G cell, SA3 needs to know whether the extra bits for SIB security can be supported by 6G RAN, and RAN can inform SA3 the transmission requirements of SIB.

Proposal 4: RAN2 studies whether it is feasible to support the SIB security in 6G, in alignment with SA3.

# Reference

[1] RP-251881, “New SID: Study on 6G Radio”

[2] SP-251132, “Study on Security for the 6G System”

[3] 3GPP TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”

[4] 3GPP TR 33.899, “Study on the security aspects of the next generation system”

[5] 3GPP TR 38.804, “Study on New Radio Access Technology; Radio Interface Protocol Aspects”

[6] 3GPP TS 33.501, “Security architecture and procedures for 5G system”

[7] 3GPP TS 37.340, “Multi-connectivity; Overall Description; Stage 2”

[8] 3GPP TS 38.331, “NR; Radio Resource Control (RRC) protocol specification”

[9] 3GPP TS 38.323, “NR; Packet Data Convergence Protocol (PDCP) specification”

[10] Meeting report of SA3#123.

[11] 3GPP TR 33.809, “Study on 5G security enhancements against False Base Stations (FBS)”

[12] 3GPP TR 22.870, “Study on 6G Use Cases and Service Requirements; Stage 1”

[13] S3-251124, “Reply LS on security handling for inter-CU LTM in non-DC cases”

[14] 3GPP TS 38.321, “NR; Medium Access Control (MAC) protocol specification”

[15] S3-252285, “Living document on NR mobility enhancement”, NR_Mob_Ph4_Sec

[16] 3GPP TS 24.501, “Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3”

[17] Yuanyuan Yang∗, Jin Cao∗, Ruhui Ma∗, Sheng Li∗, and Hui Li∗, ∗State Key Laboratory of Integrated Services Networks, School of Cyber Engineering, Xidian University, China, arXiv:2506.09502v2 [cs.CR] 12 Jun 2025, “The Security Overview and Analysis of 3GPP 5G MAC CE”

[18] RAN2#131bis meeting minutes

[19] R2-2507945, LSout, to SA3, CC RAN1/RAN3/SA2, “LS on Early Alignment on Access Stratum security aspects”