3GPP TSG-RAN WG2 Meeting #132	R2-2508112

# Dallas, USA, 17-21 November 2025

Agenda Item:	10.3.2.2

Source:	MediaTek Inc.

Title:	RRC signalling and ASN.1 aspects

Document for:	Discussion, decision

# 1	Introduction

In RAN2#131bis, RAN2 agreed to study the UE configuration structure for 6G RRC as well as how to reliably and unambiguously configure the UE while keeping the RRC signalling size small. In addition, RAN2 agreed to study how to improve the readability of ASN.1. [1]

In this contribution, we provide analysis of and proposals for improved/simplified the UE configuration structure (Chapter 2.1), delta signalling (Chapter 2.2), ASN.1 ambiguities (Chapter 2.3), and readability of ASN.1 (Chapter 2.4).

# 2	Discussion

## 2.1	UE Configuration Structure

In RAN2#131bis, we provided a list of characteristics which make the 5G UE configuration structure complex (R2-2506958) [2]:

In this section, we further discuss the 3rd bullet point above – the concept of common and dedicated configurations.

In 5G, the UE configuration for BWPs and PDCCH/PDSCH/PUCCH/PUSCH physical channels of a serving cell is split into common and dedicated configuration parts, which are signalled to the UE in separate fields within RRC signalling. The definition of a common configuration and a dedicated configuration can be seen in the descriptions of information elements in TS 38.331 as per the following:

Observation 1	In 5G, a common configuration refers to the part of the UE configuration which is cell specific, i.e., same configuration is applied for all UEs operating in the cell, whereas a dedicated configuration refers to the part of the UE configuration which is specific to this particular UE.

The exact split of the UE configuration to common and dedicated configuration in 5G RRC signalling is captured in Figure 1 (for an SpCell) and Figure 2 (for an SCell) below. Within each figure, the color coding represents the parts of the configuration that logically belong together to form UE configuration for certain BWP. We think the configuration structure captured in the figures brings a lot of complexity for both the UE and the network. As a concrete example, in order to know the PDCCH configuration for the initial DL BWP of the PCell, the UE needs to combine following fields:

- The common configuration signalled in:

  - SIB1 -> servingCellConfigCommon -> downlinkConfigCommon -> initialDownlinkBWP ->
pdcch-ConfigCommon, or

  - RRCReconfiguration -> masterCellGroup -> spCellConfig -> reconfigurationWithSync -> spCellConfigCommon -> downlinkConfigCommon -> initialDownlinkBWP -> pdcch-ConfigCommon

- with the dedicated configuration signalled in:

  - RRCSetup/RRCResume/RRCReconfiguration -> masterCellGroup -> spCellConfig -> spCellConfigDedicated -> initialDownlinkBWP -> pdcch-Config

One could ask, wouldn't it be simpler, for both the UE and the network, to signal single physical channel configuration in a single field? Furthermore, referring to the definition of common and dedicated configurations captured in Observation 1, it is irrelevant for the UE whether a part of the configuration is the same with the other UEs in the same cell or not; it simply has no functional significance for the UE.

Observation 2	The concept of common and dedicated configurations makes the 5G UE configuration structure complex for both the UE and the network. From UE's perspective, the split of the UE configuration to common and dedicated parts has no functional significance.

Based on the above, we propose to simplify the UE configuration structure in 6G by not applying the concept of common and dedicated configurations in RRC signalling.

Proposal 1	The concept of common and dedicated configurations is not applied as a defining factor for the UE configuration structure in 6G RRC signalling.

Figure 1. Common and dedicated configurations in 5G RRC signalling for an SpCell

Figure 2. Common and dedicated configurations in 5G RRC signalling for an SCell

## 2.2	Delta Signalling

The primary purpose or goal of the delta signalling mechanism is to reduce the size of the over-the-air RRC messages for UE reconfiguration. Based on the agreement in RAN2#131bis (refer to Chapter 1), companies clearly see that the over-the-air RRC message size continues to be an important aspect also in 6G for radio resources consumption and control plane signalling latency perspective. However, the delta signalling mechanism also provides another benefit, which is visible to the UE only, which we explain below.

When the UE receives RRC message for reconfiguration, one of the steps the UE needs to perform is to find out which parts of the UE configuration are actually being reconfigured. For example, if RF related configuration is being reconfigured, the UE can already in the very early phase of the configuration processing start preparations for RF reconfiguration in the UE lower layers, etc. If the delta signalling is used by the network, the UE can utilize the presence of fields in the RRC message to know which parts of the configuration are being reconfigured. This process is more streamlined and faster to execute in comparison to a scenario where the network does not use delta configuration at all (or uses it only selectively in the configuration structure), in which case the UE needs to actually compare the current and the new configurations in order to find out which parts of the configuration remain the same and which are reconfigured. To summarize, the delta signalling enables smaller over-the-air RRC message size and also faster reconfiguration procedure in the UE.

Observation 3	Delta signalling mechanism is worthwhile as it enables smaller over-the-air RRC messages for UE reconfiguration, which is beneficial for radio resource consumption and control plane latency perspective, but also significantly eases the UE in detecting which parts of the configuration are reconfigured.

Based on the analysis above, it seems reasonable to apply the delta signalling also in 6G RRC. However, the delta signalling provides its benefits only if it is actually used by the network. As explained by a network vendor (R2-2507073) [2], the delta signalling is not used in 5G networks as much as it ideally should be due to the ambiguities the delta signalling mechanism introduces. For this reason, in order to make the delta signalling more implementable in 6G, also the related ambiguities should be avoided.

Proposal 2	Delta signalling is applied in 6G RRC for UE reconfiguration. To make the delta signalling implementable in 6G, RAN2 targets to avoid the ambiguities related to the delta signalling mechanism.

In the discussion on ASN.1 ambiguities in Chapter 2.3 we also identify which of them are related to the delta signalling mechanism.

## 2.3	ASN.1 Ambiguities

### 2.3.1	Analysis

In our previous contribution for RAN2#131bis (R2-2506957) [3], we listed three main ambiguities which exist in 5G RRC signalling for UE (re)configuration. These ambiguities are listed below again and also numbered for easier referencing in the rest of the document.

- Functionally mandatory UE configuration parameters can be absent in over-the-air RRC messages for initial configuration of a feature/functionality;

- UE configuration parameters which shouldn't be modified after initial configuration of a feature/functionality can be sent in subsequent over-the-air RRC messages with new values;

- UE configuration parameters can be sent in over-the-air RRC messages with values which do not conform to other parts of the configuration (e.g., presence or values of other configuration parameters).

Analysis of the 1st Ambiguity

We carried out a study of our product development records and the 5G RRC ASN.1 (TS 38.331 v19.0.0) to identify how common this kind of ambiguity is in practice. To summarize, we easily identified dozens of occurrences by looking at TS 38.331 only; several of which we've also seen in practice in IODT/live networks. In addition, since significant amount of the UE configuration parameters originate from RAN1, many more occurrences could be revealed, if also mandatoriness of the configuration parameters for features/functionalities captured in TS 38.213 and TS 38.214 was analyzed against the ASN.1. To clarify how this kind of ambiguity looks in practice, some examples are listed in Table 1.

Table 1. Some examples of the 1st ambiguity in 5G RRC signalling for UE configuration

| IE | Field | Explanation of the ambiguity |
| --- | --- | --- |
| BeamFailureRecoveryConfig | rach-ConfigBFR RACH-ConfigGeneric OPTIONAL, -- Need M | Beam failure recovery configuration can be configured without random access parameters |
| BeamFailureRecoveryRSConfig | rsrp-ThresholdBFR-r16 RSRP-Range OPTIONAL, -- Need M | Beam failure RS configuration can be configured without RSPR threshold |
| CandidateTCI-State-r18 | pathlossReferenceRS-Id-r18 PathlossReferenceRS-Id-r17 OPTIONAL, -- Cond Joint | TCI configuration for a candidate which uses joint TCI states can be configured without pathloss reference RS ID |
| CellGroupConfig | spCellConfig SpCellConfig OPTIONAL, -- Need M | Cell group (e.g., SCG) can be configured without an SpCell |
| CondReconfigToAddMod-r16 | condRRCReconfig-r16 OCTET STRING (CONTAINING RRCReconfiguration) OPTIONAL, -- Cond condReconfigAdd | Conditional reconfiguration can be configured without RRC message to be applied |
| CSI-IM-Resource | csi-IM-ResourceElementPattern CHOICE { <omit the irrelevant part> } OPTIONAL, -- Need M | CSI IM resource can be configured without RE pattern |
| DRB-ToAddMod | cnAssociation CHOICE { eps-BearerIdentity INTEGER (0..15), sdap-Config SDAP-Config } OPTIONAL, -- Cond DRBSetup | DRB can be configured without association to EPS bearer/SDAP configuration |
| LTm-Candidate-r18 | ltm-CandidatePCI-r18 PhysCellId OPTIONAL, -- Need M | LTM candidate can be configured without PCI of the candidate cell |
| MeasConfigAppLayer-r17 | serviceType-r17 ENUMERATED {streaming, mtsi, vr, spare5, spare4, spare3, spare2, spare1} OPTIONAL, -- Need M | Application layer measurements can be configured without the type of the measurements |
| RACH-ConfigCommon | ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { <omit the irrelevant part> } OPTIONAL, -- Need M | Random access parameters can be configured without the number of SSBs per RO and the number of CB preambles per SSB |
| SchedulingRequestResourceConfig | periodicityAndOffset CHOICE { <omit the irrelevant part> } OPTIONAL, -- Need M | Scheduling request resource can be configured without periodicity and offset |
| SearchSpace | monitoringSymbolsWithinSlot BIT STRING (SIZE (14)) OPTIONAL, -- Cond Setup | Search space can be configured without slots for PDCCH monitoring |

The root cause of this kind of ambiguity is usage of an optional ASN.1 field for functionally mandatory configuration parameter. Most often, the field is defined optional just to enable more efficient delta signalling (i.e., to enable the network to leave out the field in subsequent reconfiguration of the IE), but in some cases the field is defined optional because the field is not applicable to all use cases; pathlossReferenceRS-Id-r18 in CandidateTCI-State-r18 is an example of the latter. Given the root cause of this kind of ambiguity, it could be avoided by defining ASN.1 in such way that signalling for initial configuration of a feature/functionality would contain mandatory, not optional, ASN.1 fields for functionally mandatory configuration parameters. We discuss the details of the solution in this direction in Chapter 2.3.2 below.

Observation 4	The ambiguity "Functionally mandatory UE configuration parameters can be absent in over-the-air RRC messages for initial configuration of a feature/functionality" is very common in 5G RRC signalling. Typically, an occurrence of this kind of ambiguity is a negative side-effect of the delta signalling mechanism.

Analysis of the 2nd Ambiguity

Based on a study of our product development records and the 5G RRC ASN.1 (TS 38.331 v19.0.0), also this kind of ambiguity occurs in practice. While many occurrences can be identified by looking at TS 38.331 only, most of the occurrences could be identified only by studying RAN1 specifications and finding out what kind of reconfiguration RAN1 specs actually support for each feature/functionality. To clarify how this kind of ambiguity looks in practice, some examples, several of which we've seen in practice in IODT/live networks, are listed in Table 2.

Table 2. Some examples of the 2nd ambiguity in 5G RRC signalling for UE reconfiguration

| IE | Field | Explanation of the ambiguity |
| --- | --- | --- |
| RLCBearerConfig | servedRadioBearer CHOICE { srb-Identity SRB-Identity, drb-Identity DRB-Identity } OPTIONAL, -- Cond LCH-SetupOnly | Associated radio bearer of an RLC bearer can be reconfigured, which is not supported operation |
| SCellConfig | sCellConfigCommon ServingCellConfigCommon OPTIONAL, -- Cond SCellAdd | Common configuration of an SCell can be reconfigured without release-and-add of the SCell, while it is not supported to reconfigure SCell in that way |
| PDCP-Config | pdcp-SN-SizeUL ENUMERATED {len12bits, len18bits} OPTIONAL, -- Cond Setup1 | PDCP SN size for UL of a DRB can be reconfigured on the fly, while it is not supported by PDCP specification |
| PDSCH-ServingCellConfig | pucch-Cell ServCellIndex OPTIONAL, -- Cond SCellAddOnly | PUCCH cell of non-PUCCH SCell can be reconfigured, which is not supported operation |
| SearchSpace | controlResourceSetId ControlResourceSetId OPTIONAL, -- Cond SetupOnly | CORESET applicable to a search space can be reconfigured, which is not the intention |
| ServingCellConfig | firstActiveDownlinkBWP-Id BWP-Id OPTIONAL, -- Cond SyncAndCellAdd | First active DL BWP of an SCell can be reconfigured, which is not supported |

The root cause of this kind of ambiguity is having a field for a configuration parameter which shouldn't be modified/reconfigured in the signalling for reconfiguration of a feature/functionality. It should be understood that this type of ambiguity is not related to the delta signalling mechanism, as regardless of existence of the delta signalling mechanism, a reconfiguration of a feature/functionality needs to be supported anyway. Given the root cause of this kind of ambiguity, it could be avoided by defining ASN.1 in such way that signalling for reconfiguration of a feature/functionality would not contain ASN.1 fields for configuration parameters which shouldn't be modified/reconfigured. We discuss the details of the solution in this direction in Chapter 2.3.2 below.

Observation 5	The ambiguity "UE configuration parameters which shouldn't be modified after initial configuration of a feature/functionality can be sent in subsequent over-the-air RRC messages with new values" is relatively common in 5G RRC signalling. It is independent of the delta signalling mechanism, as UE reconfiguration for features/functionalities will anyway be required.

Analysis of the 3rd Ambiguity

The 3rd ambiguity is very generic by nature, as it basically means that any part of the UE configuration collides with any other part of the configuration (i.e., value or presence of any other configuration parameter). It is very difficult to analyze or estimate the number of ways how the UE configuration can be invalid in this way, as it is heavily related to how different (RAN1) features/functionalities are designed to operate and expected to be configured; only minor part of these ambiguities can be found out by studying TS 38.331 (field descriptions and Cond's). In any case, we can safely assume there are hundreds (or even more) of occurrences of the 3rd ambiguity in 5G RRC signalling, and we have seen at least dozens of examples in IODT/live networks.

As the 3rd ambiguity is very generic, it is also not related to the delta signalling mechanism. We consider the 3rd ambiguity very difficult to be avoided by RAN2 tools (such as ASN.1 techniques) only. For this reason, we do not discuss the 3rd ambiguity further in this document.

### 2.3.2	Solution

As observed above (Observation 4), the 1st ambiguity is strongly related to the delta signalling, so it should be avoided in 6G RRC signalling in order to make the delta signalling implementable. And of course another obvious reason to avoid the ambiguity is to make the signalling less ambiguous.

Proposal 3	6G RRC signalling for UE configuration is defined in a way that the ambiguity "Functionally mandatory UE configuration parameters can be absent in over-the-air RRC messages for initial configuration of a feature/functionality" is avoided.

As already briefly described in Chapter 2.3.1, the 1st ambiguity could be avoided by carrying the functionally mandatory configuration parameters by mandatory ASN.1 fields in the RRC signalling for initial configuration of a feature/functionality. However, while this sounds a simple solution (just change all such fields from optional to mandatory), doing only so would dramatically increase the over-the-air message size for UE reconfiguration since many functionally mandatory configuration parameters are actually very large (such as embedded RRC message to be applied upon a mobility procedure, as an example); if they were always carried by mandatory ASN.1 fields they would need to be signalled again every time the feature/functionality is reconfigured. To overcome this, a practical solution needs to involve using separate ASN.1 IEs for initial configuration of a feature/functionality and for subsequent reconfiguration of the feature/functionality. In this solution, the functionally mandatory configuration parameters are carried by mandatory ASN.1 fields in the IE used for initial configuration of a feature/functionality, but are carried by optional ASN.1 fields in the IE used for reconfiguration of the feature/functionality. By this approach, the 1st ambiguity can be effectively avoided without introducing negative side-effect as increased over-the-air message size.

Considering further, the separate IEs for initial configuration and subsequent reconfiguration of a feature/functionality make it also very easy to avoid the 2nd ambiguity. What it would require is simply to leave out the fields in the IE for reconfiguration for such configuration parameters which shouldn't be modified after initial configuration of the feature/functionality. In other words, if the solution of the separate IEs is applied, it is a low-hanging fruit to avoid also the 2nd ambiguity.

Proposal 4	6G RRC signalling for UE reconfiguration is defined in a way that the ambiguity "UE configuration parameters which shouldn't be modified after initial configuration of a feature/functionality can be sent in subsequent over-the-air RRC messages with new values" is avoided.

Figure 3 shows an example how a simple configuration will look like when separate ASN.1 IEs for initial configuration and subsequent reconfiguration are defined. As can be observed in the "6G style" ASN.1, the 1st ambiguity is avoided as field1 and field2 are mandatory fields in MyConfigAdd IE. Also the 2nd ambiguity is avoided as field2 does not exist in MyConfigMod IE. Furthermore, the over-the-air RRC message size for reconfiguration remains small, as the network needs to signal only those fields in MyConfigMod for which the network actually provides new values.

Figure 3. An example of an IE definition in 5G style and 6G style

It should however be noted that the example above is simplified. In reality, the ASN.1 fields within IEs can be IEs themselves (not only basic types such as INTEGER) and these "child IEs" have their own Add and Mod IE versions. This means that the full, implementable solution requires further study to identity the best way to handle the parent IE - child IE relationship for configuration and reconfiguration, and other detailed aspects of the solution. Based on all above, we propose the following:

Proposal 5	RAN2 to study the following ASN.1 method to make the delta signalling implementable and to avoid ambiguities in 6G RRC signalling for UE (re)configuration:

- Separate ASN.1 IEs are defined for initial configuration of a feature/functionality and subsequent reconfiguration of the feature/functionality;

- In the ASN.1 IE for initial configuration of the feature/functionality, functionally mandatory UE configuration parameters are carried by mandatory ASN.1 fields;

- In the ASN.1 IE for subsequent reconfiguration of the feature/functionality, ASN.1 fields do not exist for UE configuration parameters which shouldn't be modified after initial configuration of the feature/functionality.

## 2.4	Readability of ASN.1

In RAN2#131bis, human readability of (5G) RRC ASN.1 was discussed to have room for improvement for 6G (R2-2507172) [4]. Related to the concept of readability of ASN.1, we think there are many flavours of readability and even many flavours of human readability, which we discuss below.

For readability of ASN.1, we can consider machine readability and human readability, which we can further divide into delegate readability and developer readability, shown in Figure 3. With delegate readability, we refer to ASN.1 being easy to understand, update, and maintain by the 3GPP delegates. With developer readability, we refer ASN.1 being unambiguous to understand by the developers of UEs, networks, and testing equipment. With machine readability, we refer to ASN.1 definitions themselves capturing information of intended/valid usage of signalling; captured in this way, this information is readable by the ASN.1 decoder (i.e, the machine) enabling automation of ASN.1 processing in the UE and the network. More machine readable ASN.1 has less ambiguities discussed in Chapter 2.3.

It is important to understand that these different flavours of ASN.1 readability are not necessarily the same criteria. Especially, more machine readable ASN.1 requires more complex ASN.1 definitions, which tend to be less delegate readable (but may actually be more developer readable).

Figure 4. Different flavours of ASN.1 readability

For 6G RRC ASN.1, we think the machine readability has the highest priority, the developer readability has the second highest priority, and the delegate readability has the lowest priority.

Proposal 6	For readability of ASN.1, RAN2 to consider machine readability, delegate readability, and developer readability of which the machine readability is considered the highest priority, the developer readability the second highest priority, and the delegate readability the lowest priority.

# 3	Conclusion

Based on the discussion in the previous sections we have following observations and proposals:

UE Configuration Structure

Observation 1	In 5G, a common configuration refers to the part of the UE configuration which is cell specific, i.e., same configuration is applied for all UEs operating in the cell, whereas a dedicated configuration refers to the part of the UE configuration which is specific to this particular UE.

Observation 2	The concept of common and dedicated configurations makes the 5G UE configuration structure complex for both the UE and the network. From UE's perspective, the split of the UE configuration to common and dedicated parts has no functional significance.

Proposal 1	The concept of common and dedicated configurations is not applied as a defining factor for the UE configuration structure in 6G RRC signalling.

Delta Signalling

Observation 3	Delta signalling mechanism is worthwhile as it enables smaller over-the-air RRC messages for UE reconfiguration, which is beneficial for radio resource consumption and control plane latency perspective, but also significantly eases the UE in detecting which parts of the configuration are reconfigured.

Proposal 2	Delta signalling is applied in 6G RRC for UE reconfiguration. To make the delta signalling implementable in 6G, RAN2 targets to avoid the ambiguities related to the delta signalling mechanism.

ASN.1 Ambiguities

Observation 4	The ambiguity "Functionally mandatory UE configuration parameters can be absent in over-the-air RRC messages for initial configuration of a feature/functionality" is very common in 5G RRC signalling. Typically, an occurrence of this kind of ambiguity is a negative side-effect of the delta signalling mechanism.

Observation 5	The ambiguity "UE configuration parameters which shouldn't be modified after initial configuration of a feature/functionality can be sent in subsequent over-the-air RRC messages with new values" is relatively common in 5G RRC signalling. It is independent of the delta signalling mechanism, as UE reconfiguration for features/functionalities will anyway be required.

Proposal 3	6G RRC signalling for UE configuration is defined in a way that the ambiguity "Functionally mandatory UE configuration parameters can be absent in over-the-air RRC messages for initial configuration of a feature/functionality" is avoided.

Proposal 4	6G RRC signalling for UE reconfiguration is defined in a way that the ambiguity "UE configuration parameters which shouldn't be modified after initial configuration of a feature/functionality can be sent in subsequent over-the-air RRC messages with new values" is avoided.

Proposal 5	RAN2 to study the following ASN.1 method to make the delta signalling implementable and to avoid ambiguities in 6G RRC signalling for UE (re)configuration:

- Separate ASN.1 IEs are defined for initial configuration of a feature/functionality and subsequent reconfiguration of the feature/functionality;

- In the ASN.1 IE for initial configuration of the feature/functionality, functionally mandatory UE configuration parameters are carried by mandatory ASN.1 fields;

- In the ASN.1 IE for subsequent reconfiguration of the feature/functionality, ASN.1 fields do not exist for UE configuration parameters which shouldn't be modified after initial configuration of the feature/functionality.

Readability of ASN.1

Proposal 6	For readability of ASN.1, RAN2 to consider machine readability, delegate readability, and developer readability of which the machine readability is considered the highest priority, the developer readability the second highest priority, and the delegate readability the lowest priority.

# 4	References

- R2-250xxxx: "Report of 3GPP RAN2 WG2 meeting #131bis, Prague, Czech Republic", ETSI MCC, RAN2#131bis

- R2-2507073: "RRC, ASN.1 and other signalling aspects for 6G", Ericsson, RAN2#131bis

R2-2506957: "Control plane architecture and 6G RRC protocol design", MediaTek Inc., RAN2#131bis

R2-2507172: "Views on Control Plane for 6G", Qualcomm Incorporated, RAN2#131bis