3GPP TSG-RAN WG2 Meeting #132	R2-2508116

Dallas, USA, November 17th – 21st, 2025

Agenda Item:	10.2.1

Source:	OPPO

Title:	Left Issues on On-Demand SSB and OD-SIB1 (O008, O009)

Document for:	Discussion and Decision

- Introduction

This is to discuss the remaining issues for R19 NES.

- Discussion

At 126, it was agreed that

Paging and SIB1 update in NES cell

- After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

SIB1 acquisition upon SIB change notification in NES cell

- Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

Further, at 127, it was agreed that

OD-SIB1 Validity:

- Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

So, it is agreed that NES UE would follow the legacy manner for the SIB1 reception upon SIB change, i.e., it relies on network to proactively to send out the updated SIB1, which has nothing to do with OD-SIB1.

- R19 NES UE relies on legacy manner to receive SIB1 upon SIB change notification, which has nothing to do with OD-SIB1.

Based on the latest spec, the reception of SIB1 upon SIB change notification is dependent on the availability + validity of OD-SIB1 request configuration.

CR Start

2>	else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell:

3>	if the UE has a stored valid version of od-sib1-Config for this cell as specified in clause 5.2.2.4.28:

4>	if the SIB1 acquisition is upon receiving an indication that the system information has changed or upon receiving a PWS notification:

5> acquire the SIB1 (see clause 5.2.2.2.2), which is scheduled as specified in TS 38.213 [13];

4> else:

5> if the UE is in RRC_IDLE or in RRC_INACTIVE; or

5>	if the UE is in RRC_CONNECTED while T311 is running:

6>	perform the actions as specified in clause 5.2.2.3.3b;

CR Stop

Although there is a point that availability of SIB26 may be a way to differentiate OD-SIB1 capable and incapable UE, and thus good to prevent the latter one using barring operation (5.2.2.5) from the very beginning, the gap here is this condition mixes the two cases

- OD-SIB1 incapable UE

- OD-SIB1 capable UE without stored SIB26

In other words, following the current specification, an OD-SIB1 capable UE without SIB26 on hand (e.g., due to the Kssb was set as normal values) also has to bar and leaves the camping cell, when Kssb value is changed to a value of SIB1 not being scheduled and thus short-message is delivered.

- Current spec prevents OD-SIB1 capable UE to receive updated SIBs if there is no stored SIB26 on hand.

Therefore, it is suggested to correct the spec to align with the agreement, as follows (NOTE that for an OD-SIB1 capable UE, it can always bar the cell upon SIB1 acquisition failure, as captured in 5.2.2.3.3b)

CR Start

2>	else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell:

3>	if the UE supports OD-SIB1:

4>	if the SIB1 acquisition is upon receiving an indication that the system information has changed or upon receiving a PWS notification:

5> acquire the SIB1 (see clause 5.2.2.2.2), which is scheduled as specified in TS 38.213 [13];

4> else if the UE has a stored valid version of od-sib1-Config for this cell as specified in clause 5.2.2.4.28:

5> if the UE is in RRC_IDLE or in RRC_INACTIVE; or

5>	if the UE is in RRC_CONNECTED while T311 is running:

6>	perform the actions as specified in clause 5.2.2.3.3b;

CR Stop

- [O008] R2 clarify R19 OD-SIB1 capable UE perform SIB1 acquisition upon SIB change notification, regardless of od-sib1-Config availability, and revise the spec accordingly.

In the post-131bis email discussion, the following change was proposed (although later was dropped)

| Conditional Presence | Explanation |
| --- | --- |
| 2TA-TDD-Only | The field is optionally present, Need N, for a TDD cell, in the mimoParam-v1850 if additionalPCI-ToAddModList is present in ServingCellConfig and if tag2 is present in ServingCellConfig. It is absent otherwise. |
| AsyncCA | This field is mandatory present for SCells whose slot offset between the SpCell is not 0. Otherwise it is absent, Need S. |
| MeasObject | This field is mandatory present for the SpCell if the UE has a measConfig, and it is optionally present, Need M, for SCells. For (e)RedCap UEs, this field is optionally present, Need M. For SSB-less SCell(s), this field is not present if intraF-NeighMeasForSCellWithoutSSB is not supported by the UE, otherwise this field is optionally present, Need M. |

due to the following R2 conclusion

servingcellMO is not configured for OD-SSB measurement in Case1 SCell and OD-SSB specific MO is used for both CSI-RS measurements and OD-SSB measurements after OD-SSB is activated.

However, due to the following R2 agreement, apparently the first part above is wrong

RAN2 assumes the following table describes how the CSI-RS based serving cell measurements would work when OD-SSB is configured

| Scenario | servingCellMO contains | servingCellMO-OD contains | What is measured (OD-SSB not activated) for serving cell | What is measured (OD-SSB activated) for serving cell |
| --- | --- | --- | --- | --- |
| AO-SSB and OD-SSB both configured and on same frequency | AO-SSB CSI-RS OD-SSB on same freq |  | AO-SSB, CSI-RS | OD-SSB, CSI-RS |
| AO-SSB and OD-SSB both configured and on different frequency | AO-SSB CSI-RS | OD-SSB CSI-RS2 | AO-SSB, CSI-RS | OD-SSB, CSI-RS2 |
| Only OD-SSB configured | CSI-RS | OD-SSB CSI-RS2 | CSI-RS | OD-SSB, CSI-RS2 |
| Only OD-SSB configured |  | OD-SSB CSI-RS2 |  | OD-SSB, CSI-RS2 |

- R2#131bis made two agreements which are colliding with each other, and cause problem to CR work in the post R2 meeting discussion.

So R2 should revise the agreement to make it more accurate.

- [O009] Revise the R2#131bis agreement “servingcellMO is not configured for OD-SSB measurement in Case1 SCell and OD-SSB specific MO is used for both CSI-RS measurements and OD-SSB measurements after OD-SSB is activated.” as “servingcellMO can be configured for CSI-RS measurement only in Case1 SCell and OD-SSB specific MO is used for both CSI-RS measurements and OD-SSB measurements after OD-SSB is activated.”, and revise the spec accordingly.

CR Start

| Conditional Presence | Explanation |
| --- | --- |
| 2TA-TDD-Only | The field is optionally present, Need N, for a TDD cell, in the mimoParam-v1850 if additionalPCI-ToAddModList is present in ServingCellConfig and if tag2 is present in ServingCellConfig. It is absent otherwise. |
| AsyncCA | This field is mandatory present for SCells whose slot offset between the SpCell is not 0. Otherwise it is absent, Need S. |
| MeasObject | This field is mandatory present for the SpCell if the UE has a measConfig, and it is optionally present, Need M, for SCells. For (e)RedCap UEs, this field is optionally present, Need M. For SSB-less SCell(s), this field is not present if intraF-NeighMeasForSCellWithoutSSB is not supported by the UE and if OD-SSB is not configured in IE SCellConfig, otherwise this field is optionally present, Need M. |

CR Stop

- Conclusion

Proposal 1	[O008] R2 clarify R19 OD-SIB1 capable UE perform SIB1 acquisition upon SIB change notification, regardless of od-sib1-Config availability, and revise the spec accordingly.

Proposal 2	[O009] Revise the R2#131bis agreement “servingcellMO is not configured for OD-SSB measurement in Case1 SCell and OD-SSB specific MO is used for both CSI-RS measurements and OD-SSB measurements after OD-SSB is activated.” as “servingcellMO can be configured for CSI-RS measurement only in Case1 SCell and OD-SSB specific MO is used for both CSI-RS measurements and OD-SSB measurements after OD-SSB is activated.”, and revise the spec accordingly.