3GPP TSG-RAN WG2 Meeting #132	R2-2508148

Dallas, USA, Nov. 17th-21st

Agenda Item:	10.2.2

Source: 	CATT, CUC

Title: 	Discussion on TN and NTN integration

Document for: 	Discussion and Decision

1	Introduction

In RAN2#131bis, we had an initial discussion on supporting NTN in 6G and reached the following consensus.

For next meeting companies should identify essential requirements for NTN and identify which functionality should be considered in the common design with TN and what are NTN specific and whether they are essential.

Per the guidance of RAN2#131bis’s discussion and the agenda, this contribution discusses how to support TN and NTN integration in 6G day1 and gives our proposals.

2	Discussion

2.1	Deployment/scenario assumption and essential requirements

In 5G, NR NTN and IoT NTN were introduced and the NTN study has continued from Rel-16 to Rel-20. Huge effort was spent for studying and specifying a wide range of deployment architectures and scenarios. For 6G day1, we can build on the fruit of 5G NTN, select which to support in 6G day1, and consider necessary new functionalities. Below is our view on the deployment/scenarios to support for 6G day1.

Figure 1: 6G TN and NTN integration deployment/scenario

- Orbit:

In 5G NTN, two types of orbits, GSO and NGSO, were distinguished. For 6G day1, GSO and NGSO remain supported; it is FFS whether there is a requirement to further divide NGSO into MEO and LEO, e.g., for used in multi-orbit scenarios as illustrated in section 2.2.3.

- Cell type:

5G NTN supports cell types of earth fixed cell, quasi-earth fixed cell and earth moving cell. All these cell types can be supported in 6G day1.

- Payload:

For 5G NTN, we began with transparent payload in Rel-17; then, we add support of regenerative payload with base station on-board and support of CN on-board architecture to support S&F operation in Rel-19. For 6G day1, both transparent payload and regenerative payload can be supported from the outset. The CN on-board architecture can be left for later from RAN2’s perspective, as it mainly serves delay tolerance IoT traffic, which is not the main case considered by 6G day1.

- GNSS capability of UE:

In 5G NR NTN, only UEs with GNSS capability are supported, and the UE is assumed always to have valid GNSS. For 6G, there is requirement that the UE’s GNSS may be temporarily invalid or the UE may even be GNSS-incapable. Per the agreement achieved in RAN1#122bis [1], RAN1 will consider GNSS-less/resilient/based operation. Correspondingly, RAN2 needs to take all of the GNSS-less/resilient/based operation into consideration to identified possible higher layer impacts.

Agreement - The aspects to consider for supporting NTN include, but not limited to […] - GNSS-less/resilient/based operation […]

- Beam hopping:

In 5G, beam hopping is introduced to improve the coverage ratio with limited resources, enabled by extending the SSB default periodicity. Per the agreement achieved in RAN1#122bis [1], RAN1 will consider the beam hopping operation in 6G day1. Correspondingly, RAN2 needs to take the beam hopping operation into consideration to identified possible higher layer impacts.

Agreement - The aspects to consider for supporting NTN include, but not limited to […] - Beamforming / beam management / beam hopping

- Coexistence of TN and NTN, 5G and 6G:

After 6G is specified, 5G TN, 5G NTN, 6G TN and 6G NTN will coexist; interworking among these NW types should therefore be considered.

These aspects can be treated as deployment/scenarios assumptions for identifying the essential requirement and critical functionalities of NTN for 6G day1.

Observation 1: For 6G day1, the deployments/scenarios assumption including the following aspects:

- Orbit: GSO and NGSO (including MEO and LEO);

- Cell type: earth fixed cell, quasi-earth fixed cell and earth moving cell;

- Payload: transparent and regenerative; CN on-board can left for later from RAN2’s perspective;

- GNSS capability of UE: GNSS-less/resilient/based operation;

- Beam hopping;

- Coexistence of 6G NTN, 6G TN, 5G NTN and 5G TN.

Based on the deployment/scenario assumptions in Observation 1 and industry demand, the essential NTN requirements for 6G day1 are summarized below; cover support for the interworking of 6G NTN with 6G TN, 5G TN and 5G NTN and address the following NTN specific characteristics:

- Compare with TN, NTN has long and varying propagation delay, which affects timing mechanisms, initial access, L2 timing handling and measurement, etc.

- As the satellite moves, NTN cells continuously move across the ground, impacting mobility and measurements.

- Compared with TN, the near-far effect of signal strength variation is insignificant in NTN cell, which also influences mobility and measurements;

- An NTN cell provides very large coverage. This is advantageous in some cases (e.g., MBS), and should be exploited, whereas in other cases it should be carefully handled, e.g. for cross country border case (due to regulation requirement).

- The resources of satellite networks are still relatively limited, and 6G needs to consider how to schedule resources reasonably and serve UE more efficiently. For that purpose, collaboration between multi-orbits can be considered.

- GNSS-less/resilient/based operation influences timing mechanisms, initial access, measurements and mobility. However, GNSS-less/resilient operation pending RAN1’s progress, the associated study/discussion in RAN2 can be postponed.

- Beam hopping operation may affect initial access, paging and measurements. As the detailed design depends on RAN1’s beam-hopping pattern, the associated study/discussion in RAN2 can be postponed.

Additionally, supporting NTN positioning can be considered in 6G day1. It not only assists UE access when GNSS is unavailable, but also extends PNT for satellite, which is beneficial from an economic perspective.

Proposal 1: For 6G day1, the essential requirements for NTN include:

- supporting interworking of 6G NTN with 6G TN, 5G TN and 5G NTN;

- handling the following NTN specific characteristics:

- Long and varying propagation delay;

- High-speed satellite/cell movement;

- Insignificant near-far effect of signal strength variation;

- Large cell/beam coverage;

- Efficient utilization of limited satellite network resources, considering collaboration between multi-orbits.

- GNSS-less/resilient/based operation (GNSS-less/resilient pending RAN1’s progress);

- Beam hopping operation (pending RAN1’s progress).

- Supporting NTN positioning.

2.2	Critical functionalities for 6G day1

It is commonly understood that integrating TN and NTN within the same ecosystem can reduce the cost of both network and terminal manufacturing. Harmonized radio design for TN and NTN is one of the objectives of 6G day1, we should try to use common design for TN and NTN as much as possible. While taking into account the NTN specific characteristics outlined in Proposal 1, some essential NTN specific functionalities should also be studied in 6G day1.

In this section, we discuss which functionalities can be commonly design with TN and which are specific to NTN, considering the three aspects of CP functions, UP functions, and mobility aspects, respectively.

- 2.2.1 CP functions

For the CP function, there has been some discussion and some initial agreements have achieved some progresses:

- RRC modelling and connection management;

- RRC Signalling Design and (Re)configurations, e.g., overall RRC structure, configuration improvements, improve readability of ASN.1 for RRC signalling, modular design, RRC signalling encoding used ASN.1;

- Paging design;

- System Information design and transmission, e.g., support public warning requirement, on-demand SIB and SSB, scheduling SI design, SI update mechanism, etc.;

- Access control;

- Network and UE Energy Efficiency;

- Spectrum aggregation.

It seems that the first three bullets are not be impacted by the NTN specific characteristics, for which the common designed for both TN and NTN can be considered. For system information design, in addition to the system information also used in TN, it is also necessary to broadcast some NTN specific information, and valid time based SIB acquisition mechanism may need to be supported additionally to address the issue of constantly changing ephemeris. But the basic system information design should be the same for both TN and NTN. So the common design can be considered for both TN and NTN on basic system information design and transmission, leaving the parts impacted by NTN specific characteristics discussed in NTN section. For access control, it is preferred to design unified access control mechanism for TN and NTN, and by this mechanism the UE should be able to determine the cell is NTN or TN. For Network and UE Energy Efficiency, energy efficiency is also important for NTN, common design with TN can be considered, e.g., by supporting DTX/DRX mechanism. For spectrum aggregation, it refers to CA/DC in 5G; and in 6G, the discussion includes not only CA but also single cell with multi-carriers (SCMC). Considering CA/DC is not supported for NTN in 5G (mainly due to lack of requirements and support of deployment) and how to support spectrum aggregation for TN in 6G remains to be discussed, we can come back for this until TN has more progress.

Observation 2: In 5G, spectrum aggregation is not supported for NTN (mainly due to lack of requirements and support of deployment), whether to support that in 6G can be further studied.

Proposal 2: Considering the CP aspects, at least the following functions can be common designed with TN in 6G day1:

- RRC Modelling and connection management;

- RRC Signalling Design and (Re)configurations;

- Paging design;

- System Information design and transmission, e.g., on-demand SIB and SSB, scheduling SI design, SI undate mechanism;

- Unified access control mechanism;

- Network and UE Energy Efficiency.

Based on the above analysis, on the basic of Proposal 2, some additional NTN specific designs on the system information are needed and the design of 5G can be taken as baseline.

- Broadcast specific assistance information for NTN access, including ephemeris, common TA parameters, epoch time and validity duration, stop serving time , cell reference location, , neighbour cell information, etc.

- Additional support valid time based system information acquisition mechanism.

Proposal 3: Considering the CP aspects, in addition to the common designs with TN, at least study NTN specific SIB broadcast design in 6G day1, including:

- Broadcast specific assistance information for NTN access;

- Valid time based system information acquisition mechanism.

- 2.2.2	UP functions

High-level UP functions were discussed in last meeting, and some progresses have achieved, including:

- UP architecture/ protocol stack: including the protocol stack design and which protocol layer supports each functionality;

- UP processing: UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer;

- QoS, QoE and Service-awareness framework: including a general framework and potential applications/service-awareness mechanism.;

- Scheduling: At least support dynamic grant and configured grant, to address the SR/BSR/DSR latency;

- L2 re-transmission: HARQ and L2 ARQ;

- Random Access: including RACH procedure, e.g., 4-step RACH, 2-step RACH.

It seems that these high-level functions will not be impacted by the NTN specific characteristics, the common designed for both TN and NTN can be considered on these functions.

Proposal 4: Considering the UP aspects, at least the following functions can be common designed with TN in 6G day1:

- UP architecture and protocol stack;

- UP processing;

- QoS and framework;

- Scheduling;

- L2 re-transmission;

- Random access.

As for the NTN specific aspects, the 5G NTN design can be taken as basic, which are mainly designed to adapt the long propagation delay characteristic of NTN scenario including:

- PDCP: discardTimer and t-Reordering are extended;

- RLC: t-Reassembly timer is extended;

- MAC:

  - The start of ra-ResponseWindow and ra-ContentionResolutionTimer is delayed with the UE-gNB RTT;

  - UE autonomously pre-compensate the TTA for the RTT between the UE and the RP;

  - Report Timing Advance based on configuration, used to assist in networking scheduling;

  - Support HARQ-disabled mechanism;

  - UL scheduling aspects, including BSR enhancement and sr-ProhibitTimer extension.

For the L2 timer as listed above, in 6G, they can be set to a large value range from the beginning, with considering the propagation delay characteristic of NTN. Since it is too early to discuss the detailed procedure, we can discuss how to handle these L2 timers until there is more progress. For the TA Pre-compensation, TA report, HARQ-disabled (if HARQ is supported) mechanism and UL scheduling aspects, similar mechanisms can be considered in 6G.

Proposal 5: Considering the UP aspects, in addition to the functions common design with TN, at least study the following NTN specific aspects in 6G day1:

- TA handling due to the long propagation delay, including TA Pre-compensation and TA report;

- HARQ (if supported) enable/disable.

- 2.2.3	Mobility aspects

In the last meeting, there is an initial discussion on mobility aspects which is mainly focused on 6G TN. The discussion covers mobility framework and targets, measurement framework, mobility types and inter/intra-RAT mobility, etc. More detailed, including:

- RRM measurement based handover, including type of legacy HO, CHO, (C)LTM;

- RLM/RLF;

- L3/L1 measurement and report framework;

- RRM measurement and priority based cell (re)selection mechanism;

- Inter-RAT mobility with 5G.

At present, it seems that the first four items are also applicable to NTN, so these functionalities can be common designed for both TN and NTN unless a problem is identified for NTN in future. Whether/how to support the above functionalities can be discussed in the TN mobility topic. As for the inter-RAT mobility with 5G, including interworking of 6G NTN with 5G TN and 5G NTN, some NTN specific designs may need to be further identified, but the basic schemes used to support inter-RAT mobility are common to both TN and NTN, e.g., cell reselection, handover and redirection procedures.

Proposal 6: Considering mobility aspects, at least the following functionalities are common design with TN in 6G day1:

- Measurement based handover, including type of legacy HO, CHO, (C)LTM;

- RLM/RLF;

- L3/L1 measurement and report framework;

- Measurement and priority based cell (re)selection mechanism;

- Basic schemes used to support inter-RAT mobility with 5G, e.g., cell reselection, handover and redirection.

As for the NTN specific aspects, the fruit of 5G NTN mobility can be taken as basic. In 5G, some mobility enhancements were introduced to handle the NTN specific characteristics:

For RRC_CONNECTED state:

- Time-based and location-based measurement/CHO are supported for (quasi-)earth-fixed cell and earth moving cell, which were introduced to handle the issue of RRM unsuitability caused by insignificant near-far effect of NTN cell;

- Measurement window adjustment mechanism and multiple SMTC windows per frequency, which were introduced to address measurement issues caused by the long and varying propagation delay characteristic of NTN cell;

- Satellite switch with re-sync mechanism. This mechanism can enable quasi-earth fixed cell to achieve a similar effect to earth fixed cell, and effectively reduce the frequency handover and corresponding signalling overhead.

For RRC_IDLE/INACTIVE state:

- Time-based and location-based measurement initiation for (quasi-)earth-fixed cell and earth moving cell, which were introduced to handle the issue of RRM unsuitability caused by insignificant near-far effect of NTN cell;

- Measurement window adjustment mechanism and multiple SMTC windows per frequency, which were introduced to address measurement issues caused by the long and varying propagation delay characteristic of NTN cell;

For intra-RAT mobility between NTN and TN:

- In Uu, NTN and TN belong to the same RAT, based on support of Satellite access Radio Frequency, handover. reselection and redirection between NTN and TN can be supported.

- Additionally, for cell reselection: support providing NTN assistance information in TN cells to assist UE searching NTN cells; support providing TN coverage information to assist UE determine when to start searching TN frequencies.

For inter-RAT mobility:

- In 5G, the inter-RAT mobility among NR NTN, NR TN, EUTRAN TN, EUTRAN NTN, NB-IoT TN and NB-IoT NTN are discussed and supported enhancements for many cases, e.g., LTE TN to NR NTN, NB-IoT/eMTC NTN to NR-NTN, NR NTN to NB-IoT/eMTC NTN, etc.

These above functionalities specified in 5G are designed with considering the characteristics of NTN, help ensure mobile continuity, reduce signalling overhead and save UE energy consumption in NTN, and can therefore be supported in 6G day1. For the inter-RAT mobility, in 6G day1, only 6G NTN to 5G TN/NTN are considered. The specified mechanisms in 5G can be taken as baseline.

Apart from these above aspects, SA2 is discussing the multi-orbit scenario based on SA1’s requirement. Reference to TS 22.261 [2], SA1 endorsed a multi-orbit requirement which proposed to support UE switching between orbit types depends on latency/QoS characteristic, etc. SA2 is discussing this requirement for 6G day1 and would like to coordinate and align with RAN. We observe that the multi-orbit scenario can improve network resource utilization efficiency, and it is also beneficial for UE energy conservation (e.g., an IDLE UE camps on GSO cell and perform service transmission on NGSO cell when needed, avoiding frequent reselection and measurement). It is proposed to study the mobility for multi-orbit scenario in 6G day1.

Citation from TS 22.261, 6.46.12 [2] ### 6.46.12 Multi-orbit […] Subject to regulatory requirements and operator policies, the 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g., altitude, orbital characteristics, satellite capabilities) shall support the selection of suitable satellites for the UE and the switching between satellites with orbit types having different characteristics depending on e.g. latency requirements, QoS requirements, access capabilities and availability. A 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g., altitude, orbital characteristics, satellite capabilities) shall be able to support establishing the initial UE connection to the network through satellites with a given characteristics and then having the UE data traffic using satellite with different characteristics. […]

Citation from S2#171 meeting report [3] S2-2508409 (P-CR) 23.801-01: [WT#7]: Work task and key issue for 6G satellite access. (Source: CATT, CUC) Postponed due to lack of time. Status: Postponed. # A.X Work Task 7 WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects. NOTE 1: The detailed scope for WT#7 will be coordinated and aligned with RAN, including discussing multi-orbit NTN-NTN mobility handling, in Q1 2026. A checkpoint is set to Q1 2026 to revisit WT/KI descriptions if needed before discussing solutions. NOTE 2: This WT has IWK aspects that are scoped under WT#2. NOTE 3: This WT has mobility aspects that are scoped under WT#1.1.

Proposal 7: Considering mobility aspects, at least study the following NTN specific functionalities in 6G day1:

- Reuse 5G functionalities:

- Time-based and location-based measurement/CHO for (quasi-)earth-fixed cell and earth moving cell;

- Satellite switch with re-sync mechanism;

- Measurement adjustment and enhancement (pending progress of RAN1) for all RRC states;

- Time-based and location-based measurement initiation for cell reselection for (quasi-)earth-fixed cell and earth moving cell;

- Mobility between 6G NTN and 6G TN and inter-RAT mobility between 6G NTN and 5G TN/NTN, in aspects of handover, redirection and reselection.

- Study new functionality:

- Mobility for multi-orbit scenario.

3	Conclusion

This contribution discusses how to support TN and NTN integration in 6G day1 and gives the following observations and proposals.

Deployment/scenario assumption and essential requirements

Observation 1: For 6G day1, the deployments/scenarios assumption including the following aspects:

- Orbit: GSO and NGSO (including MEO and LEO);

- Cell type: earth fixed cell, quasi-earth fixed cell and earth moving cell;

- Payload: transparent and regenerative; CN on-board can left for later from RAN2’s perspective;

- GNSS capability of UE: GNSS-less/resilient/based operation;

- Beam hopping;

- Coexistence of 6G NTN, 6G TN, 5G NTN and 5G TN.

Proposal 1: For 6G day1, the essential requirements for NTN include:

- supporting interworking of 6G NTN with 6G TN, 5G TN and 5G NTN;

- handling the following NTN specific characteristics:

- Long and varying propagation delay;

- High-speed satellite/cell movement;

- Insignificant near-far effect of signal strength variation;

- Large cell/beam coverage;

- Efficient utilization of limited satellite network resources, considering collaboration between multi-orbits.

- GNSS-less/resilient/based operation (GNSS-less/resilient pending RAN1’s progress);

- Beam hopping operation (pending RAN1’s progress).

- Supporting NTN positioning.

Critical functionalities for 6G day1

CP functions

Observation 2: In 5G, spectrum aggregation is not supported for NTN (mainly due to lack of requirements and support of deployment), whether to support that in 6G can be further studied.

Proposal 2: Considering the CP aspects, at least the following functions can be common designed with TN in 6G day1:

- RRC Modelling and connection management;

- RRC Signalling Design and (Re)configurations;

- Paging design;

- System Information design and transmission, e.g., on-demand SIB and SSB, scheduling SI design, SI undate mechanism;

- Unified access control mechanism;

- Network and UE Energy Efficiency.

Proposal 3: Considering the CP aspects, in addition to the common designs with TN, at least study NTN specific SIB broadcast design in 6G day1, including:

- Broadcast specific assistance information for NTN access;

- Valid time based system information acquisition mechanism.

UP functions

Proposal 4: Considering the UP aspects, at least the following functions can be common designed with TN in 6G day1:

- UP architecture and protocol stack;

- UP processing;

- QoS and framework;

- Scheduling;

- L2 re-transmission;

- Random access.

Proposal 5: Considering the UP aspects, in addition to the functions common design with TN, at least study the following NTN specific aspects in 6G day1:

- TA handling due to the long propagation delay, including TA Pre-compensation and TA report;

- HARQ (if supported) enable/disable.

Mobility aspects

Proposal 6: Considering mobility aspects, at least the following functionalities are common design with TN in 6G day1:

- Measurement based handover, including type of legacy HO, CHO, (C)LTM;

- RLM/RLF;

- L3/L1 measurement and report framework;

- Measurement and priority based cell (re)selection mechanism;

- Basic schemes used to support inter-RAT mobility with 5G, e.g., cell reselection, handover and redirection.

Proposal 7: Considering mobility aspects, at least study the following NTN specific functionalities in 6G day1:

- Reuse 5G functionalities:

- Time-based and location-based measurement/CHO for (quasi-)earth-fixed cell and earth moving cell;

- Satellite switch with re-sync mechanism;

- Measurement adjustment and enhancement (pending progress of RAN1) for all RRC states;

- Time-based and location-based measurement initiation for cell reselection for (quasi-)earth-fixed cell and earth moving cell;

- Mobility between 6G NTN and 6G TN and inter-RAT mobility between 6G NTN and 5G TN/NTN, in aspects of handover, redirection and reselection.

- Study new functionality:

- Mobility for multi-orbit scenario.

4	Reference

- Chair notes RAN1#122bis

- TS 22.261 Service requirements for the 5G system.

- S2#171 meeting report