3GPP TSG-RAN WG2 Meeting #132   	R2-2508186

Dallas, USA, 17th – 21st November, 2025

Source:	ZTE Corporation, Sanechips

Title:	RRC corrections (RILZ004, RILS900 and others) for R19 IoT NTN

Agenda Item:	8.9.2

Document for:	Discussion and Decision

Introduction

In this contribution, we will elaborate our RIL comments for RRC spec, e.g., RILZ004. And also give some discussion on other RIL or remaining issues for RRC spec.

Discussion

  - RILZ004: Delta configuration of CB-Msg3-EDT resources on non-anchor carrier

In the RRC running CR (Rapporteur correction on IoT NTN Ph3_v00_Rapp), the CB-Msg3-EDT resources configuration for NB-IoT is as below:

CB-Msg3-EDT resources configuration for anchor carrier: SystemInformationBlockType2-NB information element -- ASN1START SystemInformationBlockType2-NB-r13 ::= SEQUENCE { radioResourceConfigCommon-r13 RadioResourceConfigCommonSIB-NB-r13, ue-TimersAndConstants-r13 UE-TimersAndConstants-NB-r13, freqInfo-r13 SEQUENCE { ul-CarrierFreq-r13 CarrierFreq-NB-r13 OPTIONAL, -- Need OP additionalSpectrumEmission-r13 AdditionalSpectrumEmission }, timeAlignmentTimerCommon-r13 TimeAlignmentTimer, <skip> } -- ASN1STOP RadioResourceConfigCommonSIB-NB information element -- ASN1START RadioResourceConfigCommonSIB-NB-r13 ::= SEQUENCE { rach-ConfigCommon-r13 RACH-ConfigCommon-NB-r13, bcch-Config-r13 BCCH-Config-NB-r13, pcch-Config-r13 PCCH-Config-NB-r13, nprach-Config-r13 NPRACH-ConfigSIB-NB-r13, <skip> [[ ntn-ConfigCommon-r17 SEQUENCE { ta-Report-r17 ENUMERATED {enabled} OPTIONAL, -- Need OR t318-r17 ENUMERATED { ms0, ms200, ms500, ms1000, ms2000, ms4000, ms8000}, nprach-TxDurationFmt01-r17 NPRACH-TxDurationFmt01-NB-r17 OPTIONAL, -- Need OR nprach-TxDurationFmt2-r17 NPRACH-TxDurationFmt2-NB-r17 OPTIONAL, -- Need OR npusch-TxDuration-r17 NPUSCH-TxDuration-NB-r17 OPTIONAL -- Need OR } OPTIONAL -- Cond NTN ]], [[ cb-Msg3-ConfigSIB-NB-r19 CB-Msg3-ConfigSIB-NB-r19 OPTIONAL -- Need OR ]] } BCCH-Config-NB-r13 ::= SEQUENCE { modificationPeriodCoeff-r13 ENUMERATED {n16, n32, n64, n128} } PCCH-Config-NB-r13 ::= SEQUENCE { defaultPagingCycle-r13 ENUMERATED {rf128, rf256, rf512, rf1024}, nB-r13 ENUMERATED { fourT, twoT, oneT, halfT, quarterT, one8thT, one16thT, one32ndT, one64thT, one128thT, one256thT, one512thT, one1024thT, spare3, spare2, spare1}, npdcch-NumRepetitionPaging-r13 ENUMERATED { r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048, spare4, spare3, spare2, spare1} } -- ASN1STOP CB-Msg3-EDT resources configuration for non-anchor carriers: SystemInformationBlockType22-NB information element -- ASN1START SystemInformationBlockType22-NB-r14 ::= SEQUENCE { dl-ConfigList-r14 DL-ConfigCommonList-NB-r14 OPTIONAL, -- Need OR ul-ConfigList-r14 UL-ConfigCommonList-NB-r14 OPTIONAL, -- Need OR <skip> [[ cb-Msg3-ProbabilityAnchorList-NB-r19 CB-Msg3-ProbabilityAnchorList-NB-r19 OPTIONAL -- Cond cb-msg3-config ]] } <skip> UL-ConfigCommon-NB-r14 ::= SEQUENCE { ul-CarrierFreq-r14 CarrierFreq-NB-r13, nprach-ParametersList-r14 NPRACH-ParametersList-NB-r14 OPTIONAL, -- Need OR ..., [[ nprach-ParametersListEDT-r15 NPRACH-ParametersList-NB-r14 OPTIONAL -- Cond EDT ]], [[ rsrp-ThresholdsPrachInfoList-r16 RSRP-ThresholdsNPRACH-InfoList-NB-r13 OPTIONAL -- Need OR ]], [[ cb-Msg3-ConfigSIB-NB-r19 CB-Msg3-ConfigSIB-NB-r19 OPTIONAL -- Need OR ]] } <skip> CB-Msg3-ProbabilityAnchorList-NB-r19 ::= SEQUENCE (SIZE (1..maxCE-Level-CB-Msg3-NB-r19)) OF CB-Msg3-ProbabilityAnchor-NB-r19 CB-Msg3-ProbabilityAnchor-NB-r19 ::= SEQUENCE { cb-Msg3-ProbabilityAnchor-NB-r19 ENUMERATED { zero, oneSixteenth, oneFifteenth, oneFourteenth, oneThirteenth, oneTwelfth, oneEleventh, oneTenth, oneNinth, oneEighth, oneSeventh, oneSixth, oneFifth, oneFourth, oneThird, oneHalf} OPTIONAL -- Need OP } -- ASN1STOP – CB-Msg3-ConfigSIB-NB The IE CB-Msg3-ConfigSIB-NB is used to specify the CB-Msg3-EDT configuration. CB-Msg3-ConfigSIB-NB information element -- ASN1START CB-Msg3-ConfigSIB-NB-r19 ::= SEQUENCE { cb-Msg3-MinRSRP-Threshold-NB-r19 NRSRP-Range-NB-r14 OPTIONAL, --Need OR cb-Msg3-RSRP-CE-Levels-NB-r19 CB-Msg3-RSRP-CE-Levels-NB-r19 OPTIONAL, --Need OR cb-Msg3-ConfigList-NB-r19 CB-Msg3-ConfigList-NB-r19, powerRampingParameters-NB-r19 PowerRampingParameters-NB-r19 } CB-Msg3-ConfigList-NB-r19 ::= SEQUENCE (SIZE (1.. maxCE-Level-CB-Msg3-NB-r19)) OF CB-Msg3-Config-NB-r19 CB-Msg3-Config-NB-r19 ::= SEQUENCE { cb-Msg3-TBS-NB-r19 ENUMERATED {b144, b328, b408, b504, b584, b680, b808, b936}, cb-Msg3-NumOfReplicas-NB-r19 INTEGER (1..4), cb-Msg3-TimeResource-NB-r19 SEQUENCE { npusch-Periodicity-r19 ENUMERATED {ms40, ms80, ms160, ms240, ms320, ms640, ms1280, ms2560}, npusch-StartSFN-r19 INTEGER (0..1023), npusch-StartSubframe-r19 INTEGER (0..9) }, cb-Msg3-PhysicalConfig-r19 SEQUENCE { npusch-NumRUsIndex-r19 INTEGER (0..7), npusch-NumRepetitionsIndex-r19 INTEGER (0..7), npusch-SubCarrierSetList-r19 CHOICE { npusch-SubCarrierSetList-khz15 SEQUENCE (SIZE(1..12)) OF INTEGER (0..18), npusch-SubCarrierSetList-khz3dot75 SEQUENCE (SIZE(1..48)) OF INTEGER (0..47) }, npusch-MCS-r19 CHOICE { singleTone INTEGER (0..10), multiTone INTEGER (0..13) }, ack-NumRepetitions-NB-r19 ACK-NACK-NumRepetitions-NB-r13, p0-UE-NPUSCH-r19 INTEGER (-8..7), alpha-NB-r19 ENUMERATED {al0, al04, al05, al06, al07, al08, al09, al1}, npdcch-CarrierIndex-r19 INTEGER (1..maxNonAnchorCarriers-NB-r14) OPTIONAL, -- Need OP npdcch-NumRepetitions-r19 ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048, spare4, spare3, spare2, spare1}, npdcch-StartSF-CSS-r19 ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64}, npdcch-Offset-CSS-r19 ENUMERATED {zero, oneEighth, oneFourth, threeEighth} }, cb-Msg3-TxWindow-NB-r19 SEQUENCE { windowSize-NB-r19 INTEGER {3..10}, windowPeriodicity-NB-r19 ENUMERATED {n16, n32, n48, n64, n128, n256, n384 n512, n768, n1024, n1280, n1536,n1792, n2048, n2560, n3072} } OPTIONAL, --Need OP cb-Msg3-ResponseWindow-NB-r19 ENUMERATED {pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64}, cb-Msg3-MaxAttemptNum-NB-r19 ENUMERATED {n2, n3, n4, n5, n6, n7, n8, n10} OPTIONAL, --Need OP ... } CB-Msg3-RSRP-CE-Levels-NB-r19 ::= SEQUENCE (SIZE(1..2)) OF RSRP-Range NPUSCH-SubCarrierSet-r19 ::= CHOICE { khz15 INTEGER (0..18), khz3dot75 INTEGER (0..47) } PowerRampingParameters-NB-r19 ::= SEQUENCE { powerRampingStep-NB-r19 ENUMERATED {dB0, dB2, dB4, dB6}, cb-Msg3-InitialReceivedTargetPower-NB-r19 ENUMERATED { dBm-130, dBm-128, dBm-126, dBm-124, dBm-122, dBm-120, dBm-118, dBm-116, dBm-114, dBm-112, dBm-110, dBm-108, dBm-106, dBm-104, dBm-102, dBm-100, dBm-98, dBm-96, dBm-94, dBm-92, dBm-90, dBm-88, dBm-86, dBm-84,dBm-82, dBm-80} } -- ASN1STOP ------

It can be seen that, most of parameters in CB-Msg3-Config-NB are mandatory. Considering that CB-Msg3-EDT resources can also be provided on non-anchor carriers, and it is highly possible that the parameter configuration for a certain CE level on a non-anchor carrier is (mostly) identical to that of the corresponding CE level on the anchor carrier (due to that resources or timer/window length configuration may mainly adapt to the required repetitions, not the carriers), we think it is also necessary to consider allowing delta configuration for CB-Msg3 resources on non-anchor carriers, enabling some absent parameters to refer to the configuration of the corresponding CE level on the anchor carrier. This is beneficial for reducing signaling overhead.

Proposal 1a: It’s suggested to allow delta configuration for CB-Msg3 resources on non-anchor carriers.

The definition way of NPRACH-ConfigSIB-NB for NPRACH resources configuration in NB-IoT can be taken as reference on how to achieve the delta configuration among anchor and non-anchor carriers, and also among different NPRACH resources types. Please note there are several kinds of delta configuration for the NPRACH resources configured in SIB2, SIB22, SIB23 etc. We skip some too complicated delta configurations, e g., those for Fmt2 or Fmt2EDT, and mainly check the typical ones, e.g., for non-anchor carrier, for EDT and for TDD. Some observations are given as below:

- The NPRACH-Parameters-NB-r13 is used to define an entry of nprach-ParametersList-r13, e.g., to define the normal NPRACH resources for each CE level of the anchor carrier. All the parameters in NPRACH-Parameters-NB-r13 are mandatory.

- The NPRACH-Parameters-NB-r14 is used to define an entry of nprach-ParametersList-r14, e.g., to define the normal NPRACH resources for each CE level of each non-anchor carrier. The nprach-Parameters-r14 is the only embedded structure in NPRACH-Parameters-NB-r14 which can be optional and with “Need OR” code. All the parameters in nprach-Parameters-r14 are also optional and with “Need OP”, that means, the UE behavior needs to be explicitly specified when a parameter is absent in nprach-Parameters-r14.

- The NPRACH-Parameters-NB-r14 is also used to define an entry of nprach-ParametersListEDT-r15, e.g., to define the EDT resources for each CE level of anchor carrier and each non-anchor carrier.

- The NPRACH-ParametersTDD-NB-r15 is used to define an entry of nprach-ParametersListTDD-r15, e.g., to define the TDD resources for each CE level of anchor carrier and each non-anchor carrier. The nprach-Parameters-r15 is the only embedded structure in NPRACH-ParametersTDD-NB-r15 which can be optional and with “Need OR” code. All the parameters in nprach-Parameters-r15 are also optional and with “Need OP”, that means, the UE behavior needs to be explicitly specified when a parameter is absent in nprach-Parameters-r15.

- Based on the optionality of parameters, at least the following typical delta configurations:

- CEL delta configuration for a non-anchor carrier:

  - It’s possile that all the entries for all the CE levels are configured for a non-anchor carrier. Howeveve, it’s also possible that nprach-Parameters-r14 in NPRACH-Parameters-NB-r14 is whole absent, e.g., a NPRACH-Parameters-NB-r14 in the NPRACH-ParametersList-NB-r14 is configured empty. That means, it’s allowable to configure a CELwithout NPRACH resources configuration. For example, it’s possible for a non-anchor carrier, e.g., Carrier_5, only CEL0 and CEL2 are configured with normal NPRACH resources, but there is no configuration for CEL1 (e.g., empty configuration for CEL1).

  - For EDT resources configuration, it’s also possible that one or more CELs can be without EDT resources configuration for a non-anchor carrier, if nprach-Parameters-r14 is absent for a NPRACH-Parameters-NB-r14;

  - For TDD resources configuration, it’s also possible that one or more CELs can be without TDD resources configuration for a non-anchor carrier, if nprach-Parameters-r15 is absent for a NPRACH-ParametersTDD-NB-r15.

- Non-anchor carrier delta configuration:

- For a non-anchor carrier (when a parameter absent, directly refer to normal PRACH in anchor carrier): For example, if a parameter A is absent in the normal PRACH configuration for a CE level, e.g., CEL2, for a non-anchor carrier, e.g., Carrier_5, the parameter A (call it A**) of the normal NPRACH configuration for the CEL2 for anchor carrier, e.g., Carrier_0, would be applied to A.

- EDT resources delta configuration:

  - For anchor carrier: for example, if a parameter B is absent in the EDT configuration for a CE level, e.g. ,CEL2 for the anchor carrier, e.g., Carrier_0, the parameter B (call it B**) of the normal NPRACH configuration for the CEL2 for Carrier_0 would be applied to B;

  - For the same non-anchor carrier (when a parameter absent, firstly refer to normal PRACH in the same non-anchor carrier, and secondly refer to normal PRACH in anchor carrier): for example, if a parameter B is absent in the EDT configuration for a CE level, e.g., CEL2, for a non-anchor carrier, e.g., Carrier_5, and if the parameter B (call it B*) of the normal NPRACH configuration for the CEL2 for Carrier_5 is present, B* would be applied to EDT configuration for the CEL2 for Carrier_5. However, if B* is absent, the parameter B (call it B**) of the normal NPRACH configuration for the CEL2 for anchor carrier, e.g., Carrier_0, would be applied to B.

- TDD resources delta configuration:

  - All the parameters of TDD NPRACH resources configuration for anchor carrier should be mandatorily configured.

  - For the same non-anchor carrier (when a parameter absent, directly refer to TDD PRACH in anchor carrier): if a parameter C is absent in the TDD NPRACH resources configuration for a CE level, e.g., CEL2, for a non-anchor carrier, e.g., Carrier_5, the parameter C (call it C**) of the TDD NPRACH resources configuration for the CEL2 for anchor carrier, e.g., Carrier_0, would be applied to C.

Based on the above observations for all the existing delta configurations for NPRACH resources, we suggest the following simple delta configuration for CB-Msg3 resources on non-anchor carriers:

Proposal 1b: the detailed delta configuration rules for CB-Msg3 resources on non-anchor carriers are as below:

- To re-organize all the parameters in CB-Msg3-Config-NB-r19 into a new IE, e.g., CB-Msg3-Config-Parameters-r19 which is with need code “Need OR”; (To make it allowable the configuration for CEL as empty)

- To make all mandatory parameters in previous CB-Msg3-Config-NB-r19 optional, with need code “Need OP”

  - For a few parameters previously optional, to keep the field description unchanged, e.g., to follow the existing UE behaviors description;

  - For other parameters newly marked with “Need OP”, to add the following “NOTEs” to allow an absent parameter for non-anchor carrier to refer to corresponding one for anchor carrier):

NOTE:

- The field is mandatory in all the entries of CB-Msg3-ConfigList-NB in SystemInformationBlockType2-NB.

- If the field is absent in an entry of CB-Msg3-ConfigList-NB in SystemInformationBlockType22-NB, the value of the same field in the corresponding entry of CB-Msg3-ConfigList-NB in SystemInformationBlockType2-NB applies.

  - RILS900: PWS SIB accumulation across SI windows

In previous RAN2#131bis meeting, a RIL[S900] commented that, as PWS SIB may be long and sometimes may need to be segmented, contents of PWS SIB segments may change across different SI-windows.

As different SIB content via the segments will not trigger the SI notification procedures, if to allow cross-window SIB accumulation, it may cause error for PWS SIB decoding. However, in the case that PWS message not segmented, cross-window SIB accumulation provides a gain, so completely disabling cross-window SIB accumulation PWS SIB is also incorrect. Therefore, RIL[S900] suggests to introduce network control on the PWS SIB accumulation, e.g., to introduce an indication (e.g., si-NoAccumulation) to indicate whether a UE may accumulate SI decoding for PWS SIB, as below:

### 5.2.3a Acquisition of an SI message by BL UE or UE in CE or a NB-IoT UE <skip> 1> if the SI message has not been configured with si-NoAccumulation: 2> if the SI message was not possible to decode from the accumulated SI message transmissions by the end of the SI-window, continue reception and accumulation of SI message transmissions on DL-SCH in the next SI-window occasion for the concerned SI message;

However, due to the following reasons, we disagree to introduce such network indication:

- As mentioned in other contribution [R2-2507611], there are already some legacy SIBs (e.g., ETWS/CMAS for eMTC, EAB, UAC, SIB31 etc.) which have similar issue, e.g., no update notification for contents change. For a concerned SI message, UEs in different locations may experience varying reception performance—some may successfully receive it within a single SI-window, while others may not. Therefore, the current specification leaves room for UE implementation flexibility, e.g., UE is not required to accumulate several SI messages in parallel but can accumulate a SI message across multiple SI windows if needed, depending on coverage condition.

- However, if we introduce a network control indication, it will affects all relevant SIBs and all UEs support of this indication. For example, if the network configures a “si-NoAccumulation = True” solely due to that PWS SIBs are segmented, it seems to imply all the accumulation reception cross SI-windows are prohibited (e.g., there is an implicit “else” branch in above change example which may means no accumulation is performed). This is incorrect as it scarifies the possibility and benefits of accumulation reception cross-window for other SIBs.

- On the other hand, from NW perspective, there may be already some potential implementation optimizations. For example, for a concerned SI for such special SIBs, the eNB may allocate sufficient repetitions within a single window. Or, if the required repetition number is too large to fit within one SI-window, the transmission of the next changed instance of this SI could be slightly delayed.

In a summary, considering the potential risk of incorrectly prohibiting SI accumulation cross-window and some possibly existing network optimizations, we echo the Proposal 4 in [R2-2507611]:

Proposal 2: As legacy, it allows the NB-IoT UE implementation to attempt to accumulate PWS (ETWS/CMAS) SI messages across SI windows. No specification impact is expected.

  - Remaining issue: Clarification on CB-Msg3 Tx window

In current RRC running CR, the CB-Msg3 Tx window is defined as below (taking NB-IoT as example):

CB-Msg3-Config-NB-r19 ::= SEQUENCE { <skip> cb-Msg3-TimeResource-NB-r19 SEQUENCE { npusch-Periodicity-r19 ENUMERATED {ms40, ms80, ms160, ms240, ms320, ms640, ms1280, ms2560}, npusch-StartSFN-r19 INTEGER (0..1023), npusch-StartSubframe-r19 INTEGER (0..9) }, <skip> cb-Msg3-TxWindow-NB-r19 SEQUENCE { windowSize-NB-r19 INTEGER {3..10}, windowPeriodicity-NB-r19 ENUMERATED {n16, n32, n48, n64, n128, n256, n384 n512, n768, n1024, n1280, n1536,n1792, n2048, n2560, n3072} } OPTIONAL, --Need OP <skip> } <skip> n2048, n2560, n3072} cb-Msg3-TxWindow-NB CB-Msg3 transmission window configuration. The start time of the CB-Msg3 transmission window is aligned with the NPUSCH start time indicated by npusch-StartSFN-r19 and npusch-StartSubframe-r19. When cb-Msg3-NumOfReplicas-NB-r19 equals 1, this field is absent. For windowSize, value 3 corresponds to 4 PUSCH periods, 4 corresponds to 8 PUSCH periods and so on. For windowPeriodicity, value n16 corresponds to 160ms, n32 corresponds to 320ms and so on.. ------

In previous meeting, a concern has been given that, it may cause confusion on how to assume the (N)PUSCH resources within the gap between the end of the Tx window and the end of the Tx window periodicity (if windowPeriodicity is larger than windowSize). According to the more companies’ views, RAN2 gave an understanding that the PUSCH resource outside of the window is invalid for CB-Msg3-EDT.

For clarification, we suggest to capture RAN2 understanding in the field description of Tx window as below:

Proposal 3: To add a clarification for CB-Msg3 Tx window as below:

cb-Msg3-TxWindow-NB CB-Msg3 transmission window configuration. The start time of the CB-Msg3 transmission window is aligned with the NPUSCH start time indicated by npusch-StartSFN-r19 and npusch-StartSubframe-r19. When cb-Msg3-NumOfReplicas-NB-r19 equals 1, this field is absent. For windowSize, value 3 corresponds to 4 PUSCH periods, 4 corresponds to 8 PUSCH periods and so on. For windowPeriodicity, value n16 corresponds to 160ms, n32 corresponds to 320ms and so on. If the windowPeriodicity is larger than windowSize, UE assumes there is no valid PUSCH resources outside the CB-Msg3 transmission window in current CB-Msg3 transmission periodicity.

Conclusion

Based on the above discussion, the following proposals are given:

Proposal 1a: It’s suggested to allow delta configuration for CB-Msg3 resources on non-anchor carriers.

Proposal 1b: the detailed delta configuration rules for CB-Msg3 resources on non-anchor carriers are as below:

- To re-organize all the parameters in CB-Msg3-Config-NB-r19 into a new IE, e.g., CB-Msg3-Config-Parameters-r19 which is with need code “Need OR”; (To make it allowable the configuration for CEL as empty)

- To make all mandatory parameters in previous CB-Msg3-Config-NB-r19 optional, with need code “Need OP”

  - For a few parameters previously optional, to keep the field description unchanged, e.g., to follow the existing UE behaviors description;

  - For other parameters newly marked with “Need OP”, to add the following “NOTE” to allow an absent parameter for non-anchor carrier to refer to corresponding one for anchor carrier):

NOTE:

- The field is mandatory in all the entries of CB-Msg3-ConfigList-NB in SystemInformationBlockType2-NB.

- If the field is absent in an entry of CB-Msg3-ConfigList-NB in SystemInformationBlockType22-NB, the value of the same field in the corresponding entry of CB-Msg3-ConfigList-NB in SystemInformationBlockType2-NB applies.

Proposal 2: As legacy, it allows the NB-IoT UE implementation to attempt to accumulate PWS (ETWS/CMAS) SI messages across SI windows. No specification impact is expected.

Proposal 3: To add a clarification for CB-Msg3 Tx window as below:

cb-Msg3-TxWindow-NB CB-Msg3 transmission window configuration. The start time of the CB-Msg3 transmission window is aligned with the NPUSCH start time indicated by npusch-StartSFN-r19 and npusch-StartSubframe-r19. When cb-Msg3-NumOfReplicas-NB-r19 equals 1, this field is absent. For windowSize, value 3 corresponds to 4 PUSCH periods, 4 corresponds to 8 PUSCH periods and so on. For windowPeriodicity, value n16 corresponds to 160ms, n32 corresponds to 320ms and so on. If the windowPeriodicity is larger than windowSize, UE assumes there is no valid PUSCH resources outside the CB-Msg3 transmission window in current CB-Msg3 transmission periodicity.