3GPP TSG-RAN WG2 Meeting #132                                        R2-2508187

Dallas, USA, 17th – 21st November, 2025

Source:	ZTE Corporation, Sanechips

Title:	MAC corrections for R19 IoT NTN

Agenda Item:	8.9.3

Document for:	Discussion and Decision

Introduction

In this contribution, we focus on polishing the remaining issues in MAC specification, especially for the sub-topic of CB-Msg3-EDT.

Discussion

Narrow band selection for CB-Msg4 reception

In previous RAN2#131 meeting, RAN2 has sent LS [R2-2506284] to RAN1 to indicate that at most two MPDCCH narrow bands can be configured for MPDCCH monitoring and CB-Msg4 reception in eMTC NTN.

In last RAN1#122bis meeting, this issue has been discussed. However, RAN1 cannot agree on a suitable factor to be used by UE to do the narrow band selection for CB-Msg4 reception. So RAN1 gave the following answer in the response LS [R1-2508114]:

R1-2508114 (RAN1#122bis meeting) -------------------------------------------------- - Support narrow band MPDCCH configuration for CB-Msg3-EDT RAN2 ask RAN1 to check the specification impact in RAN1 to support 2 MPDCCH narrowbands for CB-Msg4 monitoring, and update the corresponding specification if needed. RAN1 response for support MPDCCH narrowband configuration for CB-Msg3-EDT: - From RAN1 perspective, introducing 2 MPDCCH narrowbands requires additional RAN1 standard effort. RAN1 would like to inform RAN2 that there are no concerns with supporting two MPDCCH narrowbands, but RAN2 should decide on a mechanism to ensure the UE is monitoring a unique narrowband.

To configure two MPDCCH narrow bands is helpful to avoid potential DL scheduling congestion caused by all UEs monitoring the same DL narrow band. This achieves the load distribution of PDCCH monitoring/scheduling. A general way to addressing this type of distributing UEs issue is to use modulo operation on the total number of DL narrow bands with a factor known to both the UE and eNB. From physical layer perspective, potential factors typically include time-domain, frequency-domain, or code-domain resources used by the UE for uplink transmission, e.g., CB-Msg3 transmission. However, as of now, RAN1 has not identified a suitable factor. Therefore, RAN2 needs to do the decision and from the RAN2 perspective, a straightforward way is to use UE ID modulo the number of DL narrow bands, similar to how the Paging Narrowband (PNB) is determined in legacy eMTC.

Regarding the CB-Msg3 transmission, the eNB can obtain the S-TMSI of a UE after successfully decoding CB-Msg3, therefore, the full S-TMSI or part of S-TMSI information (e.g., S-TMSI mod 1024) can be used as UE ID since it can be known by both UE and eNB.

Proposal 1a: For MPDCCH narrowband determination for any UE, it’s suggested to use the UE’s S-TMSI or (S-TMSI mod 1024) modulo the total number of MPDCCH narrowbands.

Proposal 1b: The following TP for MPDCCH narrowband determination can be captured in MAC spec:

R2-2507656 MAC CR: 5.1a.3 CB-Msg4 reception Once all the CB-Msg3s of a CB-Msg3 transmission window are transmitted, the MAC entity shall monitor the PDCCH of the SpCell, while CB-Msg3ResponseTimer is running, for CB-Msg4(s) identified by the CB-RNTI. For BL UE or UE in enhanced coverage, if more than one MPDCCH narrowbands (e.g., MNB) are configured, the narrowband on which the UE performs the MPDCCH monitoring and CB-Msg4 reception is determined by the following equation: MNB = UE_ID mod Nm where Nm is number of MPDCCH narrowbands, UE_ID is S-TMSI. After the CB-Msg3 transmission, the MAC entity shall: - start the CB-Msg3ResponseTimer at the subframe that contains the end of the selected CB-Msg3 transmission window plus UE-eNB RTT; - monitor the PDCCH while CB-Msg3ResponseTimer is running; - if notification of a reception of a PDCCH transmission is received from lower layers; and - if the CB-Msg4 is successfully decoded: <skip the unrelated parts>

CQI and AS RAI reporting in CB-Msg3

In previous meetings, RAN2 has made the following agreements related to Channel Quality Report (CQI or CQR) and Release Assistant Information (RAI) report for CB-Msg-EDT:

Agreement in RAN2#130: - We clarify that CQI reporting is not included in CB-Msg3 for CB-Msg3-EDT for non-anchor carrier but it can be included for anchor carrier (send LS to RAN4) Agreement in RAN2#131: - Based on NW indication, it shall be possible for the UE to indicate during CB-Msg3-EDT procedure whether DL data following the UL data in CB-Msg3 is expected or not. RAI will be reused for this.

In the contribution [R2-2507242] in RAN2#131bis meeting, based on the following description in the spec, company gave the observation that “the transmission of the “DCQR and AS RAI MAC CE” by an NB-IoT UE seems to be limited to the “regular DCQR” scenarios where Msg3 is not involved. Hence, it is not clear whether an NB-IoT UE can include a “DCQR and AS RAI MAC CE” in a CB-Msg3 and use it to inform the network of whether DL data following the UL data in CB-Msg3 is expected or not”:

TS 36.321 v19.0.0 5.25 Transmission of Downlink Channel Quality Report The MAC entity of a BL UE or UE in enhanced coverage may be configured by upper layers to report DL channel quality in Msg3. DL channel quality in Msg3 in RRC_CONNECTED is not reported. If the UE is a BL UE or UE in enhanced coverage or an NB-IoT UE, a Downlink Channel Quality Report (DCQR) shall be triggered if any of the following events occur: - DCQR Command MAC control element is received, in which case the DCQR is referred below to as "Regular DCQR"; - for BL UE or UE in enhanced coverage, transmission of DCQR in Msg3 is configured by upper layers in mpdcch-CQI-Reporting, in which case DCQR is referred below to as "Msg3 DCQR". If any type of DCQR has been triggered: - start performing DL channel quality measurements according to TS 36.133 [9]. If "Regular DCQR" has been triggered: - if an uplink grant has been received on the PDCCH for MAC entity's C-RNTI: - instruct the Multiplexing and Assembly procedure to generate a DCQR and AS RAI MAC control element as defined in clause 6.1.3.19; - cancel the triggered "Regular DCQR". If "Msg3 DCQR" has been triggered: - if an uplink grant has been received on the PDCCH for MAC entity's RA-RNTI: - if the allocated resources can accommodate a DCQR and AS RAI MAC control element plus its subheader as a result of logical channel prioritization: - instruct the Multiplexing and Assembly procedure to generate a DCQR and AS RAI MAC control element as defined in clause 6.1.3.19; - else if the uplink grant is not for EDT: - if configured by upper layers in mpdcch-CQI-Reporting, use R and F2 fields in the MAC PDU subheader, to transmit the measurement outcome, as defined in clause 6.2.1; - cancel the triggered "Msg3 DCQR".

By reviewing the standardization history in legacy NB-IoT/eMTC, we have the following observations:

Observation 1a: From R14, NB-IoT supports downlink channel quality reporting (CQI report) in Msg3 RRC signaling, e.g., via cqi-NPDCCH in RRCConnectionReestablishmentRequest-NB, RRCConnectionRequest-NB, RRCConnectionResumeRequest-NB and RRCEarlyDataRequest-NB messages. This is beneficial to Msg4 scheduling.

Observation 1b: In R16, it’s extended to eMTC to support CQI report in Msg3. However, as there is only one spare bit or even no spare bit available in the Msg3 RRC messages for eMTC, it’s not feasible to use same RRC signaling scheme as that in NB-IoT for CQI report during random access for eMTC.

Observation 1c: R16 eMTC firstly discussed the CQI report during random access for EDT case. Since EDT can allocate more resources for Msg3, it’s feasible to support CQI report via additional MAC CE (e.g., “DCQR and AS RAI” MAC CE). It’s noted that such scheme will cause additional MAC layer overhead, compared with RRC Msg3-based scheme.

Observation 1d: For eMTC non-EDT case, as generally UL grant in RAR is with the minimum possible resources, there is no sufficient room to carry additional MAC CE. Again, as mentioned above, there is also no room to contain CQI report in Msg3 RRC messages, therefore, the scheme of utilizing the R and F2 fields in the MAC PDU subheader for CQI report during random access was introduced.

Observation 1e: With the introduction of new “DCQR and AS RAI” MAC CE, it can also be leveraged for connected mode CQI report in both NB-IoT and eMTC. Such report can be triggered by another “Downlink Channel Quality Report Command” MAC CE.

So, we have sympathy with the view in [R2-2507242], e.g., for NB-IoT in idle mode, there is no trigger for CQI report via “DCQR and AS RAI MAC CE”. In other word, CQI report for NB-IoT UE in idle node still be triggered by NW indication cqi-Reporting-r14 in SIB2-NB and via RRC messages. However, based on the following description in the spec, we understand either BSR=0 MAC CE or DCQR and AS RAI MAC CE can be used to report RAI information, also for CB-Msg3-EDT:

TS 36.321 v19.0.0 5.4.8 Access Stratum Release Assistance Indication Access Stratum Release Assistance Indication is used to provide the serving eNB with information whether subsequent DL or UL transmission is expected. AS RAI uses the DCQR and AS RAI MAC Control Element. Upper layers trigger AS RAI. For EDT and transmission using PUR, if AS RAI is triggered by upper layers but is not included in the resulting MAC PDU with the MAC SDU as a result of logical channel prioritization, AS RAI is cancelled, for other transmissions if AS RAI is not included in the resulting MAC PDU as a result of logical channel prioritization, AS RAI may be cancelled. If rai-Activation is configured and a buffer size of zero bytes has been triggered for the BSR and no subsequent DL and UL data transmission is expected, and if rai-ActivationEnh is enabled and applicable as specified in TS 36.331 [8], it is up to UE to send BSR MAC control element or DCQR and AS RAI MAC control element.

Based on above observations, for CB-Msg3-EDT in NB-IoT NTN, we suggest RAN2 to check and confirm the following understandings:

Proposal 2a: RAN2 confirms that, for CB-Msg3-EDT in NB-IoT NTN, if rai-ActivationEnh is enabled and applicable as specified in TS 36.331 [8], either BSR=0 MAC CE or DCQR and AS RAI MAC CE can be used to report RAI information to NW. No spec impacts for this confirmation.

Proposal 2b: RAN2 confirms that, for CB-Msg3-EDT in NB-IoT NTN, if only cqi-Reporting-r14 is enabled as specified in TS 36.331 [8], UE send CQI report (if available for anchor carrier) via Msg3 RRC messages. If cqi-Reporting-r14 is not enabled, NW would assume no CQI report in Msg3. That means for UE, neither cqi-NPDCCH in Msg3 RRC messages nor DCQR part in “DCQR and AS RAI MAC CE” need to be set, for example, “noMeasurements” can be used. No spec impacts for this confirmation.

We can image that if both rai-ActivationEnh and cqi-Reporting-r14 are enabled, one possible optimization may be to only set the DCQR part in “DCQR and AS RAI MAC CE”, meanwhile skip the CQI report for anchor carrier in Msg3 RRC messages. Such optimization may have some impacts for RRC spec.

However, considering in some cases, the granularity for DCQR report in MAC CE may be a bit finer than that CQI report in RRC message, it may be beneficial to use MAC CE. Further considering the RAI report can already trigger using MAC CE and already incurs MAC CE overhead, it may be reasonable to consider also using MAC CE to report DCQR and disregarding the CQI report via RRC Msg3.

Proposal 3: It’s suggested RAN2 to discuss, if both rai-ActivationEnh and cqi-Reporting-r14 are enabled, whether UE can use “DCQR and AS RAI MAC CE” for CQI and RAI report alone with CB-Msg3 and skip CQI report via RRC Msg3.

Conclusion

Based on the above discussion, the following observations are given:

Observation 1a: From R14, NB-IoT supports downlink channel quality reporting (CQI report) in Msg3 RRC signaling, e.g., via cqi-NPDCCH in RRCConnectionReestablishmentRequest-NB, RRCConnectionRequest-NB, RRCConnectionResumeRequest-NB and RRCEarlyDataRequest-NB messages. This is beneficial to Msg4 scheduling.

Observation 1b: In R16, it’s extended to eMTC to support CQI report in Msg3. However, as there is only one spare bit or even no spare bit available in the Msg3 RRC messages for eMTC, it’s not feasible to use same RRC signaling scheme as that in NB-IoT for CQI report during random access for eMTC.

Observation 1c: R16 eMTC firstly discussed the CQI report during random access for EDT case. Since EDT can allocate more resources for Msg3, it’s feasible to support CQI report via additional MAC CE (e.g., “DCQR and AS RAI” MAC CE). It’s noted that such scheme will cause additional MAC layer overhead, compared with RRC Msg3-based scheme.

Observation 1d: For eMTC non-EDT case, as generally UL grant in RAR is with the minimum possible resources, there is no sufficient room to carry additional MAC CE. Again, as mentioned above, there is also no room to contain CQI report in Msg3 RRC messages, therefore, the scheme of utilizing the R and F2 fields in the MAC PDU subheader for CQI report during random access was introduced.

Observation 1e: With the introduction of new “DCQR and AS RAI” MAC CE, it can also be leveraged for connected mode CQI report in both NB-IoT and eMTC. Such report can be triggered by another “Downlink Channel Quality Report Command” MAC CE.

And also some proposals are given:

Proposal 1a: For MPDCCH narrowband determination for any UE, it’s suggested to use the UE’s S-TMSI or (S-TMSI mod 1024) modulo the total number of MPDCCH narrowbands.

Proposal 1b: The following TP for MPDCCH narrowband determination can be captured in MAC spec:

R2-2507656 MAC CR: 5.1a.3 CB-Msg4 reception Once all the CB-Msg3s of a CB-Msg3 transmission window are transmitted, the MAC entity shall monitor the PDCCH of the SpCell, while CB-Msg3ResponseTimer is running, for CB-Msg4(s) identified by the CB-RNTI. For BL UE or UE in enhanced coverage, if more than one MPDCCH narrowbands (e.g., MNB) are configured, the narrowband on which the UE performs the MPDCCH monitoring and CB-Msg4 reception is determined by the following equation: MNB = UE_ID mod Nm where Nm is number of MPDCCH narrowbands, UE_ID is S-TMSI. After the CB-Msg3 transmission, the MAC entity shall: - start the CB-Msg3ResponseTimer at the subframe that contains the end of the selected CB-Msg3 transmission window plus UE-eNB RTT; - monitor the PDCCH while CB-Msg3ResponseTimer is running; - if notification of a reception of a PDCCH transmission is received from lower layers; and - if the CB-Msg4 is successfully decoded: <skip the unrelated parts>

Proposal 2a: RAN2 confirms that, for CB-Msg3-EDT in NB-IoT NTN, if rai-ActivationEnh is enabled and applicable as specified in TS 36.331 [8], either BSR=0 MAC CE or DCQR and AS RAI MAC CE can be used to report RAI information to NW. No spec impacts for this confirmation.

Proposal 2b: RAN2 confirms that, for CB-Msg3-EDT in NB-IoT NTN, if only cqi-Reporting-r14 is enabled as specified in TS 36.331 [8], UE send CQI report (if available for anchor carrier) via Msg3 RRC messages. If cqi-Reporting-r14 is not enabled, NW would assume no CQI report in Msg3. That means for UE, neither cqi-NPDCCH in Msg3 RRC messages nor DCQR part in “DCQR and AS RAI MAC CE” need to be set, for example, “noMeasurements” can be used. No spec impacts for this confirmation.

Proposal 3: It’s suggested RAN2 to discuss, if both rai-ActivationEnh and cqi-Reporting-r14 are enabled, whether UE can use “DCQR and AS RAI MAC CE” for CQI and RAI report alone with CB-Msg3 and skip CQI report via RRC Msg3.