3GPP TSG-RAN WG2 Meeting #132	R2-2508190

# Dallas, USA, November 17 – 21, 2025

Agenda item:	10.3.1.2

Source:	Samsung

Title:	6GR QoS and service-awareness

Document for:	Discussion

1	Introduction

This paper is to discuss QoS related aspects in 6G radio such as service-awareness and AS functionality to study.

2	Discussion

2.1	QoS framework in 6GR

Figure 1 outlines the QoS architecture defined in 5G NR [1].

Figure 1 QoS architecture in NG-RAN [1]

In 5G NR, the QoS model is structured based on QoS Flow, which is the finest granularity of QoS differentiation within a PDU session. A QoS Flow is defined by a QoS profile, one or more QoS rule(s), optionally QoS Flow Level QoS parameters, and one or more UL and DL Packet Detection Rule(s). The QoS profile includes parameters such as the 5G QoS Identifier (5QI). Standardized 5QI values and their corresponding QoS characteristics are detailed in Table 5.7.4-1 of TS 23.501[2]. The 5QI is linked to QoS characteristics, providing guidelines for configuring node specific parameters for each QoS Flow. These standardized or pre-configured 5G QoS characteristics are derived from the 5QI value and are not explicitly signalled. The QoS characteristics include Priority level, Packet Delay Budget (PDB), Packet Error Rate (PER), Averaging Window, and Maximum Data Burst Volume.

The QoS model supports two types of QoS Flows: 1) GBR QoS Flows that require guaranteed flow bit rate, 2) non-GBR QoS Flows that do not require guaranteed flow bit rate. A QoS flow is classified either as GBR or Non-GBR depending on its profile. The QoS profile of a GBR QoS Flow includes parameters such as Guaranteed Flow Bit Rate (GFBR) for both uplink and downlink, Maximum Flow Bit Rate (MFBR) for both uplink and downlink, Maximum Packet Loss Rate for both uplink and downlink, Delay Critical Resource Type, Notification Control. The QoS profile of a Non-GBR QoS Flow includes parameters such as Reflective QoS Attribute (RQA) and Additional QoS Flow Information.

At NAS level, the 5GC provides the QoS profile for a QoS Flow to NG-RAN and the QoS rule(s) to UE. NG-RAN uses the QoS profile to determine how to treat traffic on the radio interface.

At AS level, the QoS Flows are linked to DRBs. The mapping of QoS Flows to DRBs by NG-RAN is based on the QFI and the associated QoS profiles. A DRB ensures uniform packet forwarding treatment for all packets belonging to it. NG-RAN maps packets belonging to different PDU sessions to different DRBs. Within each PDU session, NG-RAN decides how multiple QoS Flows are mapped to a DRB. NG-RAN and 5GC ensure QoS requirements, such as target latency and reliability, by mapping packets to appropriate QoS Flows and DRBs.

Observation 1: In 5G NR, QoS Flow is defined as the finest granularity of QoS differentiation within a PDU session. At AS level, a QoS Flow is mapped to a DRB.

Observation 2: In 5G NR, QoS profiles and QoS rules are used to map packets to appropriate QoS Flows and DRBs.

For 6GR, RAN2 can evaluate whether to reuse the principles of 5G QoS framework, such as the one-to-one mapping between QoS Flow and DRB. Reminding of RAN awareness studies in 5G Release 17, discussions on PDU set to DRB mapping were initiated to support the newly introduced concept of PDU sets for XR traffic characteristics and communication requirements. RAN2 discussed how to map PDU sets onto QoS Flows at NAS level and how to map the QoS Flows into DRBs at AS level. Concurrently, SA2 explored options, including defining sub-QoS Flows within a QoS Flow.

Given that SA2 will study the 6G QoS framework, RAN2 should await significant progress from SA2. Additionally, there are currently no identified QoS requirements necessitating changes to the 5G QoS framework. Therefore, RAN2 should discuss 6GR AS level functionalities by adopting the 5G NR QoS architecture and QoS model as a starting point. Subsequently, RAN2 can investigate potential needs for new or enhanced QoS mechanism, leveraging studies on the 6G QoS framework and service awareness conducted by SA and RAN WGs.

Observation 3: There is no specific and urgent requirements to develop new QoS framework such as new QoS granularity for 6GR.

Based on the above observations, we propose:

Proposal 1: RAN2 is asked to adopt QoS Flow based model in 5G NR as a starting point for discussing AS level QoS mechanisms in 6GR.

2.2	Service-awareness

The 6GR aims to support both new and existing services, including enhanced Mobile Broadband (eMBB), immersive communication, IoT, sensing, AI, voice, and regulatory services, outlined in the TR on 6G Scenarios and Requirements [3].

Since 5G Release 18, RAN and SA WGs have studied and defined enhanced functionalities to support use cases and services for 5G and beyond, such as eXtended Reality (XR) and Cloud Gaming (CG). XR and CG encompass various augmented, virtual, and mixed reality environments facilitated through handheld and wearable end-user devices. Initially, the primary focus of the RAN WGs was to identify XR-awareness mechanisms and XR-specific traffic handling at AS level, building on prior work from SA4 and RAN1 (refer to TR 38.838).

Observation 4: For XR services in 5G NR, RAN-level XR awareness was studied based on prior work, including traffic characteristics and traffic models developed by SA4 and RAN1.

Regarding immersive communication, it is assumed that its requirements can be aligned with those of XR, namely high data rates and low latency. By examining Table 5.7.4-1 of TS 23.501 [2], which specifies standardized 5QI values and their corresponding QoS characteristics along with example services, several services—such as Real Time Gaming, “Live” Uplink Streaming, Live Video Streaming, Augmented Reality, Interactive Service involving motion tracking data, and Low Latency eMBB applications — can fall under the category of immersive communication.

Observation 5: The requirements for XR, specifically high data rates and low latency, can be extended to certain example services associated with immersive communication.

While further investigation into the specific traffic characteristics and requirements for immersive communication can be done in SA WGs like SA4 and SA2, RAN2 can initiate studies on AS level functionalities to support immersive communication by leveraging existing 5G NR functionalities for XR. Potential AS level functionalities for immersive communication in 6GR include:

- Service awareness

  - Assistance information for sNB scheduling

    - PDU Set QoS parameters such as PSDB, PSER, and PSIHI

    - PDU information (PDU set sequence number, end PDU of PDU set, PDU sequence number within PDU set, PDU set importance (PSI), PDU set size, End of data burst)

    - Multi-modality support

    - UL traffic information such as periodicity, arrival time, and jitter range

- Delay reduction

  - Delay status reporting for urgent uplink data

  - Prioritization of urgent uplink data in LCP

  - Delay-aware RLC ARQ enhancements

- Obsolete data delivery avoidance

  - PDU Set level discard at PDCP

  - Early discarding of low importance data at PDCP

  - Discarding of obsolete packets in RLC based on PDCP discard indication

- Resource utilization optimization

  - Buffer Status Table to enable finer BSR granularity

Observation 6: AS layer functionalities developed to support XR in 5G NR can be adapted to facilitate immersive communication in 6GR.

For emerging services such as AI and sensing, further study is needed to define their traffic characteristics and corresponding communication requirements across relevant 3GPP SA and RAN WGs. While Table 5.7.4-1 in [2] lists example services related to AI/ML — such as AI/ML model download for image recognition, split AI/ML inference (DL Split AI/ML image recognition), AI/ML inference(UL Split AI/ML image recognition) — it remains unclear whether these QoS characteristics sufficiently address AS level awareness for AI/ML traffic. The traffic model for AI/ML services in 6GR is currently being studied by RAN1, as indicated in their LS to SA4 [4]. Consequently, RAN2 should await input on AI/ML traffic characteristics from SA4 before proceeding with discussion on AI/ML specific AS layer service awareness.

Observation 7: For AI/ML services, the traffic model developed by RAN1 and associated inputs from SA4 can serve as valuable resources for RAN2’s study on AS layer service awareness.

Based on the above observations, we propose:

Proposal 2: RAN2 is asked to investigate AS layer awareness and functionality for immersive communication by utilizing 5G NR XR features as a starting point.

Proposal 3: RAN2 is asked to defer discussion on AI/ML specific AS layer service awareness until receiving inputs on traffic characteristics from SA4 and insights from RAN1’s ongoing traffic model study.

3	Conclusion

Based on the discussion, we made the following observations:

Observation 1: In 5G NR, QoS Flow is defined as the finest granularity of QoS differentiation within a PDU session. At AS level, a QoS Flow is mapped to a DRB.

Observation 2: In 5G NR, QoS profiles and QoS rules are used to map packets to appropriate QoS Flows and DRBs.

Observation 3: There is no specific and urgent requirements to develop new QoS framework such as new QoS granularity for 6GR.

Observation 4: For XR services in 5G NR, RAN-level XR awareness was studied based on prior work, including traffic characteristics and traffic models developed by SA4 and RAN1.

Observation 5: The requirements for XR, specifically high data rates and low latency, can be extended to certain example services associated with immersive communication.

Observation 6: AS layer functionalities developed to support XR in 5G NR can be adapted to facilitate immersive communication in 6GR.

Observation 7: For AI/ML services, the traffic model developed by RAN1 and associated inputs from SA4 can serve as valuable resources for RAN2’s study on AS layer service awareness.

Based on the observations, RAN2 is requested to discuss and agree the following proposals:

Proposal 1: RAN2 is asked to adopt QoS Flow based model in 5G NR as a starting point for discussing AS level QoS mechanisms in 6GR.

Proposal 2: RAN2 is asked to investigate AS layer awareness and functionality for immersive communication by utilizing 5G NR XR features as a starting point.

Proposal 3: RAN2 is asked to defer discussion on AI/ML specific AS layer service awareness until receiving inputs on traffic characteristics from SA4 and insights from RAN1’s ongoing traffic model study.

4	References

[1] TS 38.300 V19.0.0

[2] TS 23.501 V19.5.0

[3] TR 38.914 V0.2.0, 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on 6G Scenarios and Requirements; (Release 20)

[4] R2-2508007, LS on traffic model study in RAN1 (RAN WG1)