3GPP TSG-RAN WG2 Meeting #132	R2-2508207

Dallas, Texas, USA, Nov 17th – 21st, 2025

Agenda item:		8.18.1

Source:		Samsung, ZTE, Nokia

Title:			Clarification on notification configuration

Document for:	Discussion & Decision

# 1 Introduction

In this paper, we propose a clarification for RRC specification on notification configuration for MBMS-dedicated cell, in line with the RAN1 conclusion [1].

# 2 Discussion

MBMS-Notification is used to notify a change of MCCH. The MCCH change notifications on PDCCH are transmitted periodically and are carried on MBSFN subframes only except on MBMS-dedicated cell or FeMBMS/Unicast-mixed cell where the MCCH change notifications are provided on non-MBSFN subframes. Further, on FeMBMS/Unicast-mixed cell, MCCH change notification can also be transmitted in subframes #4 and #9 when either is configured as legacy MBSFN subframe with control region.

MBMS-NotificationConfig-r9 ::= SEQUENCE { notificationRepetitionCoeff-r9 ENUMERATED {n2, n4}, notificationOffset-r9 INTEGER (0..10), notificationSF-Index-r9 INTEGER (1..6) } MBMS-NotificationConfig-v1430 ::= SEQUENCE { notificationSF-Index-v1430 INTEGER (7..10) } notificationSF-Index Indicates the subframe used to transmit MCCH change notifications on PDCCH. FDD: Value 1, 2, 3, 4, 5 and 6 correspond with subframe #1, #2, #3 #6, #7, and #8 respectively. Value 7, 8, 9 and 10 correspond with subframe #0, #4, #5 and #9 respectively. If notificationSF-Index-v1430 is included, UE ignores notificationSF-Index-r9. TDD: Value 1, 2, 3, 4, and 5 correspond with subframe #3, #4, #7, #8, and #9 respectively.

As described above, notificationSF-Index-v1430 will be included in case of FeMBMS/Unicast-mixed cell. However, it is not sufficiently clear as to how the notification configuration to be interpreted by implementers in case of MBMS-dedicated cell. Moreover, there can be up to 3 additional non-MBSFN subframes configurable further to the subframe carrying PBCH in MBMS-dedicated cell. However, it is not specified which one among them would carry MBMS-Notification. It is not efficient to assume any or all of the subframes could carry MBMS-Notification leading to the in-efficient and blind decoding operation by the UEs. In this regard, a clarification was proposed in RAN1#122bis and RAN1 made the following conclusion.

RAN1#122bis [1] Conclusion From RAN1 perspective, it is clarified that “for an MBMS-dedicated cell, if E-UTRAN configures a value other than "0" for additionalNonMBSFNSubframes within MasterInformationBlock-MBMS, notificationSF-Index-r9 configuration indicates the subframe pointed out by additionalNonMBSFNSubframes and MBMS-NotificationConfig-v1430 is not configured when one of the additional subframes is used to transmit MCCH change notifications on PDCCH.” No RAN1 specification impact

Given the description for notification configuration is captured in RRC specification; it is worthwhile to add this clarification in the TS 36.331 to help the 5G Broadcast implementations, which will primarily be based on Rel-19 and will help in preventing IODT issue. Hence, it is proposed:

Observation 1: Clarification on notification configuration regarding usage of additional non-MBSFN subframes is useful for 5G Broadcast implementations, which are expected to be based on Rel-19 spec, and in preventing IODT issues.

Observation 2: RAN1 agreement validated the usage of additional non-MBSFN subframes for notification configuration for 5G Broadcast; the related description belongs to RRC spec.

Proposal 1: RAN2 to adopt TP1 to clarify the notification configuration in Rel-19 RRC Spec:

- For an MBMS-dedicated cell, if E-UTRAN configures a value other than "0" for additionalNonMBSFNSubframes within MasterInformationBlock-MBMS, notificationSF-Index-r9 configuration indicates the subframe pointed out by additionalNonMBSFNSubframes and MBMS-NotificationConfig-v1430 is not configured, when one of the additional subframes is used to transmit MCCH change notifications on PDCCH.

TP1

MBMS-NotificationConfig-r9 ::= SEQUENCE { notificationRepetitionCoeff-r9 ENUMERATED {n2, n4}, notificationOffset-r9 INTEGER (0..10), notificationSF-Index-r9 INTEGER (1..6) } MBMS-NotificationConfig-v1430 ::= SEQUENCE { notificationSF-Index-v1430 INTEGER (7..10) } notificationSF-Index - Indicates the subframe used to transmit MCCH change notifications on PDCCH. FDD: Value 1, 2, 3, 4, 5 and 6 correspond with subframe #1, #2, #3 #6, #7, and #8 respectively. Value 7, 8, 9 and 10 correspond with subframe #0, #4, #5 and #9 respectively. If notificationSF-Index-v1430 is included, UE ignores notificationSF-Index-r9. TDD: Value 1, 2, 3, 4, and 5 correspond with subframe #3, #4, #7, #8, and #9 respectively. For an MBMS-dedicated cell, if E-UTRAN configures a value other than "0" for additionalNonMBSFNSubframes within MasterInformationBlock-MBMS, notificationSF-Index-r9 configuration indicates the subframe pointed out by additionalNonMBSFNSubframes and MBMS-NotificationConfig-v1430 is not configured, when one of the additional subframes is used to transmit MCCH change notifications on PDCCH.

# 3 Conclusion

Based on the discussion above, request RAN2 to consider the following proposal:

Observation 1: Clarification on notification configuration regarding usage of additional non-MBSFN subframes is useful for 5G Broadcast implementations, which are expected to be based on Rel-19 spec, and in preventing IODT issues.

Observation 2: RAN1 agreement validated the usage of additional non-MBSFN subframes for notification configuration for 5G Broadcast; the related description belongs to RRC spec.

Proposal 1: RAN2 to adopt TP1 to clarify the notification configuration in Rel-19 RRC Spec:

- For an MBMS-dedicated cell, if E-UTRAN configures a value other than "0" for additionalNonMBSFNSubframes within MasterInformationBlock-MBMS, notificationSF-Index-r9 configuration indicates the subframe pointed out by additionalNonMBSFNSubframes and MBMS-NotificationConfig-v1430 is not configured, when one of the additional subframes is used to transmit MCCH change notifications on PDCCH.

TP1

MBMS-NotificationConfig-r9 ::= SEQUENCE { notificationRepetitionCoeff-r9 ENUMERATED {n2, n4}, notificationOffset-r9 INTEGER (0..10), notificationSF-Index-r9 INTEGER (1..6) } MBMS-NotificationConfig-v1430 ::= SEQUENCE { notificationSF-Index-v1430 INTEGER (7..10) } notificationSF-Index - Indicates the subframe used to transmit MCCH change notifications on PDCCH. FDD: Value 1, 2, 3, 4, 5 and 6 correspond with subframe #1, #2, #3 #6, #7, and #8 respectively. Value 7, 8, 9 and 10 correspond with subframe #0, #4, #5 and #9 respectively. If notificationSF-Index-v1430 is included, UE ignores notificationSF-Index-r9. TDD: Value 1, 2, 3, 4, and 5 correspond with subframe #3, #4, #7, #8, and #9 respectively. For an MBMS-dedicated cell, if E-UTRAN configures a value other than "0" for additionalNonMBSFNSubframes within MasterInformationBlock-MBMS, notificationSF-Index-r9 configuration indicates the subframe pointed out by additionalNonMBSFNSubframes and MBMS-NotificationConfig-v1430 is not configured, when one of the additional subframes is used to transmit MCCH change notifications on PDCCH.

# 4 Reference

[1] R1_25XXXXX_Minutes_report_RAN1#122bis_v100