3GPP TSG-RAN WG2 Meeting #132                                                        R2-2508232

Dallas, USA, Nov 17th-21st, 2025

Source:	CATT

Title:	Discussion on Leftover Non RIL Issues

Agenda Item:	8.13.2

Document for:	Discussion and Decision

Introduction

In RAN2#131bis meeting, there was one at meeting offline discussion[1] for the non RIL issues, but due to time limitation, not all the issues were discussed, we selected some of them for further discussion. Hence, the current contribution is composed of the following three open points:

- Open Point 6: Suppressing the Notification message in RRC_CONNECTED.

- Open Point 7: Conditions for stopping discovery procedure.

- Open Point 8: Discovery Model B with no PC5 Link in multi hop scenario.

Discussion

To better accomplish the WI’s task, one face-to-face offline[1] was allocated to further discuss the RRC proposals not related to RILs, with the goal to prioritise what needs to be treated online and progress towards consensus where possible. But due to the offline time limitation, not all the issues are discussed. In the current contribution, we further share our view on some of the leftover non RIL issues.

- 2.1 Open Point 6: Suppressing the Notification message in RRC_CONNECTED

Based on the Rel-19 multi-hop SL relay discussion, as show in below agreement in RAN2#131 meeting, the intermediate Relay UE in RRC Idle/Inactive mode may not send the notification message to its child UE when relay reselection or cell selection occurs does not cause the change of the serving cell, which is used to avoid that the child UE perform the unnecessary relay reselection.

RAN2#131 Agreement: - The intermediate relay UE in RRC_IDLE/INACTIVE may omit sending the notification message if the relay reselection or cell selection (from multihop to direct path for the intermediate relay) does not cause the change of the serving cell. This does not preclude the implementation considering other parameters such as hop count if they are made visible to AS layer by implementation. Capture this in a NOTE in the stage 3 spec. - When the intermediate Relay UE receives a path switch command, the intermediate Relay UE sends a notification downstream using the handover cause value. Handling of the notification is in accordance with the existing CR and legacy behaviour. - When the RRC_CONNECTED intermediate relay UE does re-establishment as a remote UE, the intermediate Relay UE sends a notification downstream, with the cause value to be confirmed in CR implementation.

For intermediate relay UE in RRC Connected mode, as shown above, if path switching command is received, the intermediate Relay UE will send the notification message to the child UE with handover cause value relayUE-HO, then the child UE could send the notification message with the same cause value to its child UE.

The proponent company proposed that if the RRC_CONNECTED intermediate Relay UE receives the path switching command within the same cell, the intermediate relay UE may not send the notification message to child UE as the same principle used to the IDLE/INACTIVE intermediate Relay UE. In this assumption, the gNB could give proper configuration to child UEs before/after sending a path switching command to avoid the unnecessary relay path changing for child UEs. However, in our view, sometimes, the network has to send the path switching command to intermediate Relay UE immediately considering the possible quick channel degrading, it may still does not give a feasible configuration to these impacted relay/remote UE for UL/DL data transmission. Besides, the DL/UL traffic of child UE(s) has to be buffered for a while, which will further impose additional requirements on both gNB and relay UE. Hence, it is suggested to keep the legacy behavior and do not handle the suppressing notification message in RRC_CONNECTED in Rel-19 multi-hop SL Relay, i.e., when the intermediate Relay UE receives a path switch command, the intermediate Relay UE sends a notification downstream using the handover cause value.

Proposal 1: Suppressing the Notification message in RRC_CONNECTED will not be handled in Rel-19 multi-hop SL Relay.

- 2.2 Open Point 7: Conditions for stopping discovery procedure

The proponent company raised the case that in multi-hop relay, each relay UE along the path can perform discovery messaging and accept the relaying service requests form one or more remote UEs. However, a relay UE may decide to stop performing discovery procedure, for example, if serving too many Remote UEs creates excessive traffic burden or if its battery level is too low to provide additional relaying service. In such cases, the relay UE should also indicate its child UEs (both directly or indirectly connected) to stop performing discovery messaging.

Figure 1. Last Relay UE in overload situation indicating child UEs to stop performing discovery

From our point of view, we echo that the scenario raised by the proponent company is realistic. But we fail to see the need to further solve this issue under RAN2 scope. Hence, we propose:

Proposal 2: Conditions for stopping discovery procedure will not be handled in Rel-19 multi-hop SL Relay.

- 2.3 Open Point 8: Discovery Model B with no PC5 Link in multi hop scenario

The proponent company describes one possible scenario involves a UE that is capable of functioning as an intermediate relay UE (i.e., it supports the Rel-19 multi-hop relay feature through the path gNB <-> relay UE2 <-> the current intermediate relay UE as shown in Figure 2), but is not currently operating as a relay in a multi-hop configuration (i.e., it is now connected through the single-hop relay UE, the relay UE1, as illustrated in Figure 2). In this figure, the UE is capable of acting as an intermediate relay UE but is currently connected to a Rel-17 single-hop relay. In such a scenario, if a multi-hop remote UE attempts to discover a valid multi-hop path, the intermediate relay UE can participate in Model B discovery by forwarding the discovery message by evaluate the AS conditions. This enables the remote UE to identify potential relay paths.

Figure 2. Intermediate Relay UE participating in Model B multi-hop relay discovery

In above case in Figure 2, the proponent company believes that RAN2 should allow the remote UE to select the multi-hop relaying path through the candidate intermediate relay UE, since the remote UE may not find any other available candidate path. In this case, when the remote UE selects a path through an intermediate relay UE, the intermediate relay UE should be allowed to change the path form the single-hop relaying to multi-hop relaying.

From our point of view, we think the scenario raised by the proponent company had already been discussed (cross-path topologies for L2).

Agreement:

LS to SA2 to notify them that RAN2 excludes cross-path topologies for L2 and that a UE should not attempt to function as an intermediate relay UE towards two different parent nodes.  Leave to SA2 the possible spec impact to reflect this.

And one LS was sent to SA2[4] to guarantee the related topology for L2 based multi-hop U2N relay. Further, SA2’s latest LS reply states that SA2 discussed the matter and agreed the attached CR to reflect the RAN2 design considerations. RAN2 can discuss whether the raised scenario had already been captured by SA2 specification or not? If the answer is no, RAN2 can further send one LS to SA2 to exclude the raised scenario to solve the raised issue if needed.

Proposal 3: RAN2 discusses whether the raised scenario has already been captured in the latest SA2’s LS reply(R2-2508027) and sends LS to SA2 if RAN2 needs to further exclude the raised scenario.

Conclusion

According to the analysis in section 2, our proposal is summarized as follows:

Proposal 1: Suppressing the Notification message in RRC_CONNECTED will not be handled in Rel-19 multi-hop SL Relay.

Proposal 2: Conditions for stopping discovery procedure will not be handled in Rel-19 multi-hop SL Relay.

Proposal 3: RAN2 discusses whether the raised scenario has already been captured in the latest SA2’s LS reply(R2-2508027) and sends LS to SA2 if RAN2 needs to further exclude the raised scenario.

Reference

- R2-2507791	Report of [AT131bis][401][Relay] Rel-19 relay RRC non-RIL issues (Huawei)	Huawei, HiSilicon	discussion	Rel-19	NR_SL_relay_multihop-Core

- R2-2506569 LS on topologies for L2 based multi-hop U2N relay

- R2-2508027 Reply LS on LS on topologies for L2 based multi-hop U2N relay (S2-2509481; contact: Qualcomm)