3GPP TSG-RAN WG2 Meeting #132	R2-2508243

Dallas, USA, 17 – 21 November 2025

Agenda item:	10.3.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	6GR Network and UE Energy Efficiency

WID/SID:	6G_SI - Release 20

Document for:	Discussion and Decision

# 1	Introduction

Joint network and UE energy savings are one of the key aspects in the 6G design to allow energy cost efficient deployments as well keeping end user experience seamless. The following was agreed in RAN2#131bis meeting:

For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose. Focus on solutions that have RAN2 impacts.

In this paper we first discuss issues within 5G energy saving and providing overview of 5G energy saving features for both UE and NW sides. Then we will consider each feature family separately and their applicability to 6GR.

# 2	Energy efficiency in 6G

## 2.1	Energy saving issues in 5G

4G LTE focused on enhanced eMBB capacity and coverage, while maintaining a favourable and reasonable trade-off with UE complexity. Network Energy Saving (NES) was not a primary design criterion. The LTE design includes always-on signals and channels, where Cell Specific Reference Signals (CRS) are transmitted in every DL subframe and enable UE cell search, initial acquisition, and downlink channel estimation, and measurements, and the Primary and Secondary Synchronization Signals (PSS/SSS) are transmitted every 5ms and the Physical Broadcast Channel (PBCH), carrying the Master Information Block (MIB), is transmitted every 10ms.

Unlike 4G LTE, the 5G NR’s design offers improved network energy saving from the beginning. Differently from LTE’s CRSs in every DL slot, NR avoids this very frequent always-on signalling. In addition, the NR design integrates PSS/SSS and PBCH in one single SS/PBCH block, optimizing beam sweeping. The default SS/PBCH block transmission periodicity in NR is 20 ms, and this is the setting used by virtually all deployments.

In 5G-A, further work on NES has been conducted in Rel-18 and Rel-19. The Rel-18 study item phase on NES identified several beneficial NES adaptation schemes categorized by time, frequency, spatial and power domains, as summarized in TS 38.864. Time, spatial and power domain techniques were primarily addressed in the Rel-18 NES WI [3], targeting cells with RRC CONNECTED UEs. Rel-19 NES WI [4] further considers time and frequency domain techniques, targeting primarily cells with RRC IDLE/INACTIVE UEs.

The 5G and 5G-A NES features are shown in Figure 1 and detailed in Section 4.1.1. As can be seen from Figure 1, many different NES techniques are defined in 5G. However, 5G has been adding NES features on top of the NR baseline (Rel-15), which has inherent limitations on applicability of such techniques given the presence of legacy UEs in the field. For 6G, the intent should be to have NES by design from the first release, so that the entire network energy-saving toolbox, incorporating our learnings from LTE and NR, can be applied across all cells and all 6G devices from day-one.

Figure 1. List of 5G and 5G-A NES features defined by Release with the corresponding NES gain mechanism.

Energy efficiency is one of the important aspects for the design of 6G networks. However, independent design of network energy saving (NES) and UE energy saving (ES) mechanisms may limit the targeted benefits from both. In some cases, UE energy saving mechanisms may increase power consumption on the network side. Similarly, designing NES operations without taking the UE ES into account may lead to increased UE energy consumption. In previous 5G NR studies, the alignment between network and UE energy-saving functionalities in different RRC states has been deemed beneficial. For 6G, the network energy saving impact on UE energy consumption and vice versa need to be evaluated and taken into account when designing 6G NES operations and UE power saving mechanisms.

Furthermore, it is critical to enable energy saving in the most relevant practical network deployments. For example, also in scenarios where operators have limited band capability and it may not be feasible to consider multi carrier scenarios, but rather the one available band is used for all the purposes. Thus, we need to consider energy saving design and features applicable to single-band deployments in which such coverage layer cannot be switched off. This would enable operators that need to keep the single-band active all the time still to obtain energy saving compared to 4G/5G.

For initial 6G deployments, largely characterized by low or no-load conditions, the default operational mode of the 6G (coverage) cells should be NES / energy saving (6G day-1 should incorporate NES by design). The default NES operation mode means for example infrequent transmission of always-on signals and control channels, and reduced PRACH and paging occasions to ensure NES gain, also in the coverage layer which cannot be switched off. A 6G cell should be able to switch smoothly from the NES-default operation mode (reduced activity, lower broadcast frequency) to a higher-performance mode (increased activity duty cycle, more frequent broadcasting) e.g. when performance demands increase. It may be beneficial to enable a more aggressive default NES operation mode for capacity layer cells than for coverage layer cells. This could involve more aggressive relaxation or omission of broadcast channels and signals in the capacity cells, e.g. only transmitting relaxed signals not enough for camping as such or just by turning off those carriers when there is no need for extra capacity.

Also when designing 6G power saving methods we need to consider how feasible those are to be coordinated between basestations – if they are very dynamic it may not be feasible to keep all the neighbour cells up to date.

### 2.1.1	List of 5G NES features relevant to RAN2

Multi-CSI with sub-configurations

In 5G-A Rel-18, the CSI report framework was enhanced to support spatial and power adaptation for PDSCH. The network can adjust the active antenna ports/elements and/or power level based on factors such as cell load and UE radio conditions. This is facilitated by the enhanced UE-assisted CSI report, which includes multi-CSI reports for various sub-configurations corresponding to different antenna ports/elements or power levels.

Cell DTX/DRX

Introduced in Rel-18, Cell DTX/DRX is a NES feature for RRC CONNECTED mode UEs. It allows the gNB to turn off, partially or completely, transmission and reception of user data traffic, control signaling, and reference signals during cell DTX/DRX non-active periods. The gNB is then able to enter a sleep state and thereby save energy during these non-active periods. In addition, the UEs can also avoid certain transmissions/receptions when the network is non-active and sleep, resulting in better UE energy savings. To avoid non-backward compatible issues, RRC IDLE/INACTIVE UEs are not impacted by cell DTX/DRX operations.

Bandwidth adaptation (BWP and SCell usage adaptation)

In 5G, we introduced bandwidth parts in order to allow NW/UE to change operating bandwidth more dynamically than in 4G. Although 5G design of that is overly complex, it seems also an interesting candidate for power saving for 6G – especially for UE power saving as long as complexity of the design is kept in mind.

On-demand SIB1 and other SIBs

SIB transmissions (other than SIB1) triggered by UE requests are specified in Rel-15 for UEs in RRC IDLE/INACTIVE mode, eliminating the need for regular SIBs broadcast. In Release 19, this was extended also to SIB1. The feature is referred to as on-demand SIB1 and can be applied only in multi-carrier scenarios in Rel-19, in which the cell using on-demand SIB1 mode (referred to as ‘NES cell’) is assisted by an anchor cell (referred to as ‘Cell A’) for the provisioning of the WUS/PRACH resource configuration to the UEs. The UE requests an OD-SIB1 transmission by transmitting a Physical Random-Access CHannel (PRACH) preamble to the NES Cell, which then transmits the SIB1 in response.

On-demand SSB SCell and SSB adaptation

In Rel-19, on-demand SS/PBCH (OD-SS/PBCH) is defined for SCell operation for RRC CONNECTED UEs configured with intra-/inter-band Carrier Aggregation. It is supported for both FR1 and FR2 in non-shared spectrum. The network triggers OD-SS/PBCH transmissions, which can be used by the UE for SCell activation, time/frequency synchronization, and L1/L3 measurements. Furthermore, in Rel-19, the dynamic adaptation of SS/PBCH periodicity has also been specified limited to non-cell defining SS/PBCH (NCD-SS/PBCH).

SSB-less SCell operation

In legacy NR CA, a long periodicity of SS/PBCH transmissions in the SCell is configurable, i.e. via UE-dedicated RRC-signaling. Furthermore, from network energy saving perspective, the SS/PBCH-less SCell operation is a important feature that is supported since NR CA Rel-15 in intra-band contiguous scenarios, where the UE measurement, i.e. AGC tuning, T/F sync, L1-RSRP, etc. of SS/PBCH-less SCell may rely on the SS/PBCH signal transmissions from the primary or reference cell. In R18 NES WI, co-located FR1 inter-band CA scenario was further specified with stringent requirements defined by RAN4 WG.

Paging adaptation

In Rel-19, paging adaptation in time domain is defined by extending the paging frame interval, spacing out paging frames more than in legacy systems, while each paging frame may contain more paging occasions than in legacy to maintain overall paging capacity if needed. This approach enables longer cell sleeping periods, reduces overall paging transmissions during low-load conditions, and facilitates more clustered RACH responses to paging. A limitation of Rel-19 is reduced network flexibility in accommodating temporary increases in paging load. This should be considered in the 6G design.

PRACH adaptation

In Rel-19, PRACH adaptation in time domain is defined. It allows the gNB to activate additional PRACH resources in addition to legacy PRACH resources. The additional PRACH resources are semi-statically configured and activated via DCI. They remain active until the expiration of their validity duration.

## 2.2	Network energy saving for 6G

There is a general consensus among the tele-communication industry partners that network energy saving (NES) features are essential for future 6G network developments, and NES is a core requirement for 6G [1]. To our view, leveraging the NES features developed and specified in 5G NR provides a solid foundation for 6G’s first release. Further enhancements specifically for 6G can be also considered, as we are no longer constrained by 5G legacy deployments. Moreover, based on the learnings from 5G NR NES, certain NES features may benefit from assistance from the UE (e.g. UE-initiated request for network transmissions) to maximize overall network energy savings. In the following, we describe key considerations for the NES feature design in 6G.

For initial 6G deployments, typically characterized by low or no-load conditions, the default operational mode of the 6G cells should be NES/energy saving mode. As said above, the default NES operation mode means for example infrequent transmission of always-on signals and control channels, and reduced PRACH and paging occasions to increase the NES gain. A 6G cell should be able to switch from the NES-native active operation mode to a less energy conserving mode, for example by adapting the periodicity of the broadcasting signals and channels. It may be beneficial to enable a more aggressive default energy-saving operational mode for capacity layer cells than for coverage layer cells or even turning off such a cells.

### 2.2.1	Cell DTX/DRX

For 6G day-1 design with no restrictions on backward compatibility solutions, cell DTX/DRX operations can be considered for both RRC CONNECTED and IDLE/INACTIVE UEs. Moreover, to further improve the network energy saving performance, the operation of cell DTX/DRX could be jointly considered with the designs of 6G signals/channels and other features. During the non-active period of cell DTX/DRX, further “relaxation” of transmission/reception (e.g. longer periodicity of NW transmission and UE monitoring/measuring/reporting) could be specifically considered for 6G signals/channels compared with active period of cell DTX/DRX. .

Furthermore, the joint optimization between cell DTX/DRX and UE C-DRX should be carefully considered, to ensure energy efficiency to be achieved at both network and UE side.

### 2.2.2 		Common signals

Clustering of broadcasting channels and signals, e.g. SS/PBCH, SIB, paging etc. allows the NW to have longer duration of sleep, which may lead to higher NES gain. The 6G design should consider clustered broadcasting of signals and channels as NES-native design. Likely this kind of design if taken into account at early phase of 6G comes basically free. Importantly, reducing the amount of broadcast signals, not just clustering, is crucial for lowering network energy consumption, and should be aimed in the 6G design also for the coverage layer cells.

#### 2.2.2.1	SS/PBCH

SS/PBCH is one of the key signals/broadcasting channels, and it is assumed that 6G radio will have an equivalent signal for similar purposes as earlier mobile network generations.

The transmission periodicity of SS/PBCH transmission has great impact to both network and UE energy efficiency. With the shorter SS/PBCH transmission periodicity, the better UE performance can be guaranteed with also optimized UE power consumption, e.g. the UE can perform measurements, tracking, and/or cell searching in a faster manner with less power consumption. But on the other hand, it results in higher network energy consumption when more frequent SS/PBCH transmissions have to be made. The longer SS/PBCH transmission periodicity, the better network energy saving can be achieved. But on the other hand, it may require longer time and even higher complexity at the UE side, i.e. for initial cell selection, etc. It may also result in higher UE power consumption for some procedures. Thus, the SS/PBCH design for 6G should consider a trade-off between UE complexity and NES at network side.

On-demand reference signals can be considered together with the periodically transmitted SS/PBCH signals, so as to achieve the balance between NES gain and UE power consumption. Also the default periodicity of the SS/PBCH signals can be different for coverage vs capacity layer cells, where the default periodicity can be relaxed for capacity cell.

The network triggered OD-SS/PBCH operation was mainly considered for SCell operation of RRC CONNECTED mode UEs in Rel-19 NES, where it is identified to have the benefits at least for the use case of fast SCell activation. For 6G design with no restriction on backward compatibility, the OD-SS/PBCH operation can be expanded to cover more scenarios, i.e. OD-SS/PBCH operation in PCell. And the RRC IDLE/INACTIVE mode UEs can be further investigated together with RRC CONNECTED mode UEs for OD-SS/PBCH operation. The UE-triggered OD-SS/PBCH mechanism shall also be investigated on top of network triggered OD-SS/PBCH operation.

#### 2.2.2.2	SSB-less SCell operation

SSB-less SCell operation is one of the frequency domain NES techniques introduced in 5G NR since Rel-15. In Rel-19 NES, the network triggered OD-SS/PBCH mechanism have been specified that provides a more effective solution to the (de-)activation of SCell operation. t

#### 2.2.2.3	SIB1 transmission

SIB1 have a significant impact on NES, therefore its design needs to be carefully considered. Based on NR field insights:

For SIB1, the current periodicity is 160ms. Repetition of SIB1 within the 160ms window is allowed and it is up to NW implementation. However, in typical deployments in NR a repetition every 20ms is used for SIB1 to match SS/PBCH periodicity. Hence from NES and UE perspective, in NR the SIB1 is transmitted every 20ms in most practical deployments. Due to SIB1 transmissions, every 20ms and in each beam, the SIB1 network energy consumption is relatively high. This was addressed in Rel-19 by specifying OD-SIB1, which achieves a high NES gain if the SIB1 requested on-demand is transmitted only in the SSB beam where the UE transmits the request. However, on-demand SIB1 was limited to multi-cell scenarios in Rel-19, requiring a coverage cell, e.g. the anchor cell, to provide the resource configuration for the OD-SIB1 request and based on the provisioning, the UE can request SIB1 from a NES cell, (capacity cell/non-anchor cell) with the OD-SIB1 feature enabled. It is essential that 6G design supports a SIB1 transmission that is more NES friendly from the first release, without adding too much latency to initial access.  We discuss this aspect in more detail in our SI/paging specific paper in R2-2508244.

#### 2.2.2.4	Other SIB transmission

Other SIBs may also have a considerable impact on NES because of their large number and size, therefore also their design needs to be carefully considered in 6G. For SIBs other than SIB1, the current SI-periodicity is higher than 80ms, based on 38.331. This relatively long periodicity results in a small impact to NW power consumption. In addition, on-demand Other System Information (OSI) has been specified from Rel-15 and provides energy saving potential compared to always on broadcasting OSI.

### 2.2.2.5	Paging

Rel-19 NES WI introduces the clustering of paging occasions in time domain for low-load scenarios and shows that potential NES gain can be achieved. The 6G radio needs to enable paging adaptations to scale paging capacity down during low load to attain more energy efficient operation while enabling swiftly increasing the paging capacity during temporary paging load increase.  We consider these aspects in more details in our R2-2508244 paper.

In addition to pure paging adaptation system needs to be designed to support this more holistically taking into account possible need for increased RACH occasions in case of paging clustering as well as optimizing reference signal positions according to the paging occasions to allow NW energy saving as well as minimizing UE wake up times prior the paging occasions.

### 2.2.2.6	PRACH adaptation

Rel-19 PRACH time adaptation allows NW (RX chain) to enable additional PRACH resources when needed instead of enabling frequent PRACH resources by default which increases the NES gain. However, Rel-19 introduced solutions considering major limitations due to the presence of legacy NR UEs. 6G day-1 PRACH design should consider the adaptation by design, e.g. when a cell is working with low/empty load, sparse PRACH is activated by default and vice-versa, i.e. when a cell is working with medium/high load, frequent PRACH is activated. Also PRACH design should take into account more sparse/lean carrier design where likely accesses of UEs are more clustered.

### 2.2.3	Fast cell switching on/off

Cell switch-off is a widely deployed techniques on the field that obtains significant NES gains during the often occurring low-load scenarios. In 5G there are significant delays to turn off/on a cell, for example due to the slow SIB update mechanism, where each step change in the SSB transmit power (used for graceful cell shutdown) requires the SI modification procedure (i.e. at least 2x the default paging cycle) , which in turn reduces the opportunities to apply such feature. In 6G we should allow networks to turn off a cell more quickly yet gracefully so that the UE performance is not compromised. In low-load scenarios, this could be handled for example with HO type of operation, but in case one needs to move different UEs simultaneously to a different carrier then we need more fluent and effective approaches.

## 2.3	NW power saving summary

In the below table we list possible features that should be considered in 6G network energy saving design – in the table we have bolded most promising solutions that are likely affecting mostly RAN2 that would be rather simple to do and provide good gains in energy efficiency:

| NES mechanism | NES feature | (Primary) targeted RRC state | Applicable to Coverage layer | Applicable to Capacity layer only |
| --- | --- | --- | --- | --- |
| NES-native design for C-plane signalling reduction | Default SSB periodicity for initial cell selection (e.g. 40 ms) | IDLE/INACTIVE | X |  |
|  | Relaxed SSB periodicity for cells not intended to support initial cell selection (e.g. 160 ms) | IDLE/INACTIVE |  | X |
|  | SSB (periodicity) adaptation | IDLE/INACTIVE |  | X |
|  | SS/PBCH decoupling | IDLE/INACTIVE | X |  |
|  | On-demand SIB1 | IDLE/INACTIVE | X |  |
|  | On-demand other SIBs (than SIB1) | IDLE/INACTIVE | X |  |
|  | On demand Reference Signals e.g. prior to paging | IDLE/INACTIVE | X |  |
|  | Paging adaptation | IDLE/INACTIVE | X |  |
|  | RACH adaptation | IDLE/INACTIVE | X |  |
|  | Clustering SSB/Paging/RACH | IDLE/INACTIVE | X |  |
|  | Cell DTX/DRX | IDLE/INACTIVE | X |  |
|  | Faster cell shutdown with UE steering to desired layer | CONNECTED/ IDLE/INACTIVE |  | X |
| NES-native design for U-plane adaptations | Conditional HO for NES | CONNECTED | X |  |
|  | CSI-based PDSCH spatial and power domain adaptation | CONNECTED | X |  |
|  | Joint Cell DTX/DRX and UE C-DRX | CONNECTED | X |  |
|  | SCell (de)activation | CONNECTED |  | X |
|  | SSB-less for SCell | CONNECTED |  | X |
|  | On-demand SSB for SCell | CONNECTED |  | X |

This leads us to following proposal to start 6G study item for NW power saving:

Proposal 1: Study means to minimize common signal transmissions (e.g. SSB, paging, system information) and allow time-domain adaptation for common signals and channels transmissions based on the cell requirements e.g. load, coverage vs. capacity cell, including on-demand provisioning.

Proposal 2: Study efficient methods to allow fast cell shutdowns (and wakeups) with efficient steering of UEs from soon to be shutdown cells both in RRC_IDLE and RRC_CONNECTED.

Proposal 3: Study Cell DTX/DRX and UE DRX to be jointly designed, considering UEs with or without UE DRX configurations.

Proposal 4: Study means to allow energy-efficient usage of wide bands (e.g. carrier aggregation) with minimal always-on signals and efficient activation/deactivation of CA

## 2.4	UE power saving for 6G

In 6G, the UE energy saving is a high priority target for the 6GR design as indicated by the SID [3]. When designing the 6G air interface, it is anticipated that the leaner framework for enabling scalable bandwidth in single cell and multi-cell scenarios would be useful to save also UE power. While this will help the UE to perform only the measurements that are needed, such target is also improving the UE measurement framework for allowing the UE to operate in efficient low-power and low-activity modes.

### 2.4.1 List of 5G UE power saving features in RAN2 specifications

Figure 2. UE power saving mechanisms in 5G.

Discontinuous Reception (DRX) in RRC_IDLE/INACTIVE

DRX allows the UE to periodically turn off its receiver to save power while maintaining connectivity. In RRC_IDLE and RRC_INACTIVE states, DRX is used to monitor paging occasions, allowing the UE to remain in a low-power state for longer durations.

Discontinuous Reception (DRX) in RRC_CONNECTED

DRX allows the UE to periodically turn off its receiver to save power while maintaining connectivity. In the RRC_CONNECTED state, DRX enables the UE to monitor the PDCCH only during specific time intervals, reducing the need for continuous monitoring. Long DRX cycles allow the UE to stay in a low-power state for extended periods, suitable for scenarios with low data activity, while short DRX cycles enable frequent wake-ups for higher responsiveness, ideal for active data transmissions.

Bandwidth Part (BWP) Operation

BWP operation enables the UE to switch between different bandwidth parts, using narrower bandwidths during low data activity to save power. MAC and PHY/DCI signaling can be used for BWP switching. Also, timer-Based switching is supported where the bwp-InactivityTimer enables switching to the default BWP after inactivity.

RRC INACTIVE mode

The RRC inactive mode is a state where the UE maintains its RRC and security context with the network. This mode allows the UE to quickly transition with less signalling (RRC Resume procedure) back to RRC_CONNECTED while saving power during periods of inactivity.

Wake-Up Signal with DRX (DCP)

Downlink Control Information (DCI) for Power Saving (DCP) is a signal sent before the DRX cycle to inform the UE whether it needs to monitor the PDCCH. This avoids unnecessary wake-ups during DRX cycles, further reducing power consumption. The DCP is designed to be energy-efficient and minimizes the need for the UE to decode the full control channel.

Furhther PDCCH monitoring adaptation

Typically, the largest UE modem related contributor to unnecessary power consumption is Physical Downlink Control Channel (PDCCH) monitoring. Discontinuos reception, as highlighted above has been used to reduce the power consumption in this area. Other PDCCH monitoring adaptation schemes introduced in NR are SSSG switching and   PDCCH skipping. SSSG switching allows UE to be configured with SSS groups where the PDCCH monitoring activity is different enabling network to adapt UE to lower power consuming operation when e.g. data activity is low. PDCCH skipping allows the UE to skip monitoring certain PDCCH occasions when indicated by network. Network can choose to indicate skip when there is no date or scheduling expected for the UE. This schemes reduce the energy the UE spends in active mode, saving energy.

UE Assistance Information

The UE can provide assistance information to the network, such as its preferred DRX cycle length or power-saving preferences or preferred RRC state or overheating assistance information. This allows the network to optimize configurations for the UE, balancing power savings and performance.

RRM/RLM/BFD Relaxation in RRC_CONNECTED

Relaxation of Radio Resource Management (RRM), Radio Link Monitoring (RLM), and Beam Failure Detection (BFD) requirements in RRC_CONNECTED reduces the frequency of measurements and reporting. This minimizes the UE's power consumption during active connections. The network can configure these relaxations for example based on the UE's mobility and service requirements.

RRM Relaxation in RRC_IDLE and RRC_INACTIVE

In RRC_IDLE and RRC_INACTIVE states, RRM relaxation reduces RRM measurements and saves UE power. The relaxed requirements are particularly beneficial for UEs in low-mobility scenarios.

Paging Early Indication (PEI)

PEI is a mechanism where the network provides an paging early indication of an upcoming paging message. This allows the UE to prepare for paging reception without continuously monitoring the paging occasion.

TRS Availability Indication

The TRS (Tracking Reference Signal) availability indication informs the UE about the presence of TRS for mobility and beam management. This allows the UE to selectively monitor TRS signals, reducing unnecessary measurements.

Low Power Wake-Up Signal (LP-WUS) in RRC IDLE/INACTIVE/CONNECTED

The UE can be configured with LP-WUS for power saving in all RRC states. In RRC IDLE/INACTIVE, if the UE detects LP-WUS and the LP-WUS is associated with the UE, the UE monitors the associated PO. In order to reduce UE power consumption due to false paging alarms, the group of UEs monitoring the same PO are further divided into multiple subgroups. UE supporting LP-WUS, while monitoring serving cell with low power receiver, may perform further relaxed serving cell and neighbouring cell measurements on MR or serving cell measurement offloading from MR to LR. In RRC CONNECTED LP-WUS is monitored outside active-time. If LP-WUS is detected, the UE shall start the on-duration timer or LP-WUS PDCCH monitoring timer to start PDCCH monitoring and enter active-time.

### 2.4.2 UE power saving for 6G

DCP and Low Power Wake-Up Signal (LP-WUS) both serve the same purpose in the RRC_CONNECTED state, while Paging Early Indication (PEI) and LP-WUS serve the same purpose in the RRC_IDLE and RRC_INACTIVE states. To maintain efficiency and avoid redundancy, 3GPP should strive to minimize the definition of parallel solutions that achieve the same functionality. Given that LP-WUS offers better UE power-saving benefits compared to both DCP and PEI, it is justified that RAN2 specifies LP-WUS only as the unified solution for 6G.

Enhanced wake-up signal and receiver functionalities are beneficial for 6G to improve wake-up signal coverage across the entire cell, extend wake-up signal and receiver capabilities to enable neighbor cell search and measurements in IDLE and INACTIVE states, and relax or offload L1 and CSI measurements in the CONNECTED state. Additionally, the applicability of a wide range of wake-up signal and receiver-based operations in 6G should be evaluated to ensure UEs operate in an energy-efficient manner. Based on these advancements, LP-WUS should be specified as the unified solution for 6G, replacing similar features in 5G such as PEI, DCP.

Energy-efficient PDCCH operations should be designed for 6G to enable PDCCH monitoring only when necessary, ensuring optimal power usage. Similarly, the design of L1, L3, and CSI measurements and reporting in 6G should be simplified and optimized for efficiency. This involves aligning measurement activities and reporting levels with actual network conditions to provide sufficient information while minimizing UE power consumption.

Based on insights from 5G deployments, the proposed measures for enhancing UE energy efficiency in 6G include prioritizing unified solutions like LP-WUS to avoid redundancy and maximize power-saving benefits, focusing on energy-efficient designs for control channels and measurement or reporting processes, and leveraging lessons learned from 5G to inform 6G design choices that align with sustainability and performance goals. The following is proposed:

Proposal 5: Take the following 5G UE power saving features as a baseline for 6G and study whether/how to improve them: (e)DRX in all RRC states, RRM/RLM/BFD relaxation, TRS Availability Indication, RRC INACTIVE, SDT, UE assistance information for overheating/power saving(allow the UE to provide UE assistance information at least for overheating and power saving purposes such as preferred RRC state).

Proposal 6: Study further enhanced wake-up signal/receiver (as the unified solution for 6G instead of specifying all similar features in 5G such as PEI, DCP and LP-WUS).

Proposal 7: Study further Efficient measurements and reporting: the L1/L3 and CSI measurements and reporting design should be simplified and optimized for efficiency.

# 3	Conclusions

We have discussed the energy efficiency in 6GR for both UE and NW side as they cannot be designed separately in order to have efficient solutions. This resulted in the following proposals:

This leads us to following proposal to start 6G study item for NW power saving:

Proposal 1: Study means to minimize common signal transmissions (e.g. SSB, paging, system information) and allow time-domain adaptation for common signals and channels transmissions based on the cell requirements e.g. load, coverage vs. capacity cell, including on-demand provisioning.

Proposal 2: Study efficient methods to allow fast cell shutdowns (and wakeups) with efficient steering of UEs from soon to be shutdown cells both in RRC_IDLE and RRC_CONNECTED.

Proposal 3: Study Cell DTX/DRX and UE DRX to be jointly designed, considering UEs with or without UE DRX configurations.

Proposal 4: Study means to allow energy-efficient usage of wide bands (e.g. carrier aggregation) with minimal always-on signals and efficient activation/deactivation of CA

And for UE power saving:

Proposal 5: Take the following 5G UE power saving features as a baseline for 6G and study whether/how to improve them: (e)DRX in all RRC states, RRM/RLM/BFD relaxation, TRS Availability Indication, RRC INACTIVE, SDT, UE assistance information for overheating/power saving(allow the UE to provide UE assistance information at least for overheating and power saving purposes such as preferred RRC state).

Proposal 6: Study further enhanced wake-up signal/receiver (as the unified solution for 6G instead of specifying all similar features in 5G such as PEI, DCP and LP-WUS).

Proposal 7: Study further Efficient measurements and reporting: the L1/L3 and CSI measurements and reporting design should be simplified and optimized for efficiency.