3GPP TSG-RAN WG2 Meeting #132	R2-2508244

Dallas, USA, 17 – 21 November 2025

Agenda item:	10.3.2.3

Source:	Nokia

Title:	System Access and Carrier Aggregation in 6GR

WID/SID:	FS_6G_Radio - Release 20

Document for:	Discussion and Decision

# 1	Introduction

A 6G study item for RAN was first approved in June 2025 [1], and subsequently updated in September 2025 [2].

In this paper we discuss further details on what are issues with 5G design for system information and paging. Also we discuss basic principles on how to start implementing carrier aggregation in 6GR.

# 2	On System Information

In this contribution we would like to discuss the key aspects related to system information.

## 2.1	On NR MIB and SIB1 contents and size

Agreements RAN2#131bis - Study aspects related to SIB1 size. Understand the issues and desired content/size from RAN2 point of view.

### 2.1.1	MIB

MIB enables a UE to synchronize with system frame number and access the non-barred cell.  From MIB UE acquires the neccesary information to obtain remaining minimum system information (Namely, in case of NR, the neccesary information to monitor for the PDCCH scheduling the PDSCH carrying the SIB1 is provided (CORESET and SS).

Currently, the systemFrameNumber [5:0] contains 6 MSBs of the 10-bit SFN are present in the MIB. The remaining 4 bits (LSBs) of the SFN are inferred from the SSB index and timing by UE during initial sync. Although the 6 bits might be small in terms of space occupied, however, the MIB is size-limited. Thus, even saving 1 or 2 bits in MIB is valuable, and can be utilized to enable NES or some other relevant services. But most of these contents are heavily dependant on the layer 1 design and RAN2 should wait for further progress in RAN1.

MIB ::= SEQUENCE {

systemFrameNumber         BIT STRING (SIZE (6)),								 6 bits

subCarrierSpacingCommon   ENUMERATED {scs15or60, scs30or120}, 		 1 bit

ssb-SubcarrierOffset      INTEGER (0..15), 										 4 bits

dmrs-TypeA-Position       ENUMERATED {pos2, pos3},   					 1 bit

pdcch-ConfigSIB1          INTEGER (0..255), 									 8 bits

cellBarred                ENUMERATED {barred, notBarred},   		 1 bit

intraFreqReselection      ENUMERATED {allowed, notAllowed}, 		 1 bit

spare                     BIT STRING (SIZE (1)) 								 1 bit

}				        	         																Total 24 bits

Even though most of the parameters in MIB takes a few bits, they are important in all practical considerations. However, due to the limited size of MIB, it is important to discuss them and explore the possibilities of their optimization in 6G.

Observation 1: The overall MIB size and MIB contents regarding physical layer information need heavy RAN1 involvement and RAN2 needs to wait RAN1 progress.

On the other hand, we have in 5Gcell barring information which controls the UE’s attempt to camp on the cell occupies only 1 bit (or 2 bits as intraFreqReselection is heavily linked to barring). In 5G we learned that we needed to add multiple different barring bits later and those were included in SIB1. It seems likely similar will happen in 6G in time if not in first release. Thus, it might be asked whether actually we need barring information in the MIB if UE anyway needs to acquire SIB1. In our view cell being fully barred is not very common use case and using bits for that from MIB seems with pricey and should be discussed in 3GPP thoroughly whether we should really include barring in the MIB.

Observation 2: One should consider whether 6G barring can be designed in a way that all the required different barring mechanisms would be placed in SIB1.

### 2.2.1	SIB1 size and content

Further details of the physical layer configuration are provided in SIB1, such as carrier and BWP configuration for UL and DL, UL/DL slot pattern configuration for TDD and SSB periodicity and pattern. This information includes also necessary configuration for paging monitoring and other SI acquisition. Also, the configuration covers UL initial access (RACH) and UL control and data channel configuration. For access control, the cell quality related parameters are provided (to establish the suitability) as well as barring related information, e.g. per UE type, and also the PLMN(s) for RAN sharing as well UAC access control parameters.

Over each 3GPP new releases, the SIB1 size has progressively increased due to the addition of new features. The maximum size of SIB transmitted is limited by the efficiency of channel encoding scheme at physical layer. Due to physical layer evolution, NR SIB size is larger than what was allowed in LTE. It is important to note that NR SIB maximum defined sizes are variable and can even exceed the maximum allowed size for SIB transmission. SIB1 seems to have been defined with some flexibility for network, where not all information can simultaneously active or used.

Observation 3: The maximum defined size of SIB is variable and can even exceed the maximum size of SIB can take based on the limits imposed by physical layer.

The maximum size of SIB1 is not documented (in either 4G or 5G) anywhere and needs to be inferred from ASN.1. This requires some interpretation and while straightforward, is not something readily available (since one has to infer it from the maximum possible list sizes, with all optional fields present, etc., which may require manual work). Further, the physical layer adds some overhead (e.g. CRC) that need to be accounted for, and depending on which MCS network uses, the maximum size supported in practice may also vary.

The NR SIB1 size can vary from 233 to 614 bytes, excluding headers, depending on configuration. Thus, it is important to understand the SIB1 component and analyse which parts of SIB1 consumes the most space. For example, if multiple PLMNs are configured, PLMN could consume a sizeable space, thus, it would be a potential candidate to look into. Similarly, the scheduling information that provide detail information on how SIBs are scheduled, SIB mapping which gradually grows with larger number of SIBs. Besides, UAC provides details on the cell access related information also a big contributor in SIB1 size.

Further, SIB1 also carries a growing list of optional parameters – The design of NR SIB1 is such that whenever anything is changed within SIB1 it forces NW to change whole SIB1 and UEs to acquire that. Whether the SIB1 size is critical, may depend on different network implementations based on the subset of features activated. However, 6G study needs to analyse which elements of SIB1 may be required by UE and which elements may be transmitted slower rate e.g. in other system information block.

Observation 4: The monolithic structure of SIB 1 and the existing transmission scheme have not evolved in line with the continual expansion of SIB 1 contents. Every UE regardless of whether it needs only the mandatory cell access parameters or others, is forced to receive and decode entire SIB1.

SIB1 indicates using the scheduling information the time instants when SIBs can be scheduled. Since SIB1 governs the timing of all SIBs, introducing more SIBs increases the number of SI-change indications, which in turn forces UE to read SIB1 updates more often. As the list of SIBs grows, the probability of SI changes rises, thereby compelling triggering the SI change indication mechanism.  This will impact UE, which must repeatedly acquire or re-read the updated SIB1 and the updated SI. Thus, the SI modification procedure should be studied to make it more dynamic and lightweight e.g. by having more granularity and possibility to have faster changes in system information at least for some specific parts (e.g. NES related changes).

Observation 5: Discuss which parts of information need to reside in MIB and most frequently transmitted SIB and what are delay requirements for acquiring such a information.

Based on such a analysis one could consider actual placement of information in different system information blocks and after that it would be possible to consider in detail how one could indicate changes to system information in a way that minimizes delays of updated SI acquisition and UE/NW energy saving potential- we discuss this in more detail in SI update part of the paper.

## 2.2	Other System Information aspects

Agreements RAN2#131bis - System information design should consider energy efficiency and low complexity for both network and UE. - Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs)

### 2.2.1	On demand SIB and SIB1

The other system information is broadcasted to the UEs in regular periodic intervals depending on the scheduling of these SI messages. The SI messages may also be transmitted on-demand if indicated in the  scheduling information carried in SIB1. 6G SI design may continue supporting such functionality. Additionally, when multiple UEs are requesting SIBs on-demand, how the network may handle such scenarios also needs to be taken into account while designing the on-demand procedure.

Release 19 explored the on-demand SIB1 solution from a network energy-saving perspective, utilizing an uplink-WUS-configuration to define the minimal uplink configuration needed by UE to access the cell and request SIB1. While R19 considered providing this configuration from a different cell (anchor cell), 6G solution should ideally provide it within the same cell (stand alone cell) for example, the coverage cell transmitting the minimal uplink configuration for the on-demand SIB1 request. Possible issue with this kind of stand-alone cell on-demand SIB1 is how to provide UE with access information to request SIB1 as UE should get this information before acquiring SIB1. When designing such a support one should carefully consider also possible impacts to latency increases.

Proposal 1: 6G should study methods to enable on-demand SIB (including SIB1) on stand-alone cell (coverage layer) as well as (capacity layer) and involve RAN1 request feasibility for coverage layer on demand SIB1 support.

## 2.3	On SI scheduling and update

Agreements RAN2#131bis - Study how to improve flexibility and extensibility of scheduling SI design - Study mechanisms to improve SI update mechanism - System information design should ensure that the public warning requirements are met. - Study areas specific SIB design. Understand 5G pain points and what improvements can be considered.

### 2.3.1	SI update

Further, the network may be able to change the contents of the SI messages that are transmitted but this needs to be indicated to the UE. The change of the SI occurs at what is called a modification period. The same content of SI is transmitted within a modification period. Any change to the SI content is only applied in the next modification period. But the indication that the SI content would change is provided to all UEs via the Short Messages (systemInfoModification bit indicated via DCI scrambled with P-RNTI), multiple times within a modification period and in paging monitoring occasions.

When the network needs to update system information, it must wait for the next modification period boundary to activate the changes. During modification period n, the network sends one or more Short Message via paging (using systemInfoModification flag) to notify all UEs that SI will change in the next period n+1.

SIB1 indicates via the scheduling information the time instants when SIBs can be scheduled. Since SIB1 governs the timing of all SIBs, introducing more SIBs may increase the probability of SI changes notifications. This will impact UE, which must repeatedly acquire/re-read the updated SIB1 and the updated SI. The current method of system information update is suboptimal. There is a need to enable more dynamic updates with reduced delays and signalling overhead between gNB and UE.

Also, it is worth noting that not all SIBs are of interest to all the UEs. When UE receives the notification of SI modification, it is required to read the updates even if it is not of interest, (e.g., SIB1 parameters introduced in a later release, SIBs introduced in later release). There is no mechanism for UE to learn more about what is being updated and hence less efficiency from UE perspective.

Observation 6: Upon receiving the notification for SI modification, UEs are forced to read the updates although not all information (full or partial SIB contents) is of interest to all UEs.

Problem with the existing SI change notification in 5G is the quite lengthy delays it causes – especially from network energy saving point of view e.g. in case NW wants to change parameters considering energy efficiency or even turning off the cell e.g. ramping down SSB power where each step change in the SSB transmit power (used for graceful cell shutdown) requires the SI modification procedure (i.e. at least 2x the default paging cycle).

In 5G it was more understandable that SI changes mandated UE to acquire all the SIB if something was changed but in 6G we foresee need to change limited set of parameters more often – forcing UE to always read everything would cause extra power consumption. Thus we encourage RAN2 to discuss whether added complexity for more granular SIB update mechanism would

In 5G  UE may be configured by upper layers and/or RRC with an extended DRX (eDRX) cycle TeDRX, CN and/or TeDRX, RAN. In 5G, due to the introduction of the eDRX paging cycle with the maximum length of 10485.76 s (2.91 h) being well above the maximum modification period of 40.96 s, the SI update notification mechanism was extended by adding definition of eDRX acquisition period to cover new cases when eDRX cycle is longer than the modification period. The eDRX acquisition is equal to the maximum configurable eDRX cycle which is 10485.76 s (2.91 h). The eDRX acquisition period provides a time frame for the process of notifying UE with IDLE eDRX cycle longer than the modification period about the SI change and determines the moment of SI re-acquisition. The network may postpone any SI change so that it occurs at eDRX acquisition period boundary and is preceded by a full eDRX acquisition period with SI change indication transmissions, but the resulting delay (2.91 h in the best case) and resource consumption for continuous transmission of Short Messages with systemInfoModification-eDRX in every PO within the preceding eDRX acquisition period is unreasonable in cases that do not require strict simultaneous switch of all UEs to a new SI configuration.

Observation 7: The current mechanism of SI update notification for eDRX UEs based continuous transmission of Short Messages in every PO within the preceding eDRX acquisition period (2.91h) is sub-optimal.

Observation 8: The current mechanism of SI update causes long delays and is sub-optimal from the network energy saving perspective.

Proposal 2: RAN2 to study the efficient SI change mechanisms that allow change of some specific parts of system information taking UE power consumptions into account and allow faster SI changes for relevant parts while taking network energy saving into account while operating with DRX or eDRX.

### 2.3.2	PWS delivery

The Public Warning messages (PWS) are sent using the system information broadcast framework. Unlike other SIBs which broadcasts system parameters or other parameters required for operation of a certain feature or functionality, the PWS SIBs broadcast content that is to be forwarded to application layer where it is just delivered (to be displayed) to the end user. In other words, these are for use above the access stratum. Typically, such delivery of broadcast content is intended to be done by a broadcast solution where the content is delivered over user plane path.

The notification for PWS is provided via the ShortMessage (etwsAndCmasIndication bit indicated via DCI scrambled with P-RNTI). In NR network basically needs to provide this indication in every single PO as NW does not have visibility which actual PO UE will be listening.

Observation 9: In NR NW does not have visibility which POs UEs are listening and mandates NW to send PWS indication in every possible PO which is problematic from NW energy saving point of view

Some warning messages may be delivered over multiple concurrent warning notifications. UE is further required to read the SIB1 and learn about the SIB scheduling update before it can receive the SIBs for PWS. How the RRC CONNECTED UEs may obtain the updated when configured with the measurement gap is left to the UE implementation. This has been problematic in 5G to ensure achieving timing requirements for emergency messages.

Observation 10: In NR UE may miss PWS messages due to measurement gap usage freedom left to UE in RRC_CONNECTED state – possibly increasing PWS acquisition delay

Observation 11: RAN2 should design details of PWS delivery together with the other parts of SI design for 6G e.g. general SI update mechanism and carefully consider how much UE freedom from reading PWS information we leave in order to achieve timing requirements for emergency messages

### 2.3.3	Positioning information

In case of posSIBs, the broadcast content may require segmentation since they carry positioning assistance data which can be huge in terms of size, but the segmentation and reassembly functions were allocated to the end points in the LMF and UE. The gNB does not do segmentation of posSIB. Such segmentation capability is handled at the application layer or upper layer. Also, handling of change in contents is handled at the application layer or upper layer. For these reasons broadcast of posSIBs does not really fit with the existing system information broadcast paradigm. In NR, LMF is the source of the positioning assistance data from UE perspective, the LMF builds the posSIBs, performs segmentation if needed. In NR, the NRPPa protocol is used to transport the posSIB PDUs to gNB but may have metadata associated with the posSIB PDUs that gNB just uses to schedule the broadcast of posSIBs. In 6G, the question is, whether any design goals for SI broadcast will change this existing way of handling posSIBs for positioning.

Observation 12: RAN2 should consider whether positioning information is delivered through SIB transfer or transparent to control plane.

### 2.3.4	Area-specific SIB

SIB validity defines that UE stores acquired SIBs along with their associated areaScope (if present), PLMN-Identity, cellIdentity, systemInformationAreaID, and valueTag. For a stored SIB, the associated identifiers and areaScope must match the contents broadcast in the si-schedulingInfo for the SIB. A stored SIB (excluding MIB, SIB1, SIB6, SIB7 or SIB8) is considered valid for 3 hours after acquisition, after which the UE must delete it. In practice it has been quite difficult to utilize area specific SIBs but mostly all the SIBs are cell specific.

Observation 13: RAN2 should carefully study if it is possible to have system information (or part of SIB) being common among multiple cells

### 2.3.6	SIB security

In 5G there has been identified various security issues with system information and we discuss those in more details in  R2-2508697.

## 2.4	SIB design summary

In above sections we have discussed NR SI design and pain points of that. This has resulted in following proposals:

Observation 5: Discuss which parts of information need to reside in MIB and most frequently transmitted SIB and what are delay requirements for acquiring such a information.

Observation 11: RAN2 should design details of PWS delivery together with the other parts of SI design for 6G e.g. general SI update mechanism and carefully consider how much UE freedom from reading PWS information we leave in order to achieve timing requirements for emergency messages

Observation 12: RAN2 should consider whether positioning information is delivered through SIB transfer or transparently to control plane

Observation 13: RAN2 should carefully study if it is possible to have system information (or part of SIB) being common among multiple cells

Proposal 1: 6G should study methods to enable on-demand SIB (including SIB1) on stand-alone cell (coverage layer) as well as (capacity layer) and involve RAN1 request feasibility for coverage layer on demand SIB1 support.

Proposal 2: RAN2 to study the efficient SI change mechanisms that allow change of some specific parts of system information taking UE power consumptions into account and allow faster SI changes for relevant parts while taking network energy saving into account while operating with DRX or eDRX.

# 3	On 6G Paging

After the initial design on paging in 5G Rel-15, paging early indication (PEI) and TRS availability indication were introduced in Release-17, and in Release-19 Low Power Wake-Up Signal (LP-WUS) for paging monitoring was introduced. Additionally, Release-19 also introduced the paging adaptation mechanism to enable network energy savings. It would be natural to consider 5G NR paging work in 6G paging design.

Paging allows the network to reach UEs in RRC_IDLE and in RRC_INACTIVE state through Paging messages, and to notify UEs in RRC_IDLE, RRC_INACTIVE.  The system information change and ETWS/CMAS indications through Short Messages is used as notification mechanism for UEs in any RRC state. Both Paging messages and Short Messages are addressed with P-RNTI on PDCCH, but while the former is sent on PCCH, the latter is sent over PDCCH directly [3].

In the last meeting RAN2#131bis, it has been agreed that paging design should consider energy efficiency and simplicity for both UE and Network [4], as the starting point.

In this contribution, we discuss some of the key issues in 5G NR paging, specially focusing on the energy efficiency aspects of paging transmission.

## 3.1	5G NR paging design

There have been several modifications to the initial Release 15 NR paging design in progressive releases. Next, we briefly discuss about the paging enhancements specified in these different releases.

### 3.1.1	Release 15 Paging design

A UE monitors one paging occasion (PO) per DRX cycle over Paging frame (PF). A PO is a set of PDCCH monitoring occasions and can consist of multiple time slots (e.g. subframe or OFDM symbol) where paging DCI can be sent. One Paging Frame (PF) is one Radio Frame and may contain one or multiple PO(s) or starting point of a PO [5].

The main design criterion in Release 15 paging was to optimize paging capacity. This was achieved by uniformly distributing the paging frames (PFs) across the Idle state DRX cycle. That is in the legacy paging frame distribution method, the paging frames are uniformly distributed within the RRC-Idle/Inactive state DRX cycle using the legacy paging frame allocation equation shown below [5], see Figure 1,

SFN+PFoffset mod T= TN . UEID mod N.

Figure 1: Legacy PF distribution (depending on N)

It can be seen that due to the nature of the equation, the paging frames are always uniformly distributed along the DRX cycle. When the PFs are uniformly distributed, it helps to manage the paging load of the network efficiently.

### 3.1.2	Release 17 Paging early indication

To reduce the unnecessary paging message monitoring and false alarms, paging early indication (PEI) was introduced in Release 17. In order to reduce UE power consumption due to false paging alarms, the group of UEs monitoring the same PO can be further divided into multiple subgroups. If the UE detects a PEI and the PEI indicates the subgroup the UE belongs to monitor its associated PO, the UE monitors the associated PO. If the UE does not detect PEI on the monitored PEI occasion or the PEI does not indicate the subgroup the UE belongs to monitor its associated PO the UE is not required to monitor the associated PO [5], see Figure 2.

Figure 2: Release 17 PEI for paging monitoring

If PEI and subgrouping are configured, UEs monitoring the same PO can be divided into one or more subgroups. With subgrouping, the UE monitors the associated PO if the corresponding bit for subgroup the UE belongs to is indicated as 1 by PEI corresponding to its PO.

### 3.1.3	Release 19 LP-WUS based paging

In Release 19, low-power wake-up signal (LP-WUS) based paging was specified to reduce the power consumption of a UE. LP-WUS in the IDLE and INACTIVE states triggers the UE to perform paging monitoring (with the main radio (MR)) while LP-WUS is monitored with LR reducing the power consumption of a UE . Similar to PEI, to reduce the UE power consumption by reducing unnecessary paging message reception/monitoring, the group of UEs monitoring the same PO can be further divided into multiple subgroups. With subgrouping, a UE monitors paging in its PO if the subgroup to which the UE belongs is paged as indicated via associated LP-WUS. If a UE beyond the coverage range of the LP-WUS or UE  unable to monitor the associated LP-WUS occasion corresponding to its PO, it shall monitor the paging in its PO.

Figure 3: Release 19 LP-WUS based Paging

### 3.1.4	Release 19 Paging adaptation

In Release 19, the objective was to minimize the energy consumption of the network in paging transmission. Paging adaptation was specified, specifically, number of POs per PF were increased (i.e., Ns = 8) and sparsity of paging frames in DRX cycle was increased (i.e., N = T/32), see Figure 4. The UE supporting paging adaptation shall monitor PDCCH in POs separately signalled for paging adaptation, if configured. The UE supporting paging adaptation and PEI can monitor PEIs separately signalled for paging adaptation, if configured.

Figure 4: Release 19 Paging adaptation

By analysing the different paging design objectives in different releases in 5G NR, the following observations can be made.

Observations 14: In 5G NR paging design, the primary design criteria were independently optimizing the paging capacity of the network, minimizing the UE power consumption, and the network power consumption.

Observations 15: The specified approaches were not mutually supporting each other to save energy of UEs and the network together. For e.g., the proposed approaches for UE power savings, for e.g., using LP-WUS for paging and PEI were not supporting to reduce the energy consumption of a network.

### 3.1.5	CN/RAN paging

Core network handles paging procedure while the UE is in RRC-IDLE state, while the RAN handles the paging procedure while the UE is in RRC-INACTIVE state. When a UE is in RRC-IDLE state the UE location is maintained in the AMF in terms of tracking area (TA). A UE does not update about its location to the network, when it is travelling within the TA; thus, a paging message needs to be broadcast by all the gNBs within that TA.

Table 1: Summary of paging in 5G NR

| RRC state | Paging handling entity | UE paging identity | UE location tracking for paging | Paging purpose |
| --- | --- | --- | --- | --- |
| RRC-IDLE | Core network | 5G-S-TMSI | Tracking area | To initiate a mobile terminated connection, Emergency call back, |
| RRC-INACTIVE | RAN/Core Network | I-RNTI/5G-S-TMSI | RAN notification area | To initiate a mobile terminated connection, Paging to indicate the UE to initiate SDT, Emergency call back, |

In 5G we have separated paging for RAN and CN with different paging areas (TA/RNA) – During 5G specification design it was identified that in order to reduce paging latency as well as overhead of sending paging in wide area, 5G introduced RAN based paging where the context of the UE is stored in RAN instead of CN.  Drawback of this is of course increased complexity due to two different type of paging methods. As the CN/RAN paging separation is heavily dependent also on the RRC states discussion one should consider these together in the paging designs. But what should be understood from paging point of view is to design paging in such a way that it allows paging within some smaller areas than core network tracking area as well as allowing reduced latency and possible support of SDT like data transfers. Additionally, it should be understood that keeping always UE context stored in the RAN may not be feasible always.

Observation 16: In the paging design to support (or not) CN and RAN type of paging one needs study following aspects simultaneously with RRC states design: Latency,  Paging overhead in the network to transmit paging message in multiple cells/locations, NW implementation complexity (e.g. context storage), UE implementation complexity and Possible support of SDT

### 3.1.6	NB-IoT paging

NB-IoT (Narrowband Internet of Things) was specified in 3GPP as part of Release 13 to address the growing demand for IoT connectivity. NB-IoT was designed to support a massive number of IoT devices.

For NB-IoT, multi-carrier operation is supported by utilizing anchor carriers and non-anchor carriers:

Anchor carrier: in NB-IoT, a carrier where the UE assumes that NPSS/NSSS/NPBCH/SIB-NB for FDD and IoT NTN TDD or NPSS/NSSS/NPBCH for TDD are transmitted.

Non-anchor carrier: in NB-IoT, a carrier where the UE does not assume that NPSS/NSSS/NPBCH/SIB-NB for FDD or NPSS/NSSS/NPBCH for TDD are transmitted.

The UE in RRC_IDLE can, based on broadcast signaling, use a non-anchor carrier for paging reception. The UE in RRC_IDLE, based on broadcast signaling, can use a non-anchor carrier for random access. If the non-anchor carrier is not configured for the UE, all transmissions occur on the anchor carrier.

Multi-carrier operation for NB-IoT was introduced to support a massive number of IoT devices and to enable efficient load balancing across multiple carriers. It is important to note that NB-IoT devices were designed to transmit and receive data that is inherently delay-tolerant. As a result, latency was not considered as a critical factor during the specification of this technology.

Observation 17: RAN2 needs to perform analysis regarding the need for multicarrier operation (carrier(s) for paging and/or random access) considering all device categories by  e.g. taking into account latency and capacity requirements and it provides sufficient gains to warrant extra layer of complexity.

In PEI and LP-WUS based paging, the designs help to reduce the paging false alarms and energy consumption of an IDLE/INACTIVE state UE. But the network has to transmit these LP-WUS and PEI, and also needs to support the UEs who are only capable of Release 15 paging procedure; thus increasing the energy consumption of the network. In 6G, network does not need to use both these pre-indicator signals as they are used for the same purpose. RAN1 may study the impact of the pre-indicator signals that could be used for paging procedure, for e.g., OD-SS or RS, for UE power efficiency improvements, and could rely on the most efficient schemes.

## 3.2 	Summary of paging design for 6G

We have discussed in this paper various aspects of Paging design of NR and problems with that. It is possible to take care of these issues without too much effort or complexity as far as they are taken into consideration from the start of 6G standardization.

Proposal 3: RAN2 to study the need for multicarrier operation (carrier(s) for paging and/or random access) considering all device categories by e.g., taking into account latency and capacity requirements

Proposal 4: Paging in 6G should support possibility for NW to do clustered (R19 NES style) and capacity optimized (R15 style) paging approaches with taking into account need for UE power saving (e.g. by utilizing LP-WUS) simultaneously taking into account possible developments with RRC states, requirements regarding latency as well as paging overhead in the network to transmit paging message in multiple cells/locations as well as UE and NW implementation complexity (e.g. context storage) and possible support of SDT.

# 4	On Carrier aggregation

Agreements on Spectrum aggregation

- 1	Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed.

- 2	Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc).  Understand the deployment scenarios and problems to address

During the RAN2#131bis it was agreed to study and understand the issues to be addressed with spectrum aggregation. We would like to start the discussion with the concept of serving cell and delve into the multi-carrier operations.

## 4.1	The role of serving cell in 5G

The concept of serving cell has been at the heart of all cellular technologies for obvious reasons, and has several definitions in 3GPP, e.g. the following from TR21.905 and from TS36.300:

- TS36.300: Cell: combination of downlink and optionally uplink resources. The linking between the carrier frequency of the downlink resources and the carrier frequency of the uplink resources is indicated in the system information transmitted on the downlink resources.

- TR21.905: Cell: Radio network object that can be uniquely identified by a User Equipment from a (cell) identification that is broadcasted over a geographical area from one GERAN/UTRAN/E-UTRAN/NG-RAN Access Point.

In terms of network implementation and deployment, a cell allows provisioning of resources to users in a predictable manner and allows scalable deployment of radio resources in the network. This includes acting as a proxy between UE and core network (e.g. for NAS protocol), and ensuring UE can be served in a predictable manner. Thus, a cell is a logical network entity that provides (physical and logical) radio resources to users in a scalable manner and anchors the UE to the operator network. The figure 1 below shows a crude illustration of the role of the cell as a “service provider” between UE and core network.

Figure 1. Illustration of the role of a cell

The cell acts as an anchor for the UE in many ways: Providing signals that allow detection of and operation within the cell (i.e. PSS/SSS/SSB/DMRS/TRS/CSI-RS), providing information on how to access the cell resources (i.e. MIB and SIBs), providing cell and operator identification information (i.e. PCI as part of PSS+SSS and PLMN identity in SIB1) and transmitting/receiving physical channels that users can use for communication (i.e. PxxCH, PBCH, PCH) in a specific bandwidth (i.e. as determined by the SIB1 configuration). Once the UE is served by a cell, the cell can also request access from the UE (via paging) or provide information on how the UE should manage which serving cell it uses (via reselection or handovers), and act as an anchor to ensuring radio security is rooted in the actual signals UE is receiving (i.e. PCI and DL cell frequency are used in security key generation). For this reason, 3GPP has always used the concept of “cell” to aggregate how UE can access the network resources, and allows grouping of all the disparate tasks under a single, logical concept. This allows network planning and dimensioning the hardware used by the network to fit the desired operational parameters (e.g. UE density in an area), as well as efficient management of the operator frequency resources (i.e. cell can indicate how much of the operator frequencies it uses).

Observation 18: The cell is a logical concept that allows network to manage its resources and dimension the required hardware to the desired service quality.

The cell has always been considered to be a set of contiguous bandwidth for practical purposes: Typically different RF components are required to transmit/receive on non-contiguous (or wide enough contiguous) spectrum blocks, which requires network to manage how those hardware resources are used and the concept of cell helps to break down this usage into manageable pieces. Even though a logical cell concept could be used to control non-contiguous spectrum blocks, it is difficult to see any practical benefit of this since the resources of each block would anyway be handled separately. Hence, it seems reasonable to continue to assume that a serving cell in 6GR continues to be the set of resources assigned to a contiguous spectrum allocation, managed by a centralized set of network hardware components. This is especially relevant since for any non-contiguous allocations, the network is typically not in charge of the spectrum between the non-contiguous chunks – if it was, there would be no reason to use non-contiguous allocation as a single (large) bandwidth would allow better performance for networks and for the UE. This is also illustrated in Figure 2 below.

Figure 2. Difference between contiguous and non-contiguous operator bandwidths

Proposal 5: Define serving cell in 6GR as a set of contiguous spectrum resources (i.e. reference signals and physical channels) managed by a network entity.

## 4.2	Multi carrier operation

For 6G, CA remains a critical mechanism for efficient spectrum utilization and achieving high data rates, building upon its proven track record in previous generations. However, to avoid the complexities, RAN2 must prioritize a streamlined approach to spectrum aggregation for finalizing the solutions and we would like to discuss the various aspects that impact the multi-carrier operations.

### 4.2.1	Deployment aspects

For co-located intra-band contiguous carriers, the timing and synchronization requirements might indeed be similar across the carriers. However, for inter-band scenarios and/or non-contiguous carriers and non-colocated deployments, there are restrictions coming from frequency separation, UE hardware capabilities/limitations, RF preparation time, which influence the synchronization requirements and switching/activation delays for additional carriers.

These deployments have been supported in 5G CA and expected to be supported in 6G CA.

Observation 19: Multiple cell deployments like co-located cells with intra-band contiguous carriers, co-located cells with inter-band carriers, non-colocated cells have been supported in 5G CA framework and likely supported in 6G CA.

### 4.2.2	Energy Saving aspects

From our perspective, when multiple carriers or frequency layers are available at a site, the network implementation dictates how these are utilized. A primary mechanism for energy savings, involves a single primary or 'anchor' carrier which is responsible for broadcasting essential information such as SSB, SIBs and Paging messages. Critically, during periods of low traffic or when carriers are not actively used, the network can cease transmissions on additional carriers which can provide network-side energy saving.

For active data transmission on multiple carriers, energy saving is possible at the network or UE only if the solution can optimize resource utilization, reference signal availability, reduce signaling overhead and simplify resource management, and improve spectral efficiency. Such enhancements should be considered for 6G CA.

In our view, many of the proposed benefits of MCSC, including energy savings and simplified resource management, can be effectively realized through enhancements to the existing CA framework, which is a more streamlined and efficient approach to multi-carrier operations in 6G.

Observation 20: Energy savings at network and UE in multi-carrier scenario is governed by optimal resource utilization, reference signal availability, reduced signaling overhead and simplified resource management and are relevant to 6G CA solution.

### 4.2.3	Flexible DL and UL pairing

One key scenario for flexible pairing of DL and UL carriers is to overcome coverage limitations on uplink. The mid-band carriers (e.g 7GHz) promise significantly larger bandwidth than carriers in lower bands, enabling higher data rates and supporting the increased capacity demands of 6G applications. However, the uplink coverage could often be weaker for the cell-edge UEs. Hence overlaying the mid-band carriers with the low-band (e.g 3.5 GHz) carriers could improve the UL coverage for cell-edge UEs. Also, in today’s networks it has been observed that significant amount of traffic during CA operation is in the primary cell. By flexibly pairing the mid-band downlink with low-band uplink, the network can simultaneously enhance UE’s uplink coverage and maximize downlink data throughput. Flexible pairing of DL and UL carriers on different bands will also enable single carrier operations rather than UE needing to receive and/or transmit on more than one carrier. Single carrier operations will provide significant UE power savings.

When pairing the mid-band DL with low-band UL, the UE must have a DL reference signal which the UE can rely for accurate synchronization and pathloss measurements to ensure proper timing alignment for all subsequent uplink physical layer procedures. This can be achieved by using flexible DL and UL pairing only in co-sited deployments. The inherent separation across carriers and differing propagation characteristics needs RAN4 studies and careful consideration of related deployment assumptions and constraints.

Observation 21: One scenario for flexible pairing of DL and UL is to overcome coverage limitations on uplink. The DL reference signal availability is crucial for synchronization and pathloss measurement. The multi-carrier solutions in 6G support these scenarios.

### 4.2.4	Fragmented spectrum utilization

Our understanding is that the concept of Multi-carrier Single Cell (MCSC) has been proposed to address the fragmented spectrum utilization, although the fundamental problem it aims to solve remains very unclear.

The efficient use of fragmented spectrum is intrinsically linked to the UE's RF front-end design and the physical layer challenges that was studied in the CA framework by RAN4 in R19 i.e. fragmented spectrum utilization with minimal UE hardware.

The discussions on MCSC in the past have often overlooked a detailed consideration of these RF implications. It is required to precisely capture what it means to configure multiple physical carriers as a “single serving cell” and, to clearly articulate the concrete benefits this approach brings to the UE particularly in terms of UE RF requirements. It is unclear how the MCSC addresses these issues of fragmented carrier utilization. We have observed that MCSC is proposed to deliver significant energy saving and simplify resource management – although it is not clear how MCSC provides those gains we believe these are important objectives for multi carrier operation and RAN2 should study how to achieve those.

Observation 22: MCSC seems to primarily target the efficient use of fragmented spectrum, which is linked to the UE's RF front-end design. The detailed considerations of these RF implications are often overlooked. How MCSC solution circumvents the fundamental physical layer challenges already present in CA is unclear.

Proposal 6: RAN2 carrier aggregation framework shall support multi-carrier scenarios, including flexible DL and UL pairing, and fragmented spectrum utilization while avoiding redundant/complex solutions for similar problems. RAN2 studies shall also ensure the fundamental physical layer and RF implications are considered when addressing these challenges.

## 4.3	Fast activation and measurements

In 5G, RAN4 RRM has focused on reducing carrier activation time through features like SSB-less SCell and A-TRS based activation. Release 19 further introduced on-demand SSB operations for SCell, aiming to enhance network energy efficiency and accelerate UE measurements. CSI reporting remains another critical area which was also explored in R19 for early CSI acquisition and CSI reporting.

To meet the demands of future high-bandwidth networks, 6G requires to support fast SCell activation mechanisms from the initial release and must prioritize rapidly preparing carriers for data transmission, either by significantly minimizing reliance on extensive UE measurements or by ensuring faster CSI acquisition.

Observation 23: RAN2 should study the mechanisms to be supported in 6G for fast SCell activation and fast CSI acquisition.

Most of the measurement requirements in 5G are based on the LTE measurement requirements. LTE Day-1 (Rel-8) did not support CA and it was designed as a single carrier system. I.e. UE could have one serving cell on one serving carrier. Other carriers were considered as non-serving carriers. In Rel-10 (LTE), 3GPP introduced the carrier aggregation feature. The introduction of carrier aggregation led to that the UE can have multiple serving carriers on which UE can have one serving cell on each serving carrier.

Figure 7: Illustration of having one main serving cell (PCell) and one or additional serving cells (SCell). Non-serving carriers are illustrated as well.

However, introduction of CA did not lead to changes related to how the network configures the UE with measurements or how the related UE measurement requirements were defined. To facilitate CA, network often needs UE assistance measurements. Such measurements would by nature be on-need basis due to the dependency on the instant amount of data that is to be transmitted. As the need for CA is data driven triggered, measurements for the purpose of facilitating CA, can be seen as being on-need basis. Hence, measurement results for enabling CA would need to be responsive – delivered to the network when needed.

While CA measurements are important in order to enable carrier aggregation and enhanced user experience, they are not essential for maintaining the radio link between the UE and network. The lack of measurements enabling CA can lead to a temporary reduced data rate or reduced user experience etc. but it will not cause link failures. Measurements for enabling CA can therefore be considered only needed when there is a need for carrier aggregation. While if this is not the case, such measurements for facilitating CA would not be needed.

Observation 24: Measurements for CA are essentially only needed when there is a need for configuring the UE with carrier aggregation.

In our understanding, RAN4 is going to study how to do the measurements in 6GR and RAN2 should work together with them. However, RAN2 should at the same time consider how to enable more on-demand - based measurements for CA while having a unified measurement framework (as discussed in R2-2508363).

Proposal 7: Based on RAN4 guidance, RAN2 to study how to use a unified measurement framework that allows measurements to facilitate configuration of CA on a need basis.

## 4.4	Summary of multi-carrier operation

Regarding carrier aggregation and multi-carrier operation we discussed what is serving cell going to be in 6G and we propose:

Proposal 5: Define serving cell in 6GR as a set of contiguous spectrum resources (i.e. reference signals and physical channels) managed by a network entity.

Then we discussed that RAN2 should focus on the key issues of multi-carrier operation and define clear objectives to be studied to prevent the introduction of redundant or complex solutions for similar challenges:

Proposal 6: RAN2 carrier aggregation framework shall support multi-carrier scenarios, including flexible DL and UL pairing, and fragmented spectrum utilization while avoiding redundant/complex solutions for similar problems. RAN2 studies shall also ensure the fundamental physical layer and RF implications are considered when addressing these challenges.

And finally, we discussed importance to enable fast and efficient utilization of multi-carriers:

Proposal 7: Based on RAN4 guidance, RAN2 to study how to use a unified measurement framework that allows on-demand measurements to facilitate configuration of CA.

# 5	References

- RP-251881, New SID: Study on 6G Radio, 3GPP TSG RAN Meeting #108

- RP-252912, Revised SID: Study on 6G Radio, 3GPP TSG RAN Meeting #109

- TS 38.300, 5G; NR; NR and NG-RAN Overall description

- R2-250xxxx, RAN2 Chair Notes, 3GPP TSG RAN2 Meeting #131bis

- TS 38.304, 5G; NR; User Equipment (UE) procedures in Idle mode and in RRC Inactive state