3GPP TSG-RAN WG2 Meeting #132		R2-2508252

Dallas, USA, 17th – 21st Nov., 2025

Source:	vivo

Title:	Discussion on open issues for R19 mobility

Agenda Item:	8.6.2

Document for:	Discussion and Decision

- Introduction

In this paper, we will discuss the remaining MAC open issues of LTM.

- Discussion

In RAN2#131bis meeting [1], the following agreements were made:

Agreements

-	(MAC-V03) If an ltm-CSI-ReportConfig or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig is modified or removed, RRC layer should trigger MAC layer to remove the measurement reporting entry, stop the periodical reporting timer, and reset the associated information (e.g. timeToTrigger) for this ltm-CSI-ReportConfigId.

Agreements

-	(MAC-V03) If an ltm-CSI-ReportConfig or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig is modified or removed, RRC layer should trigger MAC layer to remove the measurement reporting entry, stop the periodical reporting timer, and reset the associated information (e.g. timeToTrigger) for this ltm-CSI-ReportConfigId.

And it was captured in the MAC corrections CR [2] as the following:

5.35.1	Introduction

[skipped]

For L1 measurement and event triggered L1 measurement reporting, if an ltm-CSI-ReportConfig or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig is removed or modified from the current UE configuration, as specified in TS 38.331 [5], the MAC entity shall:

1>	remove the measurement reporting entry for the corresponding ltm-CSI-ReportConfigId from the MR_LIST, if included:

1>	stop the periodical reporting timer, if running, and reset the associated information (e.g. TTT and variables defined in 5.35.3) for the corresponding ltm-CSI-ReportConfigId.

5.35.1	Introduction

[skipped]

For L1 measurement and event triggered L1 measurement reporting, if an ltm-CSI-ReportConfig or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig is removed or modified from the current UE configuration, as specified in TS 38.331 [5], the MAC entity shall:

1>	remove the measurement reporting entry for the corresponding ltm-CSI-ReportConfigId from the MR_LIST, if included:

1>	stop the periodical reporting timer, if running, and reset the associated information (e.g. TTT and variables defined in 5.35.3) for the corresponding ltm-CSI-ReportConfigId.

However, one case is missing for the open issue [MAC-V03]. If there is a triggered L1 measurement report associated with a modified or removed ltm-CSI-ReportConfig, the triggered measurement report should also be canceled to avoid unnecessary reporting.

Proposal 1: [MAC-V03] If an ltm-CSI-ReportConfig or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig is removed from or modified in the current UE configuration, the MAC entity shall cancel the triggered L1 measurement report associated with the ltm-CSI-ReportConfig.

If proposal 1 is agreed, the following TP based on the latest MAC running CR [2] can be adopted.

Proposal 2: [MAC-V03] If proposal 1 is agreed, adop the TP in Annex for MAC specification.

- Conclusion

In this paper, we discuss the remaining MAC open issues for LTM. We have the following observations and proposals:

Proposal 1: [MAC-V03] If an ltm-CSI-ReportConfig or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig is removed from or modified in the current UE configuration, the MAC entity shall cancel the triggered L1 measurement report associated with the ltm-CSI-ReportConfig.

Proposal 2: [MAC-V03] If proposal 1 is agreed, adop the TP in Annex for MAC specification.

- References

- RAN2#131bis_ChairNotes.

- R2-2508251, Miscellaneous corrections on MAC for Mob Ph4, vivo.

Annex: TP for TS 38.321

5.35.1	Introduction

[skipped]

For L1 measurement and event triggered L1 measurement reporting, if an ltm-CSI-ReportConfig or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig is removed or modified from the current UE configuration, as specified in TS 38.331 [5], the MAC entity shall:

1>	remove the measurement reporting entry for the corresponding ltm-CSI-ReportConfigId from the MR_LIST, if included:

1>	stop the periodical reporting timer, if running, and reset the associated information (e.g. TTT and variables defined in 5.35.3) for the corresponding ltm-CSI-ReportConfigId.

- cancel, if any, triggered Event Triggered L1 Measurement Report for the corresponding ltm-CSI-

ReportConfigId.

5.35.1	Introduction

[skipped]

For L1 measurement and event triggered L1 measurement reporting, if an ltm-CSI-ReportConfig or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig is removed or modified from the current UE configuration, as specified in TS 38.331 [5], the MAC entity shall:

1>	remove the measurement reporting entry for the corresponding ltm-CSI-ReportConfigId from the MR_LIST, if included:

1>	stop the periodical reporting timer, if running, and reset the associated information (e.g. TTT and variables defined in 5.35.3) for the corresponding ltm-CSI-ReportConfigId.

- cancel, if any, triggered Event Triggered L1 Measurement Report for the corresponding ltm-CSI-

ReportConfigId.