3GPP TSG-RAN WG2 Meeting #132		R2-2508254

Dallas, USA, 17th – 21st Nov., 2025

Source:	vivo

Title:	Discussion on open issues for R19 XR

Agenda Item:	8.7.2

Document for:	Discussion and Decision

- Introduction

In the last RAN2 meeting [1], RLC-H02 were discussed and the following agreements were made:

- (RLC-H02) Confirm that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. Companies are invited to bring contributions on the solutions to address it.

In this paper, we will discuss the remaining issue RLC-H02 and the window stalling issue.

- Discussion

  - [RLC-H02] How to construct a STATUS PDU when RX_Next > RX_Highest_Status

In the last meeting, RAN2 confirmed that when t-RxDiscard expires, RX_Next may be greater than or equal to RX_Highest_Status. However, whether any specification change is needed requires further discussion. The current RLC specification [2] for constructing a STATUS PDU is described as follows:

### 5.3.4	Status reporting

[…]

When constructing a STATUS PDU, the AM RLC entity shall:

-	for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer:

-	for an RLC SDU for which no byte segments have been received yet:

-	include in the STATUS PDU a NACK_SN which is set to the SN of the RLC SDU.

-	for a continuous sequence of byte segments of a partly received RLC SDU that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN, SOstart and SOend.

-	for a continuous sequence of RLC SDUs that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN and NACK range;

-	include in the STATUS PDU, if required, a pair of SOstart and SOend.

-	set the ACK_SN to the SN of the next not received RLC SDU which is not indicated as missing and is not discarded due to the expiry of t-RxDiscard in the resulting STATUS PDU.

In our view, the current RLC specification works well for both the STATUS reports triggered by t-Reassembly expiry or t-RxDiscard expiry. Taking Huawei’s contribution [3] as an example:

In the example, when t-RxDiscard expires at step 7, RX_Next = 11 and RX_Highest_Status = 9, hence RX_Next > RX_Highest_Status. According to the current specification, no RLC SDU satisfies the condition RX_Next <= SN < RX_Highest_Status because RX_Next exceeds RX_Highest_Status. Therefore, no NACK_SN is set in the STATUS PDU, and the yellow-highlighted part is skipped in this case.

For ACK_SN, since all not-received RLC SDUs with SN < RX_Next_Discard_Trigger (11) are considered obsolete and discarded, ACK_SN is set to 11, as indicated by the green-highlighted part.

[RLC-H02] The current RLC specification works well for STATUS reports triggered by t-RxDiscard expiry.

Therefore, no changes to the current specification are necessary.

- [RLC-H02] Keep clause 5.3.4 of the current specification unchanged for issue RLC-H02.

  - Window stalling issue

In RAN2#130, the following agreements were made regarding window stalling issue with polling retransmission:

RAN2#130 Agreements for RLC-11

- Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting)

- Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work

And in RAN2#131, RAN2 further confirmed that there is no need to poll or retransmit the poll in this case:

RAN2#131 Agreement for RLC-11

- [RLC-11] In case “there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with”, RAN2 confirm no need to poll or retransmit the poll for any acknowledgement (no spec change).

However, how to resolve the window stalling issue when only SDUs whose transmissions have been stopped remain in the buffer has not been fully addressed.

In our view, window stalling is already rare; combined with the condition that the buffer contains only stopped SDUs, the likelihood is even lower and this is a corner case. If it does occur, we can rely on network implementation to address it. For example, the network can trigger an RLC re-establishment for the corresponding DRB to resolve the window stalling.

- Regarding the window stalling issue when only SDUs with stopped (re)transmissions are buffered, it is up to network implementation to resolve it.

- Conclusion

In this contribution, we discuss the remaining open issues for the UP enhancement in relation to RLC for R19 XR, and we have the following observations and proposals:

Regarding RLC-H02

- [RLC-H02] The current RLC specification works well for STATUS reports triggered by t-RxDiscard expiry.

- [RLC-H02] Keep clause 5.3.4 of the current specification unchanged.

Window stalling issue

- Regarding the window stalling issue when only SDUs with stopped (re)transmissions are buffered, it is up to network implementation to resolve it.

- References

- RAN2#131bis_chair_notes.

- 3GPP TS 38.322, Radio Link Control (RLC) protocol specification, Release 19.

- R2-2507903, Discussion on the RLC STATUS PDU at t-RxDiscard expiry, Huawei, HiSilicon.