3GPP TSG-RAN WG2 Meeting #132	R2-2508295

Dallas, US, November 17th – 21st, 2025

Agenda Item:	10.2.2

Source: 	OPPO

Title:  	Discussion on TN and NTN integration

Document for:	Discussion and Decision

Introduction

In RAN2#131bis, the 6G NTN was discussed and the following agreements related to TN/NTN is achieved as below:

For next meeting companies should identify essential requirements for NTN and identify which functionality should be considered in the common design with TN and what are NTN specific and whether they are essential.

In this document, we provide the analysis on the basic NTN assumption and NTN features introduced in 5G and then provide our views on which NTN features are essential and which features can be harmonized design.

Discussion

  - NTN scenario

In NR NTN, it is supported various satellites with different orbital altitudes, including GEO, MEO, and LEO, where GEO may provide Earth fixed cell, MEO and LEO may provide Quasi-Earth fixed cell or Earth moving cell. Regarding the NTN payload type, in NR NTN, transparent payload has been supported since Rel-17, while regenerative payload was introduced in Rel-19. In 6G NTN, we think these basic NTN deployment scenarios and payload type should be supported in 6G NTN.

For GNSS capability, in Rel-17 NR NTN, GNSS capability is essential for NTN access since UEs should support uplink time and frequency pre-compensation based on their GNSS-acquired position. The dependency on the GNSS system degrades the resilience of NTN. In order to increase the robustness of NTN operation, a new SID on GNSS resilient NR-NTN operation has been approved in Rel-20, aiming to investigate enhancements to mitigate the impacts of potential GNSS unavailability. Considering the GNSS service maybe not available, in 6G NTN, UE without GNSS capability also should be considered.

- The basic assumption in 5G NTN, e.g., NTN scenarios, payload type, etc., can be adopted in 6G NTN and UE without GNSS capability can also be studied in 6G NTN.

  - Category of features for NTN

In 5G NTN, we have sufficient discussion and introduce multiple features for NTN considering the long propagation delay and satellite moving characteristics in NTN scenario. In our understanding, these features can be categorized into 3 types as below:

- Common essential features: those essential features are common for both TN and NTN. Neither TN nor NTN can work or the NTN performance will be impacted seriously without any feature in this feature category.

- NTN specific essential features: those essential features are specific for NTN. NTN can NOT work or the NTN performance will be impacted seriously without any feature in this feature category. However, TN may work without the feature in this feature category.

- NTN specific optional features: those optional features which are specific for NTN, NTN can work even without the feature in this feature category. The features in this feature categories can improve the performance of NTN, or may introduce new functionalities to NTN.

Therefore, we can identify which feature is belongs to which feature category firstly and then decide how to handle the features in each category in 6G NTN discussion. Based on the above defined three feature categories, it is benefit to process the discussion of TN and NTN integration.

- Define 3 types of features for NTN/TN feature discussions: common essential features, NTN specific essential features, and NTN specific optional features.

  - Features in each feature category

- Common essential features:

HARQ operation:

As a fundamental function, HARQ should be supported in 6G for both TN and NTN. In order to accommodate the much longer propagation delay in NTN compared to that in TN, in Rel-17 NR NTN, HARQ has been enhanced by 1) increase the maximum HARQ process number, and 2) support disabling HARQ feedback for DL and introduce two HARQ modes, i.e., HARQ mode A/B for UL. With these HARQ enhancements, HARQ stalling issue can be effectively avoided. In our view, this should be an essential feature in 6G NTN. To achieve a unified design for TN and NTN, both HARQ process number and HARQ modes should be natively supported in the HARQ design, which can be configurable as the same way in NR.

In NR NTN, HARQ operation is enhanced by increasing HARQ process number and introducing disabling HARQ feedback for DL and two HARQ modes for UL, which can effectively avoid the HARQ stalling issue.

L2 timer:

Another enhancement in NR NTN is related to L2 timer handling, for which the large propagation delay in NTN should be considered. For some MAC timers such as ra-ResponseWindow and ra-ContentionResolutionTimer, the start time of these timers is prolonged in NTN for which UE-gNB RTT should be taken into account. While for some other timers, the timer length is extended by introducing larger timer value (e.g. t-Reassembly, discardTimer) or by adding a UE-gNB RTT to the configured value (e.g., drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL). In 6G, L2 timer design should consider both TN and NTN. To achieve a unified design, timers’ value range should be able to cover both TN and NTN, and timers start point can be adapted to TN and NTN.

In NR NTN, L2 timers are enhanced by extending the timer range or postpone the start time point of timer in order to accommodate the much larger propagation delay in NTN compared to TN.

SMTC:

SMTC and measurement gap are used by UE to perform RRM measurement. The SMTC is typically configured by network based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms. In TN, this gNB-UE propagation delay difference can be negligible, thus gNB may provide the SMTC configuration based on the neighbour cell SSB period and SSB transmission timing difference between neighbour cell gNB and serving cell gNB. However, in NTN, gNB-UE propagation delay difference between the serving cell and neighbour cells should be taken into account to determine the SMTC timing. In NR NTN, SMTC have been enhanced for both RRC idle state UEs and RRC connected state UEs by introducing multiple SMTC configuration per frequency and multiple measurement gap for the UE. For RRC idle state, the UE may adjust the SMTC timing based on the actual gNB-UE propagation delay difference between the serving cell and neighbour cells. While for RRC connected state, as UE and gNB should have a common understanding on the timing for SMTC/measurement gap, PDD reporting is introduced to assist network to provide the SMTC/measurement gap configuration. In Rel-19, the default SSB periocidity can be extended to 160ms to support the beam hopping and it is allowed to configure at most 7 SMTC per frequency to UE. In 6G, SMTC and measurement gap are essential for both TN and NTN, so 6G should natively support multiple SMTC and measurement gap as well as UE autonomous SMTC timing adjustment, aiming to align the timing between SSB transmission from network and SSB measurement from UE.

In NR NTN, SMTC and measurement gap are enhancement by introducing multiple configuration and UE autonomous SMTC timing adjustment, aiming to align the timing between SSB transmission from network and SSB measurement from UE.

Measurement initiation and relaxation:

In NR, neighbour cell measurement initiation and relaxation are supported to reduce UE RRM measurement on neighbour cell when the UE is in the cell center and/or in low mobility, so that UE power consumption can be saved. In TN, measurement results is used to determine whether it is in the cell center and/or in low mobility. In NR NTN, due to the unclear near-far effect, we introduced location based neighbour cell measurement triggering on top of RSRP/RSRQ based triggering. Besides, for NTN-TN mobility, we also introduce location based measurement for TN neighbour cell. In addition, considering that the satellite motion is predictable, we also introduce time based neighbour cell measurement triggering. Due to the time limit, measurement relaxation has not been enhanced in NR NTN.

As a useful UE energy saving feature, we think both measurement initiation and relaxation should be supported in 6G Day-1, for both TN and NTN. And all of measurement based, time based and location based triggering should be supported.

Mobility in CONNECTED state:

In 6G, in order to achieve ubiquitous connectivity, all mobility scenarios including TN-TN mobility, TN-NTN mobility and NTN-NTN mobility should be supported in Day-1. 6G should target for a common mobility framework between TN and NTN as much as possible. So all the mobility features, such as CHO, (C-)LTM, RACH-less HO defined for TN and group based handover for NTN can be considered in 6G NTN.

Similar for measurement initiation, in NR NTN, for connected state mobility, we introduce location based measurement triggering, as well as location based and time based CHO triggering event. Therefore, all of measurement based, time based and location based mobility should be supported.

Location based and time based measurement trigger and mobility are useful for NTN.

TAC:

In NR NTN, in order to reduce the signalling load at cell edge, in particular for Earth moving cell, network may broadcast multiple TACs per PLMN in an NTN cell, so as to support soft TAU. We think this is an essential NTN enhancement. With a harmonized design in 6G, it is proposed to natively support multiple TACs.

Supporting of multiple TACs per PLMN in one cell can reduce the signalling load at cell edge for Earth moving cell, which is essential for NTN.

UL capacity enhancement:

In IoT NTN and NR NTN, the following UL capacity enhancements are introduced:

- CB Msg3 EDT for IoT NTN

- OCC for PUSCH for both IoT NTN and NR NTN

For the former, it allows EDT using contention based PUSCH without Msg1 transmission and Msg2 reception. The signaling reduction may improve UL capability. For the latter, PUSCH capacity is increased by multiplexing of UEs by usage of OCC.

In 6G, one of the use cases is massive IoT devices, therefore, UL capacity enhancement should be considered for both TN and NTN. From RAN2 perspective, we can study CB-Msg3 for RRC_idle state UE for UL capacity enhancement by taking CB-Msg3-EDT introduced in R19 IoT NTN as baseline. This feature can also be applied to RRC connected state UEs, e.g. introducing CB-CG resources to increase UL capacity as well as to reduce BSR/DSR latency for fast UL transmission. More details can be found in our document [1]. Regarding OCC, we can rely on RAN1 to discuss.

CB UL-SCH, e.g., CB-Msg3 or CB-CG can increase UL capacity for both TN and NTN.

Base on the above analysis, we think the above features can be considered as common essential features for TN and NTN in 6G.

- In 6G NTN, RAN2 consider the following features as common essential features:

- HARQ, i.e., natively support large HARQ process number, support disabling HARQ feedback for DL and HARQ mode A/B for UL;

- L2 timer handling, i.e., the timer value range should cover both TN and NTN, adaptive UP timer start time based on TN and NTN;

- MG/SMTC, i.e., natively support multiple MG/SMTC configuration and SMTC timing adjustment;

- Measurement initiation and relaxation, e.g. support RSRP based, location based and time based triggering;

- Mobility, e.g., support RSRP based, location based and time based mobility, support RACH-less HO;

- TAC update, e.g., support multiple TACs per PLMN in one cell;

- UL capacity enhancement, e.g., CB Msg3 for RRC idle UEs and CB-CG for RRC connected state UEs.

- NTN specific essential feature:

NTN specific SIB acquisition:

In 5G NTN, SIB19 was introduced to broadcast the satellite related information, including, Ephemeris, common TA parameters, validity duration for UL sync information, t-Service, cell reference location, Epoch time, etc. The NTN UE is necessary to acquire the NTN specific SIB and get the satellite related information to perform the related NTN procedure. In 6G, the NTN specific SIB acquisition is also necessary to NTN UE.

NTN specific SIB acquisition is necessary to NTN UE.

Uplink time and frequency pre-compensation:

In 5G NTN, the transparent payload and regenerative payload are both supported. Take the transparent payload as example, the gNB is located on the ground and a satellite relay signalling between gNB and UE. The propagation delay between UE and gNB is comprised with feeder link delay (between payload and gNB) and service link delay (between UE and satellite). Therefore, before UE transmit the UL signalling, UE should calculate the time and frequency pre-compensation based on the UE location and the broadcasted satellite information. In 6G NTN, the Uplink time and frequency pre-compensation is necessary to NTN UE with GNSS capability or UEs without GNSS capability.

Uplink time and frequency pre-compensation is essential to NTN UE due to large propagation delay and satellite moving in NTN.

TA reporting:

In 5G NTN, considering the TA pre-compensation between UE and reference location is calculated by UE based on UE location and broadcast satellite information, the UE specific TA can be reported to assist uplink scheduling. Therefore, TA reporting was supported during RACH and CONNECTED state in NTN. In 6G NTN, TA reporting is important to assist the NW to adjust the TA accurately for uplink scheduling. Therefore, it is necessary to be supported.

TA reporting is necessary to NTN UE for assisting the NW the adjust the TA.

Satellite switch with re-sync:

In 5G NTN, in case of quasi-earth fixed of transparent payload, after the satellite switching, the serving gNB may be not changed, the cell configuration can be kept without changing. So the satellite switch with unchanged PCI is supported. UE can perform the hard/soft satellite switch with re-sync with target satellite and the NW do not need to send the handover command. Therefore, the hard/soft satellite switch with re-sync can reduce the interruption time of satellite change via avoiding the L3 HO procedure. In 6G NTN, the quasi-earth fixed with transparent payload is an important deployment scenario, so the hard/soft satellite switch with re-sync is necessary to be supported.

Satellite switch with re-sync is important to NTN UE for reducing the interruption time of satellite change.

Discontinuous Coverage:

In 5G IoT NTN, in a sparse satellite constellation deployment, a UE located in the concerned geographical area may experience a situation of discontinuous coverage. To enable the UE in RRC_IDLE, to save power during periods of no coverage, the network provides NTN payload assistance information (e.g. ephemeris parameters, the start-time and/or carrier frequency of upcoming NTN payload coverage) to enable the UE to predict when coverage will be provided by upcoming NTN payloads. In 6G, at the initial NTN deployment period, the discontinuous coverage is also important to be supported for UE power saving.

In case of satellite constellation deployment, the discontinuous coverage is important to NTN UE for power saving.

Beam hopping:

In 5G NTN, to improve the coverage of NTN, the beam hopping was supported. Only a limited number of satellite beam can be active simultaneously in satellite to improve the coverage performance. But due to the legacy UE also can access this NTN cell, we only extended default SSB periodicity to 160ms and make the corresponding enhancement on SMTC. Considering the beam hopping can significantly improve the coverage, it is very important to be supported and can be enhanced further in 6G NTN.

Beam hopping is important to NTN to improve the coverage and can be further enhanced in 6G NTN.

- In 6G NTN, RAN2 considers the following features as NTN specific essential features:

- NTN specific SIB acquisition;

- Uplink time and frequency pre-compensation;

- TA reporting;

- Satellite switch with re-sync;

- Discontinuous coverage;

- Beam hopping;

- NTN specific optional features

In 5G NTN, except for above NTN essential functionalities, we also support the MBS broadcast with intended service area to support the MBS broadcast service in NTN. And geo-fencing for PWS service in NTN was also supported to enable providing the PWS service in the intended area. Besides, some features were discussed in 5G NTN but not supported, such as, dual connection between satellites, Carrier aggregation in NTN, etc. In 6G, these optional features are not urgent to be supported and can be considered in later release.

- In 6G NTN, the following features belong to specific optional features which can be considered later:

- MBS broadcast and multicast via NTN;

- PWS (Public Warning System) services over an intended area;

- Dual connection;

- Carrier Aggregation.

  - How to handle the feature category

In this section, we discuss how to handle the three feature categories in 6G NTN:

For the common essential features, considering these features are essential to both TN and NTN and  we can consider the harmonized design for these features considering both TN and NTN characteristics since 6G day1. This can avoid introducing the dedicated design for each NTN feature in later release. We can discuss features in this feature category with high priority.

For the NTN specific essential features, due to these features are essential for NTN performance, we can also discuss features in this feature category with high priority.  We need to consider whether these features can be reused directly or need some enhancement in 6G. And whether these features need to be supported from 6G day1 or can be supported from 6G day2 to achieve better NTN performance can be discussed.

For the NTN specific optional features, these features are less important and not so urgent to be supported from 6G day1. We can discuss these features in 6G later release with low priority.

- In 6G NTN, RAN2 can handle the defined three NTN feature categories with the follow principle:

- for common essential features, consider the harmonized design for TN and NTN since day1;

- for NTN specific essential features, can be discussed whether support them since day1;

- for NTN specific optional features, can be discussed in later release.

Conclusion

Based on the discussion in section2, we have following observations and proposals:

- In NR NTN, HARQ operation is enhanced by increasing HARQ process number and introducing disabling HARQ feedback for DL and two HARQ modes for UL, which can effectively avoid the HARQ stalling issue.

In NR NTN, L2 timers are enhanced by extending the timer range or postpone the start time point of timer in order to accommodate the much larger propagation delay in NTN compared to TN.

In NR NTN, SMTC and measurement gap are enhancement by introducing multiple configuration and UE autonomous SMTC timing adjustment, aiming to align the timing between SSB transmission from network and SSB measurement from UE.

Location based and time based measurement trigger and mobility are useful for NTN.

Supporting of multiple TAC per PLMN in one cell can reduce the signalling load at cell edge for Earth moving cell, which is essential for NTN.

CB UL-SCH, e.g. CB-Msg3 or CB-CG can increase UL capacity for both TN and NTN.

NTN specific SIB acquisition is necessary to NTN UE.

Uplink time and frequency pre-compensation is essential to NTN UE due to large propagation delay and satellite moving in NTN.

TA reporting is necessary to NTN UE for assisting the NW the adjust the TA.

Satellite switch with re-sync is necessary to NTN UE for reducing the interruption time of satellite change.

In case of satellite constellation deployment, the discontinuous coverage is important to NTN UE for power saving.

Beam hopping is necessary to NTN to improve the coverage and can be further enhanced in 6G NTN.

- The basic assumption in 5G NTN, e.g., NTN scenarios, payload type, etc., can be adopted in 6G NTN and UE without GNSS capability can also be studied in 6G NTN.

- Define 3 types of features for NTN/TN feature discussions: common essential features, NTN specific essential features and NTN specific optional features.

- In 6G NTN, RAN2 consider the following features as common essential features:

    - HARQ, i.e., natively support large HARQ process number, support disabling HARQ feedback for DL and HARQ mode A/B for UL;

    - L2 timer handling, i.e., the timer value range should cover both TN and NTN, adaptive UP timer start time based on TN and NTN;

    - MG/SMTC, i.e., natively support multiple MG/SMTC configuration and SMTC timing adjustment;

    - Neighbour cell measurement initiation and relaxation, e.g., support RSRP based, location based and time based triggering;

    - Mobility, e.g., support RSRP based, location based and time based mobility, support RACH-less HO;

    - TAC. e.g., support multiple TACs per PLMN in one cell;

    - UL capacity enhancement, e.g., CB Msg3 for RRC idle UEs and CB-CG for RRC connected state UEs.

- In 6G NTN, RAN2 considers the following features as NTN specific essential features:

    - NTN specific SIB acquisition;

    - Uplink time and frequency pre-compensation;

    - TA reporting;

    - Satellite switch with re-sync;

    - Discontinuous coverage;

    - Beam hopping.

- In 6G NTN, the following features belong to specific optional features which can be considered later:

    - MBS broadcast and multicast via NTN;

    - PWS (Public Warning System) services over an intended area;

    - Dual connection;

    - Carrier Aggregation.

- In 6G NTN, RAN2 can handle the defined three NTN feature categories with the follow principle:

- for common essential features, consider the harmonized design for TN and NTN since day1;

- for NTN specific essential features, can be discussed whether support them since day1;

- for NTN specific optional features, can be discussed in later release.

Reference

[1] R2-2508107, Discussion on 6G scheduling and (re-)transmission schemes