3GPP TSG-RAN WG2 Meeting #132	R2-2508315

Dallas, USA, 17 – 21 November 2025

Agenda item:	8.9.1

Source:	Nokia, Nokia Shanghai Bell

Title:	MPDCCH narrowband selection for CB-Msg3-EDT

WID/SID:	IoT_NTN_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This document is to address the issue raised by RAN1 in LS R2-2508014 (R1-2508114) [2] regarding the support of MPDCCH narrowband configuration for CB-Msg3 EDT.

# 2	Discussion

RAN2 agreed to define a set of two MPDCCH narrowbands for MPDCCH monitoring for CB-Msg4. It was captured the configuration in RRC specification as below. The intention is to follow the legacy specification of Msg2/Msg4 monitoring where two MPDCCH narrowbands are available based on SIB configuration. The two narrowbands are available in SIB to handle a potential high-capacity need if many UEs utilize the RACH procedure at the same time. Similarly, for CB-Msg3-EDT, the network will also not be able to know how many UEs will attempt to use the resources, since the configuration is shared via SIB as well. Therefore, RAN2 agreed the CB-Msg3 shall follow the legacy approach of Msg2/Msg4.

RAN2 agreements: - We configure 2 MPDCCH narrowbands as a set (inform RAN1)

MPDCCH configuration for CB-Msg3 EDT in TS36.331: … CB-Msg3-MPDCCH-Config-r19 ::= SEQUENCE { mpdcch-Narrowband-r19 SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13), mpdcch-PRB-PairsConfig-r19 SEQUENCE{ numberPRB-Pairs-r19 ENUMERATED {n2, n4, n6, spare1}, resourceBlockAssignment-r19 BIT STRING (SIZE(4)) }, mpdcch-NumRepetition-r19 ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256}, mpdcch-StartSF-CSS-r19 ENUMERATED {v1, v1dot5, v2, v2dot5, v4, v5, v8, v10}, mpdcch-Offset-CSS-r19 ENUMERATED {zero, oneEighth, oneQuarter, threeEighth, oneHalf, fiveEighth, threeQuarter, sevenEighth} }

In the LS[1], RAN2 asks RAN1 to check their specification impact to support 2 MPDCCH narrowbands for CB-Msg4 monitoring, and update the corresponding RAN1 specification if needed. In the LS response [2], RAN1 agreed there are no concerns with supporting two MPDCCH narrowbands. However, since the UE monitors only one MPDCCH narrowband at a time, RAN2 is asked to decide on a mechanism to ensure the UE is monitoring a unique narrowband and thus also ensure that the network is able to deterministically determine which narrowband the UE monitors.

RAN1 LS response for support MPDCCH narrowband configuration for CB-Msg3-EDT: - From RAN1 perspective, introducing 2 MPDCCH narrowbands requires additional RAN1 standard effort. RAN1 would like to inform RAN2 that there are no concerns with supporting two MPDCCH narrowbands, but RAN2 should decide on a mechanism to ensure the UE is monitoring a unique narrowband.

Observation 1: RAN1 has no concerns with the agreement to support two MPDCCH narrowbands for CB-Msg4 monitoring, as agreed by RAN2.

Observation 2: RAN2 is expected to decide on a mechanism to ensure that the UE monitors a unique narrowband.

In the legacy specification, the UE selects among the 2 MPDCCH narrowbands for RAR monitoring based on the preamble index the UE applied for the preamble transmission. Specifically, TS 36.213[4] defines the following:

## 6.2 Random Access Response Grant The higher layers indicate the Nr-bit UL Grant to the physical layer, as defined in 3GPP TS 36.321 [8]. This is referred to the Random Access Response Grant in the physical layer. … - given in Table 6.2-A, Table 6.2-B and Table 6.2-G is the narrow band used for first subframe of MPDCCH for Random Access Response and is determined by higher layer parameter mpdcch-NarrowbandsToMonitor-r13 if only one narrowband is configured, otherwise, it is determined by Table 6-2-E. Table 6.2-E: Narrowband () for MPDCCH RAR. Mapped Preamble Index. mod( Preamble Index, 2 )=0; First narrowband configured by high layer parameter mpdcch-NarrowbandsToMonitor-r13. mod( Preamble Index, 2 )=1; Second narrowband configured by high layer parameter mpdcch-NarrowbandsToMonitor-r13.

Since the CB-Msg3 procedure does not have a preamble index, this raises the question of how the UE can determine which of the two MPDCCH narrow bands to monitor. The most straightforward method is to replace the ’preamble index’ with another identifier and reuse the same formula as table 6.2-E above. In other words, the MPDCCH narrowband selection for CB-Msg4 monitoring is based on the value of (new identifier mod 2).

Proposal 1: For the CB-Msg3 procedure, a new identifier other than preamble index should be used to determine the MPDCCH narrowband for CB-Msg4 monitoring.

Accoring to the RAN1 discussions as captured in moderator summary [5], several identifiers have been proposed, but no final agreement reached. The proposed identifiers include CB-Msg3 resource index (e.g. provided by prb-AllocationInfoSet), UE ID (e.g., IMSI), Contention Resolution ID (i.e., CR ID) or a physical layer ID of CB-Msg3.

Firstly, among the proposed identifiers, we understand the CB-Msg3 resource index (e.g., prb-AllocationInfoSet) is not suitable. This is because CB-msg3-EDT allows the UE to transmit multiple PUSCH/replicas in the same CB-msg3-EDT transmission window. These multiple transmissions may use different CB-Msg3 resource indices and therefore result in different narrowbands to be monitored during the CB-Msg4 monitor window, which the UE cannot do.

Observation 3: The CB-Msg3 resource index is not suitable for determining the MPDCCH narrowband for CB-Msg4 monitoring, because multiple transmissions/replicas within the same CB-Msg3 window may result in different narrowbands that the UE would need to monitor, which does not satisfy RAN1 requirement.

Secondly, for the proposal to use UE ID (e.g., IMSI), we understand the identifier should be included in the RRC message for CB-Msg3 to ensure the eNB can determine the monitored MPDCCH narrowband accordingly. In other words, the proposed UE ID should be included in RRCEarlyDataRequest message for CP EDT and RRCConnectionResumeRequest message for UP-EDT. As illustrated below, the UE ID in CP EDT and UP EDT are different. For CP solution, the UE ID is the 40 bits S-TMSI, while for UP solution, it is the 40 or 24 bits resumeIdentity.

RRCEarlyDataRequest message -- ASN1START RRCEarlyDataRequest-r15 ::= SEQUENCE { criticalExtensions CHOICE { rrcEarlyDataRequest-r15 RRCEarlyDataRequest-r15-IEs, criticalExtensionsFuture CHOICE { rrcEarlyDataRequest-5GC-r16 RRCEarlyDataRequest-5GC-r16-IEs, criticalExtensionsFuture-r16 SEQUENCE {} } } } RRCEarlyDataRequest-r15-IEs ::= SEQUENCE { s-TMSI-r15 S-TMSI, establishmentCause-r15 ENUMERATED {mo-Data, delayTolerantAccess}, dedicatedInfoNAS-r15 DedicatedInfoNAS, nonCriticalExtension RRCEarlyDataRequest-v1590-IEs OPTIONAL } RRCEarlyDataRequest-v1590-IEs ::= SEQUENCE { lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension RRCEarlyDataRequest-v1610-IEs OPTIONAL } << omited part >> -- ASN1STOP -- ASN1START S-TMSI ::= SEQUENCE { mmec MMEC, m-TMSI BIT STRING (SIZE (32)) } -- ASN1STOP S-TMSI field descriptions. m-TMSI The first/leftmost bit of the bit string contains the most significant bit of the M-TMSI.. RRCConnectionResumeRequest message -- ASN1START RRCConnectionResumeRequest-r13 ::= SEQUENCE { criticalExtensions CHOICE { rrcConnectionResumeRequest-r13 RRCConnectionResumeRequest-r13-IEs, rrcConnectionResumeRequest-r15 RRCConnectionResumeRequest-5GC-r15-IEs } } RRCConnectionResumeRequest-r13-IEs ::= SEQUENCE { resumeIdentity-r13 CHOICE { resumeID-r13 ResumeIdentity-r13, truncatedResumeID-r13 BIT STRING (SIZE (24)) }, shortResumeMAC-I-r13 BIT STRING (SIZE (16)), resumeCause-r13 ResumeCause, spare BIT STRING (SIZE (1)) } << omited part >> -- ASN1STOP -- ASN1START ResumeIdentity-r13 ::= BIT STRING (SIZE(40)) -- ASN1STOP

Observation 4: If the UE ID is used to determine the MPDCCH narrowband for CB-Msg4 monitoring, it may be 40-bits S-TMSI in the CP solution and the 24-bits or 40-bits resumeIdentity in the UP solution.

Thirdly, for the proposal to use Contention Resolution ID (e.g., CR ID), the 48-bits, which the CR ID MAC CE is based on, shall always be included in the CB-Msg3 for contention resolution in both CP and UP solution. Furthermore, the 48-bits will not be changed across the multiple transmissions/replicas within the same CB-Msg3 window. Therefore, we understand it can be used to determine the MPDCCH narrowband for CB-Msg4 monitoring.

#### 6.1.3.4 UE Contention Resolution Identity MAC Control Element The UE Contention Resolution Identity MAC control element is identified by MAC PDU subheader with LCID as specified in table 6.2.1-1. This control element has a fixed 48-bit size and consists of a single field defined as follows (figure 6.1.3.4-1) - UE Contention Resolution Identity: If this MAC control element is included in response to an uplink CCCH transmission, then this field contains the uplink CCCH SDU if the uplink CCCH SDU is 48 bits long. If the CCCH SDU is longer than 48 bits, this field contains the first 48 bits of the uplink CCCH SDU. If this MAC control element is included in response to an uplink DCCH transmission (i.e. the MAC entity is configured with rach-Skip or rach-SkipSCG), then the MAC entity shall ignore the contents of this field. Figure 6.1.3.4-1: UE Contention Resolution Identity MAC control element

Observation 5: Since the Contention Resolution ID is always included in CB-Msg3 and remains unchanged across the multiple transmissions/replicas within the same CB-Msg3 window, it can be used to determine the MPDCCH narrowband for CB-Msg4 monitoring in both the CP and UP solutions.

Lastly, regarding the proposal to use a physical layer ID of CB-Msg3, we understand this is also feasible, provided that the identifier is available in CB-Msg3 and does not change across the replicas, known to the network, such that there is no ambiguity in terms of the UE’s and network’s understanding of the monitored MPDCCH narrowband. However, RAN1 did not provide the exact identifier in the moderator summary [5]. From RAN2 point of view, it may be difficult to conclude which PHY identifier is unique to a UE across multiple PUSCH transmissions/replicas within the CB-Msg3 transmission window.

Observation 6: It is feasible to use a physical layer ID of CB-Msg3 to determine the MPDCCH narrowband for CB-Msg4 monitoring. However, there is no exact PHY ID proposed from RAN1.

Proposal 2: The following three options can be considered for the new identifier used to determine the MPDCCH narrowband for CB-Msg4 monitoring:

- Option 1: The UE ID included in CB-Msg3 RRC messages. i.e., the 40-bits S-TMSI in the CP solution, or the 24-bits or 40-bits resumeIdentity in the UP solution.

- Option 2: The Contention Resolution ID in MAC layer.

- Option 3: A physical layer ID associated with CB-Msg3, while the exact ID is to be determined by RAN1.

In our understanding, Option 3 is not a good way-forward since RAN1 has asked RAN2 to decide on a mechanism to ensure the UE is monitoring a unique narrowband in LS [2] as a response to our request in LS [1]. For the other options,  both Option 1 and Option 2 are feasible but we slightly prefer Option 2. The reason is the Option 1 has different handling for CP and UP solution which may bring more but unnecessary impacts to the specifications and UE implementation.

Furthermore, since legacy MPDCCH narrowband selection for RAR is specified in TS36.213 and performed in PHY layer, we think the MPDCCH narrowband selection for CB-Msg4 monitoring should also be performed in the PHY layer to maintain consistency in both specification and UE behaviour. RAN2 may need to inform RAN1 to implement the MPDCCH narrowband determination based on the identifier agreed by RAN2.

Proposal 3: Option 2 (the Contention Resolution ID in the MAC layer) is selected to determine the MPDCCH narrowband used for CB-Msg4 monitoring.

Proposal 4: RAN2 informs RAN1 to implement the MPDCCH narrowband selection for CB-Msg4 monitoring based on Contention Resolution ID.

- The determination of the MPDCCH narrowband for CB-Msg4 monitoring is based on the value of (Contention Resolution identifier included in CB-Msg3 mod 2)

# 3	Conclusion

This document has made the following observations:

Observation 1: RAN1 has no concerns with the agreement to support two MPDCCH narrowbands for CB-Msg4 monitoring, as agreed by RAN2.

Observation 2: RAN2 is expected to decide on a mechanism to ensure that the UE monitors a unique narrowband.

Observation 3: The CB-Msg3 resource index is not suitable for determining the MPDCCH narrowband for CB-Msg4 monitoring, because multiple transmissions/replicas within the same CB-Msg3 window may result in different narrowbands that the UE would need to monitor, which does not satisfy RAN1 requirement.

Observation 4: If the UE ID is used to determine the MPDCCH narrowband for CB-Msg4 monitoring, it may be 40-bits S-TMSI in the CP solution and the 24-bits or 40-bits resumeIdentity in the UP solution.

Observation 5: Since the Contention Resolution ID is always included in CB-Msg3 and remains unchanged across the multiple transmissions/replicas within the same CB-Msg3 window, it can be used to determine the MPDCCH narrowband for CB-Msg4 monitoring in both the CP and UP solutions.

Observation 6: It is feasible to use a physical layer ID of CB-Msg3 to determine the MPDCCH narrowband for CB-Msg4 monitoring. However, there is no exact PHY ID proposed from RAN1.

And proposed the following:

Proposal 1: For the CB-Msg3 procedure, a new identifier other than preamble index should be used to determine the MPDCCH narrowband for CB-Msg4 monitoring.

Proposal 2: The following three options can be considered for the new identifier used to determine the MPDCCH narrowband for CB-Msg4 monitoring:

- Option 1: The UE ID included in CB-Msg3 RRC messages. i.e., the 40-bits S-TMSI in the CP solution, or the 24-bits or 40-bits resumeIdentity in the UP solution.

- Option 2: The Contention Resolution ID in MAC layer.

- Option 3: A physical layer ID associated with CB-Msg3, while the exact ID is to be determined by RAN1.

Proposal 3: Option 2 (the Contention Resolution ID in MAC layer) is selected to determine the MPDCCH narrowband used for CB-Msg4 monitoring.

Proposal 4: RAN2 informs RAN1 to implement the MPDCCH narrowband selection for CB-Msg4 monitoring based on Contention Resolution ID.

- The determination of the MPDCCH narrowband for CB-Msg4 monitoring is based on the value of (Contention Resolution identifier included in CB-Msg3 mod 2)

# Reference

[1] R2-2506284 LS on power ramping and RRC configuration for CB-Msg3-EDT, Nokia, Nokia Shanghai Bell

[2] R1-2508114 Reply LS on power ramping and RRC configuration for CB-Msg3-EDT, MediaTek

[3] TS36.331 Radio Resource Control (RRC) Protocol specification (Release 19)

[4] TS36.213 Physical layer procedures

[5] R1-2507987 Moderator summary on reply LS on CB Msg3 EDT, MediaTek

[6] R1-2507685 Power ramping and RRC configuration for CB-Msg3-EDT, Qualcomm