3GPP TSG-RAN WG2 Meeting #132	R2-2508322

Dallas, USA, 17 – 21 November 2025

Agenda item:	10.2.1

Source:	Nokia

Title:	6GR UE capability framework

WID/SID:	FS_6G_Radio - Release 20

Document for:	Discussion and Decision

# 1	Introduction

The contribution discusses the goals of the 6G study (RP-252912) concerning the UE capability framework, focusing on lessons learned from previous generations. By pinpointing both challenges and effective approaches, its purpose is to recognize aspects that should set the foundational principles for the design of the 6G UE capability framework.

In the interest of progressing the study and focusing the discussion, RAN2#131bis made the following agreements, which we take as the basis for further discussion:

For next meeting companies should focus on identifying the critical problems with UE capability framework and how to address them.  Identify what actions to be triggered with other WGs.

Companies can identify use cases to justify dynamic capability change in connected mode and analyse impacts to network side and network behaviour upon reception of updated capabilities.   Understand if there is impact to other WGs.

Companies can focus on the practical IODT problems in the field.   Possible ways to address them.

# 2	Discussion

## 2.1	General principles for capability framework

The 6GR SID outlines several distinct objectives related to device types. For instance: Target scalable and forward compatible design for diverse device types” [RAN1, RAN2, RAN3, RAN4], “Study UE RF capabilities considering different device types and implementations” [RAN4].

We understand the term “type” as a group of UEs sharing common characteristics, like a family of devices. Allowing ranges for certain requirements and capabilities within the same family can then enable generalization for UEs belonging to the same family. Similarly, what a device family is required to support could differ between the families. For instance, in 5G eMBB devices are required to support by default 4Rx in most bands, while RedCap devices are only required to support 2Rx or 1Rx. Such characteristics impact both the initial access and RF/PHY characteristics, coverage planning for deployments and UE configuration/capabilities.

The starting point for discussion is the fundamental PHY and RF characteristics a device family supports. While RAN1 can discuss many PHY aspects (e.g. minimum MIMO layers, UE processing time, baseband requirements, …), also RAN4 has a key role in defining minimum performance requirements, or form factor characteristics (e.g., power class, minimum/maximum channel bandwidth, Tx/Rx antennas, …). RAN2 will also naturally have to define the signalling and procedural aspects, as any device needs to be eventually recognized. Knowing the minimum set of requirements associated with each family could help to significantly reduce latency in transiting to CONNECTED while also reducing signalling overhead.

Proposal 1: A 6GR device family defines a common set of minimum requirements. This minimum set can vary across families. Within a family, requirements outside the common set can vary according to a known range.

## 2.2	Dynamic (capabilities) changes

In our understanding of the discussions from last meeting, the use cases for dynamic capabilities can be broadly categorized into two categories:

1.	UAI-like issues, e.g. UE overheating or power saving assistance requiring downgraded configuration or changes in (in)applicability of AI/ML functionality requiring configuration adjustment.

2.	Changes to device form factor, e.g. folding phones with changing hardware capabilities.

The two groups are distinct: the first relates to reconfigurations, while the second relates to modifications to RF availability/requirements.

Observation 1: RAN2 needs to ensure consistent interpretation of “dynamic UE Capabilities changes”: whether it relates to UE (in)ability to perform according to the applied configuration or previously declared capabilities.

Regarding the UAI-like issues, it was argued that some networks are not reacting to UEs preferences to downgrade the configuration; however, this needs to be understood in further detail. For example, it might be seen that networks are not modifying the RRC configuration immediately in response to the UAI, or that the network is not modifying the specific aspect of the configuration indicated in the UAI. We note that even if network RRC Reconfiguration is not modifying the specific aspect, the network may ensure prompt and reactive action to the UE assistance information by immediate and lower layers commands (e.g. in response to overheating, the network can immediately reduce the amount of resources allocated to a UE by decreasing the grant sizes and deactivating the carriers through MAC CEs).

Observation 2: The absence of RRC Reconfiguration in response to UAI does not preclude responsive network behaviour or adaptation at lower layers.

Another important distinction to draw is that UAI is only a temporary change (as it is modifying the configuration) and is local to the serving gNB(s). In contrast, UE capabilities are stored in both the RAN and CN and are considered permanent, in the sense that they stay the same as long as a UE is registered on the network.

Observation 3: The use cases associated with UAI involve only temporary modification of the configuration in RAN without impacting CN.

If dynamic capabilities are envisaged to address similar issues as UAI, it needs to be understood whether the updated capabilities are considered as temporary or permanent, and whether the change of capabilities needs to be known by the CN as well. If the capability change is considered only temporary in this scenario (and especially if CN does not need to be aware of it), then we do wonder whether it can continue to be handled as a configuration change rather than a capability change, even if that means something different than how UAI is handled in 5G NR.

Proposal 2: Study commercialized 5G UAI issues (e.g. overheating) as the first use case, including identification of the specific requirements (e.g. when the changes need to take effect, permanency of the change, configuration handling).

Form factor changes are different from UAI in that they result in outright changes to the hardware capabilities of the UE and take effect (nearly) instantaneously. Already in NR there are folding phones that modify their RF characteristics when folded, and it is our understanding that such devices are able to modify their configuration via lower layer signalling (i.e. while remaining connected), for example by reporting different rank in L1, so the motivation for dynamic capabilities is still not clear to us. Hence, when discussing this use case, it would be good to understand the specific challenges with the way folding phone hardware changes are handled in NR and why companies consider the NR way of handling folding devices to be inadequate. For this scenario, it also seems like the change only needs to be known to the serving gNB(s) and can possibly be treated as a temporary change (maybe even handled by lower layers, transparently to RRC), so the question again is whether it really needs to be treated as a UE capability change or can be considered as a type of configuration change.

Observation 4: NR already supports form factor changes (e.g. folding phones) at lower layers without change of UE capabilities.

We acknowledge that there might be other use cases not yet presented in discussions; however, as the analysis on the above cases show, when discussing the relevant use cases and requirements (such as timing requirements, permanency of the change, whether the change needs to be known outside of the RAN) RAN2 should not rule out other means of managing the change, such as handling by RRC configuration or by lower layer indication/reporting.

Proposal 3: Analysis of “dynamic capability” use cases shall also consider whether other procedures, e.g. handling by RRC configuration or lower layers, can adequately meet the requirements.

## 2.3	IODT challenges

In order to discuss interoperability device testing (IODT) problems, we think it is relevant to give some background on the IODT process.

In short, IODT involves validating UE features within (typically multi-vendor) networks, i.e. testing whether the feature “works” as expected when UE indicates a capability and network provides the corresponding configuration (see Figure 1). Test cases may be defined by RAN5, although the sheer volume of capabilities makes it impossible for RAN5 to define test cases for all features and network configurations. Hence, the IODT process often involves some degree of coordination between vendors, e.g. to verify which features they support and how they support those features and even to align on interpretation of the specifications. (Note that this is also why during 3GPP discussions on features under maintenance, companies will sometimes enquire on other companies’ product teams’ interpretations of the specs.)

Figure 1: Role of UE capabilities in IODT

IODT execution performed between UE and network vendors for an identified test case often uses initial versions of Technical Specifications, before their maturity. Therefore, IODT findings often expose ambiguities and real issues that result from interaction of UE capabilities under real signalling conditions, and validate how well individual capability bits were specified. Typically, only after successful testing, vendors declare a feature as successfully IODT’d, in which case they “enable” the feature, i.e. signal the corresponding capabilitie(s) as supported. Hence, accurate and comprehensive UE capability definitions at the early stage of a release have a direct impact on IODT.

As stated above, test cases are not fully defined (at least in the sense that they might not cover all deployment scenarios, e.g. bands, architectures, TDD/FDD, etc.), and hence one vendor’s interpretation of a feature being supported may not be identical to another vendor. The result is that there can be cases where a UE might have validated that a feature works as intended under the configurations that it has seen from a network vendor, but that the UE is still not confident enough to declare the feature as supported in that network. This is particularly true in multi-vendor networks where different network vendors might support different configurations for a given feature (e.g. a subset of the full range of configurations specified by 3GPP for that feature) due to differences in their product implementations (hardware or software).

Observation 5: IODT declaration, although based on normative work from RAN WGs, is still subject to vendor discretion outside of 3GPP.

One type of IODT issue we have observed in various iterations is when capabilities are specified with associated minimum requirements, but which UEs do not always meet in practice. As one example, we have seen cases where some UEs do not support mandatory channel bandwidths specified by RAN4 for a given band. A somewhat related example is the case of measurement gap capabilities defined by RAN4, where “no measurement gap” had originally been understood to imply that UE does not require any interruptions to make measurements, but it was later realized that some UEs declaring gapless measurements still required (small) interruptions. This resulted in enhancements being introduced in Rel-18 to distinguish whether UEs support gapless measurements with or without interruptions; but the end result is that networks are forced to assume pre-Rel-18 UEs declaring no gaps require small interruptions, leading to suboptimal scheduling.

Observation 6: Capabilities with associated minimum requirements are sometimes declared as supported despite those requirements not being fully met, risking misinterpretation by the network and invalid configuration at the UE.

As a release matures, the number of a feature options and/or optimizations often increases. While this reflects evolutionary development, it also leads to a very broad and complex conformance test scope. As 6G is becoming an evolutionary advancement of 5G, it might broaden the scope, risking early 6G IODT outcomes. Targeting early, well-defined capability baselines can help leverage this flexibility into faster, more efficient interoperability and conformance validation.

As RAN1 and RAN4 play a big role in feature definitions, it is critical that they are aware of the challenges mentioned above and commit alongside RAN2 to aiming to address these challenges.

Proposal 4: 6GR should target an early and accurate definition of minimum requirements/capability baseline for each feature. RAN2 should inform RAN1 and RAN4 of the importance of this goal.

## 2.4	Capability structure

The UE capabilities that are exchanged with RAN through RRC signalling need to provide the information to the network about what radio features the UE supports. The radio capabilities are crucial for configuring the UE correctly during connection setup and optimization procedures. The specification of UE capabilities evolves significantly over each release with the development of numerous and advanced features. Due to support for many frequency bands, band combinations, multiple radio access technologies (e.g., NR + LTE), and feature sets (e.g. for CA, MIMO, etc.), the UE capability containers can become very large, exceeding the allowable size of a single UL RRC message.

Observation 7: UE radio capabilities can be very large and can exceed the allowable size of a single UL RRC message size.

In the NR capability framework, network requests filtered UE capabilities, but, despite the use of filters, capability sizes can still be large. The different types of UE capabilities (per-UE, per-band, per-band combination, per-FeatureSet, per-Carrier), also complicated NR capabilities, and the fragmentation of features into too many capabilities via options and granularities has led to significant overhead and made it hard to distinguish the relationships between these capabilities and their corresponding configurations.

Observation 8: 5G modelling of UE Capabilities is overcomplex due to excessive fragmentation, leading to loss of coherence and understanding on how individual capabilities interconnect.

To accommodate the ever growing number of features and limited by the PDCP SDU size, RAN2 introduced UL segmentation of the UE capability container in Rel-16, allowing up to 16 capability segments to be sent by the UE during the capability enquiry; and this was further modified in Rel-17 to give more flexibility to the network to restrict the number of segments. Yet, despite these RAN-sided enhancements, there are still known to be limitations within CN interfaces.

Based on this understanding, we think that as a starting point for 6GR, RAN2 could assume that UL segmentation of the capability container from the outset might not be necessary for the first 6G release. Instead, RAN2 should try to pursue strategies to keep the capability container unfragmented. This means that we should study the overall structure of the capabilities, including the relationship between the different levels of capabilities (“per X”), aiming to limit the overall number of capabilities/options as outlined in the 6G SID description, as well as study the capability filtering mechanisms between the network and UE.

Proposal 5: Study scalable design of UE capability signalling (e.g. how to ensure reliable retrieval of decodable UE capabilities with less complex UE capability structure) while aiming to keep the UE capability container unfragmented (i.e. fitting into a single UL RRC message).

Filtering mechanisms can likely be studied within RAN2 without much coordination with other WGs. However, discussions on the capability structure (granularity and options) will inevitably require coordination with RAN1 and RAN4, as they are also responsible for defining radio features. In NR, RAN1 tended to define numerous options for features with quite fine and/or complex granularities, e.g. per-FS or per-band per-BC capabilities with numerous options that can be signalled for sub-components of the features. RAN4 NR features tended to be less complex in that regard, although there also existed other challenges with RAN4 features, such as defining RF- or RRM-related capabilities in a future-proof manner to accommodate various bands, bandwidths, BCs, etc. Hence, RAN2 needs to be proactive about coordinating with the other WGs when it comes to defining the 6GR UE Capability structure so we do not get “locked in” by decisions being made in other WGs.

Observation 9: Decisions made by other WGs early in NR had long term impacts on the UE capability structure.

To sum up: 6GR should strive to improve the capability signalling framework, considering the NR complexities from RAN2 as well as across working groups.

Proposal 6: Identify capability challenges (e.g. granularity and options of PHY capabilities, future-proofness of RF/RRM capabilities) that will require proactive coordination with RAN1 and RAN4.

## 2.5	Capabilities specification(s)

NR currently has three RAN2 specifications defining UE Capabilities: TS 38.331 (capability signalling in the RRC specification), TS 38.306 (capability descriptions based on the signalling) and TR 38.822 (feature group descriptions that are used at the end of release for creating the capability signalling).

The NR capability discussions have usually been separated from the “main” work item discussions, with separate CRs maintained since the discussions often diverge from the main technical discussions of the feature functionality. Therefore, in practice the capabilities are a separate topic from the rest of the RRC discussion (including ASN.1 review). Maintaining all this information across different specifications has shown to be laborious and has led to numerous discrepancies between the three specifications (especially since RAN2 originally did not intend to maintain TR 38.822 but later chose to start doing that). Furthermore, such discrepancies have occasionally resulted in companies implementing features differently (e.g. one product team basing their interpretation on 38.331, one basing it on 38.306, and one basing it on 38.822), which has led to multiple instances where RAN2 has debated over which spec is considered “the authoritative source” on capabilities.

Observation 10: Spreading capabilities across multiple specifications has shown to be laborious and resulted in discrepancies that led to incorrect product implementations.

The TR is good for tracking which Feature Groups are related to one another, but encoding of the Feature Groups in RRC makes it difficult to identify their relationship to one another. Furthermore, 38.306 does not capture the Feature Group IDs at all, so one is required to cross-reference between 38.822 or 38.331. Therefore, moving the capability aspects from the main RRC specification into their own specification would both reduce the complexity of specifying the UE capabilities and avoid some of the discrepancies requiring more efforts for specification maintenance. To simplify management of UE capabilities specification, it seems beneficial to re-consider the three complementary specifications approach to limit redundancy and improve efficiency, as shown in Figure 2.

Figure 2: Compacting UE capabilities to one specification

Proposal 7: Study how to capture all capability aspects in one technical specification to simplify specification, maintenance, and implementation of the 6GR UE capabilities.

# 3	Conclusion

This document has made the following observations and proposals:

General principles:

Proposal 1: A 6GR device family defines a common set of minimum requirements. This minimum set can vary across families. Within a family, requirements outside the common set can vary according to a known range.

“Dynamic” capabilities:

Observation 1: RAN2 needs to ensure consistent interpretation of “dynamic UE Capabilities changes”: whether it relates to UE (in)ability to perform according to the applied configuration or previously declared capabilities.

Observation 2: The absence of RRC Reconfiguration in response to UAI does not preclude responsive network behaviour or adaptation at lower layers.

Observation 3: The use cases associated with UAI involve only temporary modification of the configuration in RAN without impacting CN.

Observation 4: NR already supports form factor changes (e.g. folding phones) at lower layers without change of UE capabilities.

Proposal 2: Study commercialized 5G UAI issues (e.g. overheating) as the first use case, including identification of the specific requirements (e.g. when the changes need to take effect, permanency of the change, configuration handling).

Proposal 3: Analysis of “dynamic capability” use cases shall also consider whether other procedures, e.g. handling by RRC configuration or lower layers, can adequately meet the requirements.

IODT challenges:

Observation 5: IODT declaration, although based on normative work from RAN WGs, is still subject to vendor discretion outside of 3GPP.

Observation 6: Capabilities with associated minimum requirements are sometimes declared as supported despite those requirements not being fully met, risking misinterpretation by the network and invalid configuration at the UE.

Proposal 4: 6GR should target an early and accurate definition of minimum requirements/capability baseline for each feature. RAN2 should inform RAN1 and RAN4 of the importance of this goal.

Capability structure:

Observation 7: UE radio capabilities can be very large and can exceed the allowable size of a single UL RRC message size.

Observation 8: 5G modelling of UE Capabilities is overcomplex due to excessive fragmentation, leading to loss of coherence and understanding on how individual capabilities interconnect.

Observation 9: Decisions made by other WGs early in NR had long term impacts on the UE capability structure.

Proposal 5: Study scalable design of UE capability signalling (e.g. how to ensure reliable retrieval of decodable UE capabilities with less complex UE capability structure) while aiming to keep the UE capability container unfragmented (i.e. fitting into a single UL RRC message).

Proposal 6: Identify capability challenges (e.g. granularity and options of PHY capabilities, future-proofness of RF/RRM capabilities) that will require proactive coordination with RAN1 and RAN4.

Capabilities specifications:

Observation 10: Spreading capabilities across multiple specifications has shown to be laborious and resulted in discrepancies that led to incorrect product implementations.

Proposal 7: Study how to capture all capability aspects in one technical specification to simplify specification, maintenance, and implementation of the 6GR UE capabilities.