3GPP TSG-RAN WG2 Meeting #132		        R2-2508411

Dallas TX, USA, 17-21 November 2025

Agenda Item:	10.2.2

Source: 	MediaTek Inc.

Title:  	Harmonized Design for TN and NTN in 6G

Document for:	Discussion and decision

# Introduction

In last meeting, we discussed NTN in 6G. While harmonization with TN was desirable, companies raised concern that we did not know the baseline 6G TN design yet. Then the following instruction was captured in Chair’s note.

- For next meeting companies should identify essential requirements for NTN and identify which functionality should be considered in the common design with TN and what are NTN specific and whether they are essential.

In this contribution, we provide further details about TN/NTN integration in 6G, following the instruction above.

# Discussion

## Essential requirements for 6G NTN designs in RAN2

General discussions on essential requirements for NTN might better take place at plenary level. Nonetheless, here we can discuss requirements that impact RAN2 designs.

- Physical characteristics: NTN has some fundamental physical differences from TN, such as large and moving coverage and long propagation delay. These must be considered in RAN2 designs (e.g., L2 procedures and mobility management) of 6G NTN.

- Spectrum sharing: NTN operation may utilize the spectrum currently assigned to TN, especially when we consider NTN as ‘just another cell’ in a TN/NTN integration scenario. Therefore, spectrum sharing between TN and NTN should be supported.

- Mobility: From RAN2 perspective, handover between TN and NTN should be supported.

- Services provided by NTN: There may be NTN-specific services, such as positioning, navigation, and timing (PNT), multi-cast and broadcast, etc. Once we identify such procedures (may be at plenary level), RAN2 procedures should support them.

- GNSS resilience: GNSS-resilient NR-NTN operation is being discussed by RAN1 and RAN4 in Rel-20 [3]. Although the SI does not involve RAN2, such scenario may be considered in 6G and may impact RAN2 designs. For example, location-based mobility procedures may not work upon jamming or spoofing of GNSS signal. RAN2 procedural designs should ensure the support of GNSS-resilient scenario.

Proposal 1: From RAN2 perspective, we have the following essential requirements for 6G NTN:

- RAN2 procedures should accommodate physical characteristics of NTN

- Spectrum sharing between TN and NTN should be supported

- Handover between TN and NTN should be supported by RAN2 procedures

- Services provided by NTN, once identified, should be supported

- GNSS-resilient scenario should be supported

## Common vs. NTN-specific

As captured in Chair’s note, we understand that some functionalities should be considered in the common design with TN, while others are NTN-specific. We believe that the ‘commonality’ can be described at different levels.

- Common procedures with potentially different parameters: At this level, common procedures are adopted in both TN and NTN. When needed, we extend the value ranges of some parameters to account for physical characteristics of NTN. For example, we may need to extend some timers to fit longer RTT in NTN.

- Common procedures with different operation details: TN and NTN may share some procedures in general, though the detailed operations are slightly different. For example, RACH-less handover reduces handover interruption by allowing UE not to perform random access upon receiving handover command. RACH-less handover is applicable to TN in several cases (e.g., TA is reused, TA=0, or TA is obtained via early synchronization). It is also possible in NTN since TA can be derived based on UE location. Therefore, we can have a common RACH-less handover procedure for TN and NTN, with different operation details (e.g., how UE obtains TA).

- NTN-specific procedures: Despite the desire of harmonized design, there are procedures tailored for NTN’s physical characteristics and are thus only applicable to NTN. For example, time-based and location-based mobility procedures were introduced considering the RSRP variation in NTN. These procedures may not be suitable for TN and are thus considered NTN-specific.

We have the following proposal:

Proposal 2:	Regarding functionalities with common design for NTN and TN, we should design value ranges and multiplicities to ensure feasibility in NTN. Moreover, the operation details may be different for a common procedure.

## Common and NTN-specific functionalities

We now discuss major RAN2 functionalities and see which ones should be in common with TN and which ones should be NTN-specific.

Layer 2 (L2)

- L2 architecture: Although 6G L2 may be redesigned and we do not know the details at this moment, to allow TN/NTN integration, we believe that NTN and TN should share a common L2 architecture.

- L2 timers: The value ranges of some L2 timers need to be extended for NTN. For example, assuming 5G L2 architecture, the value ranges of timers in MAC (e.g., sr-ProhibitTimer, configuredGrantTimer), RLC (e.g., t-Reassembly), and PDCP (e.g., discardTimer and t-reordering) layers are extended to accommodate NTN’s long propagation delay. Similarly, DRX timers (e.g., drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL) should be extended for longer RTT of NTN.

- HARQ: To mitigate HARQ process stalling problem in NTN, we may increase the number of HARQ processes, or allow HARQ feedback disabling. The former does not change TN baseline HARQ process, and the latter should be NTN-specific.

System information

- SIB for NTN: In 5G, we introduced a new SIB (SIB19), containing NTN-specific parameters related to the serving and neighbour cells. 6G SIB structure is to be discussed and may be different from 5G, however, we prefer to have NTN-specific SIB(s).

Measurement

- SMTC: In 5G NTN, network may configure multiple SMTCs, considering the propagation delay difference and ephemeris information. For TN, the timing of SMTC is fixed, but for NTN, it needs adjustment due to the satellite movement. The timing of SMTC is adjusted by NW control in connected mode, and by UE when in idle mode. In 6G, SMTC configuration and timing adjustment for NTN should be considered. The SMTC configuration can be common for TN and NTN, although some configurations (e.g., multiple SMTC) are likely to be used by NTN.

- Measurement gap: In 5G NTN, network can configure measurement gaps based on multiple SMTCs. In 6G, the measurement gap configuration (including ‘gap-less’ feature) can be common for TN and NTN.

- Measurement configuration: TN and NTN should share common measurement configuration format. However, there may be optional NTN-specific indications, allowing UE to optimize its measurement behaviour.

Mobility in RRC idle/inactive mode

- Tracking Area: A Tracking Area corresponds to a fixed geographical area. Since the satellite is moving, the network can broadcast multiple TACs per PLMN in an NR-NTN cell, to reduce signalling overhead at cell edge, particularly for earth-moving NTN cells. Currently, we have (in PLMN-IdentityInfo) fields of trackingAreaCode and trackingAreaList for TN and NTN, respectively. In 6G, TAC configurations for TN and NTN can be integrated by allowing multiple TACs per PLMN (i.e., using a list instead of a single value).

- Cell (re)selection: In 5G NTN, we introduced time-based measurements (i.e., with configurations about service time) and location-based measurements (i.e., with configurations about reference location and distance threshold) for cell (re)selection. These procedures were developed considering that NTN’s RSRP variation is different from TN, and thus they are more likely NTN-specific.

- Discontinuous coverage: At least in early stage, NTN constellation may not have fully continuous coverage. We addressed this issue in 5G IoT NTN and introduced an NTN-specific SIB (SIB32) carrying satellite assistance information for prediction of discontinuous coverage. We may have similar design in 6G NTN, and this should be NTN-specific.

Mobility in RRC connected mode

- Measurement events: In 5G NTN, we introduced time-based event (CondEvent T1) and location-based events (Event D1/D2 and CondEvent D1/D2). These events should be considered as NTN-specific.

- Handover procedure: We expect common procedures of (network-controlled) handover in the specification, although NTN mobility may be based on different triggering conditions.

- Conditional handover (CHO): In 5G NTN, we have CHO based on event A4, time-based, or location-based conditions. We expect that CHO is supported in both 6G TN and 6G NTN, and there should be a common description for CHO procedure, though there are NTN-specific conditions.

- RACH-less handover: RACH-less handover may be adopted in both 6G TN and 6G NTN, although the methods to obtain TA may be different. We expect a common RACH-less handover procedure.

- TN-NTN handover: Mobility between TN and NTN is supported in 5G. With a harmonized design in 6G, a neighbour NTN cell should be seen as just another neighbour cell when UE is served by TN, and vice versa. However, there may be optimizations for TN-NTN handover, considering physical differences between TN and NTN.

Based on the above discussions, we have the following proposals:

Proposal 3:	TN and NTN should share a common L2 architecture, and

- L2 timers can be extended to accommodate longer propagation delay in NTN.

- A higher number of HARQ processes may be considered for NTN. HARQ feedback disabling may be introduced as an NTN-specific feature.

Proposal 4:	Introduce NTN-specific SIB(s) to carry fields that are only needed in NTN-related operations.

Proposal 5:	TN and NTN should share common measurement frameworks, and

- The network may configure multiple SMTCs with different periodicities, SMTC timing adjustment, and corresponding measurement gaps.

- There may be NTN-specific indications in measurement configurations.

Proposal 6:	Regarding mobility in Idle/Inactive mode:

- TN and NTN share the same format of tracking area configuration, and the network can broadcast a single TAC or multiple TACs per PLMN.

- Cell selection and reselection procedures for NTN UEs may be NTN-specific, e.g., time-based or location-based method.

- We should introduce NTN-specific SIB, carrying satellite assistance information for prediction of discontinuous coverage.

Proposal 7:	Regarding mobility in connected mode:

- TN and NTN may share descriptions of handover procedures at high level, e.g., network-controlled handover, conditional handover, etc.

- NTN UE should have NTN-specific handover triggering conditions, e.g., time-based and location-based events.

- TN and NTN may support RACH-less handover in common. However, NTN UE may have NTN-specific ways to obtain target cell TA.

## Standardization procedures

While a harmonized design for TN and NTN is the common goal in 6G, companies raised a major concern: We don’t know 6G TN ‘baseline’ design yet, then how can we design NTN that is harmonized with TN? However, if we wait for 6G TN design, 6G NTN will again become an ‘add-on’, as in 5G. To allow parallel development of 6G NTN, we propose the follow way forward:

- When designing 6G TN, we assume the procedures are applicable to NTN, and make sure the parameter value ranges and multiplicities are suitable for NTN. For example, some timers may need to be extended, and some fields may be implemented as lists instead of single values.

- We also identify common procedures that may require adjustments in NTN. For example, RACH-less handover can be introduced, even if TN and NTN UEs may have different ways to obtain the TA. CHO may also be introduced as a common procedure, while NTN may have NTN-specific conditions.

- Finally, we design NTN-specific procedures. We may start from new procedures introduced for NTN in 5G (see discussion above and [2]) and then introduce new designs in 6G.

Proposal 8:	To ensure a harmonized TN-NTN design in 6G, the standardization works are done in the following steps:

- Assume the TN procedures are applicable to NTN, and make sure the parameter value ranges and multiplicities are suitable for NTN.

- Identify TN procedures that require adjustments to work in NTN.

- Design NTN-specific procedures.

# Conclusion

It is proposed to discuss and decide on the following proposals:

Proposal 1: From RAN2 perspective, we have the following essential requirements for 6G NTN:

- RAN2 procedures should accommodate physical characteristics of NTN

- Spectrum sharing between TN and NTN should be supported

- Handover between TN and NTN should be supported

- Services provided by NTN, once identified, should be supported

- GNSS-resilient scenario should be supported

Proposal 2:	Regarding functionalities with common design for NTN and TN, we should design value ranges and multiplicities to ensure feasibility in NTN. Moreover, the operation details may be different for a common procedure.

Proposal 3:	TN and NTN should share a common L2 architecture, and

- L2 timers can be extended to accommodate longer propagation delay in NTN.

- A higher number of HARQ processes may be considered for NTN. HARQ feedback disabling may be introduced as an NTN-specific feature.

Proposal 4:	Introduce NTN-specific SIB(s) to carry fields that are only needed in NTN-related operations.

Proposal 5:	TN and NTN should share common measurement frameworks, and

- The network may configure multiple SMTCs with different periodicities, SMTC timing adjustment, and corresponding measurement gaps.

- There may be NTN-specific indications in measurement configurations.

Proposal 6:	Regarding mobility in Idle/Inactive mode:

- TN and NTN share the same format of tracking area configuration, and the network can broadcast a single TAC or multiple TACs per PLMN.

- Cell selection and reselection procedures for NTN UEs may be NTN-specific, e.g., time-based or location-based method.

- We should introduce NTN-specific SIB, carrying satellite assistance information for prediction of discontinuous coverage.

Proposal 7:	Regarding mobility in connected mode:

- TN and NTN may share common descriptions of handover procedures at high level, e.g., network-controlled handover, conditional handover, etc.

- NTN UE should have NTN-specific handover triggering conditions, e.g., time-based and location-based events.

- TN and NTN may support RACH-less handover in common. However, NTN UE may have NTN-specific ways to obtain target cell TA.

Proposal 8:	To ensure a harmonized TN-NTN design in 6G, the standardization works are done in the following steps:

- Assume the TN procedures are applicable to NTN, and make sure the parameter value ranges and multiplicities are suitable for NTN.

- Identify TN procedures that require adjustments to work in NTN.

- Design NTN-specific procedures.

# Reference

- RAN2#131bis Chair’s Note, October 2025

- R2-2507138, Consideration of 6G NTN, MediaTek Inc., October 2025

- RP-251933, Study on GNSS (Global Navigation Satellite System) resilient NR-NTN (Non-Terrestrial Networks) operation, Thales (Rapporteur), September 2025