3GPP TSG-RAN WG2 Meeting #132	R2-2508415

Dallas, USA, Nov. 17th-21st

Agenda Item:	10.2.2

Source:	Amazon

Title:	Overview of harmonized 6G Radio design for TN and NTN

Document for:	Discussion and Decision

# Introduction

The 6G Radio Study Item was approved at the RAN#108 meeting (RP-251881) [1]. The main objectives are:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

This document provides an overview of harmonized 6GR NTN/TN air interface design requirements.

# Lessons learned from NR/IoT NTN

NTN was introduced to NR and NB-IoT after the initial release. In another words, the NR/NB-IoT basic RAT design did not take a NTN deployment scenario into consideration. Such a progressive standardization approach results in various backward compatibility issues in later phases which is widely recognized by the industry. One example is the dependency of GNSS: 5G NR was designed to support limited RTD (Round Trip Doppler) and RTT (Round Trip Time). When NTN was introduced in later releases, the NR synchronization framework cannot support a NTN deployment scenario with large RTD and RTT. 3GPP took a short-cut to solve this problem by having NTN UEs rely on GNSS and Sat ephemeris to pre-compensate the timing adjustment and Doppler for uplink transmissions. Then a few releases later, 3GPP still needs to address the GNSS dependence issue in a GNSS-resilience study item. Such a staggered/multi-phase standardization approach is very un-friendly to the eco-system.

Observation 1: Introducing NTN as an add-on feature in later releases is not friendly to the eco-system.

# What is a harmonized 6G Radio design for TN-NTN?

In our view, a harmonized TN-NTN design means the same 6G RAT (Radio Access Technology) should support both TN and NTN deployments. While a harmonized design is not totally quantifiable, here is a list of some key design requirements:

- A harmonized 6G TN-NTN RAT supports both TN and NTN deployment in a GNSS-less environment. If 6G TN doesn’t depend on GNSS, 6G NTN should also not depend on its availability/support.

- A harmonized 6G TN-NTN RAT design shall NOT require special UE implementations for NTN deployment scenarios at least to enable basic connectivity. As a result, all 6G UE shall support NTN deployments (at least the basic communication capabilities) as a mandatory feature.

- A harmonized 6G TN-NTN RAT design should support seamless deployment of TN and NTN coverage (including mobility + multi-connectivity).

Proposal-1: A harmonized 6G TN-NTN design should support both TN and NTN in a GNSS-less environment.

Proposal-2: 6G UE should have mandatory support for the basic communication features of both TN and NTN deployments.

Proposal-3: Seamless deployment of TN and NTN coverage (including mobility + multi-connectivity) must be supported by 6G.

# NTN architecture

In the 5G radio access network architecture (NG-RAN), the gNB is responsible for all radio-related functions in one or multiple cells on the ground, including admission control, connection establishment, user-plane and control-plane routing between the 5G-RAN and the 5G core, and radio resource management (RRM) functions such as mobility and CONNECTED/IDLE mode cell selection.

In the context of NTN an additional node is introduced, the NTN Gateway (GW), which is a Transport Network Layer node. The satellite and the NTN GW communicate over the feeder link using the Satellite Radio Interface (SRI). This introduces the question of where the different radio access network functions should be deployed as shown in the figure below.

Figure 1. NTN System Architecture

In one extreme, all of the radio access and network functions may be deployed on the ground where the access link is forwarded by the NTN GW and satellite to the user (e.g. a transparent architecture). At the other extreme, all of the network and radio access functions are deployed on the satellite (e.g. a regenerative architecture).

As part of the 6G study item evaluations on the support of NTN scenarios, both architectures (and additional potential variants) should be considered. The need for flexibility in the NTN deployment architecture is a function of multiple factors such as:

- Deployment scenario: Carrier frequency, system BW, satellite altitude, constellation size, etc.

- Satellite payload compute/processing requirements vs. on-orbit power budgets

- Feeder link bandwidth/latency requirements

- Forward Compatibility: The goal would be for a design that will evolve with future 3GPP releases. Features which are HW impacting can have an especially large impact on the deployment architecture.

- User experience impact: including connection establishment, handover interruption time, and DL/UL throughput and latency KPIs

- Feeder link availability/routing: Depending on the constellation design and NTN Gateway availability, there may be the need for frequent Feeder link switching events which impact the latency of handover events and the feasibility of "anchoring" user plane and control plane functionality on the ground.

- ISL support: The overall design of the 6G NTN system should accommodate the possibility of multiple “hops” between the NTN Gateway and the UE using inter-satellite links (ISL)

Proposal-4: Both regenerative and transparent NTN payload architectures should be supported by 6G.

# Radio Resource Management

A group of cells on the ground may be served by a single beam of a satellite and experience discontinuous transmission and reception (DTX/DRX) as the beam periodically “hops” across its coverage area in a TDM manner. This may be required if, for example, the total number of cells to be served is larger than the total number of supported beams per satellite across system. An example is shown in the figure below, where an 200ms beam hopping configuration periodicity is configured for 4 cells, with each using a 50ms beam-hopping cycle duration:

Figure 2. Example of NTN beam hopping

The frequency of the beam hopping across cells may be dependent on multiple factors such as the geographic area being served as well as the traffic load distribution. As part of the 6G study item, beam hopping for NTN should be considered in the overall design of reference signals and DL and UL control and data channels and its impact on user experience KPIs (e.g. throughput and latency) evaluated for both active and inactive users.

Proposal-5: 6G should optimize the RAT design considering the use case of beam hopping for NTN.

# Mobility and NTN/TN Interworking

The 6G study item should consider different mobility scenarios for NTN, including intra- and inter-frequency handovers, mobility between NTN and TN networks, and even the possibility of multi-connectivity between NTN and TN networks for the same device. It is desirable to harmonize NTN and TN mobility procedures where possible to simplify UE and network implementations, including the use of measurement reports, time/location-based triggers, and cell-selection criteria.

However, it is important to also consider necessary enhancements for NTN mobility scenarios due to the specific challenges of NTN networks, such as very large propagation delays, Doppler variations across source and target satellites, discontinuous coverage etc. This includes the need for the indication of assistance information (e.g. satellite ephemeris, propagation delay, timing offset parameters) in dedicated or system broadcast messages which can be used for handovers, intra-RAT and inter-RAT cell selection (e.g. to/from 5G or 6G), or mobility to/from NTN and TN networks.

Proposal-6: The 6G study should consider necessary enhancements for NTN mobility scenarios due to the specific challenges of NTN networks, such as very large propagation delays, Doppler variations across source and target satellites, discontinuous coverage etc., including the need for the indication of NTN-specific UE assistance information.

# Spatial domain management

The 5G NR beam management framework is designed for TN deployments with a fixed gNB on the ground. On the other hand, for NTN deployments, especially LEO, the base-station itself is moving, which the NR beam management framework was not optimized for. For 6GR, we suggest to study the beam management framework with an NTN deployment scenario in mind.

Proposal-7: The 6GR beam management framework must support NTN with high velocity satellites.

# Conclusion

In this contribution, we made the following proposals:

Proposal-1: A harmonized 6G TN-NTN design should support both TN and NTN in a GNSS-less environment.

Proposal-2: 6G UE should have mandatory support for the basic communication features of both TN and NTN deployments.

Proposal-3: Seamless deployment of TN and NTN coverage (including mobility + multi-connectivity) must be supported by 6G.

Proposal-4: Both regenerative and transparent NTN payload architectures should be supported by 6G.

Proposal-5: 6G should optimize the RAT design considering the use case of beam hopping for NTN.

Proposal-6: The 6G study should consider necessary enhancements for NTN mobility scenarios due to the specific challenges of NTN networks, such as very large propagation delays, Doppler variations across source and target satellites, discontinuous coverage etc., including the need for the indication of NTN-specific UE assistance information.

Proposal-7: The 6GR beam management framework must support NTN with high velocity satellites.