3GPP TSG-RAN WG2 Meeting #132	R2-2508419

Dallas, US, 17th – 21st November 2025

Agenda item	: 10.4

Source	: Tejas Networks

Title	: Discussion on Mobility in 6G systems

Document for	: Discussion

# 1. Introduction

RAN#108 agreed 6G Study Item for RAN WGs and revised in RAN#109 [1]. The following key objectives related to Mobility were agreed:

Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR:

- Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4]

- Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… 
Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

RAN2 #131bis further discussed the key objectives related to mobility and agreed on few directions. This document discusses those agreed directions and additional study considerations, including deice type, NTN mobility and AI/ML based enhancements.

# 2. Discussion

RAN2 #131bis meeting agreed on the following directions for the study of 6G mobility design.

Agreement: Study mobility with the following requirements in 6G mobility design: - Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions. - Robustness of mobility procedures - Energy efficiency for both UE and NW 2 Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

### 2.1 Device Type

One of the key objectives of 6GR study is to support diverse device types through a scalable and forward compatible design framework. 6G should accommodate a wide range of devices, including eMBB, IoT, FWA devices etc. These device categories correspond to different use cases and application requirements. For example, FWA devices are primarily intended for enhanced mobile broadband applications with limited mobility needs. Therefore, RAN2 mobility procedures, such as connected state mobility and idle/inactive state cell reselection, should be designed in a unified and flexible manner to efficiently address these varying requirements. Additionally, energy efficiency considerations should be incorporated into the overall design to ensure optimized operation across diverse device types from 6G Day 1

Proposal 1: Mobility study should consider diverse device types from day 1

### 2.2 Unified mobility framework

In release15 with regards to connected mode mobility, 3GPP took principles from LTE as the baseline for designing the mobility principles in 5G. This was based on the tried mechanism of break-before-make which resulted in interruption in service during the Handover procedure.

In later releases there were several mechanisms introduced to reduce this interruption and one among them introduced in release 16 is DAPS (Dual Active Protocol Stack) where the UE retains the connection to both source and target gNBs which helped in getting zero interruption time during the mobility procedure. The major issue with DAPS type handover was that it resulted in complicated UE stack design and didn’t see much commercially deployment of the feature.

Another major issue plaguing the legacy mechanism of HO borrowed from LTE was the problem of reliability of the HO procedure. To combat this, Conditional Handover (CHO) was introduced which relied on Network configuring the UE with potential candidate target cells and conditions to evaluate for the UE to make decision to apply the target cell configuration. This helped in reduce the HO failures due to unsuccessful delivery of the HO command to the UE. This mechanism also had its own drawbacks that it didn’t solve the interruption issue and this also required significant overhead at network side as multiple “potential candidate cells” had to reserve resources even if UE will handover to only one among them.

In later releases (rel18, rel19…), the concept of Lower Layer triggered Mobility (LTM) was introduced which relied on L1 measurements to take the HO decisions at network side and took the concept of UE having set of target cell configurations just like in CHO> LTM also reduced the interruption time by allowing the UE to acquire TA earlier before the HO is triggered thus reducing the delay due to RACH procedure towards target cell. There still existed the delay of issuing the HO command (cell switch command) in LTM and to tackle this, conditional LTM was introduced which brings in advantages of both CHO and LTM.

During previous meeting, we discussed different mobility mechanisms in detail, assessing their respective benefits and limitations. A common view expressed was that 6GR should aim to unify the mobility framework, integrating the advantages of these mechanisms into a unified scalable design from day 1 of 6G.

LTM utilizes L1 measurements for faster and more reliable handover decisions, thereby reducing service interruption and latency. Furthermore, LTM incorporates the concept of pre-configured target cells similar to CHO but with reduced signaling overhead. Given its flexibility and efficiency, LTM serves as an ideal baseline for developing a common connected mode mobility framework in 6GR. Using LTM as the foundation would allow RAN2 to unify various legacy procedures, simplify UE and network implementation, and ensure scalability for future mobility enhancements.

Proposal 2: RAN2 to consider a common connected mode mobility framework by taking LTM as baseline for design principles

## 2.3 RRM related simplification

As described earlier, the different HO mechanisms were added on top of each other and this resulted in some redundant configurations and procedures for different HO types. In case of L3 RRM which was needed for legacy HO procedures and other DC/CA related actions from NW. In case of LTM, the main RRM activity was L1 based. But both the measurements are based on Reference signals and this had to be configured separately for L3 based RRM and L1 based RRM in 5G. This can be redesigned in 6G to have a common RRM framework for both L1 and L3 measurements and reporting.

Proposal 3: RAN2 to study a common RRM framework for L1 and L3 based measurement/reporting and for different use cases such as Mobility, DC/CA operations, SON/MDT etc.

## 2.4 AI/ML for mobility

RAN2 in release 20 5G is undertaking the work of specifying the framework to enable AI/ML for mobility. While this study/work item can act as a starting point for AI/ML aided enhancements in mobility, the release 19 had a limited scope for the AI/ML mobility feature. In 6G, RAN2 should consider AI/ML for more use cases such as Conditional LTM, HO from/to NTN cell etc.

Proposal 4: RAN2 to take study done in 5G AI/ML Mobility as baseline and add in support for other mobility related use cases for AI/ML.

## 2.5 Handling of TN-NTN and NTN-NTN mobility

Commercially NTN deployments are picking up and will become more prevalent in 6G timeframe and the reliability of an NTN connection will be a great factor in determining its success. One major issue in NTN is from higher latency and this can lead to longer interruption during HO cases. Enhancements to mobility procedures such as location based or time based conditional HO were added in 5G to support NTN mobility. RAN2 in 5G-A rel20 is studying the case of NTN operation without GNSS and this needs to be also extended for Connected mode NTN mobility cases in 6G.

One of the 6G SID objective is that the RAN WGs to arrive at a harmonise design for NTN and TN. RAN2 should consider the different scenarios of NTN-NTN mobility between satellites in different orbits, HAPS, drones etc and also between NTN-TN cases and mobility framework developed should try to cater to these different cases.

Proposal 5: RAN2 to study a common mobility framework for different cases of NTN-NTN mobility (satellites in different orbits/HAPS/Drones etc) and NTN-TN mobility.

# Conclusion

This section summarizes our discussion by listing all the observations and proposals discussed in this contribution.

Observation 1: The priority-based cell-reselection mechanism is complex and needs to be simplified.

Proposal 1: Mobility study should consider diverse device types from day 1

Proposal 2: RAN2 to consider a common connected mode mobility framework by taking LTM as baseline for design principles

Proposal 3: RAN2 to study a common RRM framework for L1 and L3 based measurement/reporting and for different use cases such as Mobility, DC/CA operations, SON/MDT etc.

Proposal 4: RAN2 to take study done in 5G AI/ML Mobility as baseline and add in support for other mobility related use cases for AI/ML.

Proposal 5: RAN2 to study a common mobility framework for different cases of NTN-NTN mobility (satellites in different orbits/HAPS/Drones etc) and NTN-TN mobility.

# References

- RP-252912	Revised SID: Study on 6G Radio, 3GPP TSG RAN Meeting #109 Beijing, China