3GPP TSG-RAN WG2 Meeting #132                                  R2-2508426

Dallas, USA,  Nov. 17th -21st, 2025

Agenda item:		9.3.2

Source:				Samsung

Title:					Discussion on RRM measurement prediction

Document for:		Discussion & Decision

1	Introduction

In this paper, we would like to discuss the functionality management of RRM measurement prediction for UE-sided model (i.e., applicability/capability reporting, inference configuration, inference report) according to the following objectives in the latest WID (RP-252899).

- Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release

2	Discussion

2.1 Capability/Applicability reporting

UE Capability Reporting:

Regarding UE capability reporting, RAN2 made the following agreement in Rel-19 AI/ML for mobility SI.

Agreements (from RAN2#129bis) 1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management 2 Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM) Start by discussing spec/ran2 impact of the agreed cases/subcases and then discuss functionality granularity

For the capability discussion, we agreed to discuss functionality granularity after evaluating the RAN2 spec. impact for the agreed use cases during SI phase, which remains relevant in WI phase. Additionally, in addition to granularity, we need to discuss the dependencies between different functionality/features. For instance, if indirect method is assumed for RRM event prediction, there would be a dependency in UE capability between RRM measurement and event prediction. Based on the above, we would like to propose the following.

Proposal. 1: For UE capability, RAN2 discuss granularity and dependency of functionalities/features once sufficient progress has been made on the spec. impact of AI mobility functionalities/features.

Applicability Reporting:

Regarding applicability reporting, the following agreement was made during SI phase.

Agreements (from RAN2#130) - Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case) • Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress) • Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. • Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. • Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes. • Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress). • No prohibit timer is introduced. - Associated ID should be optionally configurable for training and inference (i.e. it may not mandatorily required for all training/inference configurations). FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology). Agreements on RRM measurement prediction for UE-sided model (from RAN2#131) 1. Confirm that as a baseline following agreements on applicability reporting in AI-based beam management are applicable for AI mobility: - UE may include “release configuration” flag in applicability reporting to indicate UEs preference to release a non-applicable configuration. - Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled. - When UE indicates that an inference configuration is not applicable, the gNB is expected to release the configuration (i.e., UE autonomous release is not supported). - The UE continues to perform the inference and reporting until the configuration is released. It is up to network implementation what to do with UE reported predicted values after UE indicates that an inference configuration is not applicable. - The UE shall report when an inference configuration becomes non-applicable. - How to handle RRC configuration in IDLE/INACTIVE/RLF, follow the legacy UE behaviour in TS 38.331 on whether to release or keep the RRC configuration. - Whether Option A and Option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure. - RAN2 assumes applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A). NOTE: these agreements will be aligned with AI PHY agreements at the end of this week and will not be re-discussed

For applicability reporting, the baseline approach can follow the procedure specified for R19 AI-based beam management. However, we should remain open to discussing alternative procedures if there is any distinguishing factor.

For associated ID, we have some FFSs remaining for WI phase. From our understanding, the motivation of having associated ID in BM use case is to provide the UE with NW-side additional condition necessary for applicability check but not be explicitly provided. For example, NW can implicitly indicate the consistency of NW’s beam configuration (e.g., antenna pattern, tilting, codebook configuration that can have impact on analog beam pattern of a certain CSI-RS/SSB index) through the associated ID, allowing the UE to assume the similar properties for the DL Tx beams with the same associated ID.
Based on this understanding, we would like to discuss the FFSs on associated ID one by one as below.

- Which use case? All.
: For all use cases (Temporal Case A/B, Frequency domain prediction), it is up to UE’s implementation whether to use beam-level measurement results for cell-level prediction (e.g., whether to use sub-use cases 1/3). Therefore, we think the associated ID is relevant for the all the use cases.

- Whether it is per cell or multiple cells/frequency? Either Per cell or per frequency (MeasObject, MO).
: The same associated ID can imply the identical SSB beam configuration across cells. Therefore, as a baseline, the associated ID can be configured per cell. However, cells operating on the same frequency may have the same SSB beam configuration depending on operator’s deployment scenario. Therefore, we propose that NW could also provide the associated ID per frequency (i.e., per MeasObject).

- What absence means? All cells have the same beam configuration.
: If NW can ensure consistency in SSB beam configuration across different cells and over different time (within a specific frequency), it is allowed not to provide associated ID (in the corresponding MeasObject). Otherwise, NW should provide associated ID per cell/MO in the MeasObject configured for AI/ML prediction.

Proposal. 2: Associated ID can be relevant for all the use cases of AI/ML mobility (i.e., Temporal domain Case A, Temporal domain Case B, Inter-frequency prediction).

Proposal. 3: Associated ID can be configured either per cell or per frequency (i.e., MeasObject) depending on deployment scenario.

Proposal. 4: The absence of associated ID means UE can assume consistency of SSB beam configuration across different cells and over time.

2.2 Inference & reporting

For UE-side model inference, NW can provide some configuration for inference at UE-side. Then, the UE can perform the inference with the input data that is internally available at UE and report the outcome of the inference to NW. For the spec. impact, we can focus on the inference configuration from NW to UE and the report from UE to NW.

Common part:

During SI phase, RAN2 agreed the inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework as below.

Agreements (from RAN2#129bis) - The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

In our view, the existing RRM measurement configuration consisting of MeasObject/MeasId/ReportConfig can be easily extended/enhanced to support the use cases of AI for mobility.

Proposal. 5: For inference configuration, reuse the existing IE (i.e., MeasObjectNR, MeasId, ReportConfigNR).

Proposal. 6: For inference configuration, introduce new fields to configure inference parameters in MeasObjectNR and/or ReportConfigNR IE.

For the common part of inference configuration across the use cases (i.e., frequency/temporal domain RRM measurement prediction), NW can indicate the target MO for which the UE predicts RRM measurement results.

Proposal. 7: For inference configuration, NW can indicate the target MO (Measurement object) for which the UE performs the prediction (e.g., RRM measurement prediction).

For inference report, we also think the existing RRC message (i.e., MeasurementReport) and IE (i.e., MeasResultNR) can be reused. If UE needs to report additional information (e.g., predicted RSRP) as part of inference report, we can introduce new fields in MeasResultNR IE.

Proposal. 8: For inference report, reuse the existing message (i.e., MeasurementReport) and IE (i.e., MeasResultNR).

Proposal. 9: For inference report, introduce new fields to include inference output in MeasResultNR IE if needed.

Temporal domain Case A:

During SI phase, for temporal domain Case A, the PW length and the list of target cells are agreed for inference configuration and multiple instances of predicted measurement results in PW are agreed for inference report as below.

Agreements (from RAN2#130) - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) Agreements on RRM measurement prediction (from RAN2#131) - For temporal domain Case A, NW can indicate the list of target cells for which it expects results (if available) as part of inference configuration. This list is optional. 3. For temporal domain case A, one or more instances of predicted measurement results in PW per one cell are reported in one measurementReport message

For PW length, we can further discuss its configuration with the following two options:

- Option 1 (time duration): NW configures the length of PW as a single time duration value (e.g., xx msec).

- Option 2 (# of instances and time interval): NW configures the number of time instances to predict within PW and the time interval between consecutive time instances (similar to R19 BM case).

The choice between these options depends on who (UE or NW) can determine the interval of the future time instances to predict within PW. If NW can have control over this, Option 2 is more suitable. Conversely, if the UE can determine the interval, Option 1 seems more appropriate. In the legacy RRM, UE is required to perform the measurement at least once within the measurement period given by RAN4 requirement, leaving the exact timing/interval of measurement to UE’s implementation. Therefore, the selection of the option depends on companies’ understanding on whether the timing/interval of time instances for prediction can be up to UE’s implementation or not.

Proposal. 9: For PW length configuration in temporal domain Case A, RAN2 discuss the following two options:

- Option 1 (time duration): NW configures the length of PW as a single time duration value (e.g., xx msec).

- Option 2 (# of instances and time interval): NW configures the number of time instances to predict within PW and the time interval between consecutive time instances (similar to R19 BM case).

For the list of target cells, we understand that it refers to the cells for which UE is configured to perform prediction and report predicted measurement results. Regarding how to configure the list, two options can be considered:

- Option 1: Reuse the existing list (i.e., allowedCellsToAddModList).

- Option 2: Introduce a new list (e.g., predictedCellsToAddModList).

The existing allowedCellsToAddModList is used to indicate the applicable cells for which UE is configured to perform measurement and report results. Given its similarity to the agreed target cell list for prediction, reusing the existing list for the prediction target cell list is a viable option. However, if we consider the case where the same MeasObject is commonly used for both legacy RRM measurement configuration and RRM measurement prediction, the decision depends on whether the target cells of measurement and prediction can always be the same or not.

Proposal. 10: For list of target cells configuration in temporal domain Case A, RAN2 discuss the following two options:

- Option 1: Reuse the existing list (i.e., allowedCellsToAddModList).

- Option 2: Introduce a new list (e.g., predictedCellsToAddModList).

Temporal domain Case B:

During SI phase, for temporal domain Case B, the skipping pattern (i.e., NW's SSB Tx pattern) was agreed for inference configuration and the latest measurement results was agreed for inference report as below.

Agreements (from RAN2#130) - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI Agreements on RRM measurement prediction (from RAN2#131) 1. For the interpretation of “skipping pattern” in temporal domain Case B, RAN2 confirm that it refers to SSB configuration to indicate the timing of NW's SSB transmission—not timing of UE's SSB measurement/skipping. 4. For temporal domain Case B, the inference report can include the latest measurement results (regardless of actual results or predicted results).

Since the skipping pattern is agreed as the SSB configuration to indicate the timing of NW's SSB transmission, we can consider reusing the existing IE SSB-MTC for its configuration. However, if the periodicity in IE SSB-MTC is doubled (e.g., 40 ms -> 80 ms) for measurement reduction in Case B, the length of measurement period would be also doubled (e.g., N*40ms -> N*80 ms). This effectively relaxes the RAN4 requirement, potentially leading to the HO performance degradation. Note that the measurement period increases proportionally with the SMTC period (the periodicity in IE SSB-MTC), as it is calculated as multiples of the SMTC period according to RAN4 Specification (TS 38.133).

To address this, we can introduce a separate/new IE for the skipping pattern, which would indicate the increased SSB interval while maintaining the measurement period same as in legacy (based on the legacy IE SSB-MTC).

Observation. 1: Increasing the periodicity in IE SSB-MTC for measurement reduction in Case B would also increase the measurement period, effectively relaxing the RAN4 requirement and potentially degrading HO KPI.

Proposal. 11: For inference configuration in temporal domain Case B, RAN2 introduce a separate/new IE for the SSB configuration to indicate the timing of NW’s SSB transmission.

Frequency domain:

During SI phase, for frequency domain prediction, measured and predicted frequency was agreed for inference configuration and the lasted measurement results was agreed for inference report as below.

Agreements (from RAN2#130) - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • Measured and predicted frequency (For inter-freq domain prediction) Agreements on RRM measurement prediction (from RAN2#131) 5. For frequency domain prediction, the inference report can include the latest measurement results of the predicted cell. (No change to the existing measurement report)

From our understanding, the predicted frequency can be implicitly indicated by the target MO (e.g., MeasObject that is associated with reportConfig configured for prediction). On the other hands, the measured frequency can be up to UE implementation. For instance, if NW provides three MOs for frequency f1/f2/f3, and associate the MO for f1 with the reportConfig for inter-frequency prediction. Then, the predicted frequency (f1) can be implicitly indicated by the target MO (i.e., MO for f1). However, the measured frequency (e.g., f2 and/or f3) for which UE performs the measurement to collect input data can be left to UE’s implementation. Therefore, it seems more appropriate for NW to configure “MOs for potential input frequencies” rather than “measured frequency”.

Proposal. 12: For inference configuration in frequency domain prediction, NW can implicitly indicate the predicted frequency through the target MO (e.g., MeasObject associated with reportConfig for prediction).

Proposal. 13: For frequency domain prediction, the measured frequencies for which UE performs measurement to collect input data can be up to UE’s implementation.

Proposal. 14: For inference configuration in frequency domain prediction, NW can provide MO(s) for potential measured frequencies for which UE can perform measurement to collect input data.

As indicated in the WID, frequency domain prediction is supported only for co-located cells. This is because high channel correlation between cells on different frequencies can be expected only when those cells are co-located. Therefore, when UE selects the input cell(s) for measurement prediction for the output cell, it requires information on co-located cells to properly select input cell(s). For that, NW can provide information on co-located cells as part of inference configuration.

Proposal. 15: For inference configuration in frequency domain prediction, NW can provide information on co-located cells.

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Capability reporting:

Proposal. 1: For UE capability, RAN2 discuss granularity and dependency of functionalities/features once sufficient progress has been made on the spec. impact of AI mobility functionalities/features.

Applicability reporting:

Proposal. 2: Associated ID can be relevant for all the use cases of AI/ML mobility (i.e., Temporal domain Case A, Temporal domain Case B, Inter-frequency prediction).

Proposal. 3: Associated ID can be configured either per cell or per frequency (i.e., MeasObject) depending on deployment scenario.

Proposal. 4: The absence of associated ID means UE can assume consistency of SSB beam configuration across different cells and over time.

Inference & reporting:

Common part:

Proposal. 5: For inference configuration, reuse the existing IE (i.e., MeasObjectNR, MeasId, ReportConfigNR).

Proposal. 6: For inference configuration, introduce new fields to configure inference parameters in MeasObjectNR and/or ReportConfigNR IE.

Proposal. 7: For inference configuration, NW can indicate the target MO (Measurement object) for which the UE performs the prediction (e.g., RRM measurement prediction).

Proposal. 8: For inference report, reuse the existing message (i.e., MeasurementReport) and IE (i.e., MeasResultNR).

Proposal. 9: For inference report, introduce new fields to include inference output in MeasResultNR IE if needed.

Temporal domain Case A:

Proposal. 9: For PW length configuration in temporal domain Case A, RAN2 discuss the following two options:

- Option 1 (time duration): NW configures the length of PW as a single time duration value (e.g., xx msec).

- Option 2 (# of instances and time interval): NW configures the number of time instances to predict within PW and the time interval between consecutive time instances (similar to R19 BM case).

Proposal. 10: For list of target cells configuration in temporal domain Case A, RAN2 discuss the following two options:

- Option 1: Reuse the existing list (i.e., allowedCellsToAddModList).

- Option 2: Introduce a new list (e.g., predictedCellsToAddModList).

Temporal domain Case B:

Observation. 1: Increasing the periodicity in IE SSB-MTC for measurement reduction in Case B would also increase the measurement period, effectively relaxing the RAN4 requirement and potentially degrading HO KPI.

Proposal. 11: For inference configuration in temporal domain Case B, RAN2 introduce a separate/new IE for the SSB configuration to indicate the timing of NW’s SSB transmission.

Frequency domain:

Proposal. 12: For inference configuration in frequency domain prediction, NW can implicitly indicate the predicted frequency through the target MO (e.g., MeasObject associated with reportConfig for prediction).

Proposal. 13: For frequency domain prediction, the measured frequencies for which UE performs measurement to collect input data can be up to UE’s implementation.

Proposal. 14: For inference configuration in frequency domain prediction, NW can provide MO(s) for potential measured frequencies for which UE can perform measurement to collect input data.

Proposal. 15: For inference configuration in frequency domain prediction, NW can provide information on co-located cells.