3GPP TSG-RAN WG2 Meeting #132                                  R2-2508427

Dallas, USA,  Nov. 17th -21st, 2025

Agenda item:		9.3.3

Source:				Samsung

Title:					Discussion RRM measurement event prediction

Document for:		Discussion & Decision

1	Introduction

In this paper, we would like to discuss the functionality management of RRM measurement event prediction (i.e., applicability/capability reporting, inference configuration, inference report) according to the following objectives in the latest WID (RP-252899).

- Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release

2	Discussion

2.1 Clarification on measurement event prediction

Based on the discussion so far, the word "Measurement event prediction" has been used with two different understandings/concepts as below.

- Measurement event prediction for early report (with temporal Case A): UE predicts a future measurement event and initiates event-triggered reporting for the predicted event in advance before the actual event.

- Measurement event detection with measurement reduction (with temporal Case B or frequency domain prediction): UE can detect a measurement event and initiate event-triggered reporting as in legacy. The only difference is that some of the measurement results used for the measurement reporting are not the actual measurement results but the predicted ones.

Measurement event prediction for early report (with temporal Case A) can be regarded as temporal domain future measurement event prediction and this is aligned with the understanding during our evaluation phase. In our view, the early measurement event prediction/reporting is a new reporting procedure for which we need to discuss the spec. impact separately from RRM measurement prediction.

Measurement event detection with measurement reduction (with temporal Case B and frequency domain prediction) can be regarded as the combination of legacy event-triggered reporting and measurement reduction. From our understanding, this can be naturally done by configuring the legacy event-triggered reporting together with temporal Case B or frequency domain prediction for measurement reduction. Therefore, we don’t need to study the spec. impact for this separately from RRM measurement prediction.

Observation. 1: The word "Measurement event prediction" has been used with two different understandings/concepts as below.

- Measurement event prediction for early report (with temporal Case A): UE predicts a future measurement event and initiates event-triggered reporting for the predicted event in advance before the actual event.

- Measurement event detection with measurement reduction (with temporal Case B and frequency domain prediction): UE can detect a measurement event and initiate event-triggered reporting as in legacy. The only difference is that some of the measurement results used for the measurement reporting are not the actual measurement results but the predicted ones.

During SI phase, for measurement event prediction, the following agreement was made for inference configuration/report.

Agreements on RRM measurement event prediction (from RAN2#131) 1 For RRM measurement event prediction, the following are included in the inference configuration: - The length of PW for the associated AIML model - Event-related information (event type and event-related parameters) 2 For RRM measurement event prediction, the event prediction report can include: - RRM measurements (For normative phase to define what types of measurements) - Time-related information about the predicted event (e.g., the time/window of occurrence) 3 For measurement event determined, when the UE is configured with temporal domain case B, and frequency domain prediction, UE reports measurement event by following an approach similar legacy procedure. This can be achieved without spec impact for measurement reporting. FFS for normative phase if additional information needs to be added.

From our understanding, the agreement 1 and 2 for inference configuration and report are applicable to Measurement event prediction for early report. And the agreement 3 to reuse the legacy procedure is applicable to Measurement event detection with measurement reduction. Therefore, for measurement event prediction, we can discuss inference configuration/report, focusing on Measurement event prediction for early report (with temporal Case A).

Based on the agreement above,

Proposal. 1: For measurement event prediction, RAN2 discuss inference configuration/report, focusing on measurement event prediction for early report (with temporal domain Case A).

2.2 Inference & reporting

For UE-side model inference, NW can provide some configuration for inference at UE-side. Then, the UE can perform the inference with the input data that is internally available at UE and report the outcome of the inference to NW. For the spec. impact, we can focus on the inference configuration from NW to UE and the report from UE to NW.

During SI phase, RAN2 agreed the inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework. In our view, the existing RRM measurement configuration consisting of MeasObject/MeasId/ReportConfig can be easily extended/enhanced to support the use cases of AI for mobility. Therefore, in this part, we propose some potential enhancement based on the existing RRM measurement configuration.

Inference configuration:

For measurement event prediction, NW can configure UE to initiate the reporting procedure when the entry condition for the target event is predicted to be fulfilled for all measurements during timeToTrigger within PW. For that, NW can include target event (e.g., eventId for eventA3), and PW length in the inference configuration.

For the inference configuration from NW, the following agreement was made during SI phase.

Agreements on RRM measurement event prediction (from RAN2#131) 1 For RRM measurement event prediction, the following are included in the inference configuration: - The length of PW for the associated AIML model - Event-related information (event type and event-related parameters)

In our view, the existing ReportConfigNR IE can be reused for inference configuration for RRM measurement event prediction. Specifically, for event-related information, the existing reportType (i.e., eventTriggered) and eventId (i.e., eventAx) can be reused. For PW length, we can introduce a new field in EventTriggerConfig IE.

Proposal. 2: For inference configuration in measurement event prediction, reuse the existing ReportConfigNR IE.

Proposal. 3: For inference configuration in measurement event prediction, reuse the existing reportType (i.e., eventTriggered) and eventId (i.e., eventAx) for the “Event-related information”.

Proposal. 4: For inference configuration in measurement event prediction, introduce a new field to configure the length of PW in EventTriggerConfig IE.

Inference report:

For measurement event prediction, UE can trigger the event-triggered reporting based on the prediction. Specifically, the UE can initiate the measurement (inference) reporting procedure when the entry condition applicable for the event is predicted to be fulfilled by all measurement results for one or more applicable cells during timeToTrigger.

During SI phase, the following agreement was made for the content of event prediction report.

Agreements on RRM measurement event prediction (from RAN2#131) 2 For RRM measurement event prediction, the event prediction report can include: - RRM measurements (For normative phase to define what types of measurements) - Time-related information about the predicted event (e.g., the time/window of occurrence)

For the RRM measurement results, both the latest measurement result and the predicted results for the multiple time instances within PW can be beneficial to NW for making HO decision. Regarding the applicable cells, UE does not need to include the RRM measurement/prediction results for all detected cells. Instead, UE can report these results only for the cells for which the event is predicted.

Proposal. 5: For measurement event prediction, the UE can report the latest measurement result and/or predicted results for multiple time instances for the applicable cells. The applicable cells can be the cells for which the event is predicted.

Meanwhile, the UE can continue the measurement event prediction even after the first event prediction report, and the results can be updated or changed from “predicted” to “not predicted”. In such cases, the UE should be able to cancel the previous inference report to prevent incorrect HO decisions by NW based on the outdated information. Therefore, we see the need of supporting inference report cancellation.

Proposal. 6: For measurement event prediction, UE can continue prediction after sending the first event prediction report. If the event prediction result is updated compared to the last report, the UE can report the updated result to NW.

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Clarification on measurement event prediction:

Observation. 1: The word "Measurement event prediction" has been used with two different understandings/concepts as below.

- Measurement event prediction for early report (with temporal Case A): UE predicts a future measurement event and initiates event-triggered reporting for the predicted event in advance before the actual event.

- Measurement event detection with measurement reduction (with temporal Case B and frequency domain prediction): UE can detect a measurement event and initiate event-triggered reporting as in legacy. The only difference is that some of the measurement results used for the measurement reporting are not the actual measurement results but the predicted ones.

Proposal. 1: For measurement event prediction, RAN2 discuss inference configuration/report, focusing on measurement event prediction for early report (with temporal domain Case A).

Inference & reporting:

Inference configuration:

Proposal. 2: For inference configuration in measurement event prediction, reuse the existing ReportConfigNR IE.

Proposal. 3: For inference configuration in measurement event prediction, reuse the existing reportType (i.e., eventTriggered) and eventId (i.e., eventAx) for the “Event-related information”.

Proposal. 4: For inference configuration in measurement event prediction, introduce a new field to configure the length of PW in EventTriggerConfig IE.

Inference report:

Proposal. 5: For measurement event prediction, the UE can report the latest measurement result and/or predicted results for multiple time instances for the applicable cells. The applicable cells can be the cells for which the event is predicted.

Proposal. 6: For measurement event prediction, UE can continue prediction after sending the first event prediction report. If the event prediction result is updated compared to the last report, the UE can report the updated result to NW.