3GPP TSG-RAN2 #132	R2-2508432

# Dallas, USA, November 17th - 21st, 2025

Agenda item:	10.3.1.1 (FS_6G_Radio)

Source:	LG Electronics

Title:	Considerations on required functionalities and design objectives for 6G protocol stack

Document for:	Discussion and decision

1.	Introduction

This document discusses the required functionalities, new requirements and design objectives for 6G protocol stack.

2.	Protocol stack for 6G

2.1	Required functionalities and sublayers for 6G User plane

In 5G NR, the user plane protocol stack is comprised of SDAP, PDCP, RLC, and MAC sublayers and each sublayer has its own functions. In the rest of this subsection, it is discussed to see which function in each sublayer is essential and required for 6G UP.

The SDAP was newly introduced in 5G NR to map QoS flows from the 5G core to radio bearers and add QFI information to packets to enable per QoS flow management, then following functions are defined in the SDAP specification.

- mapping between a QoS flow and a DRB for both DL and UL

- marking QoS flow ID in both DL and UL packets

- reflective QoS flow to DRB mapping for the UL SDAP data PDUs

Reflective QoS flow to DRB mapping is anyway sort of mapping between a QoS flow and a DRB and this would be rarely used in real deployment according to company’s online comments. In our view, if mapping between QoS flow and DRB is supported, it is natural to have marking QoS flow ID in a packet as well. Thus, mapping between QoS flow and DRB is almost the only function of SDAP, but this per QoS flow management is still important and should be baseline for 6G to support various application services.

The following functions which were in 5G SDAP are required for 6G UP.

- -	mapping between a QoS flow and a DRB for both DL and UL

- -	marking QoS flow ID in both DL and UL packets

As per the PDCP specification, the following functions are defined:

- maintenance of PDCP SNs

- header compression and decompression using the ROHC protocol

- header compression and decompression using the EHC protocol

- uplink data compression and decompression using the UDC protocol

- ciphering and deciphering

- integrity protection and integrity verification

- timer based SDU discard

- PDU Set discard

- PSI based SDU discard

- for split bearers and DAPS bearer, routing

- duplication

- reordering and in-order delivery

- out-of-order delivery

- duplicate discarding

Among above functions, there should be no doubt to support header compression/decompression, ciphering/deciphering, and integrity protection/verification since those are widely used in the field, but we are not sure whether uplink data compression/decompression is used a lot on the field and it is needed from 6G day-1. It would be good to introduce this function at the later release rather than introducing it from 6G day-1. In addition, maintenance of PDCP SN/timer based SDU discard/reordering/in-order delivery/out-of-order delivery/duplicate discarding are basic functions to support data transfer of user plane/control plane, and all are definitely required. For the duplication, supporting higher reliability should be still important for 6G and this function would be needed for this purpose as well. PDU Set discard/PSI based SDU discard were introduced in Rel-18 XR to handle XR traffic characteristic.  Considering that XR services are one of important use cases in 6G, we think that all XR related enhancements introduced in 5G NR including SN gap report can be adopted. For split bearers and DAPS bearer/routing, it’s too early to discuss this aspect.

The following functions which were in 5G PDCP are required for 6G UP.

- -	maintenance of PDCP SNs

- -	header compression and decompression

- -	ciphering and deciphering

- -	integrity protection and integrity verification

- -	timer based SDU discard

- -	PDU Set discard

- -	PSI based SDU discard

- -	duplication

- -	reordering and in-order delivery

- -	out-of-order delivery

- -	duplicate discarding

- -	SN gap report

The 5G RLC has following functions:

- error correction through ARQ

- segmentation and reassembly of RLC SDUs

- re-segmentation of RLC SDU segments

Even though HARQ is used in the physical layer in 5G NR, ARQ has been used together with HARQ due to NACK-to-ACK error of HARQ. In our view, as the HARQ performance and reliability increased in 6G, ARQ may not be needed in most cases, but there may be limited use cases for lossless transmission, and it would be good to keep ARQ function for 6G UP. It is obvious to support (re)segmentation and reassembly as a basic function to support efficient data transfer.

The following functions which were in 5G RLC are required for 6G UP.

- -	error correction through ARQ

- -	(re)segmentation and reassembly of RLC SDUs

As per the MAC specification, following functions are defined in 5G MAC.

- mapping between logical channels and transport channels

- multiplexing of MAC SDUs from one or different logical channels onto transport blocks (TB) to be delivered to the physical layer on transport channels

- demultiplexing of MAC SDUs to one or different logical channels from transport blocks (TB) delivered from the physical layer on transport channels

- scheduling information reporting

- error correction through HARQ

- logical channel prioritization

- priority handling between overlapping resources of one UE, i.e., intra-UE prioritization

All functions above except “priority handling between overlapping resources of one UE” are essential for data transmission/reception and should be required for 6G UP. Of course, details of each function can be different from 5G NR, but this can be further discussed later phase of 6G SI. However, even though intra-UE prioritization was introduced in 5G to support URLLC and it is still an important use case for 6G, we think that it would be better to introduce this function after defining fundamental functions for data transmission/reception since the exact function would be different depending on what multiplexing/demultiplexing and LCP looks like in 6G.

On top of functions above, it would be good to consider power/energy saving functions from 6G day-1.

- power/energy saving, e.g., DRX/LP-WUS/cell DTX/cell DRX.

Power saving is always an important KPI for both UE and network, but the initial version of the 5G NR does not consider network energy saving and the related network energy saving function is introduced in later release. In addition, several UE power saving features for same purpose, i.e., DRX/DCP/LP-WUS/etc., are introduced over several releases and this makes specifications unnecessarily complicated. We think that RAN2 should learn lessons from this bad experience in 5G NR and 6G should consider both UE and network power saving together from 6G day-1. Thus, power/energy saving function is required for both UE and network from 6G day-1, but intra-UE prioritization function should not be considered from 6G day-1.

The following functions which were in 5G MAC are required for 6G UP.

- -	mapping between logical channels and transport channels

- -	multiplexing and demultiplexing of MAC SDUs

- -	scheduling information reporting

- -	error correction through HARQ

- -	logical channel prioritization

- -	power/energy saving for both UE and network

Based on the explanation and proposals above, the next discussion should be which layers are needed for 6G UP to support these required functions. The 5G NR UP has MAC, RLC, PDCP, and SDAP sublayers, but we doubt whether RLC and SDAP sublayers are needed for 6G.

When evolved from 4G LTE to 5G NR, the main function of RLC was shrunk and simplified to support higher data rate. More specifically, the reordering function is moved to the PDCP, and the concatenation function is moved to the MAC. The following two main functions have remained for the 5G RLC, but we don’t think that keeping these functions in the RLC is valuable for 6G.

- Error correction through ARQ;

- (re)segmentation and reassembly of RLC SDUs.

The ARQ was introduced in UMTS Rel-99. Later on, HARQ was introduced in HSDPA in Rel-5, and two similar functions have worked together from then on. However, the HARQ performance keeps going on, and now the ARQ becomes a third wheel. With the increased performance of HARQ in 6G, we think ARQ is not needed in most cases, and very limited use of ARQ for lossless transmission can be incorporated into PDCP. In this situation, keeping segmentation and reassembly in RLC would not bring meaningful gain, rather it causes large overhead since the RLC should have its own sequence number and window only for segmentation and reassembly. Considering that the RLC is not the only place to have segmentation and reassembly, it would be better to merge segmentation and reassembly functions into PDCP together with ARQ from overhead reduction perspective. This can also remove duplicated functions between RLC and PDCP such as SN allocation and window management. Then, we can remove the RLC in 6G.

In addition, we think SDAP can also be removed in 6G. As explained above, mapping between QoS flow and DRB is almost the only function of SDAP. We think such a simple function can also be incorporated into PDCP. Thus, the RLC and SDAP can be removed for 6G, and the necessary functions in those layers can be integrated into PDCP.

Remove RLC and SDAP, and the required functions which were in RLC and SDAP are integrated into PDCP.

2.2	Separate path for control signal and user data

In 5G NR, L2 control signal (i.e. PDCP Control PDU, RLC Control PDU, MAC CE) and L2 user data are transmitted through a same logical channel or a same MAC PDU, and this causes the HOL (Head-of-Line) blocking issue between control signal and user data. On the transmitting side, PDCP Control PDUs are handled in the same DRB with PDCP Data PDUs. A PDCP status report is submitted to lower layers as the first PDCP PDU for transmission, still it is processed in the same DRB. RLC sublayer operates in a similar manner. RLC Control PDUs are prioritized over data PDUs, but they are processed in the same logical channel with the data PDUs. Therefore, urgent user data may be blocked by control PDUs belong to the same DRB or the same logical channel or vice versa. It is a kind of HOL blocking issue on a DRB or a logical channel.

At the MAC sublayer, multiple logical channels are merged into a single UL-SCH transport channel for a serving cell, and UL grant to transmit on the UL-SCH are allocated to one or more logical channels based on PBR and logical channel priorities. Therefore, a PDCP Control PDU and an RLC Control PDU may be blocked by user data belonging to another logical channel. Conversely, user data may be blocked by control PDUs belonging to anther logical channel. In addition, user data may be blocked by MAC CEs because most MAC CEs are prioritized over user data by uplink scheduling, and the uplink resources are allocated to MAC CEs first. These are a kind of HOL blocking issues on the transport channel.

The above-mentioned HOL blocking issues between control signal and user data make it difficult to provide user data with QoS scheduling and to deliver control signal to the network in time.

To resolve the HOL blocking issue on L2 transmission structurally, it needs to introduce separate L2 transmission path for L2 control signal and L2 user data as shown in Fig. 2.2.1. L2 control signals such as PDCP Control PDUs and MAC CEs are transmitted using a MAC PDU which are separated from ones for user data. A HARQ process dedicated to the separate MAC PDU for control signal is also considered, and distinct handling for control signal and user data can be provided at the level of HARQ processing. To summarize, the following can be considered in 6G.

- Separate L2 transmission path for L2 control signal and L2 user data, e.g.

  - Separate HARQ process

  - Separate MAC PDU

Fig. 2.2.1. Separate transmission path for L2 control signal and user data

In addition, the separation of control signal and user data can also be considered for UL grant. If UL grants dedicated to control signal or user data are provided, this will offer a knob to schedule either urgent control signal or time-critical user data on the transport channel and the physical layer. In short, the following can be considered in 6G.

- Separate UL grant for L2 control signal and L2 user data, e.g.

  - UL grant dedicated to MAC CE

  - UL grant dedicated to user data

Design separate path for L2 control signal and user data.

3. Conclusion

Based on the above discussions, we present the following proposals:

- The following functions which were in 5G SDAP are required for 6G UP.

- -	mapping between a QoS flow and a DRB for both DL and UL

- -	marking QoS flow ID in both DL and UL packets

The following functions which were in 5G PDCP are required for 6G UP.

- -	maintenance of PDCP SNs

- -	header compression and decompression

- -	ciphering and deciphering

- -	integrity protection and integrity verification

- -	timer based SDU discard

- -	PDU Set discard

- -	PSI based SDU discard

- -	duplication

- -	reordering and in-order delivery

- -	out-of-order delivery

- -	duplicate discarding

- -	SN gap report

The following functions which were in 5G RLC are required for 6G UP.

- -	error correction through ARQ

- -	(re)segmentation and reassembly of RLC SDUs

The following functions which were in 5G MAC are required for 6G UP.

- -	mapping between logical channels and transport channels

- -	multiplexing and demultiplexing of MAC SDUs

- -	scheduling information reporting

- -	error correction through HARQ

- -	logical channel prioritization

- -	power/energy saving for both UE and network

Remove RLC and SDAP, and the required functions which were in RLC and SDAP are integrated into PDCP.

Design separate path for L2 control signal and user data.