3GPP TSG RAN WG2 Meeting #132                            R2-2508438

Dallas, USA, Nov. 17th - 21st

Agenda Item:	10.4

Source:	     KDDI Corporation

Title:	Views on Mobility for 6GR

Tdoc Type:	Discussion

Document for: 	Discussion and decision

- Introduction

At the RAN#108 meeting, new SID on Rel-20 study of 6G Radio was approved [1], and at the RAN#109 meeting, the SID was revised [2]. Mobility for 6GR is one of the objectives as follows:

(4) Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

In this contribution, we provide our initial views on mobility for 6GR in association with the guidance provided by RAN2 chair lady as follows [3]:

10.4	 Mobility

Mobility framework and targets, Measurement Framework, Intra-RAT Connected Mobility (e.g. L3, CHO, LTM, RLM/RLF), Cell Selection and Reselection, Inter-RAT

- Discussion

2.1 Mobility framework

Consideration of a Unified Mobility Framework and AI/ML Integration

The introduction of multiple connected mode mobility mechanisms in 5G NR (such as L3 HO, CHO, DAPS, and LTM) across different releases has resulted in complexity, redundancy, and inconsistent implementation at both the UE and network sides. This fragmentation leads to significant implementation load for Working Groups and increased CAPEX for operators. To address this, 6G design mandates system simplification. Therefore, RAN2 should study consolidating procedures like L3 HO, LTM, and conditional mobility into a single design. This unified framework is necessary to achieve the stringent latency and reliability requirements set for emerging applications. This single framework should support both conditional and immediate execution.

Furthermore, AI/ML is central to the 6G vision. AI/ML mobility features, such as RRM prediction, measurement event prediction, and RLF prediction, should be integrated into the 6G mobility framework from the outset. This integration aims to enhance mobility performance, including improving the accuracy of pre-configuration and reserved resources for schemes like CHO and LTM. The outcomes derived from the 5G-Advanced Rel-20 AI/ML mobility study should serve as the baseline for these enhancements in 6G.

Proposal 1: Study the feasibility of a unified framework for connected mode mobility procedures introduced in 5G NR, based on common RRC containers, signaling, and procedure descriptions. Furthermore, investigate the integration of AI/ML aspects (e.g., RRM prediction, measurement event prediction, RLF prediction) into the 6G Mobility/RRM framework, taking outcomes from 5G-Advanced Rel-20 AI/ML mobility work as a baseline.

Day-1 Support for Multi-TRP Functionality in 6GR Unified Mobility Framework

In 6G environments, particularly with the proliferation of multi-antenna and dense deployments, the concept of "Cell-free" networking is anticipated. The rise of cell-free concepts and Multi-TRP deployments makes the distinction between mobility and MIMO beam management less meaningful. Mobility procedures must be designed from Day 1 to natively accommodate Multi-TRP/cell-free deployments. By integrating Multi-TRP support directly into the unified framework, the system can improve mobility robustness and minimize issues observed in 5G, such as ping-pong effects and coverage shrinkage, especially at the cell edge. A harmonized connected-mode mobility procedure is required to reduce redundant and duplicated procedures between MIMO beam management and mobility.

Proposal 2: Day-1 support for Multi-TRP functionality (e.g., utilization of Multi-TRP for cell/beam level mobility) should be mandated within the 6GR unified mobility framework to minimize ping-pong effects and coverage shrinkage at the cell edge.

Day-1 Adoption of Low-Latency Enhancements (Early UL/DL Sync, RACH-less HO) in LTM

Minimizing latency and service interruption time is one of the most important targets for 6GR mobility. The ideal target for mobility interruption time is 0 ms. Features pioneered in 5G LTM, such as early synchronization/preparation and RACH-less Handover, have proven effective in reducing HO latency and minimizing interruption time. The unified 6G mobility framework (as proposed in Proposal 1) must ensure Day-1 support for these low-latency techniques, including both UL and DL pre-synchronization. Implementing RACH-less HO universally for relevant deployments further contributes to minimizing access latency. Introducing low-latency features from 6G Day 1 is crucial.

Proposal 3: Aiming to minimize interruption time and ensure service continuity in the initial release, 6GR should adopt the robust, low-latency functional enhancements of 5G NR LTM, such as Early UL/DL Synchronization and RACH-less Handover, as Day-1 features.

2.2 Measurement Framework

Examination of a Unified Measurement Framework for L1 and L3 Measurements

In 5G NR, L3 RRM measurements and L1 CSI/beam measurements operate under largely separate frameworks. This design deficiency leads to redundant measurement behaviors, duplication of reference signal configurations (e.g., SSB configurations), and unnecessary complexity in UE implementation and network configuration. To ensure a concise, efficient, and scalable measurement foundation for 6G, RAN2 should study implementing a single unified measurement framework. This framework should define measurement criteria, report quantities, and reference signal configurations (e.g., SSB, CSI-RS) within a unique configuration entity in the RRC structure, minimizing configuration duplication. Furthermore, this framework should study the reporting mechanisms, supporting reports via MAC CE or RRC signaling, to streamline procedures.

Proposal 4: RAN2 should study supporting a single measurement framework that unifies L1 measurements (for CSI/Beam Management) and L3 measurements (for RRM/Mobility), including consistent measurement resource configuration structure, report types, and reporting paths (via MAC CE or RRC signaling).

Investigation of Adaptive and Dynamic Measurement Gaps in Coordination with RAN4

The Measurement Gap (MG) feature in 5G NR has become increasingly complex due to the addition of various sub-features across releases. Furthermore, RRM measurements, especially for inter-frequency measurements, consume significant UE power, impacting overall energy efficiency. Fixed measurement gap periods often lead to suboptimal trade-offs, causing either under-measurement (risking RLF/HO failure) or over-measurement (wasting energy). 6G mandates energy-efficient design. RAN2 should therefore study methods to reduce measurement overhead and scheduling limitations compared to 5G. This involves evaluating the feasibility of simplification and scalability for MG dynamics (e.g., configuration, handling). Specific enhancements to be considered include studying non-uniform gap patterns and dynamic network-controlled gap use to optimize the mobility/power efficiency trade-off. Since measurement gap design is closely tied to physical layer requirements, coordination with RAN4 is essential.

Proposal 5: Study adaptive and dynamic measurement gaps to optimize the trade-off between UE mobility and power efficiency in coordination with RAN4.

Examination of Uplink Quality Measurement/Integration into Mobility Procedures

In current 5G systems, mobility decisions rely predominantly on downlink (DL) measurements, meaning the immediate uplink (UL) quality after a cell switch must be estimated indirectly. For future 6G services, such as uplink-heavy interactive AI applications, ensuring robust UL quality immediately post-mobility is crucial. RAN2 should study whether UL quality after mobility can be utilized. Studying UL-based mobility, perhaps utilizing UL Reference Signals like SRS, for connected mode could potentially reduce UE power consumption compared to constant DL measurements and reporting, especially in dense network deployments.

Proposal 6: RAN2 should investigate the feasibility of UL measurements (e.g., based on SRS) and the integration of UL quality information into mobility procedures to ensure Uplink quality post-mobility.

2.3 Pain Point Scenarios

Day-1 Introduction of Features for High-Speed Mobile Environment Adaptation

Operational experience in high-speed scenarios (e.g., high-speed railways) in 5G demonstrated that these environments require extensive manual tuning of mobility parameters (like Time-to-Trigger (TTT) or hysteresis). Static parameterization performs poorly across varying velocities, leading to operational burdens, late/early handovers, or ping-pong effects. Since high-speed scenarios risk congestion if low-mobility UEs camp on dedicated cells, 6GR should adopt adaptive features from Day 1 to improve user experience in these pain points. This includes supporting mechanisms to adaptively change the values of mobility-related parameters based on mobility state. Furthermore, reusing LTE-based mechanisms, such as handover count-based mobility state estimation, is an agreed-upon mechanism for adaptation.

Proposal 7: Study the introduction of adaptive features (e.g., ability to change mobility-related parameter values) as mandatory 6GR Day-1 functions to adapt effectively to high-speed mobile environments.

- Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1: Study the feasibility of a unified framework for connected mode mobility procedures introduced in 5G NR, based on common RRC containers, signaling, and procedure descriptions. Furthermore, investigate the integration of AI/ML aspects (e.g., RRM prediction, measurement event prediction, RLF prediction) into the 6G Mobility/RRM framework, taking outcomes from 5G-Advanced Rel-20 AI/ML mobility work as a baseline.

Proposal 2: Day-1 support for Multi-TRP functionality (e.g., utilization of Multi-TRP for cell/beam level mobility) should be mandated within the 6GR unified mobility framework to minimize ping-pong effects and coverage shrinkage at the cell edge.

Proposal 3: Aiming to minimize interruption time and ensure service continuity in the initial release, 6GR should adopt the robust, low-latency functional enhancements of 5G NR LTM, such as Early UL/DL Synchronization and RACH-less Handover, as Day-1 features.

Proposal 4: RAN2 should study supporting a single measurement framework that unifies L1 measurements (for CSI/Beam Management) and L3 measurements (for RRM/Mobility), including consistent measurement resource configuration structure, report types, and reporting paths (via MAC CE or RRC signaling).

Proposal 5: Study adaptive and dynamic measurement gaps to optimize the trade-off between UE mobility and power efficiency in coordination with RAN4.

Proposal 6: RAN2 should investigate the feasibility of UL measurements (e.g., based on SRS) and the integration of UL quality information into mobility procedures to ensure Uplink quality post-mobility.

Proposal 7: Study the introduction of adaptive features (e.g., ability to change mobility-related parameter values) as mandatory 6GR Day-1 functions to adapt effectively to high-speed mobile environments.

- References

[1] RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator), RAN#108, June 2025.

[2] RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO (Moderator), RAN#109, September 2025.

[3] R2-2508001, “Agenda for RAN2#132”, (Chair: InterDigital)