3GPP TSG-RAN WG2 Meeting #132	R2-2508448

Dallas, USA,  Nov. 17th - 21st

Agenda item:	10.2.2

Source:	Apple

Title:	Views on 6G NTN

WID/SID:	FS_6G_Radio

Document for:	Discussion / Decision

# Introduction

According to the objectives in 6GR SI (RP-252912), in 6GR we aim at a harmonized 6G radio design for TN and NTN.

In RAN2#131bis meeting, RAN2 did some initial discussion on how to approach 6G NTN study to ensure the harmonized design and made the following plan.

For next meeting companies should identify essential requirements for NTN and identify which functionality should be considered in the common design with TN and what are NTN specific and whether they are essential.

This contribution gives our initial views on 6G NTN study from the following aspects:

- 6G NTN deployment, scenario, and architecture

- 6G NTN design principle

- 6G NTN specific requirement and RAN2 directions

- Way forward on RAN2 6G NTN study.

# Background

NTN has been extensively studied in 3GPP from R15: the research on satellite access began in R15, NTN and IoT NTN were introduced in R17, and further improved in various aspects from performance, architecture, and service in subsequent releases.

Figure-1. 5G NTN evolution

For both NR NTN and IoT NTN, although 3GPP has made good research work, the commercial NTN deployment is still immature. The good news is that in recent years, more and more operators have started the trial NTN networks and plan the commercial NTN deployments.

Observation 1:  3GPP made the good research work on NR NTN and IoT NTN in 5G time window, but the commercial deployment is still at the initial stage.

# 6G NTN study

## Deployment, Scenario and RAN Architecture

NTN is supported from 6G Day 1 is to build the ubiquitous 6G network. Considering the NTN specific characteristics (i.e. large propagation delay, wide cell coverage, capacity limitation, energy limitation), TN can provide UE with rich services and better performance compared to NTN. Therefore, in areas where TN and NTN provide overlapping coverage, TN needs to be prioritized to be selected, while NTN provides supplementary coverage to TN and provides the service to UE when TN service is not available.

Proposal 1: The typical 6G NTN deployment scenario is to provide the supplementary coverage when TN service is not available.

Regarding the NTN specific architecture and deployment, the following architecture and deployment were introduced in 5G after the careful study, and there seems no new commercial deployment scenarios, we think all NR NTN specific deployment scenarios, architectures, cell types, and overall designs can be directly inherited into 6G.

| Deployment scenario | Support LEO, MEO, GEO satellite deployments |
| --- | --- |
| Architecture | Support transparent payload architecture and regenerative payload architecture |
| Cell Type | Support earth-moving cell, Quasi-earth-fixed cell, and earth-fixed cell |

Figure-2. NTN Payload Architecture

Proposal 2: 6G NTN supports all NR NTN specific deployment scenarios (i.e. LEO/MEO/GEO) and architectures (i.e. transparent payload architecture and regenerative payload architecture), and cell type (earth-moving cell, Quasi-earth-fixed cell and earth-fixed cell).

## Design Principle

According to 6GR SID, 6G study aims to provide harmonized 6G radio design for TN and NTN, including integration.

5G NR NTN design is based on 5G TN design, introducing NTN specific designs which are required to meet NTN requirements. Therefore, 6G NTN design should also use the same approach.

However, there are two differences between the current situation of 6G NTN study and the introduction of 5G NTN in R17:

- Regarding the common design (i.e. TN design)

It’s unclear for 6G NTN because all 6G study has just begun; however, this part was already quite clear when the 5G NTN was introduced in R17.

- Regarding the NTN specific design

The NTN specific characteristics/requirements are the same in both 6G and 5G, so this part is relatively clear for 6G NTN; however, the initial discussions on 5G NTN started from scratch.

Figure-3. Composition of 6G NTN design

Proposal 3: 6G NTN design consists of 6G common TN/NTN design and 6G NTN specific design, and NTN specific design is to meet the NTN specific requirements.

## NTN specific requirements and RAN2 directions

Assuming 6G design is based on the requirements from commercial network deployment, since 5G NR NTN features supported in 3GPP are far ahead of the NTN commercial situation, there seems to be no urgent new requirements for a new 6G NTN at present. The design of 5G NTN seems to be able to meet the demand of 6G NTN Day 1.

Proposal 4: 6G NTN design at least meets 5G NR NTN requirements, and 5G NR NTN design is a good starting point.

Table-1 provides the NTN specific requirements, and the corresponding 5G NTN design, and whether to inherit to 6G.

Table-1: Analysis on NTN specific challenges/requirements and NTN specific design

From RAN2 perspective, the following RAN2-led directions can be considered for 6G NTN Day 1 study:

| Direction | Description (5G NTN design as baseline) |
| --- | --- |
| NTN specific UP design | - HARQ operation enhancement - HARQ without feedback - extension of the number of HARQ process |
| NTN-NTN mobility | - NTN specific mobility condition (e.g. location based and time based condition) - The mobility for all NTN cell types (e.g. earth-fixed, earth-moving cell) - Inter-satellite specific mobility (e.g. satellite switch with resync) |
| TN-NTN mobility | - NTN->TN - TN area based neighbor TN measurement and mobility - TN->NTN - Provides NTN satellite information via TN cell |
| Area based control | - UE location-based access control (NAS feature) - PWS geofencing |
| Beam hopping deployment | - Single-beam cell (supported in NR NTN) - SSB periodicity extension - SMTC pattern based on UE location - Multiple-beam cell (not supported in NR NTN) - FFS, pending on RAN1 SSB design |

    - Proposal 5: Following RAN2-led directions shall be considered for 6G NTN Day 1 study:

- HARQ enhancement (e.g. HARQ without feedback, extension of the number of HARQ process)

- NTN-NTN specific mobility design (e.g.  location/time-based condition, mobility for earth moving cell, satellite switch with resync)

- TN-NTN mobility (e.g. TN area-based mobility, NTN satellite info provision in TN cell)

- Design to support the beam-hopping deployment (e.g. at least for single-beam cell model)

- PWS geofencing design (e.g. area-based broadcast provision).

## Way forward on 6G NTN study in RAN2

    - Based on 5G NR NTN experience, NTN study spreads across a broad range, including control plane, user plane, mobility and positioning. And it involves tight coordination with other working groups from RAN1/RAN2/RAN4, SA1/SA2/SA3/SA4, and CT1, etc.

Moving forward for 6G NTN study, we see two options for RAN2 to organize the discussion:

- Option 1: To discuss NTN specific design in CP/UP/Mobility session

- Option 2: To discuss NTN specific design in NTN specific session

As said above, since the 6G NTN specific design only focus on the NTN requirement, the study covers  multiple aspects and requires the coordination with other WGs, we think it’s better to have a separate NTN section to make the discussion more efficiently. Another point is since 5G-A NTN is about to start, it would be also helpful to coordinate the discussion between 5G-A NTN and 6G NTN during the meeting.

    - Proposal 6: RAN2 is proposed to setup the 6G NTN section to organize 6G NTN study.

# Conclusions

This contribution presents our views on 6G NTN, and we have the following proposals:

Observation 1:  3GPP made the good research work on NR NTN and IoT NTN in 5G time window, but the commercial deployment is still at the initial stage.

Proposal 1: The typical 6G NTN deployment scenario is to provide the supplementary coverage when TN service is not available.

Proposal 2: 6G NTN supports all NR NTN specific deployment scenarios (i.e. LEO/MEO/GEO) and architectures (i.e. transparent payload architecture and regenerative payload architecture), and cell type (earth-moving cell, Quasi-earth-fixed cell and earth-fixed cell).

Proposal 3: 6G NTN design consists of 6G common TN/NTN design and 6G NTN specific design, and NTN specific design is to meet the NTN specific requirements.

Proposal 4: 6G NTN design at least meets 5G NR NTN requirements, and 5G NR NTN design is a good starting point.

    - Proposal 5: Following RAN2-led directions shall be considered for 6G NTN Day 1 study:

- HARQ enhancement (e.g. HARQ without feedback, extension of the number of HARQ process)

- NTN-NTN specific mobility design (e.g.  location/time-based condition, mobility for earth moving cell, satellite switch with resync)

- TN-NTN mobility (e.g. TN area-based mobility, NTN satellite info provision in TN cell)

- Design to support the beam-hopping deployment (e.g. at least for single-beam cell model)

- PWS geofencing design (e.g. area-based broadcast provision).

    - Proposal 6: RAN2 is proposed to setup the 6G NTN section to organize 6G NTN study.

# References

- RP-252912	Study on 6G Radio