3GPP TSG-RAN WG2 Meeting #132	R2-2508449

Dallas, USA,  Nov. 17th - 21st

Agenda item:	10.3.2.1

Source:	Apple

Title:	Views 6G RRC state

WID/SID:	FS_6G_Radio

Document for:	Discussion / Decision

# Introduction

According to 6GR SID (RP-252912), the control plane design is one of the RAN2-led objectives, which includes RRC state.

The RRC state model was discussed in RAN2#131bis meeting, and some preliminary agreements were reached.

Agreements Continue study - Idle mode – - Connected mode – - Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW. Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements. We can consider the pros/cons of different modelling, after having some discussions on the functionality.

This contribution gives our initial views on 6G RRC state from the following aspects:

- Design principle

- 6G Inactive mode

# 6G RRC state

## Design principle

For NR RRC states, the key characteristics and difference are summarized as follows:

In 6G, the RRC state design needs to support the following 3 scenarios:

For scenario 1 and 2, RRC states corresponding to NR RRC_IDLE (1) and RRC_CONNECTED states are required.

For scenario 3, the RRC state can be designed like either NR RRC_IDLE (2) or RRC_INACTIVE state.

In NR, NR RRC_INACTIVE state was introduced to meet the CP latency requirement, because NAS connection is maintained in this state, and if there is data activity, only AS connection needs to be resumed. However, this state is not well deployed in commercial network, and we understand that there are several reasons for this:

- Restricted by 5G NSA architecture deployment

Many networks are NSA architecture, which prevents the deployment of the RRC_INACTIVE state.

- The same scenario can be supported by other RRC state (i.e. RRC_IDLE) that must be supported by UE and network, but the performance is not good.

The RRC_IDLE state must be supported by NW and UE, but RRC_INACTIVE state is just mandatory for UE but not for network.

When UE has no data activity for long time, network will release UE to RRC_IDLE state instead to RRC_INACTIVE state.

When UE has sparse small data traffic, network will cause UE to transition between RRC_CONNECTED and RRC_INACTIVE state for small data transmission. This is not an energy-efficient approach for UE.

But since there is no UE PS and CP latency requirements in the commercial network, operators are not very enthusiastic about deploying RRC_INACTIVE state.

- Restricted by the lack of deployment of Xn interface (NOTE: already resolved in later release)

The initial design to support RRC_INACTIVE state required the deployment of Xn interface between RAN nodes, which was difficult for some network vendors and thus affected the RRC_INACTIVE state deployment.

However, in later release, the Xn restriction has been lifted, and the RRC_INACTIVE state can also be supported when only Ng interface is available.

According to the description of 6G SID, 6G Day 1 design focuses on supporting the native energy saving design for both network and UE based on 6G standalone network. In other words, none of the factors mentioned above that prevented the deployment of the NR RRC_INACTIVE state feature in 5G are valid.

Observation 1: The reasons that prevented the deployment of RRC_INACTIVE state in 5G network (e.g. RAN architecture, network interface, energy efficiency principle) no longer exist under the design principles of 6G.

The RRC_IDLE state will be supported in 6G at least in the UE initial power on phase. To avoid network relying on RRC_IDLE state to handle no data activity scenario, and to ensure the 6G inactive state/mode can be widely deployed in Day 1 deployment, it is best to force only one state/mode to be supported in this situation.

Observation 2: When the same scenario can be supported by RRC_IDLE and RRC_INACTIVE state, it is very likely that RRC_INACTIVE state will be ignored in commercial deployment.

Regarding the CP latency requirement defined in TR38.914(as below), it can only be met by NR RRC_INACTIVE state like design. From simplicity perspective, we donot need to have two RRC states to address the same scenario.

Therefore, when network has UE context, but UE has no data activity, we should focus on using only one state to support scenario 3 while meeting the 6G CP latency requirement, which can be designed based on NR RRC_INACTIVE state.

Observation 3: Only NR RRC_INACTIVE like state can meet the 6G CP latency requirement.

Proposal 1: 6G RRC state should avoid the multiple RRC states designed for the same/similar scenario (e.g.  when network has UE context and UE has no data activity, both IDLE and INACTIVE state are supported in 5G).

Proposal 2: RAN2 study on 6G RRC state focus on supporting only one state for each of the following scenarios:

- 1) When network has no UE context, UE is in RRC IDLE state, and NR RRC_IDLE state is starting point.

- 2) When network has UE context and UE has data activity, UE is in RRC_CONNECTED state, and NR RRC_CONNECTED state is the starting point.

- 3) When network has UE context and UE has no data activity, UE is in 6G inactive mode, and NR RRC_INACTIVE design can be the starting point.

Figure 1: Illustration of the scenarios and 6G RRC states

## 6G inactive mode

Regardless of whether it a RRC state or a sub-mode under RRC connection, let’s first focus on the functionalities of this new 6G inactive mode. Our analysis of the functions that need to be supported in this mode are as follows:

- Mobility

UE has no data activity in this mode. Therefore, from both UE and NW energy efficiency perspective, we should avoid any UE-NW interaction, and UE based mobility is supported.

To ensure that 6G INACTIVE UEs are reachable in network side especially for the DL data arrival case, network needs to know approximately where UE is located so that it can determine the area to page the UE.

When UE moves within this area, it doesn't need to notify network; however, when UE moves out of this area, it needs to notify the network.

Regarding the settings for this area, if the area is smaller, network paging area will be smaller which is good from network energy efficiency perspective, but the frequency of UE notifications to the network will be higher, which is not good for UE power. Conversely, if the area is set too large, network paging efficiency will decrease, but UE power performance will be better. It's ultimately a trade-off between UE and network, and we can rely on network configuration to manage this.

Observation 4: The size of the mobile area setting in 6G inactive mode has opposite energy effects on UE and network.

When UE is not out of the mobility area, UE may still out of service in some abnormal cases (e.g. in coverage hole). If network cannot detect this, the network may continuously page the UE (triggered by DL data/signaling), causing unnecessary waste of resources. Therefore, we need to inherit the periodic notification method (i.e. periodical RNAU procedure) from NR to let the network know whether the UE is still reachable.

Observation 5: Periodical notification from UE to network is needed to handle the abnormal case.

Overall, the mobility in 6G inactive mode can directly inherit the NR RRC_INACTIVE mobility design and consider the NR RNA and RNAU design as the starting point.

Proposal 3: The mobility in 6G inactive mode can inherit the NR RRC_INACTIVE mobility design, i.e. NR RNA design and RNAU procedure can be the starting point.

- Paging/LPWUS simplification/enhancement

In NR, same paging mechanism is applied for the UE in both RRC_IDLE and RRC_INACTIVE state; but to avoid the RRC state mismatch between UE and network, UE is required to monitor both CN initiated and RAN initiated paging message.

In 6G, when UE context is stored in network, but UE has no activated dedicated AS connection, we expect the UE to be in only one RRC state (i.e. 6G inactive mode). Based on this assumption, UE only needs to monitor for one type of paging (i.e. RAN initiated paging).

Observation 6: Assuming UE is only in 6G inactive mode when network has UE context, but UE has no activated dedicated connection, UE in 6G inactive mode only monitors one type of paging.

The 6G paging design should be the unified WUS and subgroup based paging design. Under this paging reception framework, UE in 6G inactive mode can figure out whether the paging is for itself using the following methods:

- If the paging subgroup contains more than one UE

UE monitors WUS signal. If the WUS signal carries the subgroup ID to which the UE belongs, UE monitors the paging in the associated paging occasion and confirm whether the paging message is for it based on the UE ID contained in the paging message.

- If the paging subgroup contains only one UE

UE monitors WUS signal. If the WUS signal carries the UE specific subgroup ID, UE can confirm the paging is for it and UE does not need to monitor paging.

As the subgrouping can be calculated based on UE ID or configured by network explicitly. For 2), it can be achieved if network configure a specific subgroup just for a particular UE. Therefore, we can say that both 1) and 2) can be supported based on the subgrouping configuration by network.

However, if only 2) is supported, relying solely on subgrouping from each single UE brings a significant challenge to the capacity of WUS signal. This is not feasible for the NR-based WUS signal design and evaluation; and for 6G, the feasibility needs to be evaluated by RAN1 first.

      - Proposal 4: In 6G inactive mode, the unified WUS and subgroup based 6G paging scheme with inactive UE ID is used to identify the UE in inactive mode. If the paging subgroup contains only one UE, UE is only required to monitor WUS and no need to monitor paging.

- Fast state transition

The state transition latency from 6G inactive mode to 6G connected mode is analyzed based on NR RRC resume procedure. Since NAS connection is kept in 6G inactive mode, this transition only needs to consider the RRC connection resume procedure, not NAS establishment procedure. Therefore, compared to the IDLE to CONNECTED state transition, this transition is faster. Therefore, we first need to confirm that the fast state transition only requires the AS connection resumption but does not need to include NAS connection establishment procedure.

Proposal 5: The fast state transition between 6G inactive mode and connected mode should not involve NAS connection establishment procedure.

Latency analysis of the state transition based on NR RRC connection resume procedure mainly consider three parts, as shown in Figure-2:

- Latency#1: The first UL transmission latency

- Latency#2: UE context relocation latency

- Latency#3: The latency of the whole RRC Resume procedure.

Figure 2: Illustration of the state transition from inactive to connected state

(based on NR RRC resume procedure procedure)

- Latency#1: The first UL transmission latency from when data arrives to when it’s transmitted to network.

In NR RRC_INACTIVE state, the first UL transmission is performed via CBRA, and in SDT, the first UL transmission can be also configured via the UE dedicated CG resource in no cell change case.

- For the transmission via CBRA, since it’s the contention-based transmission, whole CBRA latency may be a little longer consider the potential preamble retransmission case.

- For transmission via UE dedicated CG resource, there is some restriction to apply it. It’s only applied for SDT and not supported in mobility scenario.

In 6G, some companies proposed to consider the first uplink transmission via UE dedicated resource, i.e. CFRA and CG resource.

Using the UE dedicated resource can shorten the latency of the first uplink transmission. However, if each UE is allocated a dedicated resource, RAN1 should first evaluate whether the resource capacity is feasible.

If mobility is considered, and if this UE dedicated resource can be used across cells, the capacity challenge will be even greater. Furthermore, the feasibility of resource negotiation between RAN nodes needs to be assessed by RAN3.

Observation 7: In 6G inactive mode and in mobile scenario, the first uplink transmission based on UE-dedicated resources can shorten the latency of the first data transmission, but this depends on the feasibility of the resource capacity and network negotiation, which requires the evaluation by RAN1 and RAN3.

- Latency#2: The latency in network for UE context relocation in inter-RAN node case

This delay will not occur when UE resumes the connection and performs data transmission in a cell under a RAN node that already has the UE context.

To reduce this latency and avoid the UE context fetch procedure occurring when data needs to be transmitted, network can transfer the UE context to the RAN node belonging to the latest UE location based on the RNA U procedure.

Because such operation (i.e. UE context relocation triggered RNAU procedure) is up to network implementation in NR, we understand that the network implementation can also handle this in 6G.

Observation 8: To avoid the state transition latency due to the UE context relocation, network implementation can perform the UE context relocation in advance which is based on RNAU procedure.

- Latency#3: The latency of RRC resume procedure

In NR INACTIVE to CONNECTED state transition, the user data transmission is only performed after RRC Resume procedure is completed, and during the RRC Resume procedure, network needs to use the UE ID to perform the UE identification and verification, use the resume cause to perform the access control, and use the MAC-I info to perform the security verification.

Due to the RRC resume procedure may takes 10s ms, companies would like to consider transmitting data in the first UL transmission directly in 6G to avoid delays caused by the RRC resume procedure.

If the RRC procedure latency is the concern and consider the state transition via lower layer signaling or data transmission, the following essential information in NR RRC Resume Request message still needs to be carried in the data transmitted first uplink transmission for the UE identification, access control and security protection purpose:

- UE-ID

- Resume cause

- MAC-I.

Observation 9: For the first uplink transmission in 6G inactive mode, whether via L3 or L2 signaling, needs to carry UE-ID, resume cause, and MAC-I information.

Proposal 6: In 6G state transition from inactive mode to connected mode, NR RRC resume procedure can be taken as baseline, and the following aspects need to be further study:

- The first UL transmission using the UE dedicated resource in mobility scenario require the evaluation by RAN1 (from capacity perspective) and RAN3 (from network coordination feasibility perspective).

- The first uplink transmission using the lower layer signaling but carrying the essential information (UE ID, cause, MAC-I) requires the further evaluation from the feasibility and necessity perspective.

In NR, the CONNECTED to INACTIVE transition is only via RRCRelease message. Normally network just relies on the data inactivity timer to decide whether to trigger the state transition to IDLE or to INACTIVE state.

But in UE side, UE is only configured with the data inactivity timer which can trigger the state transition from CONNECTED to IDLE, but such timer-based state transition from CONNECTED to INAVTIVE state is lacking.

As such timer-based solution can brings the following benefits, it should be considered in 6G.

- Save signaling overhead

- Save NW and UE power

- Help UE understand whether the timer value setting is appropriated and UE can provide the assistance information to help network adjust the value.

      - Proposal 6: For state transition from 6G connected mode to inactive mode, both command-based and timer-based solutions should be considered.

- Data transfer

In NR, the SDT feature is introduced to support the MT/MO small data transmission without state transition. It’s good for the APP level keep alive like packet transmission, which has small packet size and long periodicity, so it’s better to keep it in 6G.

But in NR SDT design, there are some limitations in terms of transmission efficiency and mobility support.

- From transmission efficiency perspective, in NR the CSI/SRS function is not supported during the SDT procedure, so the network cannot schedule based on the channel conditions of individual UE.

- NR SDT does not support mobility. If cel reselection occurs during SDT, UE will enter RRC IDLE state. This situation is worse than just staying in the RRC INACTIVE state.

In 6G SDT design, we should study how to improve the performance on the transmission efficiency and mobility.

      - Proposal 7: The SDT feature should be supported in 6G inactive mode, and the enhancement on mobility and transmission efficiency should be further studied.

# Conclusions

This contribution presents our views on 6G RRC state. We have the following proposals:

<Design principle>

Observation 1: The reasons that prevented the deployment of RRC_INACTIVE state in 5G network (e.g. RAN architecture, network interface, energy efficiency principle) no longer exist under the design principles of 6G.

Observation 2: When the same scenario can be supported by RRC_IDLE and RRC_INACTIVE state, it is very likely that RRC_INACTIVE state will be ignored in commercial deployment.

Observation 3: Only NR RRC_INACTIVE like state can meet the 6G CP latency requirement.

Proposal 1: 6G RRC state should avoid the multiple RRC states designed for the same/similar scenario (e.g.  when network has UE context and UE has no data activity, both IDLE and INACTIVE state are supported in 5G).

Proposal 2: RAN2 study on 6G RRC state focus on supporting only one state for each of the following scenarios:

- 1) When network has no UE context, UE is in RRC IDLE state, and NR RRC_IDLE state is starting point.

- 2) When network has UE context and UE has data activity, UE is in RRC_CONNECTED state, and NR RRC_CONNECTED state is the starting point.

- 3) When network has UE context and UE has no data activity, UE is in 6G inactive mode, and NR RRC_INACTIVE design can be the starting point.

<6G inactive mode>

Observation 4: The size of the mobile area setting in 6G inactive mode has opposite energy effects on UE and network.

Observation 5: Periodical notification from UE to network is needed to handle the abnormal case.

Proposal 3: The mobility in 6G inactive mode can inherit the NR RRC_INACTIVE mobility design, i.e. NR RNA design and RNAU procedure can be the starting point.

Observation 6: Assuming UE is only in 6G inactive mode when network has UE context, but UE has no activated dedicated connection, UE in 6G inactive mode only monitors one type of paging.

      - Proposal 4: In 6G inactive mode, the unified WUS and subgroup based 6G paging scheme with inactive UE ID is used to identify the UE in inactive mode. If the paging subgroup contains only one UE, UE is only required to monitor WUS and no need to monitor paging.

      - Proposal 5: The fast state transition between 6G inactive mode and connected mode should not involve NAS connection establishment procedure.

Observation 7: In 6G inactive mode and in mobile scenario, the first uplink transmission based on UE-dedicated resources can shorten the latency of the first data transmission, but this depends on the feasibility of the resource capacity and network negotiation, which requires the evaluation by RAN1 and RAN3.

Observation 8: To avoid the state transition latency due to the UE context relocation, network implementation can perform the UE context relocation in advance which is based on RNAU procedure.

Observation 9: For the first uplink transmission in 6G inactive mode, whether via L3 or L2 signaling, needs to carry UE-ID, resume cause, and MAC-I information.

Proposal 6: In 6G state transition from inactive mode to connected mode, NR RRC resume procedure can be taken as baseline, and the following aspects need to be further study:

- The first UL transmission using the UE dedicated resource in mobility scenario require the evaluation by RAN1 (from capacity perspective) and RAN3 (from network coordination feasibility perspective).

- The first uplink transmission using the lower layer signaling but carrying the essential information (UE ID, cause, MAC-I) requires the further evaluation from the feasibility and necessity perspective.

      - Proposal 6: For state transition from 6G connected mode to inactive mode, both command-based and timer-based solutions should be considered.

      - Proposal 7: The SDT feature should be supported in 6G inactive mode, and the enhancement on mobility and transmission efficiency should be further studied.

# References

- RP-252912	Study on 6G Radio