3GPP TSG-RAN WG2 Meeting #132	R2-2508450

Dallas, USA,  Nov. 17th - 21st

Agenda item:	10.3.2.2

Source:	Apple

Title:	Discussion on RRC Structure and Configuration

WID/SID:	FS_6G_Radio

Document for:	Discussion / Decision

# Introduction

According to 6GR SID (RP-252912), the control plane design is one of the RAN2-led objectives, which includes RRC structure and (re)configuration design.

The RRC signalling and reconfiguration were discussed in RAN2#131bis meeting, and some preliminary agreements were reached (see below).

Agreements - ASN.1 is used for encoding of RRC signaling for 6G air interface (same as NR). - Study overall RRC structure, configuration improvements, improve readability of ASN.1 for RRC signalling - Study how to efficiently, reliably and unambiguously configure UEs while keeping signalling size small (e.g. improvements to delta signaling or no delta signaling). - RAN2 will study modular design of RRC for 6G. Further study how to modularize RRC e.g. based on features, functions, verticals, etc.

The goal is to provide efficient, reliable, and explicit configuration while keeping the signaling size small.

The goal of the RRC structure and configuration design is to provide efficient, reliable, and explicit configuration while keeping the signaling size small. From this perspective, this contribution gives our views from the following aspects:

- Reference configuration

- Delta configuration

- Modular RRC design

- RRC configuration error handling.

# 6G RRC signaling design

## Reference configuration

In NR, all configurations for UE dedicated transmission are provided from RAN nodes to UE via RRC signaling. And For each RRC (re)configuration, network must provide UE with complete configuration information. With the introduction of more and more features, NR RRC (re)configuration message size has increased significantly over time, so later the RRC segmentation method was agreed to be applied to RRC (re)configuration message transmission. The transmission of a large-sized RRC (re)configuration message requires more resources and longer UE RRC processing time.

Observation 1: In NR, RRC configuration size are becoming increasingly large, requiring more and more resources and time to transmit, and the RRC processing time on the UE side is also getting longer and longer.

Because network determines UE specific configuration primarily based on UE capabilities, the triggered services and network deployments, most configuration parameters remain the same for UEs with the same capabilities, for the same services and in the same deployment. This is indeed the case in actual NR deployments.

Since the configuration set provided by network is relatively static, if UE can know the unchanged configuration in advance, network only provides the configuration of the changed part, which can greatly reduce the RRC (re)configuration message size, save the transmission resource and time, and shorten the RRC processing time in UE side.

Observation 2: For UEs with the same capabilities and service types in the same deployment, most UE-specific configurations (over 90%) are the same.

Observation 3: By providing partial/delta configuration based on the reference configuration, the RRC message size can be reduced, thereby saving the system resources and reducing UE processing time.

Figure 1: Illustration of RRC configuration size minimization based on reference configuration

Since the configuration set provided by network is relatively static, if UE can know the unchanged configuration in advance, network only provides the configuration of the changed part, which can greatly reduce the RRC (re)configuration message size, save the transmission resource and time, and shorten the RRC processing time in UE side.

In NR, we have already introduced the concept of a reference configuration in the LTM candidate configuration during LTM mobility. However, this concept is only used during LTM mobility and is not applicable to situations involving cross-RRC connections or cross-UE configurations.

    - Regarding the methods for providing the reference configuration to UE, network can provide it to UE in advance through APP layer, or network can tell UE to store (part of) the RRC configuration in the current RRC connection and use it as a reference configuration for subsequent connections. Details can be further study.

    - Proposal 1: Consider the reference configuration based RRC configuration structure and procedure, and the method for providing the reference configuration can be further studied.

## Delta configuration

To reduce the RRC configuration size, delta configuration was introduced from 4G.

In existing delta RRC configuration design, not all RRC parameters need to be present in every RRC configuration message, and the conditions and explanations for the presence or absence of parameters can be obtained through the following places in RRC spec. This approach to the RRC spec results in poor readability and is prone to errors.

- Through the parameter's Need code

- RRC procedure text description

- Explanation in the field description.

Furthermore, certain parameter configurations located in different structures are interrelated. The complex nested structure of the current RRC configuration bring significant challenges in providing correct configurations to the network side and ensuring reasonable interpretation of those configurations by the UE side.

In addition, in current broadcast configuration, almost all parameters are optional. However, some configurations are mandatory in the commercial network. For example, the configuration of ServingCellConfigCommonSIB should be presented in SIB1 in the cell which is not barred, but the need code is optional/Need R.

Due to the current complex configuration structure, the network-provided configuration is not always correct, and UEs have to perform some additional checks themselves rather than simply relying on the specification description.

Observation 4: The current complex NR RRC nested configuration structure and the diverse descriptions of delta configuration conditions have brought great difficulties to the correct interpretation of RRC.

In addition, in current broadcast configuration, almost all parameters are optional. However, some configurations are mandatory in the commercial network. For example, the configuration of ServingCellConfigCommonSIB should be presented in SIB1 in the cell which is not barred, but the need code is optional/Need R.

To optimize the delta configuration and make the RRC configuration more efficient, error-free, and readable, we can consider the following design aspects:

1) Group the configurations with the same attributes together. each group, the rules for the presence/absence of each group or parameters, as well as the corresponding UE operation, can be clearly defined.

2) Place the parameters that need to be associated in the same structure or describe the relationship in the same way/location, so that any omissions can be avoided by using a single method.

Proposal 2:  To reduce the difficulty of UE processing and network configuration provision, consider optimizing the RRC configuration structure to reflect the attributes and relationships of configuration parameters from two aspects:

- Group the configurations with the same attributes together.

- Place the parameters that need to be associated in the same structure or describe the relationship in the same way/location.

## Modular RRC design

Following issues were identified in the NR RRC configuration structure in the previous RAN2 discussion.

- Issue 1: The RRC configuration structure does not reflect the concept of feature level group.

  - Different feature configurations are mixed. For example, the configurations of positioning, on-demand SI, mobility, measurement gap configuration are mixed in the RRC configuration message.

  - One feature’s (re)configuration may result in the configuration of another unrelated features.

- Issue 2: The same configuration is duplicated in the different places of the same RRC configuration.

  - For example, for the configuration of a serving cell with multiple BWPs having the same center frequency but different bandwidths, the BWP specific common configuration (i.e. BWP-DownlinkCommon, and BWP-UplinkCommon) is the same. Such kind of duplication is inefficient and also increases the UE processing time and RRC message size.

- Issue 3: The RRC configuration structure doesnot reflect the vertical concept.

  - The configuration of SL, IAB, NTN and basic Uu configuration are mixed in RRC configuration.

Based on the above issues, the concept of modular RRC design was proposed and agreed for RAN2 study.

- RAN2 will study modular design of RRC for 6G. Further study how to modularize RRC e.g. based on features, functions, verticals, etc.

Regarding the modular RRC direction, we understand the motivation of the modular RRC design is good since the intention is to address the existing RRC configuration issues. But we need to be very cautious when making this design from the following aspects:

- The modular design needs to avoid introducing additional signaling overhead as much as possible

Overly flexible module design will introduce more signaling overhead, which can result in losses that are not worth it.

- The modular design requires the universality, scalability, and forward compatibility.

If modular design is not compatible with all potential configurations, the RRC structure will have two types of configurations (modular and non-modular) simultaneously, which will make the structure more complex.

To make the modular RRC design in feasible way, the above two aspects need to be considered.

Proposal 3: The study of the modular RRC design should consider the following aspects:

- 1) The extra signaling overhead should be avoided as much as possible.

- 2) The design should have good universality, scalability, and forward compatibility.

Considering the granularity of the modular RRC design, we think it is not easy to consider it purely based on feature, function or vertical, for the following reasons:

- Modular design based on feature

There are two types of features:

- Type-1: The higher layer features are relatively independent (e.g. QoE, SON/MDT, overheating).

- Type-2: The lower layer features mainly depend on other features. For example, the LTM is designed based on the precondition of MIMO LCBM framework, and the RAN4 RRM enhancement feature is designed under the RRM measurement framework.

For the independent features (Type-1), they can be treated as one module. But for the dependent features (Type-2), It is unrealistic to turn it into an independent module.

Observation 5: The feature specific modular design is only applicable to the independent features, but not to the dependent features.

- Modular design based on function

We understand the function specific configuration here refer to RB configuration, L1, L2 and L3 configuration, security configuration, RRM measurement configuration, mobility configuration; and in L1 configuration, it can be further split into the MIMO configuration, and the dedicated and common configuration per PHY channel per BWP/cell.

Regarding to the existing RRC configuration structure, there are some channel level configurations duplicated. In the BWP example in above Issue-2, we can use single BWP common config associated with multiple BWP configuration to avoid the duplication.  The modular RRC design is feasible in this scenario.

But the configuration in the same level may include the configuration for multiple features/vertical (e.g. IAB, MBS, NCR configuration mixed in cell group configuration), and the impact of cross feature configuration is unavoidable.

Observation 6: The function specific modular can help reduce the duplicated configuration in some cases (e.g. BWP).

- Modular design based on vertical

The vertical refers to the SL, IAB, NCR, RedCap, NTN etc.

For some verticals (e.g. SL, IAB, NCR), most of the configurations are the vertical specific configurations; but for other verticals (e.g. NTN, RedCap), most of the configuration are not specific for that vertical.

Observation 7: The vertical specific modular design is only applicable to the vertical specific configuration, but not to the configuration which are not specific for one specific vertical.

According to the above observations, it’s obvious that modular RRC design cannot cover all cases in one granularity.

Proposal 4: RAN2 study on modular RRC design should focus on the followings:

- For feature specific modular design, focus on the independent higher-layer features (e.g. MDT/SON, QOE, Overheating).

- For vertical specific modular design, focus on the modules that only contain one vertically specific configuration.

- For function specific modular design, focus on the duplicated configuration avoidance purpose.

## RRC configuration error handling

In NR RRC Reconfiguration procedure, if NW provides the RRC configuration which UE is unable to comply with (part of) it, UE will declare the reconfiguration failure and initiate the RRC connection reestablishment. As the reestablishment procedure is very costly, which will break the RRC connection and terminate the transmission, it will degrade the RRC connection performance and give worse UE experience.

Observation 8: Upon detecting the RRC reconfiguration failure, triggering the RRC connection reestablishment upon brings the significant negative impact.

The current NR RRC configuration contains various configurations for different features and functions. Some configuration errors may affect the current transmission (e.g. L1 configuration on the active BWP/active serving cell), but many have no direct impact (e.g. the L2 timer or L3 measurement configuration, SON/MDT configuration, SL/IAB configuration).

Observation 9: Not all configuration errors will affect the current transmission.

In addition, in the commercial network, the configuration provided by network is not always guaranteed to be correct or satisfactory. Sometimes the parameters configured by the network are not completely consistent with the current spec description. In some other cases, the UE and NW are not completely in sync and the NW configuration may not be suitable to the current UE circumstance.

Observation 10: Network cannot fully guarantee the configuration is always appropriate.

In order to provide better robustness and fault tolerance design in 6G network, we should try to keep the existing radio link workable as much as possible without affecting the current transmission and avoid service interruptions.

Especially in NR design, there have been some configuration procedures which does not affect current connection/transmission:

- For LTM/CHO candidate configuration error case, UE is not required to decode the candidate configuration before the cell switch to that cell is initiated. In other words, the configuration error in candidate config which does not affect the existing connection will not trigger connection reestablishment.

- For NAS procedure, it allows some UE reject, as NAS tends to have more “negotiation” between the NW and UE.

- For MUSIM and AI/ML work, the UE is allowed to report its non-compliance (e.g,, unable to apply a certain configuration) to the NW via UAI message or RRCReconfiguraitonComplete message. Then, NW may re-configure the UE with a more suitable configuration.

As we can see, there are already some precedents to not mandating UE to apply all RRC configurations in NR design.

Observation 11: There are precedents in the existing 3GPP design to allow in some cases that UE does not need to break current connection and initiate RRC reestablishment procedure.

Based on above observations and given that there will be more non-critical configurations in a variety of new 6G services expected to be included in 6G RRC design RAN2 should consider improving the reconfiguration failure procedure and have a more robust and generic framework to allow UE keep current configuration if the error has no impact to current connection. In 6G RRC, it could be a basic principle/direction for RRC design to allow different configurations to be treated differently (e.g., trigger RRCReestablishment vs not trigger), depending on how important/critical a configuration parameter (set) it is . In addition, in case the RRC reestablishment procedure is not triggered, to ensure the same understanding on the configuration applied in UE, UE can provide information about parameters that are not applied to the network.

    - Proposal 5: In RRC reconfiguration procedure, if UE is unable to comply with (part of) the configuration which does not affect the current transmission, UE is allowed to apply the good (part of) configuration and not initiating the connection re-establishment procedure.

# Conclusions

According to the analysis in section 2, we propose that:

< Reference configuration>

Observation 1: In NR, RRC configuration size are becoming increasingly large, requiring more and more resources and time to transmit, and the RRC processing time on the UE side is also getting longer and longer.

Observation 2: For UEs with the same capabilities and service types in the same deployment, most UE-specific configurations (over 90%) are the same.

Observation 3: By providing partial/delta configuration based on the reference configuration, the RRC message size can be reduced, thereby saving the system resources and reducing UE processing time.

    - Proposal 1: Consider the reference configuration based RRC configuration structure and procedure, and the method for providing the reference configuration can be further studied.

<Delta configuration>

Observation 4: The current complex NR RRC nested configuration structure and the diverse descriptions of delta configuration conditions have brought great difficulties to the correct interpretation of RRC.

Proposal 2:  To reduce the difficulty of UE processing and network configuration provision, consider optimizing the RRC configuration structure to reflect the attributes and relationships of configuration parameters from two aspects:

- Group the configurations with the same attributes together.

- Place the parameters that need to be associated in the same structure or describe the relationship in the same way/location.

< Modular RRC design>

Proposal 3: The study of the modular RRC design should consider the following aspects:

- 1) The extra signaling overhead should be avoided as much as possible.

- 2) The design should have good universality, scalability, and forward compatibility.

Observation 5: The feature specific modular design is only applicable to the independent features, but not to the dependent features.

Observation 6: The function specific modular can help reduce the duplicated configuration in some cases (e.g. BWP).

Observation 7: The vertical specific modular design is only applicable to the vertical specific configuration, but not to the configuration which are not specific for one specific vertical.

Proposal 4: RAN2 study on modular RRC design should focus on the followings:

- For feature specific modular design, focus on the independent higher-layer features (e.g. MDT/SON, QOE, Overheating).

- For vertical specific modular design, focus on the modules that only contain one vertically specific configuration.

- For function specific modular design, focus on the duplicated configuration avoidance purpose.

< RRC configuration error handling>

Observation 8: Upon detecting the RRC reconfiguration failure, triggering the RRC connection reestablishment upon brings the significant negative impact.

Observation 9: Not all configuration errors will affect the current transmission.

Observation 10: Network cannot fully guarantee the correctness of configuration.

Observation 11: In NAS procedure and CHO candidate configuration procedure, if there is any error, UE does not need to break current connection and initiate RRC reestablishment procedure.

    - Proposal 5: In RRC reconfiguration procedure, if UE is unable to comply with (part of) the configuration which does not affect the current transmission, UE is allowed to apply the good (part of) configuration and not initiating the connection re-establishment procedure.

# References

- RP-252912	Study on 6G Radio