3GPP TSG-RAN WG2#132	R2-2508462

Dallas, USA, Nov 17th – 21st 2025

Source: 	Fujitsu

Agenda item:	10.3.3.2

Title:	Discussion on network and UE energy efficiency

Document for:	Discussion and decision

Introduction

The SID of Rel-20 Study on 6G Radio was approved in RAN#108 and updated in RAN#109 [1].

(1) Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

In RAN2#131bis meeting, the design goal for energy efficiency and the guideline for the next meeting were agreed as follows [2]:

Design goal

1. 	Strive to develop solutions that consider network energy efficiency and UE power saving

For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose.   Focus on solutions that have RAN2 impacts.

In this contribution, we would like to provide our views on network and UE energy efficiency aspects.

Discussion

Energy efficiency and power savings

UE energy savings

In 5G NR, the NW and the UE had aim to target of power saving friendly design by reducing common signals and monitoring periods. However, the eMBB services were considered at the initial launch of 5G NR, thus UE power saving was studied after NR Rel-15 and several UE power saving functions were introduced during multiple releases. However, it consequently caused fragmentation of UE power saving features. Table.1 shows the UE power saving features specified in NR.

Table.1 - UE power saving features in NR

| Release | Functions in NR | RRC state | Purpose |
| --- | --- | --- | --- |
| Rel-15 | C-DRX | Connected | Reduction of the number of PDCCH monitoring in connected mode |
| Rel-15 | I-DRX | Idle/inactive | Reduction of the number of PDCCH monitoring in idle/inactive mode |
| Rel-15 | BWP switching | Connected | Switching a narrow channel bandwidth to reduce corresponding DL/UL data processing |
| Rel-16 | DCP | Connected | Skipping unnecessary PDCCH monitoring in C-DRX |
| Rel-16 | Multi slot scheduling | Connected | Reduction of PDCCH monitoring for subsequent data transmission and reception |
| Rel-16 | SCell dormancy | Connected | Reduction of PDCCH monitoring but performing CSI and RRM measurements |
| Rel-16 | RRM relaxation | Idle/inactive and connected | Reduction of RRM measurements when cell quality is good and the device is stationary |
| Rel-17 | RLM and BFD relaxation | Connected | Reduction of RLM and BFD measurements when cell quality is good and the device is stationary |
| Rel-17 | SDT | Inactive | Reduction of RRC transition from inactive to connected |
| Rel-17 | PEI | Idle/inactive | Skipping unnecessary Paging monitoring in I-DRX |
| Rel-17 | Subgroup paging | Idle/inactive | Skipping unnecessary Paging monitoring in I-DRX |
| Rel-19 | LP-WUS/WUR | Idle/inactive and connected | Switching between the low-power radio and the main radio |

When the UE is in idle/inactive mode, the UE behaviours are continuously monitoring the PDCCH for paging detection per DRX cycle and measurement the serving/neighbouring cells for cell reselection. Thus, it is observed that the PDCCH monitoring for paging is one of the primary factors of UE power consumption. In other words, it is obvious that the power consumption can be significantly reduced if the UEs can wake up only when the signalling and/or data arrives. In connected mode, the same principle can be observed if the UE is configured with C-DRX. In NR, for this purpose, to indicate the necessity of PDCCH monitoring to the UE, DCP and PEI have been introduced in different releases, for connected and idle/inactive mode, respectively. Therefore, it would be studied about the similar feature in 6G as well. Moreover, in general, there are many carriers/cells around the UE thus measurements are frequently needed for a decision of the proper idle/connected mobility. Given that, RRM measurement relaxation (including RLM/BFD measurement as well) can also consume less power as periodical measurements would be another dominant of the UE battery drain.

In addition, LP-WUS/WUR may further achieve the UE power consumption by using a separated low-power receiver, even though DCP or PEI is approved [3]. Therefore, we think it is also reasonable to consider whether the LP-WUS/WUR is included in a list of day1 features as well. Further study LP-WUS/WUR should be mandatory or optional feature depending on the device type or something.

On the other hand, the other features might depend on other aspects. For example, both BWP switching and SCell dormancy may be related to spectrum aggregation (e.g., single cell with multi-carriers), and SDT clearly depends on whether RRC inactive state is adopted or not. Hence, RAN2 should wait for further progress on the related topics.

Observation 1:	For UE power saving, it is useful to reduce the number of PDCCH monitoring for paging per DRX cycle in any RRC states.

Observation 2:	DCP and PEI are a similar function in different RRC state, both can be used to indicate skipping unnecessary PDCCH monitoring with UE DRX operation.

Proposal 1:	For UE energy saving, RAN2 to consider at least the following features in 6G day1. The features can be applied to any RRC states.

- UE DRX operation with PDCCH monitoring skip indicator

- Measurement relaxation

- LP-WUS/WUR (including whether it is optional)

NW energy savings

In 5G practical deployments network energy saving is mainly achieved by switching off the capacity carrier when the network load is very low. This is typically the case when mobile operator has multiple overlapping carriers at his disposal to be deployed as coverage and capacity layer as per the operator’s carrier deployment strategy. All UEs are moved to the always-on carrier which is typically the coverage layer, and the capacity carriers are switched-off. This is achieved in implementation-specific manner with the minimum standard support by moving the served UEs through handover, barring the new UEs from camping on the carrier for which switching off is triggered. In certain deployments, the carrier switching is feasible only in certain time periods (e.g. during night with less traffic) where the switch-off window is statically configured to realize the network energy saving. Without implementation-specific manner, NR Rel-18/19 NES was introduced to save the network power consumption.

However, there were limited applicable scenarios, and it was less efficient power saving gain because there are legacy UEs in the cell need to be considered. For example, NES techniques would be applied only for connected mode UE or SCell case. Another example is on-demand SIB1, one NES technique introduced in Rel-19. It may reduce always-on common signalling (SIB1), but it requires a configuration for a UL wake-up signal from another neighbouring cell. It means on-demand SIB1 cannot be applied to the standalone cell. In initial 6G deployment, the operator may not have multiple carriers and when a single carrier is deployed for 6G in standalone mode then network energy saving may be limited with existing implementation-specific techniques or NES techniques similar to Rel-18/19. Table.2 shows the NW power saving features specified in NR.

Table.2 - NW power saving features in NR

| Release | Functions in NR | RRC state | Purpose |
| --- | --- | --- | --- |
| Rel-15 | Intra-band SSB-less SCell | Connected | Reduction of periodical SSB transmission in intra-band SCell |
| Rel-15 | On-demand SIB | Idle/inactive and connected | Reduction of periodical broadcast message (except for MIB/SIB) |
| Rel-18 | Cell DTX/DRX | Connected | Signalling reduction and making longer DL/UL idle period |
| Rel-18 | Inter-band SSB-less SCell | Connected | Reduction of periodical SSB transmission in inter-band SCell |
| Rel-18 | Type1/2 spatial and power domain adaptation | Connected | Muting gNB transceivers and adapting transmission power |
| Rel-18 | Inter-node SSB beam activation | Idle/inactive and connected | Muting unnecessary beam transmission |
| Rel-18 | Inter-node paging beam activation | Idle/inactive and connected | Muting unnecessary beam transmission |
| Rel-19 | On-demand SIB1 | Idle/inactive | Reduction of periodical SIB1 message |
| Rel-19 | On-demand SSB SCell | Connected | Reduction of periodical SSB transmission in SCell |
| Rel-19 | SSB adaptation | Connected | Sparse SSB transmission in SCell |
| Rel-19 | PRACH adaptation | Idle/inactive and connected | Reduction of PRACH monitoring and preamble detection processing |
| Rel-19 | Paging adaptation | Idle/inactive | Making longer DL/UL idle period |

As mentioned above, some NW energy saving features in NR require the multi-carrier operation and it means it is applicable to the UE in connected state only. However, in 6G, because there is no backward compatible issue, it should be considered that a series of on-demand request techniques should be applied to any cell type. Also, to simplify the specification and UE/NW behaviours, it should be useful to study unified manner for all on-demand request. Moreover, to avoid the multiple solutions for the same purpose, intra/inter-band SSB-less SCell and SSB adaptation can be merged with on-demand SSB SCell, as on-demand SSB support non always-SSB scenario and also it can be configured with multiple SSB periodicity and switched adaptively.

Basically, a power required for a transmission process is more dominant factor of the NW power consumption than a reception one. In this sense, a technique for reducing the number of transmissions for a long time should be considered as a default/native feature in 6G day1. For this purpose, it is useful to consider common channel/signalling adaptation and cell DTX/DRX simultaneously, because both can increase an opportunity for a longer sleep period, which is helpful for low power consumption. For example. if the number of connected UEs are low in the cell, the gNB should adapt to transmit common signals like SSB, SIB1 and paging during cell DTX on-duration. Likewise, PRACH occasion can be aligned with cell DRX on-duration, The other channels and signals to/from the UEs will be handled by the NW implementation together with C/I-DRX operation as in NR NES.

Observation 3:	Some of NR network energy saving techniques require multi-carrier operations.

Observation 4:	Intra/inter-band SSB-less SCell and SSB adaptation can be covered by on-demand SSB SCell.

Observation 5:	Common channel/signalling adaptation and cell DTX/DRX have a similar purpose which can increase an opportunity for a longer sleep period in NW side.

For inter-node SSB or paging beam activation, both can be achieved by inter-node coordination without involving the UEs therefore it is natural to leave it to RAN3.

Observation 6:	Inter-node SSB and paging beam activation should be studied in RAN3.

In summary, to achieve more energy efficiency than NR, we propose 6G NW energy saving techniques should be simple and try to adopt a unified framework.

Proposal 2:	For NW energy savings, RAN2 to consider at least the following features in 6G day1. The features can be applied to any cells.

- Unified framework for on-demand broadcast signalling request

- Cell DTX/DRX with common channel/signal adaptation

Joint energy savings

Further, in NR, joint power saving mechanism may potentially increase power saving gain, but it was not clearly specified (up to NW implementation), because such power saving techniques have been specified as independent feature. One can argue that joint power saving is desirable but the motivations for network energy saving and UE battery conservation may have orthogonal objectives. For instance, network energy saving applies at cell level (i.e. applicable to all served UEs) whereas connected mode DRX is UE-specific depending on the traffic scheduled to the UE. NES techniques and UE battery conservation techniques may be independent or joint as along as the combination of the two can co-exist to serve the respective objectives.

Proposal 3:	RAN2 to study efficient mechanism to enable independent and joint power saving between UE and NW.

- For joint power saving, Cell DTX/DRX and UE I/C-DRX alignment can be a good starting point.

Conclusion

In this paper we presented some views on network and UE energy efficiency aspects and have come up with the following observations and proposals:

UE power savings:

Observation 1:	For UE power saving, it is useful to reduce the number of PDCCH monitoring for paging per DRX cycle in any RRC states.

Observation 2:	DCP and PEI are a similar function in different RRC state, both can be used to indicate skipping unnecessary PDCCH monitoring with UE DRX operation.

Proposal 1:	For UE energy saving, RAN2 to consider at least the following features in 6G day1. The features can be applied to any RRC states.

- UE DRX operation with PDCCH monitoring skip indicator

- Measurement relaxation

- LP-WUS/WUR (including whether it is optional)

NW power savings:

Observation 3:	Some of NR network energy saving techniques require multi-carrier operations.

Observation 4:	Intra/inter-band SSB-less SCell and SSB adaptation can be covered by on-demand SSB SCell.

Observation 5:	Common channel/signalling adaptation and cell DTX/DRX have a similar purpose which can increase an opportunity for a longer sleep period in NW side.

Observation 6:	Inter-node SSB and paging beam activation should be studied in RAN3.

Proposal 2:	For NW energy savings, RAN2 to consider at least the following features in 6G day1. The features can be applied to any cells.

- Unified framework for on-demand broadcast signalling request

- Cell DTX/DRX with common channel/signal adaptation

UE/NW joint power savings:

Proposal 3:	RAN2 to study efficient mechanism to enable independent and joint power saving between UE and NW.

- For joint power saving, Cell DTX/DRX and UE I/C-DRX alignment can be a good starting point.

References

- RP-252912, Revised SID: Study on 6G Radio

- RAN2#131bis RAN2 chair’s note

- TR 38.869 v18.0.0