3GPP TSG-RAN WG2 Meeting#132	R2-2508468

Dallas, United States, 17th – 21st Nov. 2025

Source:	vivo

Title:	Discussion on remaining issues of AIoT MAC

Agenda Item:	8.2.2

Document for:	Discussion and Decision

- Introduction

In this contribution, the following open issues identified in [POST131bis][017][AIoT] MAC CR (Huawei) will be discussed:

- Issue 3-7 no upper layer data available

- Issue 6-2 delayed inventory response

- Discussion

## Issue 3-7 no upper layer data available procedure

In RAN2#131bis, there were the following agreements regarding this issue:

Agreements

- RAN2 confirms, in addition to delayed response, it is valid that in some cases A-IoT NAS doesn’t provide a response at all.

- For cases other than integrity failure, AS will indicate no NAS response expected to reader.   FFS how (e.g. using 0 SDU & MDI, or new indication).

- For integrity failure, for now RAN2 assumes that there is no AS response to the reader.  Ask SA3 ccCT1 whether a similar mechanism (e.g. AS response to the reader) can be used to indicate to reader no NAS response due to integrity failure.

During post RAN2#131bis meeting, RAN2 received an LS [1], from CT1, regarding this issue. This LS confirmed that there is no NAS response in case the received NAS message is ignored at device side and NAS layer will inform AIoT MAC that the received NAS message is ignored.

The AIoT device shall indicate to the lower layers that the AIoT NAS message was ignored, so that the lower layers stop AIoT data transmission.

The LS also informed RAN2 the corresponding agreed change request to TS 24.369 provided below as a reference:

### 6.2.1 Message too short When a message is received that is too short to contain a complete message type information element, that message shall be ignored, c.f. 3GPP TS 24.007 [3], and the AIoT device shall provide an indication to the lower layers that the message was ignored.

For RAN2, the main issue is how the device indicates to the reader whether the NAS response is delayed or not provided in D2R Upper Layer Data Transfer message. The related impact on the reader is as follows:

- If the NAS response is not provided, the reader will not schedule future D2R transmissions

- If the NAS response is delayed, the reader shall schedule future D2R transmissions for the delayed NAS response.

During the discussion in RAN2#131bis, the following two options were identified to indicate the reader whether the NAS response is delayed or not provided:

- Option 1: Using MDI to 0 without SDU, i.e.

  - “0 SDU & MDI =0” represents “no upper layer data available”

  - “0 SDU & MDI =1” represents “no upper layer data available due to delay NAS”

- Option 2: introduce a new indicator (single bit) in D2R data MAC PDU.

From our perspective, both Option 1 and Option 2 can work but Option 2 adds one fixed bit overhead, in addition to the MDI, in D2R MAC PDU for data transmission. To avoid unnecessary overhead, Option 1 should be adopted. Hence, we propose:

- (Issue 3-7) Reuse MDI to indicate whether NAS response is delayed or ignored, i.e.

- “0 SDU & MDI =0” represents “no upper layer data available”

- “0 SDU & MDI =1” represents “no upper layer data available due to delay NAS”.

Based on Proposal 1, a text proposal has been made based on the TS 38.391 v19.0.0.

- (Issue 3-7) If Proposal 1 is agreeable, adopt the text proposal in the annex.

## Issue 6-2 delayed inventory response

The description regarding this open issue is provided here for reference:

- New issue 6-2: Confirm to apply the no data available case to more general delayed NAS cases, e.g., inventory response message.

The proponent thought that the delayed inventory response may occur for CFA as the device may take a long time for authentication process upon reception of a paging message. The description of the proponent for this issue is provided below for reference:

The timing between CFA R2D and responding D2R message is discussed in RAN1. During the discussion, time consumption for authentication is not considered. As a consequence, the timing between CFA R2D and responding D2R message (is about 200us for some chip duration based on TS38.291) is too small for authentication, that there may be no upper layer data, i.e., inventory report NAS message, generated by upper layer at the time to transmit D2R Upper Layer Data Transfer message after CFA paging message. However, even the delayed inventory report message is generated later, it will not be retransmitted to reader based on current RAN2 MAC procedure. In such case, D2R Upper Layer Data Transfer message retransmission due to delayed upper layer data available for initial transmission should be supported for CFA inventory. The retransmission could be scheduled by R2D Upper Layer Data Transfer message. The following TP could be considered to solve this issue.

For CBRA, there is no such issue, as the device can process authentication during the period from the end of paging message reception to the end of Msg2 reception. This period is expected to be sufficiently for the device to complete the authentication procedure.

The processing delay depends on the computational complexity. For authentication procedure specified in [2], “Upon receiving the paging request message, if the device determines it needs to respond based on the device identification information, AIoT device shall generate RANDAIOT_d, calculate RESAIOT using KAIoT_root and RANDAIOT_n for network authenticating AIoT Device.” the device mainly needs to generate random number and compute a response through logical operations. The computational complexity is expected to be low, and the computation time should be very short.

- (Issue 6-2) For the authentication procedure, the device mainly needs to generate random number and calculate a random number, the associated processing delay should therefore be small.

Then let’s review the background of issue 3-7 (no upper layer data available) which has been discussed in several RAN2 meetings. As understood, this issue arises from read/write operations on large messages (e.g., close to 1000 bits) in the upper layer. Due to both integrity/security processing and memory read/write operations on such large messages, the device may require significant time to complete the processing and generate the NAS response.

- (Issue 6-2) the NAS response delay is mainly due to processing of the large NAS msg (e.g., close to 1000 bits), including the related integrity/security procedures and read/write operations.

As understood, upon receiving a PRDCH intended for the device in an R2D transmission, the device shall determine the D2R transmission starting time. For example, the D2R transmission time corresponds to a contention-free random access procedure can be impacted by the channel coding and chip duration. Even if D2R switching time requirement (i.e. the timing restriction for AIoT device to switch from RX mode to TX mode) is tough in case of CFA, the reader can adjust the configuration to receive the inventory responses over a slightly long period, ensuring successful transmission.

- (Issue 6-2) Even if D2R switching time in CFA is tough, the reader can address it by implementation.

- (Issue 6-2) Leave the handling of delayed inventory responses (if any) to reader implementation, i.e. no specification change is needed.

- Conclusion

On a basis of above analysis, we make the following observations and proposals regarding A-IoT paging:

For Issue 3-7:

- (Issue 3-7) Reuse MDI to indicate whether NAS response is delayed or ignored, i.e.

- “0 SDU & MDI =0” represents “no upper layer data available”

- “0 SDU & MDI =1” represents “no upper layer data available due to delay NAS”.

- (Issue 3-7) If Proposal 1 is agreeable, adopt the text proposal in the annex.

For Issue 6-2:

- (Issue 6-2) For the authentication procedure, the device mainly needs to generate random number and calculate a random number, the associated processing delay should therefore be small.

- (Issue 6-2) the NAS response delay is mainly due to processing of the large NAS msg (e.g., close to 1000 bits), including the related integrity/security procedures and read/write operations.

- (Issue 6-2) Even if D2R switching time in CFA is tough, the reader can address it by implementation.

- (Issue 6-2) Leave the handling of delayed inventory responses (if any) to reader implementation, i.e., no specification change is needed.

- Annex：Text proposal for issue 3-7 based on TS 38.391 v19.0.0

************************************change start************************************

## 5.4	A-IoT upper layer data procedure

### 5.4.1	General

The purpose of this procedure is for a device to transmit or receive upper layer data.

### 5.4.2	D2R message transmission

Upon initiation of the procedure corresponding to the A-IoT access procedure or reception of an R2D Upper Layer Data Transfer message which contains either the Data SDU field or the Received Data Size field set to 0, the A-IoT MAC entity shall:

1>	apply the D2R Scheduling Info, received in the A-IoT Paging message with Access Type set to CFA or in the Random ID Response message or in the R2D Upper Layer Data Transfer message containing either the Data SDU field or the Received Data Size field set to 0, whichever triggered the initiation of the procedure;

1>	if upper layer data is available to be transmitted:

2>	if the size of the resulting MAC PDU including the total upper layer data is smaller than or equal to the resource size given by the D2R TBS in the D2R Scheduling Info:

3>	generate the D2R Upper Layer Data Transfer message, as follows:

4>	set the More Data Indication field to value 0;

4>	include SDU Length field and Data SDU field;

4>	if the size of the resulting MAC PDU including the total upper layer data is smaller than the resource size given by the D2R TBS in the D2R Scheduling Info:

5>	include the MAC Padding field;

3>	instruct the physical layer to transmit the D2R Upper Layer Data Transfer message and indicate the L1 parameters to the physical layer, as specified in clause 6.2.1.6;

2>	else (the size of the resulting MAC PDU including the total upper layer data is larger than the resource size given by the D2R TBS in the D2R Scheduling Info):

3>	initiate the segmentation procedure for the upper layer data SDU as specified in 5.4.4;

1>	else (i.e., upper layer data is not available to be transmitted):

2>	generate the D2R Upper Layer Data Transfer message, as follows:

3>  If upper layer provides an indication that the received message was ignored as specified in TS 24.369 [x]:

4>	set the More Data Indication field to 0;

3> else:

34> set the More Data Indication field to 01;

3>	set the SDU Length field to 0;

3>	if the size of the resulting MAC PDU including no upper layer data is smaller than the resource size given by the D2R TBS in the D2R Scheduling Info:

4>	include the MAC Padding field;

2>	instruct the physical layer to transmit the D2R Upper Layer Data Transfer message and indicate the L1 parameters to the physical layer, as specified in clause 6.2.1.6.

NOTE:	It is up to reader’s implementation to avoid segmentation for the D2R Upper Layer Data Transfer message for inventory response.

************************************change end************************************

- Reference

- C1-256623 LS on no Command Response in NAS

- TS 33.369 V19.0.0 Security aspects of Ambient Internet of Things (AIoT) services for isolated private networks