3GPP TSG-RAN WG2 Meeting #132	R2-2508501

Dalla, USA, Nov. 17th-21st

Agenda item:	10.3.2.3

Source: 	China Telecom

Title:	Discussion on Initial and System Access in 6GR

WID/SID:	Study on 6G Radio

Document for:	Discussion and Decision

- Introduction

In RAN #108 meeting, the New SID for “Study on 6G Radio” was approved [1] aiming to develop 6GR to meet a broad range of use cases, and requirements as defined during the RAN requirements study. In RAN2#131bis meeting, 6G SI was discussed for the first time and some high level agreements on control plane aspects for 6GR were reached.

In this contribution, we would like to share our opinions on initial and system access in 6GR, including system information, paging and aspects related to spectrum aggregation.

- Discussion

System information

RAN2#131bis meeting made the following agreements on system information:

- System information design should consider energy efficiency and low complexity for both network and UE. - System information design should ensure that the public warning requirements are met. - Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs) - Study how to improve flexibility and extensibility of scheduling SI design - Study mechanisms to improve SI update mechanism - Study aspects related to SIB1 size. Understand the issues and desired content/size from RAN2 point of view. - Study areas specific SIB design. Understand 5G pain points and what improvements can be considered. NOTE: this is a starting list

According to the agreements, system information design should consider energy efficiency. In 5G, system informations are divided into Minimum SI (i.e. MIB and SIB1) and Other SI. MIB and SIB1 are periodically broadcast, and Other SI are provided on demand. In 5G-A, SSB-less SCell operation for inter-band CA, on-demand SSB operation and SSB periodicity adaptation are introduced to reduce network energy consumption on SCells in Rel-18 and Rel-19. On-demand SIB1 for UEs in idle/inactive mode is introduced to reduce network energy consumption due to periodic SIB1 transmissions in Rel-19. However, since the network energy saving features are not adopted from day one, to minimize the impact to legacy UEs, the network energy saving techniques are applied to limited scenarios, such as SCell, RRC state.

For better network energy saving, in 6G, longer default SSB/SIB1 periodicity can be taken into account. To ensure user experience, flexible additional SSB/SIB1 transmission mechanism such as on-demand SSB/SIB1 transmission can be the supplement. In 5G, on-demand SSB transmission can only be triggered by the network. And the on-demand SSB operation is only applied for UE in RRC_CONNECTED. If on-demand SSB is introduced for UE in RRC_IDLE, more sufficient network energy saving can be achieved. However, the network may not know how many UEs camping on a serving cell and SSB acquisition requirement from UEs in RRC_IDLE. On-demand SIB1 requested by UEs has been introduced in 5G, and it can also be considered as the starting point in 6G. Therefore, on-demand SSB/SIB1 transmission triggered by UEs can be considered in 6G system information design.

Proposal 1: On-demand SSB/SIB1 transmission triggered by UEs can be considered in 6G system information design.

Paging

RAN2#131bis meeting made the following agreements on paging:

Paging design should consider energy efficiency and simplicity for both UE and Network.

In 5G-A, adaptation of paging (i.e. the value range for parameter N and the number of POs within one PF are extended to have increased interval between PFs but increased number of POs within one PF) is supported for network energy saving in Rel-19, considering minimum specification impact. However, the distribution of PFs/POs is still even. The PFs and the POs in one PF are determined by UE_ID, UE DRX cycle, and parameters (i.e. parameter N and the number of POs within one PF) configured by the network. From the perspective of the network, the distribution of the PFs/POs of UEs may be dispersive and not friendly to base station sleep. Then the energy saving gain may be limited. Now it is an opportunity to redesign the distribution of PF/PO. Uneven distribution of PF/PO should be considered to enable more sleep opportunity for the base station.

Proposal 2: Support uneven distribution of PF/PO in 6G paging design.

Spectrum aggregation

In last RAN2 meeting, Spectrum aggregation in 6GR was discussed and the following agreements were reached.

Agreements on 6GR Spectrum aggregation in RAN2#131bis:

1. 	Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed.

2	Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc).  Understand the deployment scenarios and problems to address

Regarding the spectrum aggregation of multiple pieces of spectrum, CA is considered as a beneficial feature in previous generations, and TR 38.914 requires the 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

However, the fragmented spectrums are not utilized well based on legacy CA framework, there are many drawbacks and limitations for the CA or SUL based solutions.

- The management of multiple cells based on legacy CA framework leads to the signaling overhead, delay, low spectrum utilization efficiency on account of SCell addition/release, and inflexible carrier selection before entering RRC_CONNECTED state.

- SUL operation has the limitation on supporting only one available carrier in UL of a single cell at one time, which could not meet the demands to aggregate more than two carriers for the fragmented spectrum.

Therefore, new multi-carrier operation and cell management technologies could be studied to overcome the limitations and utilize the spectrum effectively and efficiently in 6G. From our perspective, potential solutions on multi-carrier operation and cell management in 6GR are discussed as follows.

- A single “Super” cell with multiple carriers

In order to utilize the fragmented spectrums more efficiently, one potential solution is to enable multiple UL/DL carriers configured and managed in one serving cell, which could be treated as one single cell and called a “Super” cell. These multiple carriers within a “Super” cell can be intra-band, and/or inter-band. Multiple Super cells are supported to be aggregated based on CA, for example when the carriers associated to different Super cells belong to different frequency ranges. Fig. 1 is an illustration for the association between multiple carriers and cells.

Fig. 1 Schematic diagram for a single “Super cell” with multiple carriers

Based on the concept of a single “Super cell” with multiple carriers, the following benefits related to initial and system access could be achieved.

- Less synchronization signal and broadcast information on carriers within a “Super cell” for common signaling overhead reduction and energy saving: Synchronization signal can be transmitted only on part of the carriers within a Super cell, e.g. synchronization carrier(s). Other carrier(s) with frequency adjacent to synchronization carrier(s) in the super cell can share the synchronization signal. It extends applicable scenario of SSB-less/on demand SSB/SSB adaptation operation for SCell in RRC connected mode supported in 5G NR to carriers in a Super cell as well as UEs in idle/inactive (if supported) state. Common broadcast information for the carriers such as some information in MIB/SIB can be transmitted on only one carrier in the Super cell, e.g. a MIB/SIB carrier, avoiding the overhead of broadcasting the same information on multiple different carriers.

- Random access carrier selection to reduce the congestion and increase the successful rate: The multiple carriers of a Super cell are visible by a UE in RRC_IDLE/ RRC_INACTIVE (if supported) state, so that the UE can select a more suitable UL/DL carrier for random access procedure within multiple carriers of the super cell, rather than accessing from only UL/DL carrier associated with PCell as the NR legacy behaviour.

As analysed above, it is worth to study a new framework supporting a single “Super cell” with multiple carries for initial and system Access in 6GR.

Proposal 3: Regarding initial and system Access in 6GR, a new framework supporting a single “Super cell” with multiple carries could be studied for further reduction of the access latency and the system overhead.

- Conclusion

In this contribution, we discuss initial and system access in 6GR, including system information, paging and aspects related to spectrum aggregation. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: On-demand SSB/SIB1 transmission triggered by UEs can be considered in 6G system information design.

Proposal 2: Support uneven distribution of PF/PO in 6G paging design.

Proposal 3: Regarding initial and system Access in 6GR, a new framework supporting a single “Super cell” with multiple carries could be studied for further reduction of the access latency and the system overhead.

- Reference

RP-251881	Study Item: Study on 6G Radio