3GPP TSG-RAN WG2 Meeting #132                                                                            R2-2508509

Dallas, USA, Nov. 17th - 21st 2025

Agenda item:	10.2.1

Source: 	ZTE Corporation, Sanechips

Title: 	Consideration on the 6G UE Capability

Document for:	Discussion and Decision

Introduction

In the last meeting, we reached the following agreements for the further 6G UE capability study:

For next meeting companies should focus on identifying the critical problems with UE capability framework and how to address them.  Identify what actions to be triggered with other WGs.

Companies can identify use cases to justify dynamic capability change in connected mode and analyse impacts to network side and network behaviour upon reception of updated capabilities.   Understand if there is impact to other WGs.

Companies can focus on the practical IODT problems in the field.   Possible ways to address them.

In this paper, we will share our views on the 6G UE capability based on these agreements.

Discussion

In this chapter, we will first discuss the UE capability framework, and then analyze the dynamic UE capability update, at last we share our views on the IODT problems.

  - UE Capability Framework

In this section, we will discuss UE capability reporting framework and the UE capability signaling framework.

    - 2.1.1 UE Capability Reporting Framework

In legacy 4G and 5G networks, UE capability reporting is triggered by a network request message. This request typically includes a capability filter that instructs the UE to report only the capabilities the network is interested in, as illustrated in Figure 1.

Fig 1: Legacy UE Capability Reporting Procedure

In 5G, a significant issue is the growing number of band combinations (BCs) that UEs are required to report. This results in capability signaling that exceeds two segments, particularly when the filter includes both FR1 and FR2 bands. For 6G, this problem should be addressed from both the network request and UE reporting perspectives.

From the UE reporting side, optimizations to the capability signaling structure should be considered—a topic that will be further discussed in the next chapter. From the network side, additional filtering granularity may be necessary. During early 5G discussions, there was also a lot of discussion on how to configure the filter, such as whether to include band combination information or MIMO layer parameters. However, no consensus was ultimately reached. For 6G, these issues should be re-evaluated in light of existing UE capability reporting challenges and the potential structure of 6G networks.

Proposal 1: The basic UE capability reporting procedure can be retained from 4G/5G, where the UE reports capabilities based on a network request. But RAN2 to study how to optimize the filtering mechanism to enable additional granularity for the filtering of UE capabilities.

In 5G, in addition to explicit UE capability reporting, the RACS concept was introduced in Release 16 as an implicit reporting method. It aims to reduce signaling overhead by defining UE capability templates and identifying UE capabilities through a capability ID. However, its implementation in 5G requires a template that covers all UE capabilities, which has hindered its commercialization. For 6G, the template-based concept can be revisited. RAN2 should discuss how to optimize the RACS mechanism.

Based on the companies’ contributions from the last meeting, there are two optimization directions regarding how to define the UE capability templates:

- Direction 1: The Capability ID is defined to cover all UE capabilities as in 5G, with optimizations performed through coordination with SA2/CT1 [1].

- Direction 2: Capability IDs are defined for parts of the UE capability set (e.g. for different modular or different functions), with each part associated with a separate ID [2].

For the direction 1, the issue of not being commercially viable under 5G still remains. In 6G, corresponding solutions must be found to facilitate its commercialization. For the direction 2,  by defining some commonly used capability templates for specific modules or functions, the UE can indicate the capabilities of the corresponding module or function by reporting the template ID, thereby reducing signaling overhead. Compared to Direction 1, Direction 2 is more straightforward to implement and is therefore easier to commercialize.

Proposal 2: In addition to the explicit UE Capability report, RAN2 should also support UE Capability ID based reporting and should study optimizations for the UE capability ID based reporting (e.g. enhanced RACS mechanism by defining templates for parts of UE capability sets).

    - 2.1.2 UE Capability Signaling Framework

As in the WID, one objective of the 6G UE capability is to improve and simplify the Radio signaling framework compared to 5G NR. To achieve this objective, first we need to identify the potential issues in the current 5G structure, and then discuss the potential improvements or simplifications based on these issues. During discussions on 5G UE capability, many new concepts were introduced, such as the decoupling of RF and BPC, as well as UL and DL. To decouple RF and BPC capabilities, the concepts of Band Combination and Feature Set were proposed. To decouple DL and UL, the feature set was further divided into DL feature set and UL feature set. These solutions have contributed to signaling simplification. However, the potential benefits of decoupling of RF and BPC, as well as UL and DL, have not been fully realized under the current architecture.

RF and BPC Decoupling

The intention of RF and BPC decoupling is to reduce the repeated reporting for Band Combinations (BCs) that share the same BPC capabilities. To implement this, one BC is linked to one FeatureSetCombinationID. However, due to the current FeatureSetCombination architecture (as shown in Fig. 2) and the fact that a single FeatureSet includes both UL and DL parts, the FeatureSetCombination reuse rate is very low in actual networks.

Fig 2: FeatureSetEntry List in 5G UE Capability Structure

Based on capability reporting from some commercial UEs, one UE reports 58 FeatureSetCombinations, but only 4 are reused; another reports 158, with only 33 reused. This means the UEs report 191 Band Combinations (BCs) using 158 FeatureSetCombinations.

Furthermore, even when BCs share the same BPC capability, the current structure does not support reporting capabilities for a group of BCs. This limitation further diminishes the potential benefits of RF and BPC decoupling.

Observation 1: The intention of the FeatureSetCombination was to implement RF and BPC decoupling. However, the reuse rate of FeatureSetCombinations is low with the current structure, and it does not support reporting capabilities for a group of BCs.

DL and UL Decoupling

The purpose of decoupling DL and UL was to prevent redundant DL reporting for BCs that share the same DL capabilities. However, this has not been effectively implemented in the current structure as follows.

FeatureSetCombination information element -- ASN1START -- TAG-FEATURESETCOMBINATION-START FeatureSetCombination ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FeatureSetsPerBand FeatureSetsPerBand ::= SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSet FeatureSet ::= CHOICE { eutra SEQUENCE { downlinkSetEUTRA FeatureSetEUTRA-DownlinkId, uplinkSetEUTRA FeatureSetEUTRA-UplinkId }, nr SEQUENCE { downlinkSetNR FeatureSetDownlinkId, uplinkSetNR FeatureSetUplinkId } } -- TAG-FEATURESETCOMBINATION-STOP -- ASN1STOP

With the current structure, each FeatureSet includes both featureSetDownlink and featureSetUplink, the UE can’t report more than one featureSetUplink for the same featureSetDownlink. Taking Table 1 from a commercial UE log as an example, the UE reports BCs for the combination of bands 28, 41, and 79, with up to 10 BCs listed. Among these 10 BCs, there are three different DL combinations. For each DL combination, there are three or four UL combinations. In other words, 3 or 4 UL combinations share the same DL combination. However, the UE still need to report 10 BCs for this case.

Table 1: Multiple reporting of the same BC

| Index | Band Numbers | DL BW class | DL FeatureSet ID list | UL BW class | UL FeatureSet ID list | FeatureSet Comb |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 28/41/79 | a,a,a | 4,1,9 | a,a,a | 8,7,0 | 6 |
| 2 | 28/41/79 | a,a,a | 4,1,9 | _,a,_ | 0,7,0 | 12 |
| 3 | 28/41/79 | a,a,a | 4,1,9 | _,a,_ | 0,2,0 | 20 |
| 4 | 28/41/79 | a,a,c | 4,1,5 | a,a,_ | 6,7,0 | 4 |
| 5 | 28/41/79 | a,a,c | 4,1,5 | _,a,a | 0,7,7 | 14 |
| 6 | 28/41/79 | a,a,c | 4,1,5 | _,a,_ | 0,2,0 | 1 |
| 7 | 28/41/79 | a,c,a | 4,10,1 | a,a,a | 8,7,0 | 16 |
| 8 | 28/41/79 | a,c,a | 4,10,1 | a,a,a | 0,7,7 | 8 |
| 9 | 28/41/79 | a,c,a | 4,10,1 | a,a,_ | 0,2,0 | 13 |
| 10 | 28/41/79 | a,c,a | 4,10,1 | a,c,_ | 0,9,0 | 7 |

Even if a super BC (with DL bandwidth classes a, c, and c, and UL bandwidth classes a, c, and a) were used with 10 FeatureSetEntries under the current FeatureSetCombination structure, the DL repetition in each FeatureSetEntry would still be unavoidable.

Observation 2: A FeatureSet inherently contains both downlink and uplink components, which undermines the potential gains of DL and UL decoupling.

If Observations 1 and 2 are correct, a more flexible structure for RF/baseband decoupling and DL/UL decoupling should be considered for 6G. Furthermore, 6G is expected to support more flexible network architectures (e.g., UL-only cells). This will introduce a wider variety of band combinations, which will also require a more flexible BC reporting structure.

Proposal 3:  RAN2 to study a more flexible 6G capability structure to better support RF/baseband decoupling  and DL/UL decoupling (e.g. to solve the known problems of the current 5G UE capability signaling structure and to support new 6G use cases / network architectures such as multi-carrier and UL-only cells).

Based on proposal 3, RAN2 also requires input from RAN1 and RAN4 on RF/baseband and DL/UL decoupling. Typically, RAN1 and RAN4 discuss UE capability at a later stage. However, from RAN2's perspective, the UE capability signaling structure is a critical component that should be studied as early as possible, particularly for issues requiring inter-WG cooperation.

For the RF/Based band decoupling, based on the degree of decoupling, it can be divided into the following two types:

- Type 1: Light decoupling, which requires explicit methods to indicate the coupled baseband and RF capabilities, e.g. Current 5G signaling structure, the UE indicates the FeatureSetCombinationID for each band combination.

- Type 2: Deep decoupling, where the coupling relationship between RF and baseband capabilities is defined only through a set of rules. E.g. as shown in Fig 3 and agreed in [3], baseband capability is applicable in band combinations (RF) which consist of bands 1) with equal bandwidth class, and 2) equal or higher MIMO layer capability.

Fig 3: 5G Deep Decoupling Method [4]

Compared with Type 1, Type 2 reduces the signaling overhead required to indicate the linkage between RF and baseband capabilities, as well as between DL and UL capabilities. However, the Type 2 solutions were not actually implemented in 5G, as companies had significant concerns about the testing burden. For example, it might require UEs to pass tests on all possible combinations of RF and baseband capabilities. For 6G, whether this test burden issue persists can be further studied. If solutions emerge to address this concern, the deep decoupling mechanism could significantly reduce capability signaling overhead.

Observation 3: During the 5G discussion, two types of RF and baseband capability decoupling were identified:

- Type 1: Light decoupling, which requires explicit methods to indicate the coupled DL and UL capabilities.

- Type 2: Deep decoupling, where the coupling relationship between DL and UL capabilities is defined only through a set of rules.

Similar to RF/baseband decoupling, in the current architecture, the UL and DL are tightly coupled for a BC, as noted in Observation 2. However, from a chipset design perspective, the DL and UL can be designed separately. Therefore, it is worth considering whether it is necessary to maintain a tight explicit linkage between the DL and UL feature sets, or whether a deeper level of decoupling could also be feasible for 6G. In the 6G, more flexible network architecture is expected (e.g., UL-only cells), which also requires more flexible UL and DL decoupling. Furthermore, RAN1 would also start the DL/UL decoupling discussion from the Feb meeting next year, and thus for this issue, it is suggested to first consult RAN1 and RAN4.

Proposal 4: Send an LS to RAN1 and RAN4 to study the RF/baseband and DL/UL decoupling structure.

Redundant BC Reporting Structure

Furthermore In the current 5G UE capability structure, two separate Band Combination (BC) reporting branches exist: one for normal BC and the other for Uplink Transmit (UL Tx) Switching.

BandCombinationList-v1840 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1840 BandCombinationList-UplinkTxSwitch-r16 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-r16 BandCombinationList-UplinkTxSwitch-v1630 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1630 BandCombinationList-UplinkTxSwitch-v1640 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1640 BandCombinationList-UplinkTxSwitch-v1650 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1650 BandCombinationList-UplinkTxSwitch-v1670 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1670 BandCombinationList-UplinkTxSwitch-v1690 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1690

This kind of structure leads to signalling redundancy and complexity. For example, the difference between BCs that support TxSwitching and those that do not may only lie in parameters at the UL FSPC level, while many parameters at the PerBC or even PerFeatureSet level may be consistent. However, for normal CA under a BC and CA implemented through the UL TxSwitching method, the UE has to report the common capabilities part repeatedly in both 2 braches.

This dual-branch architecture introduces signaling redundancy and complexity. For instance, the difference between BCs that support Tx Switching and those that do not might only involve parameters at the UL FSPC level, while many parameters at the PerBC or even PerFeatureSet level could be identical. Nevertheless, for capabilities common to both normal CA under a BC and CA implemented via the UL Tx Switching method, the UE is required to report them redundantly across both branches.

Observation 4: The dual BC reporting branch structure (for normal BC and UL Tx switching CA, respectively) in the 5G UE capability framework leads to signaling redundancy and increased complexity, as common capabilities often need to be reported repeatedly.

Besides the problems in the above observations, the other issue of the current UE capability structure is about the high repetition rate of capability parameters. In 5G, each capability parameter has a defined granularity (e.g., per UE, per band, per band combination, etc.). However, in practice, different UEs may support different granularities for the same capability parameter. According to logs from commercial UEs, some devices support coarser granularities than specified for certain finely granular parameters. For example, in the case of per-BC CSI-RS-related capability reporting (Table 2), some UEs report the same capability values across all band combinations. Even in such cases, the UE is still required to report these capabilities per BC, resulting in a high repetition rate. A similar issue occurs with other fine-grained reporting schemes, such as per-band capabilities or per FeatureSet capabilities.

Table 2: High Repetition Rate of Capability Parameters

| BC | DL and UL cc number | CSI-RS port Num | CSI-RS beam Num | CSI report num |
| --- | --- | --- | --- | --- |
| N1-N41-N77-N79 | DL4CC;UL2CC | 164 | 16 | 16 |
| N1-N28-N41 | DL3CC | 164 | 16 | 16 |
| N1-N28-N41-N77 | DL4CC | 164 | 16 | 16 |
| N28-N41 | DL3CC;UL2CC | 164 | 16 | 16 |
| N28-N41-N77-N79 | DL4CC;UL2CC | 164 | 16 | 16 |
| N40-N41-N79 | DL3CC | 164 | 16 | 16 |
| … | … | … | … | … |
| N41-N41-N41-N41 | DL4CC | 164 | 16 | 16 |

Observation 5: With 5G structure, although some UEs support a coarser granularity in practice for certain capability parameters, they are still required to report them at the defined finer granularity, resulting in a high repetition rate.

Proposal 5: For the 6G UE capability structure, RAN2 should study mechanisms to avoid the repeated reporting seen in 5G.

  - Dynamic Capability Update

In the last meeting, there is a hot discussion on the dynamic capability update and thus the following agreements was achieved for the further study.

Companies can identify use cases to justify dynamic capability change in connected mode and analyse impacts to network side and network behaviour upon reception of updated capabilities.   Understand if there is impact to other WGs.

In this chapter, we will first discuss the following issues individually and then present our proposal based on the analysis.

- Issue 1: Whether enhancements to the semi-persistent capability update procedure are needed.

- Issue 2: How to handle the "plan to change" and "already changed" cases.

- Issue 3: How to reduce the processing complexity on the network side.

Issue 1: Whether enhancements to the semi-persistent capability update procedure are needed.

According to companies' contributions from the last meeting, two high-level scenarios were mentioned:

- Semi-persistent capability update, which requires the CN to store the updated capabilities.

- Temporary capability update, which has no impact on the CN.

Scenario 1 is implemented via the re-attach procedure in 4G/5G, where the UE cannot perform a semi-persistent capability update while in the connected state.

24501 5.5.1.3.2: If the UE is in the 5GMM-CONNECTED mode and the UE changes the radio capability for NG-RAN or E-UTRAN, the UE may locally release the established N1 NAS signalling connection and enter the 5GMM-IDLE mode.

However, if this restriction were lifted, it would allow the UE to perform a capability update during the connected state and request the Core Network (CN) to update the stored UE capabilities accordingly. In such a case, the RAN node would also need to synchronize with the updated capability in a timely manner. These changes would pose significant challenges to the implementation of both the CN and RAN. Furthermore, since this type of capability update is not expected to occur frequently, we don’t think there is a need to modify the current mechanism for semi-persistent capability update unless a strong motivation is identified.

Observation 6: For semi-persistent capability updates that require the CN to store the updated capabilities, there is no strong motivation to enhance the existing mechanism (i.e., the re-attach procedure in 4G/5G).

Issue 2: How to process “plan to change” and “already changed” cases

Regarding temporary capability updates, as summarized in Table 3, 5G already covers scenarios such as overheating, power saving, and MUSIM. A foldable phone case has now been proposed for 6G. Although XR and UE aggregation scenarios were also mentioned in the contributions, it remains unclear whether they would impact the dynamic capability update procedure. Therefore, these two scenarios are not included in the table.

Table 3: 5G UAI with Capability Update

| 5G | Scenario | Parameter Type | Configuration/Timer |
| --- | --- | --- | --- |
|  | Overheating | ReducedMaxCCs(DL/UL) | T345 prohibit timer : s0~s600 |
|  |  | ReducedMaxBWDL/UL |  |
|  |  | reducedMaxMIMO-LayersDL/UL |  |
|  | Power Saving | MaxBW-Preference | T346B prohibit timer: s0~s30 |
|  |  | MaxCC-Preference | T346C prohibit timer: s0~s30 |
|  |  | MaxMIMO-LayerPreference-r16 | T346D prohibit timer: s0~s30 |
|  |  | MinSchedulingOffsetPreference | T346E prohibit timer: s0~s30 |
|  |  | DRX-Preference-r16 | T346A prohibit timer: s0~s30 |
|  | MUSIMR18 (Reactive) | MUSIM-MaxCC-r18 | T348 (musim-WaitTimer) ms10~ms100 |
|  |  | SCG/Cell release |  |
|  |  | MIMO layer and bandwidth |  |
|  | MUSIMR18 (Proactive) | MUSIM affected band/BC and forbidden band/BC with affected MIMO layer and bandwidth | T346n,Prohibit timer s0~s10 |
| 6G | Foldable Phone | MIMO layer | N/A |

During the last meeting discussion, companies sought to clarify whether the "dynamic capability update" refers to a scenario where the UE capability is planned to change or has already changed. For issues such as 5G overheating, power saving, and MUSIM, they fall under the "plan to change" category. In such cases, the UE would indicate its preferred capability restriction to the network and await a response.

However, for the foldable phone case, it is unclear whether the UE has already changed its capability when it triggers the update indication. According to company contributions, foldable phones may support different MIMO layers when folded versus unfolded. If this is considered an "already changed" scenario, the UE could indicate the updated MIMO layer capability via the CSI report. This approach allows the UE to signal the change immediately, enabling the network to perform scheduling based on the latest CSI report.

Observation 7: The dynamic capability update may include both "plan to change" and "already changed" cases. For certain capabilities (e.g., MIMO layers), the issue could also be resolved at the lower layers especially for the “already changed” case.

Issue 3: How to reduce the processing complexity on the network side

From the network side, determining the available capabilities for a UE is usually the first step when configuring a UE, and thus if the capabilities are updated, the network needs to go back to the first step for the configuration. If UEs were to signal a larger number of parameters, it would introduce significantly higher processing complexity on the network side. Furthermore, the number of scenarios would also impact the network side’s processing, because the network needs to determine the processing priority and strategy for each scenario by taking all of the UE into consideration. Additionally, as described above, some capability updates can be addressed through the lower layers. Thus, if the issue can be resolved by the lower layers, it can also help to offload part of the processing complexity.

Observation 8: The number of updated capabilities as well as the number of scenarios would significantly affect the processing complexity on the network side.

Meanwhile, in live networks, we have also observed scenarios where terminals misuse cause values, which needs to be addressed in 6G as well. For example, some MUSIM devices may report UAI with “overheating” to get a fast response from the network.

Additionally, the prohibit timer mechanism allows the network to control the frequency of capability updates. This is also a key factor that enables the practical implementation of features like overheating protection and power saving in commercial 5G networks.

Furthermore, the legacy 5G architecture does not require the network to respond to the UE immediately, even for overheating, which facilitated the commercialization of the UAI functionality.

For existing 5G use cases, the network is typically designed to respond to UE overheating indications. For power saving, the effectiveness depends on the network's scheduling algorithm. Requiring an immediate response would be a very strict and even impossible requirement for the network side implementation.

Based on the discussion of the above three issues, we propose the following Proposal:

Proposal 6: For dynamic capability update mechanisms, the following principles should be considered:
(1) No impact on the CN;
(2) Focus first on the scenarios in which the UE plans to change its capabilities;
(3) Only focus on critical parameters in critical scenarios that cannot be resolved by other WGs/layers;
(4) Prevent cause/scenario misuse;
(5) Avoid frequent requests;
(6) Do not require an immediate network response.

  - IODT

In the last meeting, IODT issues were discussed based on [1][5], and the following agreement was reached for further study.

Companies can focus on the practical IODT problems in the field.   Possible ways to address them.

In the field, IODT issues occur frequently for the following two reasons:

- A UE cannot conduct IODT tests with every network version from every vendor before setting a capability to "supported"; similarly, a network cannot perform IODT tests with every software version from every UE vendor before enabling a feature.

- Even if such exhaustive testing were conducted, it could not possibly cover all scenarios encountered in live networks, especially the cases that happened only when multiple conditions are satisfied simultaneously.

For example, we encountered a handover failure issue involving a Secondary Cell in a live network: the affected UE initiates reestablishment when the primary cell has a multi-BWP configuration and the serving SSB index changes due to a handover, with the old and new SSBs located in different slots.

To resolve these kinds of issues, different companies have proposed varying directions in online discussions, for example:

- Direction 1: UE-Driven Solution: Address the issue on the UE side using network information (e.g. network vendor or version information). For instance, the UE could be informed of the network vendor/version and then adjust its reported capabilities accordingly.

- Direction 2: Network-Driven Solution: Address the issue on the network side using UE information (e.g. UE/Chip vendor or version information). For instance, the network could be informed of the UE vendor/version and then interpret or handle the UE's capabilities accordingly.

For Direction 1, challenges remain in inter-vendor handover scenarios. For example, if a UE has passed the IODT test with network vendor 1 but has not completed the test with network vendor 2, it will report a feature as supported while connected to vendor 1. However, after handover to network vendor 2, that network vendor 2 will also assume the capability is supported, potentially leading to IODT failures. Furthermore, even if a UE has passed IODT tests with a particular network, interoperability issues may still occur under specific network conditions, especially those involving multiple concurrent factors. Once this issue occurs, the network still needs UE information for further processing in the live network.

Observation 9: Direction 1 (i.e. UE-Driven Solution) has limitations in handover scenarios and cannot fully address interoperability issues that arise under certain special conditions in the live network, even if the UE has passed the corresponding IODT test.

For Direction 2, the network has knowledge of which UE types have passed IODT testing. With UE information available, the network can handle different cases more effectively. Take the handover case above as an example: a UE that passed IODT with network vendor 1 but not network vendor 2 may still report a feature as supported. The network vendor 1 would consider it supported, but after handover to the network vendor 2, the latter could treat the capability as unsupported if it is aware of the UE’s information.

Moreover, in cases where interoperability issues arise under rare conditions despite previous IODT validation, the network can temporarily disable the related feature for the affected UEs based on their information. This step is critical for the controlled commercialization of new features.

Proposal 7: For IODT issues, RAN2 to study a Network-Driven approach in which UE information (e.g., vendor/version) is used to better manage UE feature behavior for affected UE types and avoid errors and signalling overhead in case of inter-vendor handovers.

Conclusion

Based on the above discussion, we have the following observations and proposals:

---- UE Capability Framework

Proposal 1: The basic UE capability reporting procedure can be retained from 4G/5G, where the UE reports capabilities based on a network request. But RAN2 to study how to optimize the filtering mechanism to enable additional granularity for the filtering of UE capabilities.

Proposal 2: In addition to the explicit UE Capability report, RAN2 should also support UE Capability ID based reporting and should study optimizations for the UE capability ID based reporting (e.g. enhanced RACS mechanism by defining templates for parts of UE capability sets).

Observation 1: The intention of the FeatureSetCombination was to implement RF and BPC decoupling. However, the reuse rate of FeatureSetCombinations is low with the current structure, and it does not support reporting capabilities for a group of BCs.

Observation 2: A FeatureSet inherently contains both downlink and uplink components, which undermines the potential gains of DL and UL decoupling.

Proposal 3:  RAN2 to study a more flexible 6G capability structure to better support RF/baseband decoupling  and DL/UL decoupling (e.g. to solve the known problems of the current 5G UE capability signaling structure and to support new 6G use cases / network architectures such as multi-carrier and UL-only cells).

Observation 3: During the 5G discussion, two types of RF and baseband capability decoupling were identified:

- Type 1: Light decoupling, which requires explicit methods to indicate the coupled DL and UL capabilities.

- Type 2: Deep decoupling, where the coupling relationship between DL and UL capabilities is defined only through a set of rules.

Proposal 4: Send an LS to RAN1 and RAN4 to study the RF/baseband and DL/UL decoupling structure.

Observation 4: The dual BC reporting branch structure (for normal BC and UL Tx switching CA, respectively) in the 5G UE capability framework leads to signaling redundancy and increased complexity, as common capabilities often need to be reported repeatedly.

Observation 5: With 5G structure, although some UEs support a coarser granularity in practice for certain capability parameters, they are still required to report them at the defined finer granularity, resulting in a high repetition rate.

Proposal 5: For the 6G UE capability structure, RAN2 should study mechanisms to avoid the repeated reporting seen in 5G.

---- Dynamic Capability Update

Observation 6: For semi-persistent capability updates that require the CN to store the updated capabilities, there is no strong motivation to enhance the existing mechanism (i.e., the re-attach procedure in 4G/5G).

Observation 7: The dynamic capability update may include both "plan to change" and "already changed" cases. For certain capabilities (e.g., MIMO layers), the issue could also be resolved at the lower layers especially for the “already changed” case.

Observation 8: The number of updated capabilities as well as the number of scenarios would significantly affect the processing complexity on the network side.

Proposal 6: For dynamic capability update mechanisms, the following principles should be considered:
(1) No impact on the CN;
(2) Focus first on the scenarios in which the UE plans to change its capabilities;
(3) Only focus on critical parameters in critical scenarios that cannot be resolved by other WGs/layers;
(4) Prevent cause/scenario misuse;
(5) Avoid frequent requests;
(6) Do not require an immediate network response.

---- IODT

Observation 9: Direction 1 (i.e. UE-Driven Solution) has limitations in handover scenarios and cannot fully address interoperability issues that arise under certain special conditions in the live network, even if the UE has passed the corresponding IODT test.

Proposal 7: For IODT issues, RAN2 to study a Network-Driven approach in which UE information (e.g., vendor/version) is used to better manage UE feature behavior for affected UE types and avoid errors and signalling overhead in case of inter-vendor handovers.

Reference

- R2-2506988	 Considerations on UE capability signalling in 6G	  Qualcomm

- R2-2506797	 Considerations on 6GR general aspects	                  vivo

- R2-1801701      RAN2#100_Meeting_Report_Final                            RAN2

- R2-1712369	  UE capability structure of NR and MR-DC        	        Qualcomm

- R2-2507607      Consideration on the 6G UE Capability                      ZTE Corporation