3GPP TSG-RAN WG2 Meeting #132                                                         R2-2508520

Dallas, US, Nov 17th - 21st 2025

Agenda item:	8.2.2

Source: 	ZTE Corporation, Sanechips

Title: 	Remaining issues in R19 Ambient-IoT

Document for:	Discussion and Decision

Introduction

Based on the post email discussion on AIoT MAC spec after RAN2#131bis meeting, a list of open issues have been identified[1], e.g., regarding no data available case, MDI handling, indication of ID type. In this contribution, we discuss the remaining issues in R19 ambient-IoT as listed in the below and provide our considerations.

- Issue 3-7: Checking whether there is new case for “no upper layer data available” other than long writing operation which may impact to how to set MDI field.

- New issue 6-1: Whether there is RAN2 further impact regarding CT1 LS C1-256624 on handling of inventory and command collision.

- New issue 6-2: Confirm to apply the no data available case to more general delayed NAS cases, e.g., inventory response message.

- New issue 6-3: Whether there is AS impact to support Temp ID, e.g., including upper layer ID type in paging message. Or, assume CT4 will address this based on SA2 LS S2-2509655.

Discussion

  - (Issue 3-7) More data indication handling

During RAN2#131bis meeting, new case with no NAS response has been discussed. And it was agreed that for cases other than integrity failure, AS will indicate no NAS response expected to reader. And integrity failure case, RAN2 assumes that there is no AS response to the reader for now, and an LS was sent to SA3 [2] to ask whether AS response to reader can be sent due to integrity failure.

- Agreements RAN2 confirms, in addition to delayed response, it is valid that in some cases A-IoT NAS doesn’t provide a response at all. For cases other than integrity failure, AS will indicate no NAS response expected to reader. FFS how (e.g. using 0 SDU & MDI, or new indication). For integrity failure, for now RAN2 assumes that there is no AS response to the reader. Ask SA3 ccCT1 whether a similar mechanism (e.g. AS response to the reader) can be used to indicate to reader no NAS response due to integrity failure.

On the other hand, an LS from CT1 was received in [3] to inform that CT1 has agreed a pCR that the AIoT device shall provide an indication to the lower layer when a received AIoT NAS message was ignored (e.g. when the received NAS message is too short) as agreed in the CR [4]. According to [4], the motivation is to differentiate from the delayed NAS response case and prevent the reader from continuously sending R2D messages while waiting for a D2R response. And based on TS 24.369, device shall discard the AIoT NAS message which fails the integrity check and provide an indication to the lower layers. And whether AS response can be sent to reader due to integrity failure still needs to wait for the reply LS from SA3.

C1- 256640 - 6.2.1 Message too short When a message is received that is too short to contain a complete message type information element, that message shall be ignored, c.f. 3GPP TS 24.007 [3], and the AIoT device shall provide an indication to the lower layers that the message was ignored. TS 24.369: In case of an AIoT NAS message fails the integrity check by the AIoT NAS, the AIoT device shall discard the AIoT NAS message and provide an indication to the lower layers.

As we can see, no matter whether AS response can be sent to reader due to integrity failure, device may send D2R upper layer data transfer message without data SDU to reader in the following two cases. In order that reader can determine whether to send another R2D upper layer data transfer mesasge after receiving a D2R message without data SDU, the MDI field can be used to indicate whether there will be NAS response later. When there is more data to be sent, the MDI field is set to “1”; while the MDI field is set to “0” when there will be no more data to be sent to the reader.

- Case 1: the received NAS message was ignored by the device and there is no NAS response, e.g. received message is too short.

- Case 2: NAS response is delayed due to the long “write” operation time.

Proposal 1: MDI field is used to indicate whether there will be NAS response later. When there is more data to be sent, the MDI field is set to “1”; while the MDI field is set to “0” when there will be no more data to be sent to the reader.

  - (Issue 6-2) Apply the no data available case to more general delayed NAS cases

During the post email discussion,  NEC raised the issue that the timing between CFA R2D and responding D2R message (is about 200us for some chip duration based on TS38.291) is too small for authentication, that there may be no upper layer data, i.e., inventory report NAS message, generated by upper layer at the time to transmit D2R Upper Layer Data Transfer message after CFA paging message. And it was proposed that the procedure text in TS 38.391 on R2D message reception to allow same mechamism as delayed NAS response case can be applied to this case, i.e. another R2D Upper Layer Data Transfer message is sent to reschedule the delayed response message for CFA paging.

In our understanding, if inventory report NAS message is not available at the time to transmit D2R Upper Layer Data Transfer message after CFA paging message, device will send a D2R Upper Layer Data Transfer message with 0 SDU. In this situation, reader can re-transmit the CFA paging mesage to reschedule the device to send the NAS inventory report message, which is already supported in current specification. And device can determine that the paging message is the same as the previous one by implementation, so that it can directly include the corresponding NAS inventory report mesasge in the data SDU.

Proposal 2: If inventory report NAS message is not available at the time to transmit D2R Upper Layer Data Transfer message after CFA paging message, reader can re-transmit the CFA paging mesage instead of R2D Upper Layer Data Transfer message to reschedule the device.

  - (Issue 6-1) Handling of inventory and command collision

An LS was received from CT1 on handling of inventory and command collision in [5]. In the LS, it is informed that CT1 concluded that  there can be an attack scenario of false AIoT pagings that would abort any ongoing valid paging or command, so CT1 asks RAN2 whether it is possible for the AIoT device to respond to the new AIoT paging after completion of an ongoing inventory or command when collision happens.

C1-256624 LS on handling of inventory and command collision Based on above RAN2 agreement, CT1 has discussed the following two scenarios: a) Collision between inventory procedure and inventory procedure: The AIoT NAS layer receives a new inventory from lower layer when the AIoT NAS layer is performing the inventory procedure b) Collision between inventory procedure and command procedure: The AIoT NAS layer receives a new inventory from lower layer when the AIoT NAS layer is performing the command procedure CT1 concluded that there can be an attack scenario of false AIoT pagings that would abort any ongoing valid paging or command, and thereby temporarily or continuously block valid inventory procedure and command procedure execution by the AIoT devices. Hence, CT1 kindly ask RAN2 to provide their view on whether it is possible for the AIoT device to respond to the new AIoT paging after completion of an ongoing inventory or command when collision happens.

This issue has been discussed in RAN2 in previous meetings for overlapping multi-reader scenario and in spite of many companies highlighting this issue, RAN2 has decided to go with the solution which allows the device to respond to any new paging even if there is an ongoing procedure. The only way to fix this in Rel-19 would be to revert this agreement, but this is too big a change at this last minute. So, RAN2 should respond to CT1 acknowledging the issue and to confirm that RAN2 design does not solve this issue in Rel-19 but this can be solved in Rel-20.

Proposal 3: RAN2 replies to CT1 as following: RAN2 has discussed the highlighted scenario in Rel-19 and made a conscious decision that the device should respond to any paging message in Rel-19 under the assumption that overlapping reader case in Rel-19 is a rare case. It is too late to change this design decision as it will have significant impact to RAN2 design. Enhancements to this can be considered in R20. The handling of inventory and command collision issue will be address in R20.

  - (Issue 6-3)Indication of paging ID type and T-ID handling to device

New issue 6-3: Whether there is AS impact to support Temp ID, e.g., including upper layer ID type in paging message. Or, assume CT4 will address this based on SA2 LS S2-2509655.

During RAN2#131bis meeting, the issue of indication of paging ID type was briefly discussed, and it was concluded that RAN2 wait for SA2 and discuss in November. And there is an overall guidance that after December, NBC changes should be avoided as much as possible similar to NR MAC process.

Based on TS 33.369, the mechanisms to protect AIoT device identifier privacy during the inventory procedure are defined based on the use of a Temporary ID. And when privacy protection is not used during the inventory procedure, AIoT device permanent ID is included in the paging message as a device identification information. So the paging ID included in the paging message can be a device permanent ID, a temporary ID, or filtering information. Furthermore, there are two types of T-ID captured in TS 33.369 , including concealed type or stored type. So there are four types of paging ID in paging message. And different behaviours need to be adopted at device upon reception of paging message including different types of paging IDs. Therefore, device needs to distinguish the ID type of the paging ID included in the paging message, so that it can determine whether to respond to the paging message properly:

- If paging ID is device permanent ID, device checks whether its stored device permanent ID matches the paging ID in the paging message to determine whether to respond to the paging message.

- If paging ID is filtering information, device checks whether to respond to the paging message based on the filtering information in the paging message.

- If paging ID is concealed type T-ID, device generates a new T-ID based on received RANDAIOT_n in the paging message, and then checks whether the generated T-ID matches the T-ID contained in the paging message to determine whether to respond to the paging message.

- If paging ID is stored type T-ID, device checks whether the stored T-ID matches the T-ID contained in the paging message to determine whether to respond to the paging message.

Furthermore, based on TS 33.369, device needs to be indicated whether the stored T-ID type shall be updated with or without a command. And in case the stored T-ID update shall be done without a command, the AIoT Device generates a new Temp_ID_n+1.

Excerpts from TS 33.369: - 5.4.3 Procedure for AIoT Device identifier protection with Temp ID update during Individual inventory For the protection of AIoT device permanent identifier during the inventory procedure with AIoT device identifier described in clause 5.2.2, the following changes shall apply: - In step 1, AIOTF shall retrieve a T-ID in addition to the RANDAIOT_n from ADM. The ADM shall, based on T-ID type, either fetch the stored T-ID in the AIoT device profile or generate the T-ID as specified in Annex B.1. - In step 2, 3 and 4, the T-ID shall be used as a device identification information. - In step 2 and 3 the AIOTF includes indication of type of T-ID handling. T-ID can be either concealed type or stored type. The concealed type can be based on either the stored T-ID or the permanent identifier. If needed the handling also indicates whether the stored T-ID type shall be updated with or without a command. NG-RAN includes the T-ID handling in the paging message. - In step 4, the AIoT device, based on the T-ID handling indication in the paging message, generates the T-ID in the same way as the ADM did in step 1. The AIoT device determines it needs to reply to the NG-RAN if the generated T-ID matches with the received T-ID. In case the stored T-ID update shall be done without a command, the AIoT Device generates a new Temp_ID_n+1 as specified in Annex B.1 and stores the new Temp ID_n+1.

As analyzed above, the paging ID type (i.e. device permanent ID, concealed T-ID, stored T-ID, or filtering information) and T-ID handling type (i.e. whether the stored T-ID type shall be updated with a command) needs to be informed to devoce. During the post email discussion, companies have different views on how to inform the ID type and T-ID handling type to device. One way is to indicate in MAC layer, i.e. AIOTF indicates the paging ID type and T-ID handling type to NG-RAN, and then NG-RAN includes them in the paging message, as captured in TS 33.369. Another way is to indicate in NAS layer, e.g. paging ID type and T-ID handling type are indicated in the paging ID.

As we can see, currently all NAS information (i.e. paging ID, security parameter) in paging message is visible at A-IoT MAC layer, i.e. not encapsulated as upper layer data SDU. So no matter whether the type is indicated in NAS or AS layer, there is no much difference to the device complexity. And we are not sure no sure whether CT4 would address this based on SA2 LS S2-2509655, considering that the indicating paging ID type and T-ID handling type is not mentioned or requested in the LS. In order to avoid potential NBC changes in future, it is suggested that RAN2 adopts that paging ID type and T-ID handling type are indicated in the MAC layer in paging message to align with SA3 spec.

Observation 1 (Issue 6-3): Device needs to be informed of paging ID type (i.e. device permanent ID, concealed T-ID, stored T-ID, or filtering information) and T-ID handling type (i.e. whether the stored T-ID type shall be updated with a command) in the paging message.

Proposal 4 (Issue 6-3): A 3 bit field is introduced in paging message to indicate the paging ID type (i.e. device permanent ID, concealed T-ID, stored T-ID, or filtering information) and T-ID handling type (i.e. whether the stored T-ID type shall be updated with a command).

Another issue is about the presence of the paging ID length field. According to the A-IoT MAC rapporteur CR [6], the presence of Paging ID Length field is indicated by the Paging ID Presence Indication field, i.e. Paging ID Length field shall be included if Paging ID field is present. On the other hand, based on the agreed CT4 CR C4-254398 [7], the length of device temporary ID is 32 characters as copied in the below. Therefore, if device temporary ID is included as paging ID, the paging ID length field can be omited, considering that the length of device temporary ID is fixed. In this way, unnecessary overhead can be avoided, i.e. around 10bits can be reduced if device permanent ID is extended as indicated by SA2. And this is independent on whether paging ID type is indicated in MAC or NAS layer, since A-IoT Device Identification Requested in the NGAP INVENTORY REQUEST message is not included as NAS PDU and it was agreed in RAN2 that the paging ID is visible to the reader.

A-IoT MAC rapporteur CR: - Paging ID Presence Indication (PIPI): The length of the field is 1 bit. This field indicates whether Paging ID and Length of Paging ID fields are present (when set to 1) or absent (when set to 0). - Paging ID Length: This field is optionally present, as indicated by Paging ID Presence Indication field. If present, the length of the field is 8 bits. This field indicates the length of the Paging ID field in unit of bits when Paging ID field is present. C4-254398 Table 6.1.6.3.2-1: Simple data types Type Name; Type Definition; Description; Applicability. AfId; string; Identifier used to identify the AF.. Tid; string; pattern: "^[A-Fa-f0-9]{32}$".

So we suggest to update the field description of Paging ID Presence Indication (PIPI) and Paging ID Length field as below:

Proposed text proposal to TS 38.391: - Paging ID Presence Indication (PIPI): The length of the field is 1 bit. This field indicates whether Paging ID field is present (when set to 1) or absent (when set to 0). - Paging ID Length: This field is optionally present when the Paging ID is not AIoT Device Temporary Identifier, as indicated by Paging ID Presence Indication field. If present, the length of the field is 8 bits. This field indicates the length of the Paging ID field in unit of bits when Paging ID field is present.

Proposal 5: The paging ID length field is absent when device temporary ID is included as paging ID, considering that the length of device temporary ID is fixed, to avoid unnecessary overhead in paging. This is applicable no matter whether paging ID type is indicated in MAC or NAS layer since paging ID received from AIOTF is visible to the reader.

Conclusion

In this paper, we discussed remaining issues in R19 Ambient-IoT. And we have the following observations and proposals:

Proposal 1: MDI field is used to indicate whether there will be NAS response later. When there is more data to be sent, the MDI field is set to “1”; while the MDI field is set to “0” when there will be no more data to be sent to the reader.

Proposal 2: If inventory report NAS message is not available at the time to transmit D2R Upper Layer Data Transfer message after CFA paging message, reader can re-transmit the CFA paging mesage instead of R2D Upper Layer Data Transfer message to reschedule the device.

Proposal 3: RAN2 replies to CT1 as following: RAN2 has discussed the highlighted scenario in Rel-19 and made a conscious decision that the device should respond to any paging message in Rel-19 under the assumption that overlapping reader case in Rel-19 is a rare case. It is too late to change this design decision as it will have significant impact to RAN2 design. Enhancements to this can be considered in R20. The handling of inventory and command collision issue will be address in R20.

Observation 1 (Issue 6-3): Device needs to be informed of paging ID type (i.e. device permanent ID, concealed T-ID, stored T-ID, or filtering information) and T-ID handling type (i.e. whether the stored T-ID type shall be updated with a command) in the paging message.

Proposal 4 (Issue 6-3): A 3 bit field is introduced in paging message to indicate the paging ID type (i.e. device permanent ID, concealed T-ID, stored T-ID, or filtering information) and T-ID handling type (i.e. whether the stored T-ID type shall be updated with a command).

Proposal 5: The paging ID length field is absent when device temporary ID is included as paging ID, considering that the length of device temporary ID is fixed, to avoid unnecessary overhead in paging. This is applicable no matter whether paging ID type is indicated in MAC or NAS layer since paging ID received from AIOTF is visible to the reader.

Reference

- A-IoT MAC open issues summary

- R2-2507915 LS on integrity failure

- C1-256623 LS on no Command Response in NAS

- C1-256640 Indication to lower layers when the AIoT NAS message is ignored

- C1-256624 LS on handling of inventory and command collision

- A-IoT MAC rapporteur CR

- C4-254398 AIoT support for T-IDs