3GPP TSG-RAN WG2 Meeting #132                                                       R2-2508521

Dallas, US, Nov 17th - 21st 2025

Agenda item:	9.2.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on Ambient-IoT topology 2

Document for:	Discussion and Decision

Introduction

During RAN2#131bis meeting, Ambient-IoT in topology 2 was discussed and some preliminary agreements were achieved on resource allocation. In this contribution, we continue to discuss the support of Ambient-IoT in topology 2, e.g. regarding deployment scenarios, resource allocation, Uu procedure for A-IoT operation, etc.

Discussion

  - Deployment scenario

According to [2], both device 1 and active device needs to be considered for the support of topology 2. And the work on active device/DO-A will start based on RAN1 study after March 2026. So we can focus on the support of topology 2 according to D2T2-B for device 1 for DO-DTT and DT traffic types, to support  indoor inventory and  indoor command. According to TR 38.769,  Deployment Scenario 2 with Topology 2 according to D2T2-B is illustrated as in the below. When an intermediate UE acts as a Reader, the UE reader relays and transfers AIoT data between BS and the AIoT device.

Figure 1. D2T2-B

According to the WID, FR1 licensed spectrum in FDD is used for topology 2, with R2D in UL spectrum, and D2R and CW in UL spectrum. In other words, UE reader needs to transmit and receive in UL FR1 spectrum in AIoT radio interface with AIoT device, and meanwhile communication with base station in Uu air  interface as illustrated in figure 2.

Figure 2. Topology 2 with R2D in UL spectrum, and D2R and CW in UL spectrum

Observation 1: In AIoT topology 2, both D2R and R2D uses FR1 UL spectrum. UE reader needs to transmit and receive in UL FR1 spectrum in AIoT radio interface for communication in R2D and D2R direction with AIoT device, and also maintain communication with base station in Uu air  interface.

In our view, two deployment cases can be considered for topology 2, i.e. in-band and out-of-band.

- Case 1: in-band

If in-band is deployed for topology 2, operation of Uu and A-IoT at the UE reader are in the same band. According to the WID, simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band is not supported. In this deployment case, non-time overlapping resources need to be allocated for the Uu and A-IoT operation.

- Case 2: out-of-band

In case 2, out-of-band deployment is used for topology 2, i.e. operation of Uu and A-IoT at the UE reader are in different bands. In this deployment, simultaneous (time overlapping) operation of Uu and A-IoT at the UE reader may be supported depending on the radio capability of the UE reader. If simultaneous operation is supported at the UE reader, both overlapping and non-overlapping time resources can be used by the UE reader for Uu and A-IoT operation. Otherwise, only non-overlapping time resources can be used by the UE reader for Uu and A-IoT operation.

Observation 2: If in-band deployment is used for topology 2 (i.e. operation of Uu and A-IoT at the UE reader are in the same band), non-time overlapping resources need to be allocated for the Uu and A-IoT operation.

Observation 3: For out-of-band deployment, if simultaneous operation is supported at the UE reader, both overlapping and non-overlapping time resources can be used by the UE reader for Uu and A-IoT operation. Otherwise, only non-overlapping time resources can be used by the UE reader for Uu and A-IoT operation.

Proposal 1: RAN2 to discuss whether to support both in-band and out-of band deployment for AIoT topology 2.

Proposal 2: If out-of band of AIoT and Uu operation is supported, UE reader’s radio capability on simultaneous AIOT and Uu operation needs to be informed to gNB.

  - Resource allocation to the UE Reader

Interference between R2D and CW transmission

As defined in R19 AIoT, D2R transmission is based on backscattering on a carrier wave for AIOT device 1. So CW signal needs to be transmitted during the time duration for D2R transmission. And according to the WID, FR1 licensed spectrum in FDD is used for topology 2, with R2D in UL spectrum, and D2R and CW in UL spectrum. So both CW and R2D transmission uses FDD UL spectrum. Therefore, if CW signal is transmitted during the time when R2D is transmitted, the CW signal will interfere the R2D reception at the device, and device may not be able to receive R2D message successfully. To ensure that R2D transmission is received successfully, CW signal shall not be transmitted during the time when R2D message is transmitted, as illustrated in figure 1 below.  As agreed in last meeting, the reader schedules the AIoT interface transmissions within the resources allocated by the gNB, which is unlike in R19 where gNB reader schedules AIoT transmission. So it is suggested that how to avoid the potention potential interference between CW and R2D transmission needs to be taken into account when designing AIoT resource allocation scheme in RAN2.

Figure 1. R2D/D2R transmission and CW transmission in topology 2

Observation 4: CW signal needs to be transmitted during the time duration for D2R transmission, while it shall not be transmitted during the time when R2D message is transmitted, otherwise CW signal will interfere R2D reception at the device.

Proposal 3: RAN2 to investigate AIoT resource allocation scheme considering how to avoid the potention potential interference between CW and R2D transmission.

Resource allocation and assistance information

During RAN2#131bis meeting, AIoT resource allocation for the UE reader was discussed and the following agreements were achieved.

RAN2#131bis agreements

2.	gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling.   The resource allocation can be triggered by CN request.   UE reader may provide assistance information related to AIoT transmissions.  FFS what the information may be useful to be provided to gNB.  It is up to gNB how the information is used.  It is understood that assistance information is not mandated for the gNB to determine resource allocation.

3. 	the reader schedules the AIoT interface transmissions within the resources allocated by the gNB.

Two resource allocation mechanisms were discussion, including CN-request based and UE-request based. And it was agreed that the resource allocation can be triggered by CN request. Based on the discussion, companies have different views on whether to support UE-request based AIoT resource allocation. And there was some discussion on the how does the AIoT resource provided to UE-reader looks like, e.g., periodic, one-shot, continuous. In our view, the AIoT resource allocated by gNB to UE-reader could be periodic which is similar as configured grant resources, or one-shot resource, depends on the AIoT service/procedure:

- Case 1) CBRA procedure and follow-on command procedure

CBRA procedure is used for group inventory procedure, i.e. a group of devices are paged and triggered to perform the random access procedure with the reader. Take MSG1 transmission for example, a subset of devices are triggered to send MSG1 upon reception of paging or one access trigger message. And access trigger message can be sent periodically to trigger the corresponding subset of devices to perform random access procedure.  So periodic resources can be allocated for R2D/D2R transmission (, e.g. paging/access trigger message, MSG1, MSG2, MSG3) to UE reader to avoid frequent resource allocation signaling.

After CBRA procedure, command procedure can be performed between UE reader and individual devices. Periodic resources can be allocated for this case, similar as in CBRA procedure.

Proposal 4: Periodic AIoT resources are allocated to UE reader for CBRA procedure and follow-on command procedure.

- Case 2) CFA procedure and follow-on command procedure

CFA procedure is defined for inventory for a single device, where basically one CFA paging message and one D2R upper layer transfer message needs to be transmitted. And if inventory response message is not received successfully, the UE reader may retransmit the CFA paging message. For the follow-on command procedure, multiple R2D/D2R transmisstions may be needed due to segmentation and re-transmission. The following two resource allocation schems can be considered for CFA procedure and follow-on command procedure:

- Option 1) periodic resources

In option 1, periodic AIoT resources are allocated to UE reader, after the periodic resource can be release after the completion of the corresponding procedure, e.g. after receiving inventory response message for CFA paging, or receiving command response message for command procedure.

- Option 2) one-shot resource

For CFA procedure, one-shot resource is allocated by gNB to the UE reader, which may include R2D/D2R transmission for one or multiple times. In this one-shot resource allocation scheme, if more resources are needed (e.g. due to re-transmission or segmentation), the UE reader needs to request for more resources from the gNB.

Proposal 5: for CFA procedure and follow-on command procedure, RAN2 discusses whether periodic resource or one-shot resource is allocated. If one-shot resource is allocated, UE reader may need to request for more resources from the gNB, e.g. due to re-transmission or segmentation.

During RAN2#131bis meeting, it was agreed that UE reader may provide assistance information related to AIoT transmissions, and it is FFS what the information may be useful to be provided to gNB. As agreed in last meeting, the reader schedules the AIoT interface transmissions within the resources allocated by the gNB. Specifically, UE reader determines R2D resources and D2R scheduling information (e.g. number of time/frequency domain resource, Bit Duration, Block Repetition number, etc) according to the AIoT radio link condition.

For AIoT resource allocated to UE reader, the following two alternatives can be considered:

- Alt 1: gNB doesn’t provide R2D and D2R resources seperately

In alt 1, a set of resources are allocated from gNB to UE reader, and then UE reader determines which resource is used for R2D and schedules D2R transmission on its own. In this case, gNB is not aware of the actual usage of the allocated resources, i.e. cannot differentiate the resources used for R2D and D2R. In this case,,how to avoid the inteference between R2D and CW transmission needs to be discussed as analyzed above. According to the R20 AIoT WID,  there is no specification support for CW control. Then it can be assumed that inteference between R2D and CW transmission can be addressed by implementation in this case.

In alt1, UE reader may provide time duration of continuous AIoT resources and periodicity of the resources (for periodic resource) to gNB to assist the AIoT resource allocation.

Proposal 6: If gNB doesn't provide R2D and D2R resource separately, time duration of continuous AIoT resources and the periodicity (for periodic resource) may be helpful for AIoT resource allocation at gNB. Inteference between R2D and CW transmission can be addressed by implementation in this case.

- Alt 2: gNB provides R2D and D2R resources separately

In alt 2, gNB allocates R2D and D2R resources to UE reader. UE reader selects R2D resource to send R2D message within the R2D resources, and schedules D2R transmission within the allocated D2R resources. In this way, the interference between R2D and CW transmission can be avoided by implementation, which is similar as in R19. However, the problem is that how could gNB obtain the accurate timing resources for D2R transmission, considering that the D2R scheduling is performed by UE reader instead of gNB. In this case, assistance information related to D2R/R2D scheduling determined by the UE reader based on radio condition can be provided to gNB, e.g., time duration of continuous D2R/R2D resources and periodicity of the resources.

Proposal 7: If gNB provides R2D and D2R resource separately, assistance information related to D2R/R2D scheduling based on radio condition can be provided to gNB, e.g., time duration of continuous D2R/R2D resources and periodicity of the resources.

  - Uu procedure for AIOT operation

An example of the overall procedure for AIoT resource allocation is described below as illustrated in figure 2.

- Step 0: The cell supporting AIoT topology 2 broadcasts topology 2 supported indicator, so that UE which intends to acts as an UE reader can access the cell supporting TP2 if needed.

- Step 1: A UE sends an indicator indicating that it intends to act as an UE reader for AIoT service via RRC message.

- Step 2-3: After receiving AIoT service request from CN, gNB sends RRC AIoT service request message to the selected UE reader.

- Step 4: If the UE reader is able to perform the corresponding AIoT service in the AIoT air interface, it responds with an RRC AIoT service response message. Meanwhile, some assistance information for the AIoT resource allocation can be sent from UE reader to gNB via the RRC AIoT service response message.

- Step 5: gNB sends AIoT service response message to CN.

- Step 6: gNB configures AIoT resources to UE reader based on the assistance information.

- Step 7: UE reader initiates AIoT inventory/command procedure in AIoT air interface, based on the allocated  AIoT resources.

- Step 8-9: During the AIoT inventory/command procedure, if the UE reader determines that the allocated resource is not sufficient (e.g. when one-shot resource is allocated and more resources are needed due to segmentation or retransmission), it may request for more AIoT resource from the gNB. On the other hand, if the UE completes the inventory and command procedure, UE may request for releasing AIoT resource from the gNB.

- Step 10-11: After receiving AIoT upper layer data from device, UE reader forwards the upper layer data to gNB via RRC message, which is further forwarded to CN.

Figure 2. overall Uu procedure for AIoT service

Proposal 8: A cell supporting AIoT topology 2 broadcasts topology 2 supported indicator, so that UE which intends to act as an UE reader can access the cell.

Proposal 9: A UE sends an indicator indicating that it intends to act as an UE reader for AIoT service via RRC message.

Proposal 10: A UE may request for releasing AIoT resource from the gNB.

Regarding the RRC AIoT service request message, it was agreed in RAN2#131bis that gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN. And it is FFS what information needs to be provided to the UE reader. Here we analyze the information contained in the NGAP inventory request and command request messages and discuss what information needs to be provided to the UE reader.

According to TS 38.413, the following information is included in the NGAP inventory request message:

- AIOTF Identifier, A-IoT Correlation Identifier

- Inventory Request Transfer, which further includes the following information:

- A-IoT Correlation Identifier

- A-IoT Device Identification Requested

- Requested Service Area Information

- Inventory Assistance Information, which further includes Expected D2R Message Size, Approximate Number of Target A-IoT Devices (optional), and Time Interval (optional)

- Follow on Command Indication (optional)

In our view, the Device Identification Requested, Inventory Assistance Information (i.e. Expected D2R Message Size, Approximate Number of Target A-IoT Devices, and Time Interval) needs to be transmitted from gNB to the selected UE reader. However, the Requested Service Area Information is used for reader selection, which could be done in CN/gNB, depends on progress in SA2 and RAN3, so the Requested Service Area Information is needed to be provided to UE reader.

And according to TS  38.300, the purpose of Follow on Command Indication is to indicate that at least for one of the targeted A-IoT device(s) a Command procedure will follow. And if a follow-on Command indicator was received by gNB, the Inventory Report message also includes a RAN A-IoT device NGAP ID for each A-IoT device which was inventoried. In other words, the Follow on Command Indication IE is used for determination of whether RAN A-IoT device NGAP ID needs to be allocated and reported to CN. In our view, the device NGAP ID shall be allocated by gNB instead of UE reader, which is similar as in R19. Otherwise, there will be collision issue between device NGAP IDs allocated by different UE readers served by the same gNB. So the Follow on Command Indication doesn’t need to be provided to the UE reader from the gNB.

Proposal 11: For inventory request, Device Identification Requested and Inventory Assistance Information (i.e. Expected D2R Message Size, Approximate Number of Target A-IoT Devices, and Time Interval) need to be sent from gNB to UE reader. While the Requested Service Area Information and Follow on Command Indication don’t need to be sent from gNB to UE reader.

On the other hand, the following information is included in the NGAP command request message based on TS 38.413:

- AIOTF Identifier, A-IoT Correlation Identifier

- RAN A-IoT Device NGAP ID

- Command Request Transfer, which includes the following information:

- A-IoT Correlation Identifier

- RAN A-IoT Device NGAP ID

- A-IoT NAS PDU

- Command Assistance Information, which further includes Estimate of Expected D2R Message Size.

In our view, the A-IoT NAS PDU and Command Assistance Information (i.e. Estimate of Expected D2R Message Size) need to be sent from gNB to UE reader. Beside, identity of device (e.g. AS ID) needs to be sent to UE reader along with the A-IoT NAS PDU.

Proposal 12: For command request, A-IoT NAS PDU and Command Assistance Information (i.e. Estimate of Expected D2R Message Size), and identity of device (e.g. AS ID) needs to be sent from gNB to UE reader.

  - Uu SRB for AIoT operation

Based on the WID, RRC-based architecture needs to be specified for the AIoT data transmission between UE reader and BS. That means A-IoT upper layer data is forwarded via UE reader’s NR Uu RRC message. The protocol stack for RRC-based architecture is coped in the below.

TR 38.769 Upon receiving XXAP: A-IoT related message from A-IoT CN, the A-IoT-enabled gNB transmits the related information towards the A-IoT-enabled UE via NR Uu RRC, and vice versa. Figure 6.4.2.1.1-1: RRC based solution of Topology 2

In our view, there are two types of Uu RRC messages for AIoT operation between UE reader and gNB.

- Type 1: RRC message with AIoT upper layer data

After receiving D2R upper layer data transfer message with SDU data from AIoT device, or R2D upper layer data transfer message with SDU data from AIOTF, UE reader sends the corresponding AIoT upper layer data to gNB or AIoT device via RRC message, e.g., via RRC inventory report, RRC command request, RRC command response messages.

- Type 2: RRC message without AIoT upper layer data

The following RRC messages without AIoT upper layer data are required to be transmitted between UE reader and gNB for AIoT service:

- RRC inventory request, RRC inventory response, RRC inventory failure;

- RRC command failure;

- RRC AIoT session release request, RRC AIoT session release command, RRC AIoT session release complete

In our view, it needs to be discussed whether new SRB needs to defined for AIoT operation, or existing SRB can be shared between AIoT operation and normal UE operation. Furthermore, it needs to be discussed whether different SRBs are used for the RRC messages with or without AIoT upper layer data, considering that there may be different QoS requirments on the two types of RRC message.

Proposal 13: RAN2 to discuss whether existing SRB can be shared between AIoT operation and normal UE operation, and whether different SRBs are used for the RRC message with or without AIoT upper layer data.

  - Temporary out-of-connection scenarios

According to the WID, A-IoT radio resource of the UE reader is only valid within the serving cell, and the scenarios of temporary out-of-connection (including RLF and handover) needs to be considered. The temporary out of connection scenarios have been discussed during R19 AIoT study phase, and two options were captured in TR 38.769 on whether UE reader considers the resource as temporarily invalid or valid for a time period.

TR 38.769 - Scenario 2: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface, upon its temporary out of connection scenarios (e.g., RLF and handover cases): - Option 1: The UE reader considers the resources as temporarily invalid during the temporary out of connection condition. The resource may become valid again after the UE recovers from the condition. - Option 2: The UE reader considers that the resources remain valid for a time period. It needs to be further discussed on whether this time period belongs to the one in Option 2 for Scenario 1 or is a different time period. It needs to be further discussed on whether the duration of this time period is related to the ongoing A-IoT operations.

Take RLF case for an example, the estimated time for RLF detection and RLF recovery procedure for a UE is illustrated in figure 4. As we can see, the maximum time period for RLF detection and RLF recovery procedure is about 34s.

Figure 4. UE RLF recovery procedure

If UE reader considers the AIoT resources as invalid during the temporary out of connection condition, there would be waste of AIoT resource and unnecessary power consumption, since AIoT device is not aware of the status of UE reader, and the device will keep on monitoring R2D message or sending D2R message to the reader, and it will bring additional latency to the AIoT service. On the contrary, if UE reader considers the AIoT resources remain valid for a time period during the temporary out of connection condition, the resource waste, unnecessary power consumption, and additional latency due to UE reader mobility can be avoided. Moreover, it is very likely that a UE re-establishes in the same cell as the previous one, there is no interference issue at all in this case. So it’s preferred that UE reader considers the AIoT resources remain valid for a time period during temporary out of connection condition.

Proposal 14: When UE reader is temporary out of connection, if it has the capability to perform AIOT/Uu operation simultaneously, it is preferable to use the previously obtained AIOT resource to continue the AIOT service for a time period.

Then it needs to be discussed for how long the UE reader can consider the AIoT resources remain valid when temporary out of connection. In our view, the following options can be considered:

- Option 1: After UE reader complete the current AIoT service/procedure identified by the transaction ID

- Option 2: Upon reception of new AIOT resource configuration/release indication from the new/target cell.

- Option 3: Upon expiration of the validity timer configured by the serving cell

- Option 4: Upon transmission of RRCReestablishment request message to the new cell

- Option 5: Upon transmission of RRCreconfigurationcomplete message to the target cell

Proposal 15: If UE reader considers the AIoT resources remain valid for a time period when it’s temporary out of connection, the following options can be considered on when UE reader considers the previous AIoT resources as invalid:

- Option 1: After UE reader complete the current AIoT service/procedure identified by the transaction ID

- Option 2: Upon reception of new AIOT resource configuration/release indication from the new/target cell.

- Option 3: Upon expiration of the validity timer configured by the serving cell

- Option 4: Upon transmission of RRCReestablishment request message to the new cell

- Option 5: Upon transmission of RRCreconfigurationcomplete message to the target cell

Conclusion

In this paper, we discussed the support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader. And we have the following observations and proposals:

Deployment scenario

Observation 1: In AIoT topology 2, both D2R and R2D uses FR1 UL spectrum. UE reader needs to transmit and receive in UL FR1 spectrum in AIoT radio interface for communication in R2D and D2R direction with AIoT device, and also maintain communication with base station in Uu air  interface.

Observation 2: If in-band deployment is used for topology 2 (i.e. operation of Uu and A-IoT at the UE reader are in the same band), non-time overlapping resources need to be allocated for the Uu and A-IoT operation.

Observation 3: For out-of-band deployment, if simultaneous operation is supported at the UE reader, both overlapping and non-overlapping time resources can be used by the UE reader for Uu and A-IoT operation. Otherwise, only non-overlapping time resources can be used by the UE reader for Uu and A-IoT operation.

Proposal 1: RAN2 to discuss whether to support both in-band and out-of band deployment for AIoT topology 2.

Proposal 2: If out-of band of AIoT and Uu operation is supported, UE reader’s radio capability on simultaneous AIOT and Uu operation needs to be informed to gNB.

Resource allocation to the UE Reader

Observation 4: CW signal needs to be transmitted during the time duration for D2R transmission, while it shall not be transmitted during the time when R2D message is transmitted, otherwise CW signal will interfere R2D reception at the device.

Proposal 3: RAN2 to investigate AIoT resource allocation scheme considering how to avoid the potention potential interference between CW and R2D transmission.

Proposal 4: Periodic AIoT resources are allocated to UE reader for CBRA procedure and follow-on command procedure.

Proposal 5: for CFA procedure and follow-on command procedure, RAN2 discusses whether periodic resource or one-shot resource is allocated. If one-shot resource is allocated, UE reader may need to request for more resources from the gNB, e.g. due to re-transmission or segmentation.

Proposal 6: If gNB doesn't provide R2D and D2R resource separately, time duration of continuous AIoT resources and the periodicity (for periodic resource) may be helpful for AIoT resource allocation at gNB. Inteference between R2D and CW transmission can be addressed by implementation in this case.

Proposal 7: If gNB provides R2D and D2R resource separately, assistance information related to D2R/R2D scheduling based on radio condition can be provided to gNB, e.g., time duration of continuous D2R/R2D resources and periodicity of the resources.

Uu procedure for AIOT operation

Proposal 8: A cell supporting AIoT topology 2 broadcasts topology 2 supported indicator, so that UE which intends to act as an UE reader can access the cell.

Proposal 9: A UE sends an indicator indicating that it intends to act as an UE reader for AIoT service via RRC message.

Proposal 10: A UE may request for releasing AIoT resource from the gNB.

Proposal 11: For inventory request, Device Identification Requested and Inventory Assistance Information (i.e. Expected D2R Message Size, Approximate Number of Target A-IoT Devices, and Time Interval) need to be sent from gNB to UE reader. While the Requested Service Area Information and Follow on Command Indication don’t need to be sent from gNB to UE reader.

Proposal 12: For command request, A-IoT NAS PDU and Command Assistance Information (i.e. Estimate of Expected D2R Message Size), and identity of device (e.g. AS ID) needs to be sent from gNB to UE reader.

Uu SRB for AIoT operation

Proposal 13: RAN2 to discuss whether existing SRB can be shared between AIoT operation and normal UE operation, and whether different SRBs are used for the RRC message with or without AIoT upper layer data.

Temporary out-of-connection scenarios

Proposal 14: When UE reader is temporary out of connection, if it has the capability to perform AIOT/Uu operation simultaneously, it is preferable to use the previously obtained AIOT resource to continue the AIOT service for a time period.

Proposal 15: If UE reader considers the AIoT resources remain valid for a time period when it’s temporary out of connection, the following options can be considered on when UE reader considers the previous AIoT resources as invalid:

- Option 1: After UE reader complete the current AIoT service/procedure identified by the transaction ID

- Option 2: Upon reception of new AIOT resource configuration/release indication from the new/target cell.

- Option 3: Upon expiration of the validity timer configured by the serving cell

- Option 4: Upon transmission of RRCReestablishment request message to the new cell

- Option 5: Upon transmission of RRCreconfigurationcomplete message to the target cell

Reference