3GPP TSG-RAN WG2 #132	R2-2508526

Dallas, US, November 17th – 21st, 2025

Agenda Item:	9.3.3 RRM measurement event prediction

Source:	Ericsson

Title:	Discussion on event predictions

Document for:	Discussion, Decision

# Introduction

Rel-20 Work Item RP-252899 [1] lists the objectives which will be treated during the Rel-20 activity. In this contribution the objective related event prediction configuration is discussed:

- For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]:

- UE capability request and response procedure

- Applicability report for both full and partial configuration approach

- Inference configuration and inference report

- Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality

- UE sided data collection

- For measurement event prediction:

  - Measurement event A1~A6 are in the scope in this release

Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation.

# Discussion

## Designing a single solution for event prediction

The direct and indirect approach for event prediction have been discussed during the Rel-19 Study Item, and the following is stated in TR 38.744 [2] Ch.6.1.3:

It is up to UE’s implementation to decide on choice between indirect and direct event prediction methodology.

The same chapter also includes the following note:

NOTE 2:	A single framework is aimed for direct and indirect event prediction.

Additionally, the WID [1] includes the following objective:

- For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]:

  - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction

  - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction

- For direct event prediction, study feasibility of core requirement in Q2 2026.

- Specify LCM-related core requirements to support the LCM functionality management , if any

Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions

Although the text in the TR is a positive step towards a methodology-independent RRC specification, we think that there might be differences between direct and indirect approaches when it comes to the content of the report generated by the UE, which results in different standardized solutions at the RRC level.

In the direct approach, the report generated by the UE contains the probability of an event occurrence within a time window, while in the indirect approach the content of the report can potentially be the predicted RRM measurements and the time related information about the predicted event.

Hence there might be fundamental differences between the UE’s report to the network based on its model choices, which also results in a separate set of configurations, and in general it implies a different implementation of the two solutions.

Direct event prediction might lead to a different report (e.g., probability of event in a future time window) than indirect event prediction (e.g., RSRP predictions in the future) with the consequence of different configurations and, in general, different implementation of the solutions.

We also note that RAN2 already agreed to have a functionality level management, and model level LCM is ruled out. Hence, RAN2 should not spend time on designing solutions working for a specific model type.

RAN2 already ruled out model level LCM and should not design solutions per model type.

Based on the WID objective, RAN4 will study the feasibility of core requirements for direct event prediction in Q2 2026. While waiting for the outcome of this study, RAN2 should proceed with the specification of the indirect approach solution.

RAN2 focuses on designing a signaling solution for the indirect event prediction while waiting for the outcome of the RAN4 feasibility study on the direct approach.

## Event prediction configuration

As specified in TS 38.331 Ch.5.5.4, an event (e.g. A3) is triggered by the UE once the evaluation of the relevant entry conditions of the serving and neighboring cell measurements are fulfilled for a period of Time-To-Trigger (TTT). Then the UE sends a report to the network including the available measurements.

In TR 38.744 [2], Ch. 6.1.2.1.2, the document states the following:

UE can be configured with periodic or event triggered reporting of predicted and/or actual measurement result(s). For each reported cell in the measurementReport message:

-	For intra-frequency temporal domain case A, one or more instances of predicted measurement result(s) in PW can be reported;

-	For intra-frequency temporal domain case B, the latest actual or predicted measurement result can be reported;

-	For inter-frequency prediction, the latest predicted measurement result is reported.

Chapter 6.1.3 further indicates this:

UE can be configured with event triggered reporting based on predicted and/or actual measurement result(s). As baseline event type A1~A6 can be predicted and reported. UE can report measurement results, predicted measurement event together with its timing related information.

Based on above, the TR distinguishes two event evaluation approaches:

- Event evaluation based on measurements: the event entry condition is evaluated using conventional (legacy) RRM measurements.

- Event evaluation based on predictions: the event entry condition is evaluated using predicted RRM measurements (indirect approach).

Currently, the legacy events (used for RRM measurements and mobility procedures) are configured in ReportConfigNR IE within in the MeasConfig. We propose to retain the existing event identifiers (A1 through A6) but introduce an explicit flag to distinguish if the event is evaluated using actual measurements or predictions.

Reuse the definitions of events A1 through A6 and include a flag within ReportConfigNR IE indicating whether the event should be evaluated using RRM measurements or RRM predictions.

Regardless of how an event is evaluated, the measurement report sent to the network may include predicted measurements, as indicated in the TR. This opens other possibilities about how measurements are sorted within the report.

At present, the report orders measurements according to a chosen sorting quantity, such as RSRP, RSRQ, or SINR. The selected metric determines which cells or beams are reported first and which candidates are considered best for handover, beam selection or cell reselection.

If predicted measurements are included, the network may use the prediction quantity (for example, predicted RSRP) as an additional sorting criterion.

This can be useful when the network needs to identify the best cell/beam candidates at a future time. In such cases, cells/beams could be sorted by predicted RSRP, ensuring that the report highlights candidates with high predicted RSRP values (even if their current RSRP would be lower at the moment the report is generated).

RAN2 to discuss to include the predicted quantity (e.g. predicted RSRP) as sorting quantity for the reported measurements.

## Configuration of event prediction based on temporal domain Case A and B

The configuration of the event prediction based on Case A may include the prediction window size (how far into the future the UE should predict) and how many samples in such prediction window that should be predicted. However, in case of event prediction based on temporal prediction Case B, the configuration can be different than Case A. In case B, the network may need to instruct the UE to use specific skipping pattern or the measurement reduction rate or maybe both can be left to UE implementation and the network just configures the UE whether it is allowed to use temporal prediction Case B for specific measurement object or not. Besides the prediction configuration, it is important to pay attention to where such configuration should be placed in the RRM measurement procedure.

In NR the following principle holds for the RRM measurements:

- Measurement object indicates to the UE the physical signal to sample/measure on (type and period).

- Quantity Config indicates what signal processing to be done on samples before the reporting/event evaluation .

- Report Config indicates how (periodic, event) and what quantities (RSRP, RSRQ, SINR) to report.

In our understanding, the temporal prediction Case A configuration is closer to the report config as it addresses the question of how to report and what to report. In other words, it does not impact the UE’s measurement and signal processing algorithm. However, the temporal prediction Case B configuration is closer to the measurement object configuration as it addresses the question how to sample/measure and it also replaces some of the measurement samples with predictions. In other words, temporal prediction Case B changes how the UE performs the measurements and applies the signal processing algorithm. Based on that we propose the following:

reportConfigNR is used as a baseline to include the additional configurations concerning temporal prediction Case A for event prediction (and RRM measurement prediction), including prediction window and the number of samples to be predicted.

measObjectNR is used as baseline to include the additional configuration concerning temporal prediction Case B for event prediction (and RRM measurement prediction).

# Conclusion

In the previous sections we made the following observations:

Observation 1	Direct event prediction might lead to a different report (e.g., probability of event in a future time window) than indirect event prediction (e.g., RSRP predictions in the future) with the consequence of different configurations and, in general, different implementation of the solutions.

Observation 2	RAN2 already ruled out model level LCM and should not design solutions per model type.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 focuses on designing a signaling solution for the indirect event prediction while waiting for the outcome of the RAN4 feasibility study on the direct approach.

Proposal 2	Reuse the definitions of events A1 through A6 and include a flag within ReportConfigNR IE indicating whether the event should be evaluated using RRM measurements or RRM predictions.

Proposal 3	RAN2 to discuss to include the predicted quantity (e.g. predicted RSRP) as sorting quantity for the reported measurements.

Proposal 4	reportConfigNR is used as a baseline to include the additional configurations concerning temporal prediction Case A for event prediction (and RRM measurement prediction), including prediction window and the number of samples to be predicted.

Proposal 5	measObjectNR is used as baseline to include the additional configuration concerning temporal prediction Case B for event prediction (and RRM measurement prediction).

# References

RP-252899 - Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, OPPO, Interdigital, RAN#109, September 2025

TR 38.744, v.19.0.0 - Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR