3GPP TSG-RAN WG2 Meeting #132	R2-2508544

Dallas, USA, Nov. 17th – 21st 2025

Agenda item:	10.3.1.3

Source:	Sony

Title: Scheduling enhancement to address latency – initial simulation results

Document for:	Discussion and Decision

# 1 Introduction

At RANP#108, the 6G study item was agreed [1], where Radio interface architecture related part of the study item is captured as follows:

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3].

At RAN2#131bis, RAN2 reached the following initial User Place (UP) agreements [4]:

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2. UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

In this paper, we provide our views of UL scheduling enhancement to address latency and initial performance simulation results.

# 2 Discussion

## 2.1 Scheduling for DL/UL Resources

Scheduling is the heart of gNB functionality to realise the link adaptation and resource assignment in multi-user scenario within a cell. Dynamic DL/UL scheduling via DCI from gNB provides full flexibility of allocating resources and can handle well with most of the traffic profile. But this results in significant control overhead and UE power consumption for receiving and decoding DCI(s). In addition, for UL scheduling, there is initial delay caused by scheduling request (SR) from a UE which is not adequate for some services like HRLLC, XR, future services, etc.

For the legacy dynamic UL scheduling, as shown in Figure 1, a UE first transmit scheduling request (SR) to the serving gNB. Then the gNB sends DCI scheduling PUSCH with limited resources as gNB is not aware how much data is available in the UE’s buffer for transmission. Next the UE transmits PUSCH including Buffer Status Report (BSR) to inform gNB about data sitting in its buffer. Further, gNB schedules additional DCI scheduling PUSCH with probably large resources allocated for the UE. Subsequently, the UE transmits PUSCH to the gNB, and probably engages with further transmissions as needed, depending on actual data size at the UE side. The total delay as shown on Figure 1 is significant for some services.

Note that the configuration of SR (PUCCH) for a given UE also generates more overhead in the system depending on the SR periodicity, taking into account number of UEs with such configurations in a cell.

Figure 1. Scheduling Request (SR) Procedure.

For DL/UL scheduling, in order to reduce the latency, control overhead and power consumption for periodic traffic with predictable packet sizes, it was specified semi-persistent scheduling (SPS) for the DL data and configured grant (CG) scheduling for UL data. However, in some services, the traffic may not be exactly periodic and there may be a jitter. In addition, for CG scheduling in the uplink, the packet sizes may not predictable (i.e., variable packet sizes) due to the nature of the traffic profile, and there is also radio channel variability. Instead of adapting to these variabilities, in 5GNR, the network pre-configures CG resources with a fixed packet size and conservative MCS level which limits actual traffic demands. As these fixed packet size and MCS level cannot handle multiple services with different QoS requirements (e.g., different reliability and latency), in 5GNR, the network configures multiple CG resource configurations which consumes a lot of resources. This leads to inefficiency use of the resources, for example, resources are allocated but may not be used for some time.

Furthermore, in Rel-18 XR WI [2], it was specified multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration and dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE to the network. The motivation was to handle more data in short period of time, and to free the resources immediately if not required anymore. However, the issue here is that it is still employing a fixed packet size and conservative MCS level. There is also an issue of multiple CG resource configurations to handle different services with different QoS requirements which consumes a lot of resources.

Therefore, in 6G, there should be a new design principles/requirement for UL scheduling, especially to support potential new services/applications, as follows:

- Immediate transmission of the UL data to reduce the latency, for example for heavy UL data with low latency.

- Choosing appropriate link adaptation parameters, for example the best frequency resources (PRBs), MCS and transmission power.

- Flexible resource allocation scheme, for example FDRA and or TDRA in a slot

- Efficient way of identifying UE and its resource allocation dynamically at the gNB receiver

- Improving spectral efficiency of the cell, so when a UE is not using the resources, another UE can be assigned the resources with minimum latency.

In our view, the above design requirement leads to somewhat combine CG and dynamic scheduling where a UE controls/decides its scheduling assignments autonomously within pre-configured large uplink resources (CG), termed as UE-based UL scheduling. The UE can allocate just enough PUSCH resource for itself each time within the pre-configured resource depending on the current packet size and channel condition, taking into account QoS requirement. However, further investigation is needed how to allocate orthogonal resources among users, or to allow users to contend resources in case of overlapping frequency resources. Note that the number of MU-MIMO antenna ports may be significantly increased in 6G (e.g., up to 48) and that may facilitate to allocate orthogonal resources among users.

Observation 1: For scheduling in the uplink, a new scheduling scheme is necessary that combines pre-configured uplink resources and dynamic scheduling where the UE controls/decides its scheduling assignments autonomously within the pre-configured large uplink resources.

Proposal 1. Study a new UL scheduling scheme to instantaneously adapt the scheduling parameters and resource size for multiple services with different QoS requirements.

Proposal 2. Support a new design principles/requirement for a UL scheduling scheme as follows:

- Immediate transmission of the UL data to reduce the latency, for example for heavy UL data with low latency.

- Choosing appropriate link adaptation parameters, for example the best frequency resources (PRBs), MCS and transmission power.

- Flexible resource allocation scheme, for example FDRA and or TDRA in a slot

- Efficient way of identifying UE and its resource allocation dynamically at the gNB receiver

- Improving spectral efficiency of the cell, so when a UE is not using the resources, another UE can be assigned the resources with minimum latency.

## 2.2 Initial Simulation Results

In this section, we present our initial performance results comparing different UL scheduling schemes as follows:

- Dynamic UL Grant Scheduling

- Configured Grant (Type 1)

- Configured Grant with UTO-UCI

- UE-based scheduling – contention free.

The traffic model and simulation assumptions are based on Rel-18 XR study [3]. We employed XR traffic based on UL video (10 Mbps and 60 fps) for all schemes (DG, CG, CG-UTO and UE-based scheduling) where the detailed simulation assumptions are captured on Appendix 1 and 2.

Based on the earlier XR study [3], the system capacity is defined as the maximum number of users per cell such that at least 90% of UEs being satisfied, and where the UE satisfaction is measured as the packet success rate (PSR) of 99%. If packet delivery delay exceeds a given PDB, then, the packet is counted as failure.

Details of the schemes evaluated in this contribution:

DG scheme: Baseline DG scheme is employed where a UE first triggers SR (PUCCH) when XR traffic packet arrives at the UE buffer, and then gNB provides initial grant/DCI for UL scheduling. The total SR+DG delay implemented in this performance evaluation is 5ms.

CG scheme (CG): Legacy CG scheme is utilized where network provides the CG configuration for the XR traffic. In this performance evaluation, the periodicity of normal CG scheme is 2.5ms for 10ms PDB and 15ms PDB, and 5ms for 30ms PDB. The retransmissions are scheduled dynamically via DG.

eCG2 scheme (CG-UTO): Similar to legacy CG scheme with intermediate occasions, but UE indicates unused PUSCH occasions via UCI to the network. So that network can assign these unused resources to other active users. The retransmissions are scheduled dynamically via DG.

eCG1 scheme (UE-based scheduling): This is enhanced scheduling scheme where network provides a large CG resource configuration for XR traffic. The periodicity of the scheduling scheme is 2.5ms for 10ms PDB and 15ms PDB, and 5ms for 30ms PDB. But in addition, the UE adjusts transmission parameters dynamically: MCS, number of PRBs, number of layers within the large CG resource. The UE indicates these scheduling parameters to the gNB via UCI transmitted separately in the same resource, and whether the subsequent resources will be used/unused until next XR data arrival (for example XR frames arrive at every 16.67ms). The retransmissions are scheduled dynamically via DG.

Figure 1: Capacity performance results for the normal DG, CG, eCG2 (CG-UTO) and eCG1 (UE-based scheduling), showing the percentage of the satisfied UEs against the number of UEs per cell.

Table 1. Summary of the capacity performance results for the normal DG, CG, eCG2 (CG-UTO) and eCG1 (UE-based scheduling).

The system capacity simulation results are shown on Figure 1 which depicts PDB = 10ms, PDB = 15ms and PDB = 30ms from left to right.

When PDB = 10ms, the observation is as follows:

- The observation is that the capacity for normal CG has decreased by 28.6%, eCG2 (CG-UTO) has increased by 128%, and eCG1 (UE-based scheduling) has significantly increased by 471%, all compared to dynamic scheduling (DG).

- The capacity performance of the baseline DG suffers (i.e., a smaller number of satisfied UEs) because it has limited time to deliver the packets, i.e., most of the packets exceed PDB and then dropped due to SR+DG delay and retransmission delays.

- The capacity performance of the normal CG also suffers due to limited transmission occasions and less time available for retransmissions which limits actual traffic demands, hence the packets exceed PDB and then dropped.

- The enhanced CG (eCG2/CG-UTO) has better capacity performance compared to the baseline DG and normal CG, due assigning unused resources to other users, i.e., more resources are available, contributing more UEs to be satisfied.

- The capacity performance of UE-based scheduling (eCG1) is much superior to all other schemes because this scheme can modify/adjust the scheduling parameters dynamically (i.e., MCS, number of PRBs, number of layers and releasing the unused resource in advance) and as a result most of the packets are transmitted within PDB, contributing more UEs to be satisfied.

When PDB = 15ms, the observation is as follows:

- The observation is that the capacity for normal CG has decreased by 16.7%, eCG2 (CG-UTO) has increased by 8.3%, and eCG1 (UE-based scheduling) has significantly increased by 47%, all compared to dynamic scheduling (DG).

- The performance of the baseline DG and normal CG has improved (i.e., more satisfied UEs) because the PDB is slightly longer and there is more time to deliver the packets.

- The enhanced CG (eCG2/CG-UTO) has better capacity performance compared to the baseline DG and normal CG, due assigning unused resources to other users, i.e., more resources are available, contributing more UEs to be satisfied.

- The capacity performance of UE-based scheduling (eCG1) is still much superior to all other schemes and , because this scheme can modify/adjust the scheduling parameters dynamically (i.e., MCS, number of PRBs, number of layers and releasing the unused resource in advance) and as a result most of the packets are transmitted within PDB, contributing more UEs to be satisfied.

When PDB = 30ms, the observation is as follows:

- The observation is that the capacity for normal CG has decreased by 11.8%, eCG2 (CG-UTO) has decreased by 5.6%, and eCG1 (UE-based scheduling) has increased by 0.8%, all compared to dynamic scheduling (DG).

- The performance difference between the baseline DG and UE-based scheduling (eCG1) narrows due to longer PDB where there is more time for scheduling users and as a result the number of satisfied UEs has improved significantly for all schemes. Normal CG and eCG2 have underperformed, it is believed, due to lack of adjusting link adaptation parameters.

Furthermore, Table 1 above captures the summary of the capacity performance results for all schemes. It can be observed that the UE-based scheduling (eCG1) provides higher capacity than all other schemes when there is a tight deadline (PDB) for delivering packets. Hence, the higher capacity performance presented here by this scheme motivates to study a new UL scheduling scheme for 6G.

Observation 2: Initial capacity performance results comparing different UL scheduling schemes, such as A) Dynamic Grant Scheduling B) Normal Configured Grant C) Configured Grant with UTO-UCI D) UE-based scheduling, have shown that the new scheduling scheme (UE-based scheduling (eCG1)) outperforms all other schemes, and  motivates to study new UL scheduling scheme to instantaneously adapt the scheduling parameters and resource size for multiple services with different QoS requirements.

We have also studied the pros and cons of the different UL scheduling schemes and summarized in Table 2.

Table 2. Summary table for UL scheduling schemes:

| Grant type | Pros | Cons |
| --- | --- | --- |
| Dynamic UL Scheduling | Full flexibility of allocating resources and can handle well with most of the traffic profile. | Control overhead, UE power consumption, initial delay caused by scheduling request (SR) for some services like HRLLC, XR, future services, etc. |
| Configured Grant | Reduces the latency, control overhead and power consumption for periodic traffic with predictable packet sizes. | Cannot handle variable packet sizes and radio channel variability. Multiple CG resource configurations to handle multiple services with different QoS requirements (e.g., different reliability and latency) leads to inefficiency use of the resources. |
| Configured Grant with UTO-UCI | Reduces the latency, control overhead and power consumption for periodic traffic with predictable packet sizes. Free resource immediately if not required anymore. | Cannot handle well variable packet sizes and radio channel variability. Multiple CG resource configurations to handle multiple services with different QoS requirements (e.g., different reliability and latency) leads to inefficiency use of the resources. |
| UE-based UL scheduling | Reduces the latency, control overhead and power consumption for all kind of traffic (periodic and aperiodic). Free resource immediately if not required anymore. Handles variable packet sizes and radio channel variability. Single resource configuration to handle multiple services with different QoS requirements (e.g., different reliability and latency). | Further investigation is needed how to allocate orthogonal resources among users, or Users may contend resources in case of overlapping frequency resources. |

Based on the above discussion and performance results, if a UE provides initial UL data that contains only BSR information in the first transmission, then further latency would be encountered where gNB must issue additional DCI scheduling PUSCH for more data transmission. It would be desirable that if the UE transmits immediately as much data as it can in the first/initial transmission based on its link adaptation parameters, not only BSR information.

Proposal 3. Study how UE can transmit immediately as much data as it can in the first/initial transmission based on its link adaptation parameters, not only BSR information.

# 3 Conclusion

In this contribution, we have provided our views of UL scheduling enhancement to address latency and initial performance simulation results, and we have the following observations and proposals.

Observation 1: For scheduling in the uplink, a new scheduling scheme is necessary that combines pre-configured uplink resources and dynamic scheduling where the UE controls/decides its scheduling assignments autonomously within the pre-configured large uplink resources.

Observation 2: Initial capacity performance results comparing different UL scheduling schemes, such as A) Dynamic Grant Scheduling B) Normal Configured Grant C) Configured Grant with UTO-UCI D) UE-based scheduling, have shown that the new scheduling scheme (UE-based scheduling (eCG1)) outperforms all other schemes, and  motivates to study new UL scheduling scheme to instantaneously adapt the scheduling parameters and resource size for multiple services with different QoS requirements. Summary table for UL scheduling schemes as follows:

| Grant type | Pros | Cons |
| --- | --- | --- |
| Dynamic UL Scheduling | Full flexibility of allocating resources and can handle well with most of the traffic profile. | Control overhead, UE power consumption, initial delay caused by scheduling request (SR) for some services like HRLLC, XR, future services, etc. |
| Configured Grant | Reduces the latency, control overhead and power consumption for periodic traffic with predictable packet sizes. | Cannot handle variable packet sizes and radio channel variability. Multiple CG resource configurations to handle multiple services with different QoS requirements (e.g., different reliability and latency) leads to inefficiency use of the resources. |
| Configured Grant with UTO-UCI | Reduces the latency, control overhead and power consumption for periodic traffic with predictable packet sizes. Free resource immediately if not required anymore. | Cannot handle well variable packet sizes and radio channel variability. Multiple CG resource configurations to handle multiple services with different QoS requirements (e.g., different reliability and latency) leads to inefficiency use of the resources. |
| UE-based UL scheduling | Reduces the latency, control overhead and power consumption for all kind of traffic (periodic and aperiodic). Free resource immediately if not required anymore. Handles variable packet sizes and radio channel variability. Single resource configuration to handle multiple services with different QoS requirements (e.g., different reliability and latency). | Further investigation is needed how to allocate orthogonal resources among users, or Users may contend resources in case of overlapping frequency resources. |

Proposal 1. Study a new UL scheduling scheme to instantaneously adapt the scheduling parameters and resource size for multiple services with different QoS requirements.

Proposal 2. Support a new design principles/requirement for a UL scheduling scheme as follows:

- Immediate transmission of the UL data to reduce the latency, for example for heavy UL data with low latency.

- Choosing appropriate link adaptation parameters, for example the best frequency resources (PRBs), MCS and transmission power.

- Flexible resource allocation scheme, for example FDRA and or TDRA in a slot

- Efficient way of identifying UE and its resource allocation dynamically at the gNB receiver

- Improving spectral efficiency of the cell, so when a UE is not using the resources, another UE can be assigned the resources with minimum latency.

Proposal 3. Study how UE can transmit immediately as much data as it can in the first/initial transmission based on its link adaptation parameters, not only BSR information.

# 4 References

- RP-251881, New WID: Study on 6G Radio, RAN#108.

- RP-223502, “New WID on XR Enhancements for NR”, RANP #98e, December 2022

- TR38.835, “Study on XR enhancements for NR”, Release 18.

- Minutes of RAN2#131bis meeting, Prague, Czech Republic.

# Appendix 1. Grant procedure

# Appendix 2.  Evaluation Assumptions

Traffic model:

- Characteristics of XR traffic - Model 1: one stream model

Table A.1: System simulation parameters