3GPP TSG-RAN WG2 Meeting #132	R2-2508553

Dallas, USA, Nov 17th – Nov 21st, 2025

Agenda item:	9.3.3

Source:	Qualcomm Incorporated

Title:	Measurement Event prediction

WID/SID:	NR_AIML_Mob – Release 20

Document for:	Discussion and Decision

# Introduction

- Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]:

- Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported.

- L3 Cell level predication is supported

Note 1: For UE sided model, spatial domain prediction is not supported.

Note 2: For UE sided model, L3 beam level prediction is not supported.

- For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]:

- UE capability request and response procedure

- Applicability report for both full and partial configuration approach

- Inference configuration and inference report

- Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality

- UE sided data collection

- For measurement event prediction:

  - Measurement event A1~A6 are in the scope in this release

Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation.

In the WID document for Rel-20 AI/ML for Mobility [1], for Measurement Event prediction for UE-side models, the following are the stated objectives:

In this contribution, we discuss the following aspects of Measurement Event prediction for UE-side models:

- UE capabilities,

- Inference configuration and reporting.

# UE capabilities, Inference configuration and reporting

The following proposal gives the basic UE capabilities for Event prediction. These capabilities are for Indirect as well as Direct Event prediction. It is up to the UE to select the kind of Event prediction, Indirect or Direct, to use.

Proposal 1: The following are the basic UE capabilities for Event prediction:

- The UE indicates whether it supports event prediction for each event type (pA1, …, pA6), where, e.g., pA1 denotes predicted event A1.

- For each event type, the UE indicates the maximum prediction window (PW) length for which the UE supports event prediction.

For an event, based on the UE capability regarding the maximum PW length it can support, the gNB configures the UE for Event prediction reporting, with a range of values for the predicted time of occurrence of event, i.e., UE transmits an Event prediction report if the predicted time of occurrence of event lies within the range. As an example, in case of handover related events, if the predicted time of occurrence of event is close to the current time, then it is not of much value to transmit an Event prediction report.

Proposal 2: For event prediction reporting, the gNB configures the UE with:

- a window for the predicted time of occurrence of event, i.e., the UE transmits an event prediction report if the predicted time of occurrence of the predicted event lies within the configured window,

- whether the UE can transmit an event prediction report when the entering condition of the event is met by actual measurements, and the UE predicts occurrence of the event at a future time

The following proposal gives the conditions for UE triggering transmission of an Event prediction report to the gNB.

Proposal 3: The UE transmits an event prediction report to the gNB if its AI/ML model predicts occurrence of the event at some future time within the network configured window, based on actual measurements of serving cell and/or neighbor cell and other information available at the UE, and:

- Optionally, if the actual measurements at the current time satisfy the entering conditions of the event.

- RAN4 may also define additional conditions, e.g., for prediction accuracy.

The contents of the event prediction report are the same for both direct and indirect event prediction.

Proposal 4: An event prediction report transmitted from the UE to the gNB includes:

- The event type (pA1, …, pA6),

- Any available (actual) measurements of the serving cell and/or the neighbor cell at the current time,

- The expected time of occurrence of the event,

- Optionally, the probability of occurrence of the event within a time window (TBD) around the expected time.

Proposal 5: If the UE also supports RRM measurement prediction, the network can configure the UE to report predicted measurements in the event prediction report.

# 3 Conclusions

Proposal 1: The following are the basic UE capabilities for Event prediction:

- The UE indicates whether it supports event prediction for each event type (pA1, …, pA6), where, e.g., pA1 denotes predicted event A1.

- For each event type, the UE indicates the maximum prediction window (PW) length for which the UE supports event prediction.

Proposal 2: For event prediction reporting, the gNB configures the UE with:

- a window for the predicted time of occurrence of event, i.e., the UE transmits an event prediction report if the predicted time of occurrence of the predicted event lies within the configured window,

- whether the UE can transmit an event prediction report when the entering condition of the event is met by actual measurements, and the UE predicts occurrence of the event at a future time

Proposal 3: The UE transmits an event prediction report to the gNB if its AI/ML model predicts occurrence of the event at some future time within the network configured window, based on actual measurements of serving cell and/or neighbor cell and other information available at the UE, and:

- Optionally, if the actual measurements at the current time satisfy the entering conditions of the event.

- RAN4 may also define additional conditions, e.g., for prediction accuracy.

Proposal 4: An event prediction report transmitted from the UE to the gNB includes:

- The event type (pA1, …, pA6),

- Any available (actual) measurements of the serving cell and/or the neighbor cell at the current time,

- The expected time of occurrence of the event,

- Optionally, the probability of occurrence of the event within a time window (TBD) around the expected time.

Proposal 5: If the UE also supports RRM measurement prediction, the network can configure the UE to report predicted measurements in the event prediction report.

# References

[1] RP-252899, WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR (Oppo, Interdigital).