3GPP TSG-RAN WG2 Meeting #132	R2-2508562

Dallas, USA, 17th – 21st November, 2025

Agenda Item:	10.3.2.1

Source:	Huawei, HiSilicon

Title:	RRC state: requirements and functions discussion

Document for: Discussion and Decision

1	Introduction

Some RAN2 agreements were achieved about the RRC state modelling:

Agreements Continue study - Idle mode – - Connected mode – - Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW. Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements. We can consider the pros/cons of different modelling, after having some discussions on the functionality.

In this contribution:

- We highlight the 5G lessons about NR inactive state in section 2.1;

- The UE based mobility is also discussed in section 2.2 for the agreed energy efficiency requirement, where:

  - UE cell switch behaviors are clarified in section 2.2.1, and

  - The corresponding “area” concept is clarified in section 2.2.2;

- The UE identification further improvement is discussed in section 2.3 for the agreed fast transition requirement, where:

  - The lower-layer procedure (see 2.3.1) using lower-layer UE ID (see 2.3.2) and lower-layer WUS (see 2.3.3) are illustrated.

- After the discussion on the key UE functions above, we also provide some comparisons between different RRC modelling, in section 2.4.

- As to the other RAN2 study aspects, we also provide some analyses in section 2.5, where:

  - The small data transfer use case is clarified in different modelling (see section 2.5.1);

  - Some alternatives are provided for the issue about paging simplification in different modelling (see section 2.5.2);

  - Some other aspects to reduce the latency in the preparation phase for data transmission are listed for the agreed fast transition requirement (see section 2.5.3).

2	Discussion

2.1	5G lessons for 6G justifications

NR inactive state is nearly not commercialized. This is not just due to late standardization of SDT and mandatory with capability signalling of inactive state. There are more reasons.

In the last RAN2 meeting, several issues or problems are mentioned about the inactive state. For example, it has some impact to the core network to handle both RAN paging and CN paging; it requires the UE context management among RNA, which has not been planned by the network.

Observation 1: 	The complexity of RAN paging, impacts on RNA, and CN-related impacts are the primary reasons why the NR inactive state has not been successfully commercialized.

Moreover, the operators may be lack of the motivations to use this NR inactive state. It has the similar power saving performance as the IDLE state, while there is no significant difference about the state transition latency compared to IDLE state.

Observation 2:	The state transition latency of the NR inactive state is not compelling, compared to NR idle state.

Therefore, RAN2 needs to study a "mode/sub-state" that offers lower latency and simpler paging/wake-up mechanism compared to the inactive state, while maintaining comparable power-saving gains.

2.2	UE based mobility for energy efficiency

2.2.1	Energy efficient cell switch behaviors

In RAN2#131bis meeting, RAN2 agreed to study “UE-based mobility” for the inactive mode or sub-state in which the UE has no data to transmit or receive. When the UE moves from one cell to another cell during this “no-data” period, an energy efficient cell switch procedure should be studied, since energy saving is one main target to design this inactive mode or sub-state.

In the “no-data” period, if the UE continues to perform RRM measurement and sends the report to the network for the network side handover decision, the UE power consumption would be high, which is unfavorable. Therefore, UE-based mobility procedure can be studied without the handover command and notification with the network.

In the UE-based mobility during the “no-data” period, the UE performs DL measurement to select a target cell, and does not need to send the measurement result to the network. Once the UE switches to a new cell, the key issue is that whether the UE notifies the network about the selection result.

Option 1: when the UE selects a new target cell, the UE notifies to the network.

- The advantage of this option is that the network can transfer the UE context to the target base station timely, and once the UE has data to send or receive, the network is able to schedule the UE faster with the fetched UE context. Another benefit is that the network can easily reach the UE when DL traffic arrives, without the need of paging the UE in multiple cells. To achieve these advantages, the UE power consumption for transmitting the notification for cell switch should be further studied.

Option 2: when the UE selects a new target cell, the UE does not notify the network.

- In this option, when the UE needs to transit to the “data-transmission” mode, for UL services, the UE sends a notification to the target cell, and the target cell fetches the UE context then begins the data transfer. For DL services, since the network does not know the exact UE location as to the cell-level, paging/wake-up procedure in several cells may be needed. Then the cell/base station which successfully finds the UE will fetch the UE context and begin the data scheduling. More UE power saving gain is obtained in this option, however, how to reduce the UE context fetching latency and reduce the paging/wake-up signalling overhead should be further studied

Figure 2.2.1 UE based mobility for energy efficiency

In order to target the similar UE power saving performance, the option 2 can be the starting point. To overcome the drawback of the option 2, some directions, e.g., designing a relatively small area in which the UE does not need to notify the network, can be further studied. In this small area, preparing the UE context in advance at the network side to reduce the latency may be feasible because of the easier interaction among network nodes within the small area. In addition, the paging/wake-up signalling overhead is controllable if the number of cells in this area is limited to a certain number.

Proposal 1:	RAN2 study the UE based mobility:

When UE is in “DRX-off”/“no-data” period, UE can perform cell switch based on its RRM measurement results, rather than being triggered by network HO command;

Upon this cell switch within a small area (e.g., several cells), UE may not notify the network.

If the UE selects a new target cell, upon UL traffic arrival, the UE should switch back to the “data transmission mode” and access the target cell either via common UL access resources or via dedicated UL access resources (e.g., dedicated preamble of the new target cell), if allocated. If the target cell decides to reconfigure the UE, the RRC reconfiguration procedure is applied. If the target cell provides some pre-configuration to the UE before, this pre-configuration can be applied as well. This aligns with the normal RRC procedures to manage the UE handover.

Observation 3:	Upon UL traffic arrival, UE can switch back to “data transmission mode” in connected state, via common or dedicated UL access resource in new/target cell. The follow-up procedure for data transmission can be aligned with the RRC procedure like RRCReconfiguration or handover.

The above part is discussing cell switch during “no-data” mode. If the UE is in “data transmission” mode, the UE anyway needs to maintain the Tx/Rx link with the network, so the UE still can send the measurement result and the network command triggered mobility is still used when cell switch happens.

Observation 4:	Network HO command triggered mobility is still used when there is ongoing data/traffic.

2.2.2	“Area” concept and management

In the above UE based mobility procedure, there could be one area of several cells, among which the UE can perform the UE based mobility rather than network-controlled mobility.

Note this "area" concept does not have to be specified as "Area". It is just one list of several neighbor cells, among which:

- the network has the UE context and configuration relation information, and

- the network will try find or wake-up the UE once DL traffic arrives, and

- Some kind of UE identification is maintained (see below section).

Those coordination among cells will add some complexity to the network side, even though large area will bring more UE power saving gains, e.g., for relative high speed UEs mobility. For some typical use case, e.g., Pedestrian: 0 km/h to 10 km/h as in TR 38.914, we may consider one typical BS deployment with 3 ~ 8 cells, where the ISD (Inter-Site Distance) may be 400m or 500m as some examples. This UE moving across 3 ~8 cells may take about 10mins. For such kind of example for “no-data sleep” period or longer than it, the network could also release this to IDLE state. In our understanding, enabling the UE sleep for about 10mins during mobility will be sufficient for UE power saving.

Therefore, in practical network, to balance the network complexity on UE context/configuration management, UE power saving and short transition latency, one reasonable small area with several cells can be the good use case for 6G study. Of course, this is not to exclude other possible network implementation using large area to manage the UE context.

Observation 5:	In practice, to simplify network-side area management, the design should target the "RAN-controlled area" as a small area with a reasonable size.

2.3	UE identification for fast transition

2.3.1	Lower-layer transition procedure for fast transition

In NR, when the UE transits from an energy saving state (e.g., RRC IDLE or RRC INACTIVE) to RRC CONNECTED state, the transition latency is still large, including the paging latency (for DL traffic), the random access latency and the RRC processing latency. Taking RRC INACTIVE as an example, the transition procedure and the latency analysis is shown in the following discussion.

Figure 2.3.1 Resume procedure from RRC_INACTIVE to RRC_CONNECTED

Paging latency:

For MT traffic, the UE monitors paging message, once its paging is received, and initiates CBRA to access the network. To be able to receive paging, the UE is required to periodically monitor PDCCH scrambled by P-RNTI in paging occasions, and then decoding Paging message in the corresponding PDSCH. NR introduced a paging early indication (PEI) for UE power saving, where the typical delay value is around 50ms, e.g., including 20ms for transition time from Deep Sleep, 20ms for reception of 2 SSB bursts and PEI, and 10ms for monitoring of paging occasions.

Random access latency:

For 4-step CBRA procedure, the typical random access delay value is around 20~30 ms when there is no collision, which includes SSB detection (~10 ms) if the UE has no available SSB measurement results, RACH occasion waiting, RAR reception (~10 ms), and Msg3 and Msg4 transmission and reception (~10 ms). If collision happens, the random access delay value is longer, e.g., more than 30 ms. With the 2-step CBRA, although the delay is shorter than 4-step CBRA, collision may still happen which increases the random access delay.

RRC procedure latency:

In NR, RRC state transition comes with RRC procedures. For example, for the RRC resume procedure, the RRCResumeRequest message is included either in Msg3 (4-step RA) or in MsgA (2-step RA) of the CBRA procedure. If the contention resolution and UE context retrieval procedures are successful, the UE processes the RRCResume message sent from the gNB. The UE applies the configuration and resumes the RRC connection. According to the RRC procedure delay requirement in TS 38.331, this RRC resume processing delay is around 6 ms or 10 ms. In inter-gNB resume scenario, 10 ms should be added for the UE context retrieval procedure on the Xn interface.

To some extends, the NR separate RRC states constrains the even further latency reduction due to the key bottleneck of the RRC/L3-level procedure. If the real short-latency switch is the target, the lower layer switch is necessary to remove the constrain/limitation caused by RRC/L3 kind of procedure.

Remark:	RRC layer paging and resume procedure cause some latency.

Observation 6:	For fast transition, the lower-layer transition procedure can be considered, to enable the fast switching to DL data transmission status.

2.3.2	Low-layer UE identification in one area

UE identification is essential for the transition from “no-data” mode to “data transmission” mode, since the network needs to wake up a particular UE for DL-service triggered transition, and the UE needs to show the UE identity to the network for UL-service triggered transition.

For DL services (MT traffic), one-step paging/wake-up is a promising direction to reduce the transition latency, where the UE monitors a UE dedicated DL WUS signal (e.g., a UE dedicated sequence). This UE dedicated DL WUS replaces the NR group based WUS + Paging DCI + Paging PDSCH procedure, which can achieve faster mode switch. At the same time, it provides better UE energy saving gain. In this case, the DL UE identification is achieved by the UE dedicated DL WUS. In the inter-cell scenario, as discussed in the above section, the cells in a small area can coordinate the WUS sequence and use it to wake up the UE in this area.

Agreement Study and evaluate DL WUS of OFDM based sequence and corresponding mechanisms for 6GR EE improvement, regarding at least the following aspects: - Coverage target for DL WUS (e.g., same as PDCCH, common sync signal, or other) - Measurements and/or synchronization. - System overhead and network energy consumption/UE energy saving for UE operation with the DL WUS. - RRC states - Other functionalities

Based on the RAN1 agreements listed above, RAN1 is studying the DL WUS aiming to carry more information, e.g., to provide finer granularity of UE wake-up. Especially considering that DL WUS configuration is under network's control in CONNECTED mode, it is feasible and beneficial to use UE dedicated DL WUS for UE identification/wake-up.

Remark:	For DL UE identification, UE dedicated DL WUS can be used to wake up UE in an area for fast transition and better UE energy saving gain.

For UL services (MO traffic), reducing random access latency is an important direction to study. If a UE can be configured with a UE-dedicated RACH resource, the network knows the UE identity directly by the Msg1 transmission without collision, which accelerates UE identification and starts data transfer faster. In the intra-cell transition case, the UE dedicated RACH resource can be allocated by the cell. In this case, the UE dedicated RACH resource can be one format of UE identification.

However, for the inter-cell case but still within the area, in addition to the UE dedicated RACH resource method, RAN2 could also study other lower layer identification method (e.g., region-specific unique UE ID) when the UE dedicated RACH resource is not allocated due to resource limitation. Note that the number of the region-specific unique UE ID should be the same as the aforementioned dedicated WUS, and the ID is associated with the anchor cell, so when a new cell receives this ID, it can further fetch the UE context from the anchor cell if the new cell has no prepared UE context.

Remark:	For UL UE identification:

Within the same serving cell, UE-dedicated RACH resources may be utilized to accelerate UE identification and achieve low uplink access latency.

Among different cells in an area, RACH and/or a unique UE ID within one area (with a 1:1 mapping to the aforementioned dedicated WUS and association with the anchor cell) can serve as the baseline for UE identification. Further enhancements may be studied.

Proposal 2:	For UE identification, lower-layer UE identification mechanisms should be studied to enable fast transition, based on lower-layer identifications unique within a single area.

2.3.3	Low-layer DL wake-up/paging

There is the fast transition benefit to use the lower-layer wake-up, for example the DLWUS, to trigger the UE for data transmission:

- On the one hand, as analyzed above, the DCI scheduling and PDSCH decoding for paging message can be saved;

- On the other hand, if the WUS monitoring occasion is decoupled with paging occasion, it can be designed as somehow more frequent WUS occasion, via network configuration.

Therefore, it is observed that the UE dedicated WUS can directly replace the RAN paging to wake-up one specific UE. This is one important aspect to study for fast transition.

Proposal 3:	For fast transition, RAN2 to study the DL wake-up via UE dedicated layer-1 signalling, e.g., DL WUS, to replace RAN paging, which can save the latency of paging message scheduling and reception.

2.4	Summary for the differences among RRC modelling

The above section 2.2 and 2.3 clarify how the UE based mobility and UE identification work in the connected sub-state. For sure, there is no need for RAN2 to make any down-selection on the different RRC modelling for now. The key problems/requirements and UE functions should be studied first. To help the discussion on the functionalities study, we give some comparison on the key differences among RRC modellings just for information.

Remark:	RAN2 will first study the requirements and functions first and postpone the down-selection for RRC modelling. Some comparisons on the key differences among RRC modellings are still given just for information.

Below table is the analyses based on current discussion.

Table 2.4-1 Comparison on key differences among different RRC modellings

|  |  | Delta part compared with NR connected state and connected sub-state modelling |  |
| --- | --- | --- | --- |
|  | Comparison with NR inactive state and connected sub-state modelling |  |  |
| Functions | NR Inactive state | Connected sub-state | NR connected state (supporting DRX) |
| DL wake-up/paging | RAN paging message | UE dedicated (LP-)WUS (in one area) | UE dedicated (LP-)WUS in the cell, or DRX cycle-based wake-up |
| UE identification | I-RNTI | Some ID unique in one area | C-RNTI |
| State/sub-state transmission | RRC layer involved | Low layer procedure | N/A |
| UL access | Random access | Dedicated access resource (e.g., CFRA, RACH-less, CG) |  |
| UE based mobility | UE cell switch without notification to network |  | CHO or C-LTM |
| RRM behavior | More relaxed RRM |  | Current RRM requirement for connected state |

2.5	Other RAN2 study aspects

2.5.1	Small data transfer use case

In Release 17, small data transfer was introduced in NR to allow infrequent (periodic and/or non-periodic) data transmission while remaining in RRC Inactive state, i.e. without transition to RRC connected state, thereby reducing signaling overhead and unnecessary power consumption. During the last RAN2 meeting, “data transfer” received a few attention for 6G RRC “inactive/sub-state” state which seems to be essential functionality to support in 6G. In this subsection, we intend to provide our analysis of how “data transfer” works in 6G and the relation with RRC state.

Figure 2.5.1-1 “Small data transfer” in NR

As illustrated in above Figure 2.5.1-1, the basic procedure of “small data transfer” in NR can be concluded into the following 4 steps (we take MO-based SDT as example):

- Step 0: Small data transfer configuration

The resource used for “small data transfer” which is either RA-based resource or CG-based resource can be delivered in in SIB and RRC release message before initiating data transfer procedure, and both UE and NW need to maintain the relevant configuration in RRC inactive state. For 6G, we think small data transfer configuration should be naturally supported and the detailed configuration can be further studied.

- Step 1: RRC Resume Request + UL data transfer

In NR, small data transfer use RRC Resume procedure to delivery UL data while the UE keeps in RRC Inactive state. The key technique is that the UE uses pre-configured SDT resource and relevant configuration in Step 1 (e.g. dedicated radio bearer) to transmit “UE ID” and “UL data” where the former one is to identify the UE carried in RRC Resume Request message. For 6G, regardless of RRC Inactive or Sub-state, such UE identification and “stored” context for data transfer should be supported, and the main difference would be the UE uses RRC procedure or lower-layer procedure to deliver the required “UE ID” from the UE.

- Step 2: UE Context Retrieve

In NR small data transfer, both SDT with and without UE context relocation are supported, which depends on which gNB is supposed to store the UE context. In our opinion, both options can be studied in 6G regardless of RRC Inactive or Sub-state. In case of RRC Sub-state, as we discussed in subsection 2.2,

1) UE-based mobility without notification to the network: the UE context may not be relocated along with the UE mobility, and thus it is necessary for the Receiving gNB to receive the (partial) UE context which is stored in the Last Serving gNB.

2) UE-based mobility with notification to the network: the UE context can be relocated along with the UE mobility, which is indeed similar to UE Connected mode, and thus UE context Retrieve can be performed before/during the data transfer procedure.

- Step 3: UL data transfer

After retrieving the UE context, the Receiving gNB can transmit UL data to UPF using the maintained UP tunnel between gNB and UPF, this procedure should be supported in 6G with no difference between RRC Inactive or Sub-state, which can be further studied in RAN3.

- Step 4: RRC Release

In case the Receiving gNB decides to terminate the data transfer procedure, it will generate RRC Release message with suspend config and deliver to the UE. For 6G sub-state, it can be supported with the spirit of UE context management in 6G RRC Connected state.

Note that for MT-based SDT, RAN paging with SDT indication is initiated before triggering data transfer procedure. Similar to our analysis in Step 1 above, the main point is how to identify the UE and the main difference in RRC Inactive and Sub-state modelling is it is RRC paging or “low-layer DL paging and wake-up” as described in Subsection 2.3.

Therefore, we don’t see any significant difference in the support of “small data transfer” between RRC Inactive and Sub-state modelling, and it can be supported by switching from “no-data” to “data transmission” sub-state by use of “low-layer UE identification” with the natural benefit of UE context maintenance in sub-state in 6G RRC Connected state.

Remark:	Inactive state modelling uses RRC resume SDT procedure to support small data transfer use case.

Proposal 4:	RAN2 to clarify that “small data transfer” use case does not have to be supported within a particular “inactive” state, while it can also be supported by switching from “no-data” sub-state to “data transmission” sub-state within connected state.

2.5.2	Paging/wake-up simplifications/enhancements and area management

In the last meeting, there were some contributions to propose unify the CN paging area (TA) and the RAN paging area (RNA) to simplify UE behavior. For this direction, there are two approaches: one is making RNA as large as TA, and another is making TA as small as RNA.

The first approach expands the RNA coverage, leading to significant large paging delay, as the base station needs to coordinate with other base stations in a larger area to retrieve the UE’s context. That may also cause signaling storm when massive UEs need to be paged, and the false alarm rate for paging will also increase. Even the network may adopt some optimization strategies to gradually expand the paging area, the latency will increase significantly. Moreover, this may result in more scenarios where there is no Xn interface between some base stations in this big RNA. Addressing these cases will require more sophisticated coordination and signaling mechanisms.

Observation 7:	Enlarging the RNA same as TA will increase DL latency and network signaling for reaching the UE, and suffer more from Xn unavailability.

Base on the assumption of RNA with Xn unavailability, the NG-based UE context retrieval procedure is a potential solution. But such solution will introduce more CN impact, which has been discussed for several times and was not supported in NR due to the complexity. From commercial perspective, that solution will make inactive state even more difficult to deploy. Thus, we believe it is not a good way to enlarger RNA same as TA.

Observation 8:	NG-based UE context management/retrieval requires CN impact, which makes inactive state not easy to deploy.

Another approach to unify TA and RNA is making TA as small as RNA. For this approach, TA needs to be re-planed by operator. From commercial perspective, it is not practical to reorganize the network topology completely. And also, if TA is defined too finely, the UE needs to frequently wake up and inform CN, which increases the UE power consumption (especially for UE in Idle state) and additional NAS signaling overhead.

Remark:	Making TA as small as RNA will requires operator to re-plan the TA, which is not practical.

In our opinion, to simplify paging/wake-up design and consider area management aspects, the RAN-controlled area for DL wake-up may be still used in 6GR. However, the RAN controlled area should be small as discussed in section 2.2.2. It will be easily for RAN to reach the UE within the area and also beneficial to reduce the paging latency.

For energy saving and paging efficiency, some optimization mechanisms instead of legacy DCI+ paging message, i.e., the connected sub-state modelling does not need any RAN paging reception, should be studied.

Proposal 5:	As to the paging/wake-up simplification and area management aspect:

In any RRC modelling, to address issues related to the RAN area, the "RAN-controlled area" can be used as a small area in practical deployments.

The connected sub-state modelling does not need any RAN paging reception, which can be one study direction for paging/wake-up simplification.

2.5.3	Preparation latency for fast transition

When UE switches from “no-data” sub-state to “data transmission” sub-state in connected state, there are some preparation steps which will have impact on latency of data transmission.

- UL access procedure: Before the uplink transmission, UE need to acquire the TA and uplink grant. In NR, RAN2 specified the CFRA, 2-step RA, configured grant, RACH-less to reduce the UL access latency for some features. In the last meeting, some companies also propose to study the UL access latency reduction for the fast transition during the state switch [1][2][3]. One way to reduce random access latency is to use CFRA during the sub-state switch procedure. In order to support CFRA for most UEs, 6GR should consider a larger preamble sequence pool, so that most UEs can be allocated a dedicated preamble. Therefore, UE dedicated RACH resource can enable fast access.

- Measurement procedure: Early CSI acquisition was introduced in Rel-19 for LTM to enable high-throughput data transmission and reception immediately after a cell switch. In the “no-data” sub-state, UE will only measure the necessary reference signal in order to reduce the UE power consumption. Therefore, RAN2 can also study the early measurement/CSI acquisition considering the UE power consumption.

- Context retrieve procedure: For the UL/DL data transmission, network need to send the configuration of each protocol layer to UE and know the keys for the ciphering and integrity protection. Therefore, the target node needs to retrieve the UE context from the source node. It will increase the latency of data transmission. In the last meeting, some companies also propose to study the context retrieve latency reduction for the fast transition during the state switch [1][2]. For example, the network can pre-configure the UE context/configuration in advance in the target node and the data transmission can directly start without the latency of the context retrieve procedure.

Proposal 6:	For fast transition, RAN2 to study the aspects to reduce the preparation latency for data transmission:

UL access latency reduction, e.g., CFRA, 2-step RA, configured grant, RACH-less;

Measurement related latency reduction, e.g., early measurement/CSI acquisition;

Configuration related latency reduction, e.g., pre-configured UE context/configuration.

# 3	Conclusion

This contribution makes the following proposals:

5G lessons for 6G justifications

Observation 1: 	The complexity of RAN paging, impacts on RNA, and CN-related impacts are the primary reasons why the NR inactive state has not been successfully commercialized.

Observation 2:	The state transition latency of the NR inactive state is not compelling, compared to NR idle state.

UE based mobility for energy efficiency

Proposal 1:	RAN2 study the UE based mobility:

When UE is in “DRX-off”/“no-data” period, UE can perform cell switch based on its RRM measurement results, rather than being triggered by network HO command;

Upon this cell switch within a small area (e.g., several cells), UE may not notify the network.

Observation 3:	Upon UL traffic arrival, UE can switch back to “data transmission mode” in connected state, via common or dedicated UL access resource in new/target cell. The follow-up procedure for data transmission can be aligned with the RRC procedure like RRCReconfiguration or handover.

Observation 4:	Network HO command triggered mobility is still used when there is ongoing data/traffic.

Observation 5:	In practice, to simplify network-side area management, the design should target the "RAN-controlled area" as a small area with a reasonable size.

UE identification for fast transition

Observation 6:	For fast transition, the lower-layer transition procedure can be considered, to enable the fast switching to DL data transmission status.

Proposal 2:	For UE identification, lower-layer UE identification mechanisms should be studied to enable fast transition, based on lower-layer identifications unique within a single area.

Proposal 3:	For fast transition, RAN2 to study the DL wake-up via UE dedicated layer-1 signalling, e.g., DL WUS, to replace RAN paging, which can save the latency of paging message scheduling and reception.

Other RAN2 study aspects

- Small data transfer use case

Proposal 4:	RAN2 to clarify that “small data transfer” use case does not have to be supported within a particular “inactive” state, while it can also be supported by switching from “no-data” sub-state to “data transmission” sub-state within connected state.

- Paging/wake-up simplification

Observation 7:	Enlarging the RNA same as TA will increase DL latency and network signaling for reaching the UE, and suffer more from Xn unavailability.

Observation 8:	NG-based UE context management/retrieval requires CN impact, which makes inactive state not easy to deploy.

Proposal 5:	As to the paging/wake-up simplification and area management aspect:

In any RRC modelling, to address issues related to the RAN area, the "RAN-controlled area" can be used as a small area in practical deployments.

The connected sub-state modelling does not need any RAN paging reception, which can be one study direction for paging/wake-up simplification.

- Preparation latency for fast transition

Proposal 6:	For fast transition, RAN2 to study the aspects to reduce the preparation latency for data transmission:

UL access latency reduction, e.g., CFRA, 2-step RA, configured grant, RACH-less;

Measurement related latency reduction, e.g., early measurement/CSI acquisition;

Configuration related latency reduction, e.g., pre-configured UE context/configuration.

# 4	References

- R2-2506857	Energy efficient and unified RRC state modelling, Huawei, HiSilicon, 3GPP RAN2 meeting #131bis

- R2-2507232 Considerations for 6G Control Plane, Samsung, Verizon, 3GPP RAN2 meeting #131bis

- R2-2507340 Consideration on general aspects for 6G, LG Electronics Inc., 3GPP RAN2 meeting #131bis