3GPP TSG-RAN WG2 Meeting #132	R2-2508571

Dallas, USA, 17 – 21 November 2025

Agenda item:	8.7.2

Source:	Nokia, Nokia Shanghai Bell, Apple, LG Electronics Inc., Samsung, Vivo

Title:	Discussion on RLC-N02

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we address the open issue RLC-N02 that remained after the previous meeting, as follows.

Agreements for RLC - … - (RLC-N01) Try to capture it in the procedure “If t-RxDiscard is configured and expires, the receiving side of an AM RLC entity consider the corresponding SDUs as received.”, if problem is identified or companies have strong concern during CR phase, it will be postponed. - (RLC-N01) Add “or t-RxDiscard expires” at the end of definition of RX_Next. - (RLC-N02) In NOTE in 5.3.3.3 we add “If stopReTxDiscardedSDU is configured” at the beginning of the sentence. - (RLC-N02) FFS whether any other changes are needed, e.g. as in R2-2507159 - … - (RLC-N02) Companies will bring contribution in next meeting to explain the use cases and the TP. - (RLC-H02) Confirm that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. Companies are invited to bring contributions on the solutions to address it. - …

# 2	Discussions

The following NOTE was introduced in clause 5.3.3.3 to capture an agreement from RAN2#130:

NOTE: When all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

In RAN2#131bis, based on RLC-N02, RAN2 agreed to clarify that the NOTE is applied only when stopReTxDiscardedSDU is configured. This agreement is handled by the rapporteur’s CR.

One thing left from RLC-N02 is whether we should allow stopping t-PollRetransmit when all RLC SDUs with SNs up to and including POLL_SN are already negatively acknowledged. We explained in R2-2507159 that the current NOTE could lead to an undesirable stopping of the t-PollRetransmit timer even though a valid and not yet discarded SDU may be under retransmission and the transmitter is awaiting the result of this transmission. However, it wasn’t well understood which specific case could be problematic. For example, it was claimed that the NOTE was intended to cover the case when all RLC SDUs with SNs up to and including POLL_SN are already indicated as discarded, in addition to what is already specified in the normative text, which was not our understanding. Due to lack of consensus based on different understanding of the current NOTE, it was decided to discuss further.

In our understanding, the normative text in clause 5.3.3.3 is for the case when all the SDUs are either positively or negatively acknowledged while the NOTE is for the case where there is at least one SDU which is discarded. Having this in mind, the case concerned can be - for example, consider that there are only 4 RLC SDUs in the initial transmission buffer of the transmitting RLC AM entity with SNs 1, 2, 3, & 4.

- At time T0, an initial transmission of all the SDUs is completed. The POLL_SN will now be set to 4, while the transmitting RLC AM entity starts the t-PollRetransmit timer.

- At time T1, a status report is received with ACK_SN = 3, & NACK_SNs = 3,4. With this reception, the SDU with the POLL_SN(4) is negatively acknowledged, and the t-PollRetransmit timer is stopped and reset as per the normative text in clause 5.3.3.3

Following this update, the transmitter RLC AM entity will retransmit the negatively acknowledged SDUs (SN=3 & 4) and restart the t-PollRetransmit timer. The retransmission buffers, and the initial transmission buffers are now empty.

- At this point in time, if the SDU with SN = 4 is indicated as discarded by the upper layer, the transmitting RLC AM entity can stop the t-PollRetransmit timer as per the NOTE (since all RLC SDUs with SNs up to and including POLL_SN have been either positively or negatively acknowledged, or discarded).

However, the desired behavior in this case should be that the t-PollRetransmit timer remains running, since there is a retransmission in progress for SDU with SN=3 and no technical harm is identified in keeping the t-PollRetransmit timer running.

Hence, we propose to modify the NOTE as below:

NOTE: If stopRetxDiscardedSDU is configured, when all RLC SDUs with SNs up to and including POLL_SN have been positively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

Proposal 1: [RLC-N02] The NOTE in clause 5.3.3.3 is updated as below:

NOTE: If stopRetxDiscardedSDU is configured, when all RLC SDUs with SNs up to and including POLL_SN have been positively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

# 3	Conclusion

This document has made the following proposals:

Proposal 1 [RLC-N02]: The NOTE in clause 5.3.3.3 is updated as below:

NOTE: If stopRetxDiscardedSDU is configured, when all RLC SDUs with SNs up to and including POLL_SN have been positively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.