3GPP TSG-RAN WG2 Meeting #132	R2-2508574

# Dallas, USA, 17th-21st November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 1380 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of UE capability to support early CSI acquisition for L3 handover [EarlyCSI_L3HO] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, China Unicom, Sony, Turkcell, NTT Docomo INC., Meta, Ericsson, Reliance Jio, Vodafone, ZTE Corporation, BT Plc., Deutsche Telekom, Vivo, LG Electronics Inc., Xiaomi, NEC, Samsung, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of UE capabilities to support early CSI acquisition for handover. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following two capabilities are introduced: - Optional per band UE capability to indicate whether the UE supports early CSI-RS and CSI-IM measurement and CSI reporting - Optional per band UE capability to indicate whether the UE supports early CSI-RS measurement and CSI reporting without CSI-IM resource configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Early CSI acquisition for handover is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 38.331 CR 5587 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### 4.2.7.2	BandNR parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| ack-NACK-FeedbackForMulticastWithDCI-Enabler-r17 Indicates whether the UE supports DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signalling via DCI format 4_2. A UE supporting this feature shall also indicate support of ack-NACK-FeedbackForMulticast-r17 and dynamicMulticastDCI-Format4-2-r17. | Band | No | N/A | N/A |
| < Unchanged parts are omitted > | Band | No | N/A | N/A |
| earlyCSI-AcquisitionWithCSI-IM-r19 Indicates whether the UE supports CSI-RS and CSI-IM measurement and CSI reporting after reception of handover command based on periodic CSI-RS(s) and CSI-IM of cell indicated in earlyCSI-Acquisition-r19 of ReconfigurationWithSync. This capability comprises the following parameters with maximum values that are defined within the resource set: - maxNumOfCSI-RS-Resource-r19 indicates the maximum number of CSI-RS resources for CMR associated with CSI report configuration for target cell; - maxNumOfCSI-RS-Ports-r19 indicates the maximum number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for target cell; - maxNumOfNZP-CSI-RS-Ports-r19 indicates the maximum number of ports in one NZP CSI-RS resource; - maxRank-r19 indicates the max rank for CSI reporting for target cell; - maxNumOfCSI-IM-Resource-r19 indicates the maximum number of CSI-IM resources for interference measurement associated with CSI report configuration for target cell. | Band | No | N/A | N/A |
| earlyCSI-AcquisitionWithoutCSI-IM-r19 Indicates whether the UE supports CSI-RS measurement and CSI reporting without CSI-IM resource configuration after reception of handover command based on periodic CSI-RS(s) only. A UE supporting this feature shall also indicate support of earlyCSI-AcquisitionWithCSI-IM-r19. | Band | No | N/A | N/A |
| enhancedChannelRaster-r18 Indicates whether the UE other than (e)RedCap UE supports the requirements for UE channel bandwidths located on the enhanced channel raster of a band as specified in TS 38.101-1 [2] and TS 38.101-5 [34]. Indicates whether the (e)RedCap UE supports the requirements for UE channel bandwidths located on the enhanced channel raster of a band as specified in TS 38.101-1 [2], clause 5.4I. It is mandatory with capability signalling for UEs other than (e)RedCap UE for certain bands (as defined in TS 38.101-1 [2] and TS 38.101-5 [34]) from Rel-18. It is mandatory with capability signalling for all (e)RedCap UEs for all bands supported by the UE. Otherwise, it is optional. | Band | CY | N/A | FR1 only |
| < Unchanged parts are omitted > |  |  |  |  |

END OF CHANGE

Annex 1: Introduction of TEI19 Early CSI Acquisition UE capabilities to 3GPP TS 38.822 V19.0.0

## 8.2.X	TEI19 [EarlyCSI_L3HO]

| Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] or TS 37.355 [9] or TS 38.355 [23] | Features | Parent IE in TS 38.331 [2] or TS 37.355 [9] or TS 38.355 [23] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X-1 | CSI-RS and CSI-IM measurement and CSI reporting | Indicates whether the UE supports CSI-RS and CSI-IM measurement and CSI reporting based on periodic CSI-RS(s) and CSI-IM of cell indicated in earlyCSI-Acquisition-r19 of ReconfigurationWithSync. |  | earlyCSI-AcquisitionWithCSI-IM-r19 | X. TEI19 and Others | BandNR | No | No |  | Optional with capability signaling |
| X-2 | CSI-RS measurement and CSI reporting without CSI-IM reception | Indicates whether the UE supports CSI-RS measurement and CSI reporting without CSI-IM resource configuration based on periodic CSI-RS(s) cell indicated in earlyCSI-Acquisition-r19 of ReconfigurationWithSync. | earlyCSI-Acquisition-r19 | earlyCSI-AcquisitionWithoutCSI-IM-r19 | X. TEI19 and Others | BandNR | No | No |  | Optional with capability signaling |