3GPP RAN WG2 Meeting #132                                                                               R2- 2508600

Dallas, USA, Nov. 17th-21st

Agenda Item:	9.7.2

Source:	HONOR

Title:	Discussion on IoT-NTN to support IMS voice call

Document for:	Discussion and Decision

- Introduction

In Rel-20 Non-Terrestrial Networks (NTN) for Internet of Things (IoT) WID [1], the following objectives were achieved:

The aim of this work item is to specify the enhancement of NB-IoT-NTN to support IMS voice call over GSO with the following objectives: - Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2] - Coordination with SA is required - The objective can be updated with further details based on the outcome of the above down-selection - Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1] - Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2] - Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2]

As shown in the following, the agreements in RAN2#131bis meeting were concluded:

- Agreements regarding the reply LS to SA2: 1. Regarding SA2 Q1, RAN2 is not able to make any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC, and thinks the implementation of ROHC functions (e.g., compression and decompression) falls outside of RAN2 scope 2. Regarding SA2 Q3, RAN2 responds to SA2 that scheduling methods may handle the voice packets of different sizes (e.g., via semi-persistent scheduling and dynamic scheduling, including over-provisioning of resources in UL), but RAN2 will further study and discuss the details of the scheduling mechanism to avoid the potential issues. 3. Regarding SA2 Q6, RAN2 responds to SA2 that it is technically feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20. 4. Regarding SA2 Q9 and Q10, RAN2 responds to SA2 that it is technically feasible to introduce SRB with RLC UM for voice packet 5. We add a sentence in the same reply LS saying that majority of companies in RAN2 prefer the UP over CP solution (because of the higher impacts on RAN2 specs of the CP solution). RAN2 also understands that the final decision will be taken at the checkpoint in December plenary. - Agreements regarding the reply LS to SA4: 1. RAN2 assumes the MAC header size may be 1 to 3 bytes. However, in most common scenario, the total MAC header size is likely to be 3 bytes. 2. For transmitting voice packets via the UP solution RAN2 expects 1 byte for PDCP + 1 byte for RLC UM + the MAC header size 3. For transmitting voice packets via the CP solution RAN2 expects 2 bytes for RRC + 2 bytes for RLC AM (the only mode currently specified for CP) or 1 byte for RLC UM (if RLC UM will be introduced for SRB in case a CP based solution will be selected) + the MAC header size 4. RAN2 will provide further feedback on the expected average RoHC header size 5. We indicate to SA4 that RAN2 has not started work on SPS for voice but thinks that SPS periodicities like 120ms and 240ms periodicities that do not divide 10240 would require additional specification work in RAN2 to resolve the issues

In this contribution, we will discuss some issues needed to be addressed for supporting IMS voice call over GSO.

- Discussion

2.1 CP solution vs UP solution

An IMS-based session requires at least two distinct EPS bearers with different reliability requirements, mapped to specific RLC transmission modes.

- IMS Signalling Bearer (Default EPS Bearer): This bearer carries SIP messages (TCP-based), which require high reliability. The loss of a single SIP message can cause call failure. Therefore, this bearer must be configured with RLC Acknowledged Mode (AM).

- Media Bearer (Dedicated EPS Bearer): This bearer carries real-time voice packets (UDP-based). It prioritizes low latency over perfect reliability and should be configured with RLC Unacknowledged Mode (UM) to minimize transmission delay.

Observation 1: IMS Signalling Bearer and Media Bearer have different QoS requirement, which is different in RLC transmission mode.

During the previous discussions, some companies proposed to utilize the Control Plane (CP) solution for IMS signaling and the User Plane (UP) solution for media transmission for IMS Voice over NB-IoT NTN. We failed to find the benefit for the hybrid approach based on the following considerations.

- It requires the network simultaneously supports both CIoT EPS optimizations: CP solution for IMS signaling and UP solution for media. This introduces significant complexity and overhead in network implementation and operation.

- In LTE, IMS signaling packets are treated as standard user data with the default eps bear (e.g., QCI=5). They do not require the highest priority of SRBs. The requirement for reliable delivery for IMS signaling can be perfectly met within the UP by configuring RLC in Acknowledged Mode (AM) on the DRB.

we see no noticeable advantage of the CP solution for IMS signaling transmission. We think the UP solution is sufficient, simpler, and more aligned with the standard architecture.

Proposal 1:  Both IMS signalling and media should be supported over the one solution.

The current CP or UP solution all needs enhancement to cover the requirement for both IMS Signalling Bearer and Media Bearer.

The Control Plane CIoT EPS Optimization (CP solution) is designed for infrequent, small, and delay-tolerant data transmission. It encapsulates user data within NAS PDUs, which are transported over Signalling Radio Bearers (SRBs). If we use CP solution for both IMS signalling and media, there are following disadvantages:

- As per 3GPP specifications (TS 36.331), SRBs must operate in RLC Acknowledged Mode (AM) to guarantee the delivery of critical control signalling. This mandatory use of RLC-AM is fundamentally incompatible with the low-latency requirements of real-time voice traffic.

- Attempting to reconfigure an SRB to use RLC Unacknowledged Mode (UM) for voice packets would be a fundamental violation of legacy design principle.

The User Plane CIoT EPS Optimization (UP solution) establishes standard Data Radio Bearers (DRBs), allowing for:

- Appropriate RLC Mode Selection: A dedicated DRB for voice media can be configured with RLC Unacknowledged Mode (UM), minimizing transmission delay by avoiding retransmissions. While a dedicated DRB for signalling can be configured with RLC AM.

- Distinct QoS Handling: Separate DRBs can be established for IMS signalling (RLC-AM) and media (RLC-UM), enabling the network to apply differentiated scheduling and prioritization.

- Protocol Efficiency: Direct transmission over DRBs avoids the excessive header overhead associated with encapsulating voice packets within NAS PDUs.

- Spec Impact: The limitation of 2 DRBs for IOT device needs to be removed and the spec impact is limited.

Thus, the UP solution is more suitable for the transmission for both IMS signalling and media. To handling different QoS and transmission requirement for IMS signalling and media, Separate DRBs can be established.

Proposal 2:  Support UP solution with separate DRBs for IMS voice service.

  - Scheduling voice packets of different sizes

In the previous SA2 discussion, the issue over/under scheduling was identified, which will have a significant impact on RAN2. It is believed that this scheduling issue is caused by the variation in packet size. We think that the variation in packet sizes is usually due to the transitions between talk and silence phases and the changes in the RoHC state.

Regarding the variation in packet size caused by the transition between talk and silence phases, it will usually bring about the significant difference in size between the voice packets transmitted during the talk phase and the silence packets transmitted during the silence phase. How to allocate radio resource will be a problem that has to be considered when applying SPS to IMS voice call. We know that SPS is typically used for the traffic with periodic characteristics, and it is assumed that the size of the packet is relatively fixed. If SPS is introduce for IMS voice call, the issue we need to solve is how to use SPS when the packet size varies in order to improve the efficiency of resource scheduling. Based on the current mechanism, we can reserve the SPS resource based on the largest packet size. However, it will cause over-scheduling, which will result in inefficiency on radio resource utilization.

One solution is to allocate different SPS configuration for transmitting silence packet in silence phase and voice packet in talk phase respectively. How to dynamically switch between the two SPS configurations should be taken into account in the subsequent discussion.

Proposal 3: It is suggested to define two SPS configurations for silence packets and voice packets transmission respectively.

Due to the fact that the changes in the RoHC state usually have the characteristic of suddenness，a dynamic scheduling is more suitable for scheduling the packet with size variation caused by the changes in the RoHC state. In this regard, we believe that dynamic scheduling is only a supplement to SPS scheduling. When and how to trigger dynamic scheduling should be taken into consideration. The fundamental reason for dynamic scheduling is to provide appropriate scheduling grant to meet the variation in packet size. The network needs to know the change of packet size to allocate the appropriate scheduling grant. Since there are many issues involved, how to use dynamic scheduling combined with SPS needs further discussion.

Proposal 4: RAN2 to discuss whether to use the dynamic scheduling for the change of packet size due to RoHC state.

In the current spec, for the purpose of coverage enhancement, the repetition transmission mechanism has been introduced for IOT terminals. When considering that SPS configuration is used for the transmission of voice packet, the impact of the repetition transmission on SPS configuration needs to be taken into account. In addition, usually, a voice service will last for a relatively long period of time. During this period, there is a possibility that the coverage level may change, and it might affect the use of SPS configuration.

Proposal 5: RAN2 to study SPS configuration for voice packet transmission with consideration of the impact of coverage enhancement.

  - Others

Enhancements related to RRC connection setup are introduced considering the characteristic of NB-IoT. For example, massive NB-IoT UEs could trigger access at the same time which may cause access congestion. Extended Access

Barring mechanism enables the NW to control the access for these delay-tolerant NB-IoT UEs. However, based on the performance requirements for satellite access from SA1, the call set up time needs to be less than 30s. The voice service for IoT NTN is not delay-tolerant which is quite different from other IoT services. Considering the RTT for the NTN network (especially for communication using GSO satellite access), the RRC connection setup latency should be reduced for voice service compared with other services for NB-IoT UEs.

It is important to increase the success probability and reduce the latency for the access triggered by voice service (e.g., IMS voice call and emergency call),

Proposal 6: RAN2 to study how to reduce the latency for the access triggered by voice service (e.g., IMS voice call and emergency call).

One possible solution is that the UE's initial access attempt could be recognized and prioritized by the network. It is straight forward to introduce a new value in the establishmentCause for the voice service. As mentioned during the discussion in RAN meeting, a NB-IoT UE initiating a voice service could set the specific establishmentCause in the RRC connection request message (MSG3). Upon receiving an RRC connection request with the specific establishmentCause, the eNB could prioritize this request over other access attempts.  But we are still open to the solutions about whether and how UE informs the NW about if the access is triggered by voice service during the random-access procedure.

Proposal 7: RAN2 to discuss whether and how UE informs the NW about if the access triggered by voice service during the random access procedure.

In terrestrial networks, UEs indicate the need for emergency service establishment during RRC connection and NAS signaling. The network responds by prioritizing the access procedure and establishing EPS bearers with appropriate QoS.

For IoT-NTN, characterized by long delays caused by large RTT and potentially limited resources, explicit signaling and prioritization mechanisms are crucial to ensure emergency calls can be established reliably. Without these mechanisms, a IoT device's emergency request may be delayed or blocked.

An IoT-NTN device must be able to quickly determine if the cell it is camped on supports emergency services. Without this indication, the device may waste energy attempting registration or service request procedures on a cell that cannot fulfil its emergency request. We propose introducing an emergencyServiceSupport indicator in the relevant IoT-specific SIB. A NB IoT UE requiring emergency service shall only attempt access on a cell where the relevant SIB indicates emergencyServiceSupport = true.

Proposal 8: Introduce an emergencyServiceSupport indicator in the relevant IoT-specific SIB.

- Conclusions

In conclusion, we discuss the issues related to semi-persistent scheduling, RRC connection setup and emergency call. Based on the discussion, the following observation and proposals are concluded:

Observation 1: IMS Signalling Bearer and Media Bearer have different QoS requirement, which is different in RLC transmission mode.

Proposal 1:  Both IMS signalling and media should be supported over the one solution.

Proposal 2:  Support UP solution with separate DRBs for IMS voice service.

Proposal 3: It is suggested to define two SPS configurations for silence packets and voice packets transmission respectively.

Proposal 4: RAN2 to discuss whether to use the dynamic scheduling for the change of packet size due to RoHC state.

Proposal 5: RAN2 to study SPS configuration for voice packet transmission with consideration of the impact of coverage enhancement.

Proposal 6: RAN2 to study how to reduce the latency for the access triggered by voice service (e.g., IMS voice call and emergency call).

Proposal 7: RAN2 to discuss whether and how UE informs the NW about if the access triggered by voice service during the random access procedure.

Proposal 8: Introduce an emergencyServiceSupport indicator in the relevant IoT-specific SIB.

- References

- RP-251867 Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4