3GPP RAN WG2 Meeting #132                                                                                R2-2508601

Dallas, USA, Nov. 17th – 21st, 2025

Agenda Item:	9.3.2

Source:	HONOR

Title:	Discussion on RRM measurement prediction

Document for:	Discussion and Decision

- Introduction

In Rel-20 NR AI/ML for mobility WID [1], the following objectives were achieved:

- Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: o Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. o L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: o UE capability request and response procedure o Applicability report for both full and partial configuration approach o Inference configuration and inference report o Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality o UE sided data collection o For measurement event prediction:  Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation. - For RRM measurement prediction, specify signalling and protocol aspects to enable LCM functionality management for network sided model including [RAN2]: o Inference input report for at least RRM sub-case 2 of intra-frequency temporal domain case A for L3 cell prediction o Inference input report for RRM sub-case 5 of all scenarios for L3 beam level prediction o Network sided data collection for all scenarios and all RRM sub-cases of both L3 cell level and L3 beam level prediction, at least  UE is configured to log RRM measurement results.  UE supports periodical logging and L3 event triggered logging.  UE can indicate logged data availability. Note 4: All scenarios in this objective refer to intra-frequency domain case A and case B, inter-frequency prediction and spatial domain prediction.

In this contribution, we will discuss inference configuration/reporting of RRM measurement prediction for UE sided model.

- Discussion

During the Rel-19 AI/ML for mobility study, significant performance gains were demonstrated for AI/ML-assisted mobility through RRM measurement prediction and measurement event prediction. To realize these gains in normative specifications, a structured framework for configuring and reporting AI inference operations is essential. In this contribution, we will focus on discussing inference configuration/reporting.

  - Inference Configuration

The current L3 measurement configuration framework provides a robust foundation for managing UE measurement behavior. Extending this framework for AI inference ensures backward compatibility and minimizes implementation complexity.

We propose integrating inference configuration as an inherent part of the existing MeasConfig structure. The association between inference configuration and measurement objects could be established through the existing MeasId mechanism.

This approach has the following advantages:

- One-to-one mapping between a specific measurement object and its associated inference configuration.

- Flexible combinations where the same measurement object can be associated with different inference configurations through different MeasId. For example, serving cell can be associated with temporal domain case A and frequency domain prediction.

Proposal 1: Integrating inference configuration as an inherent part of the existing MeasConfig structure.

The SI phase has reached agreements on inference parameters per use case:

- Temporal Domain Case A (handover performance enhancement): Requires configuration of Prediction Window (PW) length to define the future time horizon for prediction.

- Temporal Domain Case B (measurement relaxation): Requires configuration of Measurement Reduction Rate in Temporal domain (MRRT) to control the trade-off between prediction accuracy and measurement effort reduction.

- Frequency Domain Prediction: Requires configuration of both measured frequency and predicted frequency to establish the frequency layer relationship.

In our view, we should use these agreements as a baseline for further discussion on inference parameters.

Proposal 2: Using agreements on inference parameters in SI phase as a baseline for further study.

For frequency domain prediction, simply configuring the frequency pair is insufficient for optimal UE operation. We think additionally configuring the correlation coefficient between the two frequencies may benefit for UE inference. UE may select appropriate inference models based on the correlation coefficient. For example, the lower the correlation coefficient, the more complex the model can be used to achieve higher prediction accuracy.

Proposal 3: RAN2 to discuss if correlation coefficient is needed for frequency domain prediction.

  - Inference Reporting Configuration

In our views, Inference reporting serves different purposes versus conventional measurement reporting. Simultaneous execution of both legacy measurement and inference reporting for the same measurement object creates unnecessary overhead. Networks should be able to choose between legacy measurement-based mobility and AI-enhanced mobility per UE and per measurement object. We propose introducing a separate InferenceReportConfig structure, which is similar to the existing ReportConfigNR but is specifically tailored for inference reporting. The association between inference configuration and inference reporting should be established through an enhanced MeasId structure. Upon receiving a MeasurementReport message from the UE, gNB uses the included MeasId as the key to look up the original reporting configuration.

Proposal 4: Introducing a separate InferenceReportConfig structure that parallels the existing ReportConfigNR, gNB can distinguish the type of the received report by MeasId field.

  - When to perform inference

We propose that the initiation of UE-side inference is governed by network-configured triggering conditions. This provides the gNB with precise control, enabling it to enable inference tasks only in specific scenarios. For instance:

- To save measurement overhead, the gNB can configure inference for "Time-domain Case B" to start only when the serving cell's RSRP is above a certain threshold.

- To enhance Handover (HO) performance, the gNB can configure inference for "Time-domain Case A" to start only when the serving cell's RSRP falls below a certain threshold.

In addition, in order to flexibly control the UE to start inference, the trigger conditions should be optional. When the inference configurations are configured without trigger conditions, UE should start inference immediately.

Proposal 5: An optional triggering conditions can be included in inference configuration to control UE when to perform inference.

In the current framework, when a measurement gap is configured, the gNB abstains from scheduling any data during the gap period, as it expects the UE to be tuned away to perform inter-frequency measurements. With the introduction of frequency domain prediction:

- During some gap occasions, the UE will not perform actual RF measurements but will instead predict the inter-frequency quality. During these occasions, the UE remains available for data transmission.

- During other gap occasions, the UE will perform actual measurements (e.g., performance monitoring) as per legacy behavior, requiring it to tune away.

If the gNB cannot distinguish between these two types of gap occasions, it will assume that every configured gap occasion is used for actual measurement and will avoid scheduling data. This would nullify the throughput benefit that frequency domain prediction is designed to achieve. Therefore, a mechanism for the gNB and UE to align the pattern of prediction gap occasions is essential.

Proposal 6: RAN2 to discuss a mechanism for the gNB and UE to align the prediction gap occasions for frequency domain prediction.

- Conclusions

In conclusion, we discuss inference configuration/reporting of RRM measurement prediction for UE sided model, the following proposals are concluded:

Proposal 1: Integrating inference configuration as an inherent part of the existing MeasConfig structure.

Proposal 2: Using agreements on inference parameters in SI phase as a baseline for further study.

Proposal 3: RAN2 to discuss if correlation coefficient is needed for frequency domain prediction.

Proposal 4: Introducing a separate InferenceReportConfig structure that parallels the existing ReportConfigNR, gNB can distinguish the type of the received report by MeasId field.

Proposal 5: An optional triggering conditions can be included in inference configuration to control UE when to perform inference.

Proposal 6: RAN2 to discuss a mechanism for the gNB and UE to align the prediction occasions for frequency domain prediction.

- References

- RP-252899 Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR