3GPP TSG RAN WG2 #132	R2-2508603

Dallas, USA, Nov 17th-21st, 2025

Agenda Item:	7.0.2.21

Source: 	Xiaomi

Title:  	Discussion on UE autonomous TA adjustment for the positioning SRS in the RRC Inactive state

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss UE autonomous TA adjustment for the positioning SRS in the RRC Inactive state and provide corresponding suggestions.

Discussion

In Rel-18, UE autonomous TA adjustment for positioning SRS in the RRC Inactive state was introduced, and we summarize how the UE performs autonomous TA adjustment according to the RAN2/RAN1/RAN4 specifications.

TS 38.331 [1]

#### 5.3.13.6	Cell re-selection or cell selection or L2 U2N relay (re)selection while T390, T319 or T302 is running or SDT procedure is ongoing (UE in RRC_INACTIVE) or SRS transmission in RRC_INACTIVE is configured

The UE shall:

1>	if cell reselection occurs while T319 or T302 is running or while SDT procedure is ongoing; or

1>	if relay (re)selection or cell selection by a L2 U2N Remote UE or by L2 Intermediate U2N Relay UE occurs while T319 is running; or

1>	if cell changes due to relay reselection or cell selection by a L2 U2N Remote UE or by L2 Intermediate U2N Relay UE while T302 is running:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'RRC Resume failure';

1>	else if cell selection or reselection occurs while T390 is running, or cell change due to relay selection or reselection occurs while T390 is running:

2>	stop T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4.

1>	else if cell reselection occurs when srs-PosRRC-Inactive is configured:

2>	indicate to the lower layer to stop inactivePosSRS-TimeAlignmentTimer;

2>	release the srs-PosRRC-Inactive.

1>	else if cell reselection occurs when srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured and if there is an on-going SRS for positioning transmission procedure in RRC_INACTIVE:

2>	if the selected cell is not included in the srs-PosConfigValidityArea:

3>	indicate to the lower layer to stop inactivePosSRS-ValidityAreaTAT;

3>	initiate RRC connection resume procedure in 5.3.13.2;

2>	else if the cell is included in the srs-PosConfigValidityArea:

3>	if the selected cell and the previously camped cell are in the same srs-PosConfigValidityArea:

4>	if autonomousTA-AdjustmentEnabled is configured and if the Timing Advance validation requirements specified in clause 5.6.6.3 of TS 38.133 [14] is met:

5>	indicate to the lower layer to update Timing Advance and stored RSRP;

4>	instruct lower layers to continue transmitting SRS if Timing Advance validation condition as specified in TS 38.321 [3] are satisfied;

3>	if the selected cell and previously camped cell are in the different srs-PosConfigValidityArea:

4>	initiate RRC connection resume procedure in 5.3.13.2;

4>	indicate to the lower layer to stop inactivePosSRS-ValidityAreaTAT.

Observation 1：The RRC layer indicates the lower layer to update Timing advance and stored RSRP when the Timing Advance validation requirements specified in clause 5.6.6.3 of TS 38.133 [14] are met.

TS 38.321 [2]

- 5.26.2	TA validation for SRS transmission in RRC_INACTIVE

RRC configures the following parameters for validation for SRS transmission in RRC_INACTIVE:

-	inactivePosSRS-RSRP-ChangeThreshold: RSRP threshold for the increase/decrease of RSRP for time alignment validation.

The MAC entity shall:

1>	if the UE receives configuration for SRS transmission in RRC_INACTIVE:

2>	store the RSRP of the downlink pathloss reference with the current RSRP value of the downlink pathloss reference as in TS 38.331 [5].

1>	else if the UE is configured with SRS transmission in RRC_INACTIVE:

2>	if Timing Advance Command MAC CE is received as in clause 5.2, or;

2>	if Timing Advance Command or Absolute Timing Advance Command is received for Random Access procedure that is successfully completed:

3>	update the stored the RSRP of the downlink pathloss reference with the current RSRP value of the downlink pathloss reference.

2>	if the UE is configured Positioning with SRS with validity area and the upper layer indicates the MAC to update the stored RSRP:

3>	update the RSRP of the downlink pathloss reference with the current RSRP value of the downlink pathloss reference of the camped cell as specified in TS 38.331 [5].

The MAC entity shall consider the TA to be valid when the following conditions are fulfilled:

1>	compared to the stored downlink pathloss reference RSRP value, the current RSRP value of the downlink pathloss reference of the camped cell as specified in TS 38.331 [5] has not increased/decreased by more than inactivePosSRS-RSRP-ChangeThreshold, if configured; and

1>	inactivePosSRS-TimeAlignmentTimer is running or inactivePosSRS-ValidityAreaTAT is running when positioning validity area is configured.

Observation 2: The MAC layer updates the stored RSRP based on the upper layer (RRC layer) indication.

TS38.213 [3]

- 4.2	Transmission timing adjustments

If the received downlink timing changes and is not compensated or is only partly compensated by the uplink timing adjustment without timing advance command as described in [10, TS 38.133], the UE changes NTA accordingly. If a UE operates with two TAGs on an active UL BWP of a serving cell, the UE expects that a difference between a first downlink timing associated with a first TAG and a second downlink timing associated with a second TAG is not larger than the CP length for the active UL BWP unless the UE indicates rxTimingDiff. If a UE indicates posUE-TA-AutoAdjustment, and transmits SRS based on a configuration by SRS-PosResourceSet in SRS-PosRRC-InactiveValidityAreaConfig in RRC_INACTIVE state,

-	if the UE is provided autonomousTA-AdjustmentEnabled, the UE may autonomously update NTA at cell reselection as described in [10, TS 38.133]

-	if the UE is not provided autonomousTA-AdjustmentEnabled, the UE maintains the NTA of a last serving cell prior to the release of a dedicated RRC connection [11, TS 38.321].

Observation 3: The UE physical layer autonomously updates NTA at cell reselection according to the RAN4 requirements.

TS38.133 [4]

#### 7.1.2.4	UE transmit timing for positioning measurements

If cell reselection occurs in RRC_INACTIVE within srs-PosRRC-InactiveValidityArea area, the UE shall have capability to follow the frame timing change of the reference cell when transmitting SRS for positioning. The uplink frame transmission takes place NTA+NTA offset×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. The UE shall use the current camping cell as reference cell for deriving the UE transmit timing.

When autonomousTA-AdjustmentEnabled-r18 is configured, the following requirements apply to the UE supporting posUE-TA-AutoAdjustment-r18 upon cell reselection to a new camping cell within the SRS validity area (srs-PosRRC-InactiveValidityArea):

If the absolute value of the DL timing difference between the new camping cell and the previous camping cell immediately after cell reselection is ≥ CP/4, UE shall autonomously adjust the TA as

TAadjusted=TAold+2*Tnew-Told

where TAadjusted and TAold are the TA after adjustment in the new camping cell and TA applied in previous camping cell, respectively, Tnew is the DL timing of the new camping cell immediatedly after cell reselection and Told is the DL timing of the previous camping cell immediately before cell reselection.

-	The UE UL transmission timing error after autonomous TA adjustment shall be less than or equal to Te in clause 7.1.2.

-	If the TA value would be negative after applying the autonomous TA adjustment, the TA is set to zero. i.e. the TA applied in the new camping cell shall be set to maxTAadjusted, 0.

If the DL timing difference is < CP/4, UE follows the DL timing of the new camping cell by performing gradual timing adjustment as defined in clause 7.1.2.1.

Observation 4: RAN4 defines the requirements of UE transmit timing for positioning measurements.

Observation 5: Physical layer measurements and parameters are used to define the requirements of UE transmit timing for positioning measurements.

According to the above observations, we can conclude that:

- The physical layer updates the timing advance according to the RAN4 requirements not based on RRC layer indication.

- The MAC layer updates the stored RSRP based on indications from the RRC layer, but does not update Timing Advance

Therefore, we have the following observation:

Observation 6: There is a misalignment between the higher layer and the physical layer regarding UE autonomous TA adjustment for positioning SRS.

Therefore, according to the above observations, we have the following proposal:

Proposal 1: The physical layer updates the Timing Advance according to the RAN4 requirements as specified in the existing specification.

Proposal 2: The physical layer indicates to the MAC layer to update the stored RSRP when the Timing Advance is updated.

Proposal 3: The MAC layer updates the stored RSRP according to the indication from the physical layer.

Proposal 4: Remove the RRC layer indicating the lower layer to update Timing advance and stored RSRP.

Considering that proposal 2 relates to the RAN1 spec, we suggest sending an LS to RAN1 if RAN2 agrees on P2.

Proposal 5: Send an LS to RAN1 to indicate the RAN2 agreements if Proposal 2 is agreed.

Conclusions

In this contribution, we discuss UE autonomous TA adjustment for the positioning SRS in the RRC Inactive state and provide the following observations

Observation 1：The RRC layer indicates the lower layer to update Timing advance and stored RSRP when the Timing Advance validation requirements specified in clause 5.6.6.3 of TS 38.133 [14] are met.

Observation 2: The MAC layer updates the stored RSRP based on the upper layer (RRC layer) indication.

Observation 3: The UE physical layer autonomously updates NTA at cell reselection according to the RAN4 requirements.

Observation 4: RAN4 defines the requirements of UE transmit timing for positioning measurements.

Observation 5: Physical layer measurements and parameters are used to define the requirements of UE transmit timing for positioning measurements.

Observation 6: There is a misalignment between the higher layer and the physical layer regarding UE autonomous TA adjustment for positioning SRS.

Proposal 1: The physical layer updates the Timing Advance according to the RAN4 requirements as specified in the existing specification.

Proposal 2: The physical layer indicates to the MAC layer to update the stored RSRP when the Timing Advance is updated.

Proposal 3: The MAC layer updates the stored RSRP according to the indication from the physical layer.

Proposal 4: Remove the RRC layer indicating the lower layer to update Timing advance and stored RSRP.

Proposal 5: Send an LS to RAN1 to indicate the RAN2 agreements if Proposal 2 is agreed.

References

[1] 3GPP TS 38.331 V19.0.0，Radio Resource Control (RRC) protocol specification

[2] 3GPP TS 38.321 V19.0.0，Medium Access Control (MAC) protocol specification

[3] 3GPP TS 38.213 V19.1.0，Physical layer procedures for control

[4] 3GPP TS 38.133 V19.2.0，Requirements for support of radio resource management