3GPP TSG-RAN WG2 Meeting #132	R2-2508617

Dallas, USA, 17th –21st November, 2025

Agenda Item:	10.2.3

Source:	Huawei, HiSilicon

Title:	Discussion on device types and forward compatible design

Document for: Discussion and Decision

1	Introduction

In this contribution, we provide our views on design approaches for 6GR.

2	Discussion

According to 6GR SID, one objective is to support diverse device types as follows:

From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz). - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] …… g) Target scalable and forward compatible design for diverse device types.

It is expected to have a number of device types supported in 6GR, for example, eMBB devices, IoT devices, and advanced device types supporting advanced services. The design target for each device types will be discussed in RAN plenary and RAN1 WG, such as the maximum bandwidth, Tx/Rx number, coverage, and so on. In our view, RAN2 can wait for the conclusions from RAN plenary and RAN1 on the definition of device types.

From RAN2 perspective, it is needed to guarantee the scalability and future compatibility to support diverse device types from 6G DAY-1. The design on UE capability framework, modular RRC signalling, and system information should consider the impact of device types.

Regarding UE capabilities for different device types, defining a common capability set across these types requires careful evaluation. Such common set encompasses basic features defined by RAN1, RAN2, and RAN4. Notably, it will be difficult for RAN2 to finalize a common capability set for diverse device types until two prerequisites are met: first, clear definitions of the device types themselves; second, clear specifications of device type-specific requirements and features. Building on top of this common capability set, further mandatory or optional capabilities for each individual/specific device type can then be defined.

The modular RRC design is a key focus area in 6GR research. In our opinion, a basic RRC module should be defined to support essential functions across all device types. Building upon this basic module, the supported functions and features can vary not only among different device types but also within the same device type (e.g., eMBB UE) due to differences in UE vendors, models, and versions.

The modular RRC design should be both scalable and efficient to address the diverse requirements of various device types. A comprehensive analysis of this concept is provided in our accompanying paper [2]. However, similar to discussions on UE capabilities, it is not possible to determine which features can be integrated into the basic modular design until device types and their specific requirements and features are clearly defined.

Lastly, the design of system information is also impacted by device types. To support initial access for device types, essential information should be defined in SIB1 and some other necessary information may be needed in other SIBs. Again, at the current stage, it is difficult for RAN2 to make significant progress on what information should be included in SIB1 or other SIBs.

In our view, the approach for discussing device types in RAN2 should be as follows: first, we should await the conclusions from TSG RAN and RAN1 regarding the definition of device types. Once these definitions are established, RAN2 can proceed to discuss specific requirements and features for each device type. Following this, RAN2 can then address solutions for the UE capability signalling design, RRC signaling modular design, and system information design.

Proposal 1: The approach for device type discussion in RAN2 can be as follows:

  - RAN2 starts discussing specific requirements and features for each device type only after RAN and RAN1 define the device types.

  - UE capability signalling design, RRC signaling modular design, and system information design are only addressed after RAN2 and other WG completed the requirement and features for each device type.

Based on the learning from 5G, extending certain features (especially those related to idle mode) in subsequent versions has become difficult, hard to read, and even challenging to maintain backward compatibility. Therefore, considering forward compatibility during the initial design phase of 6G can help avoid potential issues that may arise in the future.

One of the aspects is cell bar control, in 5G we have cell bar in MIB in first release and then introduces cell bars in SIB1 for REDCAP(RedCap1Rx/RedCap2Rx and further extension to halfDuplexRedCap /eRedCap1Rx/eRedCap2Rx) /2RxXR/NTN(further extsion to FixedVSAT and MobileVSAT)/ATG/IAB/NES in later releases. Some cell barring functions are mutually exclusive (for example, when using NTN cell barring, MIB cell barring is ignored), while others can be used together (e.g., MIB cell barring and REDCAP cell barring). We have to use field explanation to restrict illegal combinations or indicate which ones can be used together. This will reduce the readability and compatibility of the specification. It would be very attractive if there were a unified cell barring mechanism that could be applied to all feature specific functions and support further extension easily, just as we have done for UAC in 5G.

Another aspect is SI scheduling related to positioning. Since the positioning function in Release 16 requires a large number of SIBs, an additional dedicated SI scheduling list for positioning has been introduced in 5G. However, as discussed in [3], this additional dedicated SI scheduling list can cause some issues when PWS/ETWS starts or stops. Therefore, even if the positioning function may not be a first-day feature in 6G, we still need to consider the possibility that the positioning function might require a large amount of SIB when designing the SI scheduling mechanism for 6G, in order to avoid such problems.

The transmission of SIB1 is also affected by the increasing number of functions (such as REDCAP, NPN, etc) in subsequent releases. In Release 15, we evaluated that the required size of SIB1 was 3000 bits and submitted this evaluation request to RAN1. However, as more and more functions have an impact on SIB1, leading to a continuous increase in its size, we encountered some issues where certain functions could not be enabled due to the size of SIB1. Therefore, when designing the essential information (SI) transmission for 6G, we need to consider this issue to ensure that it can easily accommodate future expansion requirements.

Proposal 2: Forward-compatible design should be given special consideration for idle-mode features, such as system information scheduling/transmission and cell barring.

# 3	Conclusion

This contribution makes the following proposals:

Proposal 1: The approach for device type discussion in RAN2 can be as follows:

  - RAN2 starts discussing specific requirements and features for each device type only after RAN and RAN1 define the device types.

  - UE capability signalling design, RRC signaling modular design, and system information design are only addressed after RAN2 and other WG completed the requirement and features for each device type.

Proposal 2: Forward-compatible design should be given special consideration for idle-mode features, such as system information scheduling/transmission and cell barring.

# 4	References

- Draft meeting report of RAN2#131-bis

- R2-2508618, Discussion on RRC signaling design, Huawei, HiSilicon

- R2-2508619, Discussion on cell management and system access, Huawei, HiSilicon