3GPP TSG-RAN WG2 #132	R2-2508642

Dallas, USA, Nov. 17th – 21st, 2025

Agenda Item:	10.3.1.2

Source:	InterDigital, Inc.

Title:	Requirements for QoS, QoE and Service-Awareness

WID/SID:	FS_6G_Radio – Release 20

Document for:	Discussion

# Introduction

RAN approved a new SID on 6G Radio at RAN#108 [1] to study the next generation 3GPP radio. During RAN2 #131bis meeting, RAN2 discussed QoS framework and made the following agreement:

“Study potential benefits and standardized mechanisms for application/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic). Understand the issues and shortcoming with current NR QoS. Aim to look at a general framework”.

This document focuses on QoS design targets and considerations, building on the agreement from last RAN2 meeting. In addition, this document provides an overview of the new traffic characteristics the 6GR needs to be able to effectively support; overview and constraints of the 5G QoS framework; design approach for the 6GR QoS framework as well as discusses service-awareness and QoE aspects.

# QoS, QoE and Service-Awareness

## Traffic Characteristics for 6GR

Immersive communication/XR enhanced with AI-assisted contextual awareness (e.g., XR or AI systems that adapt intelligently to the user’s environment) or conversational assistance (e.g., low-latency, AI-driven interactive services) represent a next-generation class of applications that integrate real-time sensor inputs, media streams e.g., video/audio/haptic, AI data, and cloud/edge AI inference. These emerging applications generate traffic patterns with distinct characteristics compared to traditional broadband data. These include uplink-heavy transmissions, variable burstiness, and tight latency and reliability requirements. They place stringent requirements on data delivery, as they combine both high-throughput multimodal data transmission and low-latency AI-driven interactions with bounded end-to-end round-trip time. In addition, the prompt or AI input data used to express user intent needs to be transmitted reliably for correct processing by the AI model. Current NR mechanisms are insufficient to handle this diversity efficiently As such, it is recommended to design a network with uplink-optimized capacity, ultra-low latency scheduling, synchronized multimodal flows, high reliability, and adaptive QoS, to optimize the end-user’s Quality of Experience (QoE) while ensuring efficient resource utilization across wireless systems.

Observation 1: Emerging AI and immersive traffic introduce new patterns (bursty UL, delay-sensitive, adaptive QoE).

[Dynamically changing traffic characteristics]

An immersive multimodal XR application can adapt its data rate e.g., in terms of modalities, scene composition, resolution and compression ratio while maintaining acceptable QoE level and subjectively satisfactory end user perception. Similarly, scene-driven bursts may cause sudden uplink bursts when the encoder faces high motion/texture or changing scene. Hence, the resulting data traffic characteristics, as driven by application needs and QoE targets, is one that is varying over the course of the user data session.

Observation 2: Advanced immersive/XR applications can adapt in rate and/or modalities and/or scene composition to implement graceful QoE adjustments leading to dynamically changing traffic characteristics.

[Irregular uplink bursty data]

A Mobile Generative AI application can exhibit mid-term, contextually driven changes in its dominantly uplink data rate for its periodic stream component while also generating aperiodic or irregular delay-bounded bursts of varying data volumes and inter-burst intervals where the timely and reliable transfer of data directly impacts the applications task’s success rate. More specifically, AI-assisted tasks may trigger irregular context-based uplink bursts of data when, for example, a new object or feature is detected in the user environment. Additionally, AI-assisted tasks may be periodic or irregular and trigger uplink data bursts for inference e.g., interactive AI prompting, agentic AI. These uplink data bursts may correspond, for example, to images, audio, or video used as input to a Cloud/Edge AI model.

Observation 3: Interactive AI-based services are uplink heavy, latency sensitive, and data sessions are characterized by periodic and/or irregular data bursts with varying volumes and inter-burst intervals.

[Delay-critical low latency constraints]

Real-time services require low end-to-end latency to ensure quality interactive experiences. For example, real-time context-aware AI average end-to-end round-trip delay should be within <50ms (ultra-low latency), lower than 100ms (low latency), and lower than 200ms (moderate latency) such that timely feedback be provided e.g., for object recognition, environmental understanding, and user intent prediction. In this context, AI-assisted tasks require that the input data used for inference be transmitted within a delay-bound to enable inference and related output to meet the expected QoE e.g., time to first token (TTFT). Besides average end-to-end round-trip delay, real-time services may require that the uplink transmission latency be no worse than a certain delay threshold to meet a target QoE.

For many interactive XR/AR scenarios, end-to-end latency budgets follow common motion-to-photon (MTP) guidance for AR with a round-trip time (RTT) <~20-30ms for strong presence and XR control loop exchanges.

Observation 4: Interactive real-time services with AI-assistance consist of delay-critical UL data bursts.

[Real-time wireless platform measurements on providing QoS Burst Guarantees for uplink]

We did several experiments using commercially available generative AI applications over a 5G real-time wireless platform with real-time channel emulation. We collected measurements to characterize the UL transmission latency under several network conditions from single UE to loaded cell. In the loaded cell scenario, a single UE is running the generative AI application, other UEs are set with constant UL bitrate. Then we enabled an UL data burst detection mechanism at the UE running the generative AI application. The UL data burst detection mechanism is configured with parameters from a ‘QoS burst profile’ allowing it to detect the start and end of a data burst. The UE reports detected (or qualified) burst data to the scheduler. The scheduler is configured with QoS burst parameters also from a ‘QoS burst profile’ to provide QoS burst guarantees.

As shown in Figure 2.1-1, the experiments consist of three scenarios where UL burst duration is measured for the UE running the generative AI application, at UE ingress, then measured at gNB egress corresponding to the UL transmission latency of burst data units. In the first scenario, with a single UE, the average UL transmission latency is measured as 88ms. Then in the second scenario, with a loaded cell (4 UEs @ 2Mbps UL + 1 UE with UL data bursts), the average UL transmission latency is measured as 258ms. Finally, in the third scenario, with a loaded cell (4 UEs @ 2Mbps UL + 1 UE with UL data bursts), QoS burst guarantees is enabled, the average UL transmission latency is measured as 96ms. See Table 2.1-1 for detailed measurements and platform related information.

Observation 5: UL data burst guarantee reduces burst transmission latency without GBR over-provisioning by enabling UL data burst differentiated treatment and scheduling for data bursts.

In addition, in the case of cell edge, enabling burst guarantees is also beneficial as the limited resources can be optimized for the delay-critical data burst effectively limiting the impact on the burst transmission latency.

Figure 2.1-1 Measured uplink data burst transmission latency (with and without UL QoS burst guarantees)

Table 2.1-1 Measured UL data burst characteristics with and without UL QoS burst guarantees

| Uplink burst characteristics (Note 1) | 5G baseline single UE (Note 2, 3, 4) |  | 5G loaded cell (Note 2, 3, 4, 5) |  | 5G loaded cell with UL QoS burst guarantees (Note 2, 3, 4, 5, 6) |  |
| --- | --- | --- | --- | --- | --- | --- |
| Measurement Endpoint | UE ingress | gNB egress | UE ingress | gNB egress | UE ingress | gNB egress |
| Duration (ms) | 23 | 88 | 40 | 258 | 29 | 96 |
| Avg Burst Size (Bytes) | 160k | 160k | 160k | 160k | 160k | 160k |
| Avg Bits/s | 56M | 15M | 32M | 5M | 44M | 13M |
| Note 1: Measured uplink data burst characteristics from multiple samples of similar burst size Note 2: TDD format 7D-1S-2U, SU-MIMO, single transmission layer, SCS 30kHz, 40MHz bandwidth Note 3: TDD Schedule Period (dl-UL-TransmissionPeriodicity) is 5ms Note 4: Real-time channel emulation TDLA30-10, average UL MCS 14, average BLER 9% Note 5: 5 UEs, 1 UE with Live AI data traffic, 4 UEs with 2Mbps uplink data traffic Note 6: Data units belonging to an UL data burst are detected at the UE and reported to the scheduler as such |  |  |  |  |  |  |

In summary, the traffic characteristics for new emerging applications and associated data types exhibit rapidly varying bandwidth, latency and burstiness requiring finer granularity of QoS differentiation compared to 5G (e.g. per-packet or per-packet group QoS differentiation) and dynamic RAN adaptation to the application requirements (e.g. dependencies between application packets, QoS performance as well as QoE as seen by the application). These dynamically changing application requirements and traffic characteristics such as adaptive data flows, and irregular delay-bound UL data bursts, pose a challenge to the current QoS framework.

The objective would be to identify RAN-level techniques and QoS requirements that will support QoE-driven framework including studying QoS adaptation and ranging, sub-flow QoS granularity and Delay Critical Guaranteed Burst Rates.

## Overview of 5G QoS framework

The NR QoS framework introduced a flexible model with QoS Flows, QFIs, and mapping to DRBs via SDAP. While effective for 5G, it exhibits several limitations: coarse granularity, static configuration, limited adaptability, and weak linkage to service-level performance. For example, certain packets of a data flow need to be treated differently than others. These limitations restrict optimization for diverse and dynamic traffic types. One example is XR PDU sets, where even though both sets can belong to the same flow, one set can be treated differently than another or one PDU within the set may be more important than others in the same set. A PDU set in 5G-NR represents a DRB that is finite in time and in the number of packets. In a video flow for example, I-frames or meta data or volumetric video can be more important than other packets or frames within the same flow. Limitation of DRB to meet QoS was discussed in NR XR, where mappings of a QoS flow to multiple DRBs was discussed, but deemed too complex to change at a later stage of 5G.

Observation 6: NR’s bearer and QFI-based QoS framework provides coarse granularity and limited dynamic adaptation e.g. in support of service awareness or change in network or UE conditions.

In EPC, an EPS bearer is the level of granularity for bearer level QoS control. CN can aggregate traffic flows onto the same EPS bearer and all traffic flows mapped to the same EPS bearer receive the same bearer level packet forwarding treatment. CN assigns the bearer level QoS parameter to AN and AN establishes/modifies/releases RBs (Radio Bearer) and performs RB level QoS control on the air interface correspondingly.

One of the key requirements of 5G system design is to achieve the functional decoupling of CN and AN domains. As a result, SA2 introduced a Flow based QoS framework in 5G CN. Instead of the EPS bearer level QoS control and enforcement as in EPC, in this Flow based QoS framework, the 5G CN performs flow level QoS control and enforcement. CN transfers the per-Flow QoS information to AN and AN performs RB-based QoS Control. LTE and NR also use DRBs to differentiate data to different PDU sessions

It is expected that the same key requirement of functional decoupling of CN and AN domains will be maintained for 6G CN QoS framework design.

Observation 7: 5G CN Flow based QoS framework is likely to be the baseline for 6G CN QoS framework.

Observation 8: NR and LTE use radio bearers to differentiate data to different PDU sessions.

## 6GR design targets

6GR aims to evolve the QoS framework to support service differentiation, modularity, extensibility.  Considering the dynamically changing application requirements, traffic characteristics and limitations of 5G QoS framework, the 6GR QoS framework should support the following design targets: a) service awareness, and support for finer QoS granularity for a more precise QoS differentiation; b) Dynamic RAN QoS adaptation e.g. to maintain QoE or application level observed QoS at an acceptable target level; c) Dynamic RAN QoS adaptation with varying QoE or varying application level observed QoS.

Furthermore, 6G aims to support a variety of data types and applications (as discussed in Section 2.1), including best effort data, video, XR and augmented reality applications, AI traffic, and better service awareness for example with various latency and reliability requirements that can vary on a per-packet basis. In some applications (e.g. video, XR, or AR), data packets within a stream can have different QoS parameters or different QoS characteristics, for example priorities, timelines, or PDB, even though they may come from the same data flow. Similar to LTE and NR as described above, 6GR also needs to support differentiation of data to different PDU sessions.

Observation 9: 6G aims to support a variety of data types and applications, where data packets within a stream can have different QoS parameters or different QoS characteristics, for example priorities, timelines, or PDB, even though they may come from the same data flow.

Observation 10: 6GR needs to differentiate data to different PDU sessions.

During NR specification, one aspect of the discussion on QoS framework, the radio protocols and architecture is whether radio bearer concept should be used to model the logical interconnection between the different protocol layers (i.e. the modeling of the logical channels), the procedure to associate data to a specific transmission (i.e. logical channel prioritization) and its control such that QoS levels targeted by NR can be fulfilled over the radio access. We anticipate this discussion to happen for 6G QoS framework as well. A bearer can be abstracted as a configurable logical combination of processing steps applicable to data such that transport of such data can be performed while meeting specific QoS requirements. From that perspective, a radio-bearer like concept may need to be introduced in 6GR.

Observation 11: 6GR QoS framework may need to support a radio bearer-like concept.

Proposal 1: Establish baseline assumptions for 6GR QoS evolution and identify enhancement areas (granularity, flexibility, adaptation).

### Support for Service Awareness

Service-awareness in the RAN context refers to the ability of the Access Stratum to identify, interpret, and act upon information about the service or application generating traffic. NR has been largely application-agnostic, with packet treatment determined by QFIs and DRB mappings. For 6G, native service-awareness aims to allow RAN protocols to utilize metadata and application-level indicators to optimize packet treatment for example scheduling, buffering, and retransmission. Service-awareness implies that the RAN can: a) differentiate packet or group of packets within a data flow (associated with a given QoS flow) based on application type or metadata; b) Identify dependencies between packets within a data flow or across data flows;  c) use QoE-related indicators and application level QoS performance-related indicators to adjust packet handling dynamically; d) adapt QoS parameters (e.g., delay budgets, reliability) based on feedback from the UE or service layer.

The 5G QoS framework operates on the network and transport layer, while many applications requirements reside at the service and application layers. Without application-level context, QoS enforcement is mostly based on flow attributes rather than semantic service knowledge. The 5G QoS framework cannot distinguish between different media segments or adapt based on QoE. Dependency on external enablers e.g., network exposure function, AFs for service-awareness adds complexity and latency.

In summary, the 5G QoS framework enforces generic QoS policies rather than application or service-optimized behavior, reducing effectiveness for adaptive, AI-driven delay-sensitive applications, or applications with dynamically changing traffic characteristics.

Observation 12: Lack of service-awareness limits optimization for AI and immersive applications.

Service-awareness introduces new requirements for Layer 2 protocols for example: a) enable metadata-based mapping to protocol configuration (or device profile) and flexible sub-flow treatment; b) allow selective retransmission and discard based on packet relevance; c) prioritize latency-critical traffic with service-aware scheduling indicators.

Observation 13: Service-awareness imposes L2 requirements for finer granularity, dynamic behavior control, and cross-layer signaling.

Proposal 2: Extend L2 protocols to incorporate service-aware functions while maintaining modularity and backward compatibility.

[Support for dynamically changing traffic characteristics, finer QoS Granularity]

The traffic characteristics for new emerging applications and associated data types exhibit rapidly varying bandwidth, latency, and burstiness. These dynamically changing traffic characteristics, adaptive data flows, and irregular delay-bound UL data bursts pose a challenge to the current QoS framework. In NR, the smallest QoS granularity is the QoS Flow (QFI) which is semi-statically mapped to a DRB for treatment in the RAN, with all packets carried on the same DRB receiving uniform treatment in the protocol stack. This flow-level model does not provide any finer differentiation within a data flow (e.g., per subflow or equivalently per packet or per packet group). In this contribution, we referred to QoS subflow as the per-packet or per packet-group QoS differentiation. As a result, the NR cannot prioritize the level of packet groups. It also cannot handle groups of packets that occur within a single application flow without creating additional QoS flows and DRBs. This coarse granularity of handling packets in NR limits, for example, multimodal synchronization of packets that need to be delivered together to the application especially when the delay budget is tight. Furthermore, the 5G QoS framework does not support dynamic mapping between upper layer packets and QoS treatment in RAN, for example it doesn’t support dynamic mapping between media adaptation layers and QoS parameters. These remain static or policy-driven, not context-aware. Without real-time feedback mechanisms between the UE/application and QoS functions, the QoS framework cannot anticipate or assist in bitrate adaptation, resulting in oscillating performance or degraded QoE.

Observation 14: NR QoS model lacks per-packet or per packet-group granularity

Observation 15: NR QoS model lacks dynamic mapping of packets to packet treatment in RAN

Proposal 3:	6GR QoS framework support service-based differentiation and sub-flow granularity e.g., based on L2 application-level awareness, to allow differentiated QoS treatment on per-packet or per-packet-group basis.

[Support delay-critical uplink data bursts]

The 5G QoS framework does not support time-sensitive uplink bursts with low and deterministic latency. In addition to the grant latency, there are no application triggers to indicate the presence of uplink data burst packets and there is no data burst QoS profile that would provide delay-critical guaranteed burst transmission latency. Using GBR to provide delay-critical guaranteed burst transmission would lead to over-provisioning of system resources.

Observation 16: Delay-critical UL bursts require differentiated treatment and efficient resource use.

Proposal 4: The 6GR QoS framework shall support QoS burst profile parameters for QoS burst guarantees of delay critical bursts of varying volumes and inter-burst interval in a resource efficient manner (e.g., without over-provisioning of radio resources).

[Support of dependencies between PDUs]

Further, there may be also dependencies to manage in the QoS/QoE framework between not only data of the same service, which make QoS vary over time depending on other flows and/or data. For example, there can be inter-flow UP dependencies (e.g., a multi-modal XR service, an XR service that has AR, or a subset of the data such as AR objects, metadata for scene description and/or updates thereof), dependency between user plane data and system data (e.g., sensing of real objects + position to help anchoring the AR objects coherently with the real world), or dependency between UP data and CP data e.g., in case of mobility triggers that initiates from sensing activity that is associated to mobility. This points to the fact that QoS may need to be treated differently, e.g. temporally, for some packets of a data stream.

The 5G system exposes QoS only at the QoS Flow granularity, i.e., all packets with the same QFI receive the same forwarding treatment, so the RAN has no standardized means to enforce inter-PDU dependencies within a flow. At the user plane, the SDAP conveys just the QFI (and flags like RQI) and performs QoS-flow-to-DRB mapping, there is no SDAP field or procedure to signal application-level dependencies between PDUs. Current specification of PDU Set has several limitations for example: a) identification of PDU set or PDUs of a PDU set over the air interface protocols that is left to implementation, b) hierarchical PDU set structure where there could be multiple levels of dependencies between PDUs, with a PDU possibly belonging to more than one PDU set. Furthermore, support for PDU set QoS is limited since differentiated QoS treatment of PDU set mapped to the same QoS Flows is limited with the assumption of mapping of only one PDU set to a QoS Flow. Similarly, support for multi-modality awareness in RAN and particularly over the air interface protocols is limited and left to implementation Meanwhile, PDCP and RLC handle transport functions such as reordering, in-order and out-of-order delivery, segmentation and retransmission, but remain agnostic to application requirements. These mechanisms can change relative arrival times of PDUs without any standardized way to ensure that dependent packets are delivered together or within a short delay.

Observation 17: NR QoS framework mechanisms have no standardized way to ensure dependent packets are delivered together.

Observation 18: Dependencies between data flows, data types, or modalities may require QoS to be treated differently, e.g. temporally, for some packets of a data stream

Proposal 5: 6GR QoS framework supports standardized way to ensure dependent packets are delivered together.

[Support for varying QoE]

The appreciation of QoE from the service consumer perspective (whether it human or not) may vary for various reasons, for example the context in which the service or application is being used, the usage scenario, the relative cost placed on the resources being consumed by the service (e.g. relative importance of UE battery consumption function of the context in which the service is being used), or other subjective factor if the user is human. With the current 5G QoS framework, RAN has limited visibility of the QoE performance and cannot adjust real-time to varying QoE.

Observation 20: NR QoS framework lacks dynamic adaptation to varying QoE

An L2 interface should enable elastic QoE level varying (e.g. somewhere between the min data rate/GBR and max data rate), while QoS is met. QoE must meet a min requirement. For example, the UE may start transmission of data with a given QoE level that is higher, then as UE moves out of coverage, experiences cell congestion, or when data rate drops, the UE may then reduce the QoE level or reduce the number of data packet sets that are transmitted (e.g. if there are multiple/multi-modal streams, the UE may drop one stream that is not necessary to meet the min QoE level while still maintaining the QoS).

Elasticity of meeting QoS requirement means that the QoE level may change (e.g. some where between the min data rate/GBR and max data rate), though QoS is met. QoE must meet a min requirement. For example, the UE may start transmission of data with a given QoE level that is higher, then as UE moves out of coverage, experiences cell congestion, or when data rate drops, the UE may then reduce the QoE level or reduce the number of data packet sets that are transmitted (e.g. if there are multiple/multi-modal streams, the UE may drop one stream that is not necessary to meet the min QoE level while still maintaining the QoS).  Such may be done transparent to RAN, e.g. realized by NW configuration. Another way is that the application itself may adjust the rate of the packet set, and in such case the UE does not change the underlying QoE level (e.g. when the video rate is decreased by the application). This may mean that the UE may or may not be aware of the application layer (no awareness in the first case, but awareness in the second way).

Observation 21: While a QoS requirement can either be met or not met, QoE levels can be changed while meeting QoS to adapt to changing radio conditions or application needs (and vice-versa).

Proposal 6:	6GR QoS framework supports QoS ranging for QoE and/or rate-based adaptations e.g., the UE autonomously selects parameters within the NW-configured range of values.

Proposal 7: 6GR QoS framework supports dynamic adaptation of QoS with varying QoE.

### Support for adaptation to network/UE conditions

This essentially relates to RAN QoS adaptation to change in network or UE state/conditions in order to maintain a given QoE level or application level QoS performance. 5G QoS framework is coarse and only design to react to relatively long-term averages of changes in the network state (e.g. congestion level). With emerging applications and related traffic characteristics, network and UE could see more frequent changes in e.g. load conditions, radio channel conditions (e.g. bursty interference), dynamic device capability change as function of e.g. change in traffic characteristics. 5G QoS is not designed for the type of dynamic adjustment that is required to effectively adapt to these changes in the network or in the UE and maintain a consistent application level QoE and QoS.

Observation 23: NR QoS framework lacks dynamic adaptation to frequent change in NW or UE conditions.

# Conclusion

Observation 1: Emerging AI and immersive traffic introduce new patterns (bursty UL, delay-sensitive, adaptive QoE).

Observation 2: Advanced immersive/XR applications can adapt in rate and/or modalities and/or scene composition to implement graceful QoE adjustments leading to dynamically changing traffic characteristics.

Observation 3: Interactive AI-based services are uplink heavy, latency sensitive, and data sessions are characterized by periodic and/or irregular data bursts with varying volumes and inter-burst intervals.

Observation 4: Interactive real-time services with AI-assistance consist of delay-critical UL data bursts.

Observation 5: UL data burst guarantee reduces burst transmission latency without GBR over-provisioning by enabling UL data burst differentiated treatment and scheduling for data bursts.

Observation 6: NR’s bearer and QFI-based QoS framework provides coarse granularity and limited dynamic adaptation e.g. in support of service awareness or change in network or UE conditions.

Observation 7: 5G CN Flow based QoS framework is likely to be the baseline for 6G CN QoS framework.

Observation 8: NR and LTE use radio bearers to differentiate data to different PDU sessions.

Observation 9: 6G aims to support a variety of data types and applications, where data packets within a stream can have different QoS parameters or different QoS characteristics, for example priorities, timelines, or PDB, even though they may come from the same data flow.

Observation 10: 6GR needs to differentiate data to different PDU sessions.

Observation 11: 6GR QoS framework may need to support a radio bearer-like concept.

Proposal 1: Establish baseline assumptions for 6GR QoS evolution and identify enhancement areas (granularity, flexibility, adaptation).

Observation 12: Lack of service-awareness limits optimization for AI and immersive applications.

Observation 13: Service-awareness imposes L2 requirements for finer granularity, dynamic behavior control, and cross-layer signaling.

Proposal 2: Extend L2 protocols to incorporate service-aware functions while maintaining modularity and backward compatibility.

Observation 14: NR QoS model lacks per-packet or per packet-group granularity

Observation 15: NR QoS model lacks dynamic mapping of packets to packet treatment in RAN

Proposal 3:	6GR QoS framework support service-based differentiation and sub-flow granularity e.g., based on L2 application-level awareness, to allow differentiated QoS treatment on per-packet or per-packet-group basis.

Observation 16: Delay-critical UL bursts require differentiated treatment and efficient resource use.

Proposal 4: The 6GR QoS framework shall support QoS burst profile parameters for QoS burst guarantees of delay critical bursts of varying volumes and inter-burst interval in a resource efficient manner (e.g., without over-provisioning of radio resources).

Observation 17: NR QoS framework mechanisms have no standardized way to ensure dependent packets are delivered together.

Observation 18: Dependencies between data flows, data types, or modalities may require QoS to be treated differently, e.g. temporally, for some packets of a data stream

Proposal 5: 6GR QoS framework supports standardized way to ensure dependent packets are delivered together.

Observation 20: NR QoS framework lacks dynamic adaptation to varying QoE

Observation 21: While a QoS requirement can either be met or not met, QoE levels can be changed while meeting QoS to adapt to changing radio conditions or application needs (and vice-versa).

Proposal 6:	6GR QoS framework supports QoS ranging for QoE and/or rate-based adaptations e.g., the UE autonomously selects parameters within the NW-configured range of values.

Proposal 7: 6GR QoS framework supports dynamic adaptation of QoS with varying QoE.

Observation 23: NR QoS framework lacks dynamic adaptation to frequent change in NW or UE conditions.

# References

- RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator), Prague, CR, June 9-13, 2025.