3GPP TSG-RAN WG2 #132	R2-2508672

Dallas, USA, 17 – 21 November 2025

Agenda item:		10.3.2.3 Initial and System Access and Others

Title:		Discussion on initial access and system information

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN2#131bis, RAN2 discussed the CP aspects including the system information design and paging framework. The following agreements were made [1]. In this contribution, we discuss the initial access procedure, the system information and the paging framework. We also provide our views on these aspects.

Agreements on System Information

- -	System information design should consider energy efficiency and low complexity for both network and UE.

-	System information design should ensure that the public warning requirements are met.

-	Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs)

-	Study how to improve flexibility and extensibility of scheduling SI design

-	Study mechanisms to improve SI update mechanism

-	Study aspects related to SIB1 size.  Understand the issues and desired content/size from RAN2 point of view.

-	Study areas specific SIB design.  Understand 5G pain points and what improvements can be considered.

NOTE: this is a starting list

Agreements

Paging design should consider energy efficiency and simplicity for both UE and Network.

# 2. Discussion

2.1	Initial access procedure

This is the most important aspect involving the transmission/reception of the essential information and signal for the cell access. L1 (physical layer) signaling like synchronization signal (SS) and random access preamble in the 5G NR is the RAN1 scope, while it would be good to consider it in RAN2 from higher layer perspective, e.g. SSB, RACH procedure with taking into account the RAN1 progress.

In the NR, the SSB and the essential SI (MIB, SIB1) are transmitted periodically in PCell. The other SI can be transmitted by On-demand. By introducing the On-demand SI transmission, the signaling overhead and thus network energy can be saved. This could be reused for 6G basically. Furthermore, On-demand SIB1 transmission for Idle/Inactive in a capacity cell (i.e. its coverage is fully covered by other cell (Cell A)) is introduced for network energy saving purpose in Rel-19 NES.

For the NR, there were some more aggressive proposals as the WI scope for NES, although those were not selected as the objectives. For example, On-demand SIB1 in PCell as stand-alone (e.g. coverage) cell. This is also interesting, but RAN2 should consider baseline approach at first by taking NR as the starting point..

Proposal 1: RAN2 study initial access procedure by assuming that in the NR as starting point (i.e. always-on SSB, essential SI from broadcasted MIB and SIB1, CBRA procedure).

The initial access procedure includes the following aspects assuming the contention-based 4 Step RA:

- MIB/SIB1 reception

- MSG1: Random access preamble

- MSG2: Random access response (RAR)

- MSG3: RRC Setup Request

- MSG4: Contention Resolution (with or followed by RRC Setup)

- MSG5: RRC Setup Complete

RA preamble transmission should be common for all the UEs in a cell, i.e. RRC_Idle, RRC_Inactive (if supported) and RRC_Connected regardless of the purposes of the RA. Similarly, the RAR design should be common with flexibility for the contents. The RA preamble partition per use case or feature as in the NR may be studied. On one hand, the partitioning is technically useful to have differentiation among use cases or features. On the other hand, it unfortunately causes the resource fragmentation. For 6G, it would be good to study its applicability, while it should be carefully designed from future-proof (forward compatibility) perspective.

For the MSG3 transmission, it is always important what information should be included, because it tends to be size limited due to its design principle. As this issue is highly related to the RAN1 study, RAN2 should try to identify essential information and consider possibility of some additional information which will be useful in the following steps. Another aspect for the MSG3 is the early identification of specific type of UE via a special LCID (e.g., RedCap indication). This is useful for the network side to handle the accessing UE properly. However, this would impact on the LCID pool design. Considering the benefits and the drawbacks in this approach of the NR, 6G design should be smarter rather than just reusing such the special LCID as it is in the NR.

For the MSG4, at least a certain mechanism of contention resolution has to be supported for the CBRA. If there are any critical issue in the field, RAN2 should reconsider this as well. We do not see any yet for now. For the MSG5, this is more flexible message than the others. RAN2 can study its usage after having more progress.

Proposal 2: RAN2 study the following aspects for the initial access procedure:

- MSG1:

  - whether to support preamble partition per use case(s) or feature(s)?

- MSG3:

  - what information to be essential?

  - whether to support early identification of specific UE types?

There is one different aspect related to the initial access. In the NR, the network can provide SCell only cell without SIB1 transmission. Further, there is the concept of SSB-less SCell. These implies that the network can configure some cells (e.g. Group A) for cell camping and some other cells (e.g. Group B) for data transmission purpose only. For examples, a UE in RRC_IDLE can camp on a cell #1 of the Group A. The UE can perform the initial access procedure on the cell #1. While the UE in RRC_CONNECTED, the UE may use a cell #2 of the Group B as the SCell of the carrier aggregation. Also, it is up to network whether the UE should stay in this cell #1 or move to the cell #2 as the PCell, unless the cell #2 is SSB-less cell.

For 6G, it would be good to study this kind of cell (or carrier) type differentiation a bit more generic, as the fundamental design of a cell or a carrier is under study from scratch. For example, one type of cells may be mainly used for the Idle mobility purpose, while the other type of cells may be used only for the Connected purpose including the data transmission and the mobility. With this, the essential signal and/or information on each type of the cell or carrier can be optimized to fit (main) purpose of each type, respectively. However, there should be comparison of pros and cons, e.g. a drawback may be that many UEs may camp on the same cells, while the paging transmission can be limited to those cells.

Proposal 3: RAN2 study possibility of the cell/carrier type differentiation to optimize the cell/carrier design to fit its purpose(s) (e.g., cell camping, data only).

2.2	System information details

In the last meeting, RAN2 agreed some study directions, e.g. design target (energy efficient, low complexity), requirement (PWS support), and potential improvement areas (e.g., SI scheduling flexibility, SI update).

Energy efficiency

For the energy efficient and low complexity, we assume other SI than the essential SI (MIB, SIB1) should be broadcast via on-demand manner like the NR. The main question is whether to support the on-demand SIB1. Our view is that at least on-demand SIB1 in a capacity cell overlapped by other cell(s) like Cell A in 5G NES　can be considered. This would give network energy saving gain without causing any critical issue. On the other hand, it should be studied whether on-demand SIB1 can be applied for a standalone cell, which is used by the RRC_Idle UEs as camping cell and by the RRC_Connected UEs as PCell.

Proposal 4: RAN2 study feasibility/need of on-demand SIB1 transmission in a standalone cell, in addition to that in a capacity cell covered by a coverage cell.

SIB1 contents

Regarding the SIB1 contents, the Table 1 below roughly shows the SIB1 contents and their essentiality in 5G NR. In SIB1, there must be at least information which are essential to cell camping and cell access from the UE perspective, for example the Cell Selection Info, Cell Access Related Info, SI Scheduling Info for other SI and Serving Cell Config Common for SIB. In high-level discussion, 6G will be the same as 5G. In a certain condition (e.g., cell is overloaded due to massive accesses from too many UEs gathered for a special event), the system must have a way to solve the congestion. For this purpose, the access control mechanism (e.g., UAC Barring Info) should be included, although it may not always be necessary. In addition, it is also important to include the indication of emergency call support (e.g., IMS Emergency Support).

In 5G NR, some device type specific or the use case specific information (e.g., RedCap, NTN) are also included due to late introduction of such device types or use cases. Regarding the device types in 6G, RAN plenary is discussing under the 6G RAN SI and RAN2 should wait for the progress. In general, we assume multiple device types should be supported by the common system design as much as possible in 6G. It is important to study possibility of the way to avoid the device type specific or use case specific indication in SIB1 including a future proof, once we see what or how many device types should be expected.

Observation 1: SIB1 contents of 6G will be similar to those in the SIB1 of 5G NR, while it should be studied further after some progress, e.g., on functionality, device types, or use cases in 6G.

Table 1. SIB1 message contents in 5G NR

| Information (examples) | Contents (examples) | Essentiality |
| --- | --- | --- |
| Cell Selection Info | Parameters associated with evaluation of quality and Rx level of serving cell | Essential for cell camping |
| Cell Access Related Info | PLMN info (e.g. PLMN ID, TAC, RAN area code, Cell identity), cell reservation info, NPN info | Essential for cell access |
| SI Scheduling Info | Information for acquisition of SI messages (e.g. SIB mapping, window, periodicity, SI request config) | Essential for cell access, service |
| Serving Cell Config Common | Cell specific parameters (e.g. PCI, DL/UL common configuration, SSB positions within a burst, TDD config, NTN config) | Essential for cell camping and cell access |
| UAC Barring Info | Access control parameters | Crucial under certain condition (e.g., cell overload) |
| IMS Emergency Support | A flag for support of IMS emergency call | Vital for emergency |
| HSDN Cell | A flag for support of HSDN | Important in HSDN cell deployment |
| RedCap Config Common | A flag for support of RedCap, RedCap specific cell barring, cell configuration | Essential for RedCap UEs |
| Cell Barred NTN | NTN specific cell barring | Essential for NTN UEs |
| Cell Barred NES | NES Cell DTX/DRX specific cell barring (only sent in notBarred case) | Important to make feature beneficial |
| Idle Mode Measurements | A flag of support of HSDN | Important to make feature beneficial |
| . . . |  |  |

SI scheduling

For the potential improvements, SI scheduling flexibility would be good to think about. In the NR, each SI message is basically transmitted from the beginning of the SI window. Within the SI window for one SI, other SI cannot be sent. This way is very simple, and it is clear when each SI transmission is started. However, increasing the number of SIBs and thus number of SIs would cause inefficiency for UEs to receive multiple SI messages which are distributed in time. In NR Rel-17, the new concept to indicate the SI window position is introduced to explicitly indicate when the corresponding SI is transmitted. The network can decide its position as long as there is no overlap among SI messages.

Proposal 5: RAN2 study efficient SI scheduling mechanism, which is better than the basic mechanism in NR.

2.3	Paging framework

The paging framework is one of essential system design. Usually, the paging messages can be distributed by using the paging frame (PF) and paging occasion (PO). With sparse PO, the paging overhead can be reduced, while the paging latency and capacity tend to be smaller. There should be reasonable and flexible balances between the paging latency/capacity and the overhead.

On top of that, in Rel-19 NR NES, paging adaptation is introduced in time domain by extending the PF interval (i.e., N) and increasing the number of POs within a PF (i.e., Ns) to compensate for the reduced number of PFs. This allows the network to have longer consecutive sleep periods, thereby reducing power consumption. To ensure backward compatibility with legacy UEs, a dedicated paging configuration, which is signalled separately in the system information, is introduced for UEs supporting paging adaptation.

Unfortunately, Rel-19 paging adaptation can only be based on semi-static configuration and is not applicable to legacy UEs, which limits its effectiveness and reduces flexibility in updating paging parameters. In 6G, paging adaptation may be studied from Day1 without such constraint, and more dynamic approaches—such as L1-based mechanisms—can be considered to enable faster and more efficient paging adaptation by considering the paging load. For example, a paging adaptation mechanism triggered by LP-WUS or DCI (e.g., short message) can be introduced to dynamically specify the applicable paging parameter set.

Proposal 6: RAN2 to study paging adaptation from 6G Day1, and more dynamic approaches (e.g., L1-triggered) can be considered to enable faster and more efficient paging adaptation.

There is an inherent association between SSBs and paging monitoring occasions. Specifically, each PDCCH monitoring occasion for paging (PMO) is associated with an SSB for beam determination. Furthermore, prior to performing paging monitoring, the UE shall acquire 2 or 3 SSBs to achieve finer synchronization. This implies that any change in the SSB periodicity or SSB pattern may impact the paging monitoring. For example, if the SSB periodicity is extended to 160ms, keeping a dense paging cycle with a periodicity smaller than the SSB periodicity may not be optimal. Therefore, to achieve a more energy-efficient system, 6G could study the joint design or adaptation of common channels and signals, such as coordination between SSB and paging. Specially, extending the SSB periodicity would lead to a corresponding extension of the paging cycle.

Basically, the common channel design (e.g. SSB) is led by RAN1, so RAN2 should collaborate with RAN1 for system-wide optimization for the paging.

Proposal 7: RAN2 to study joint design or adaptation of common channels and signals (e.g., between SSB and paging) by collaborating with RAN1.

# 3. Conclusion

In this contribution we discussed the initial access procedure, the system information and the paging framework. We made the following proposals.

Initial access procedure

Proposal 1: RAN2 study initial access procedure by assuming that in the NR as starting point (i.e. always-on SSB, essential SI from broadcasted MIB and SIB1, CBRA procedure).

Proposal 2: RAN2 study the following aspects for the initial access procedure:

- MSG1:

  - whether to support preamble partition per use case(s) or feature(s)?

- MSG3:

  - what information to be essential?

  - whether to support early identification of specific UE types?

Proposal 3: RAN2 study possibility of the cell/carrier type differentiation to optimize the cell/carrier design to fit its purpose(s) (e.g., cell camping, data only).

System information

Proposal 4: RAN2 study feasibility/need of on-demand SIB1 transmission in a standalone cell, in addition to that in a capacity cell covered by a coverage cell.

Proposal 5: RAN2 study efficient SI scheduling mechanism, which is better than the basic mechanism in NR.

Paging framework

Proposal 6: RAN2 to study paging adaptation from 6G Day1, and more dynamic approaches (e.g., L1-triggered) can be considered to enable faster and more efficient paging adaptation.

Proposal 7: RAN2 to study joint design or adaptation of common channels and signals (e.g., between SSB and paging) by collaborating with RAN1.

# References

[1] RAN2#131bis Chair notes