3GPP TSG-RAN WG2 #132	R2-2508744

Dallas, US, November 17th – 21st, 2025

Agenda item:	  9.3.2

Source: 	Ericsson

Title:  	Discussion on RRM Measurement Prediction

Document for:	Discussion and Decision

# Introduction

Rel-20 Work Item RP-252899 [1] includes the objectives which will be treated during the Rel-20 activity. In this contribution we will focus on the part related to the partial and full inference configuration configurations, toggling between inference and legacy RRM configuration as well as L3 filtering process when measurements and predictions are used together for RRM purpose.

# Discussion

## Considerations on applicability and inference configuration handling

In the following we discuss different aspects concerning the LCM procedure in particular handling inference configurations in conjunction with its applicability status.

Full and partial inference configuration

Full configuration refers to the scenario when the UE receives the full inference configuration from the network and based on that, the applicability is reported. Alternatively, for the partial configuration scenario, the intention is that the UE receives a subset of the inference configuration (so-called as partial configuration) and determines whether the AI/ML model is applicable or a change in configuration is required.

We understand that full and partial configuration is supported in Rel-19 AI/ML PHY beam management use case, however in our understanding there is not much difference on configured parameters between full and partial configuration for AI/ML PHY beam management. Therefore, it is necessary to clarify the rationale for adopting the partial configuration for AI/ML for mobility. It is expected that an agreement on the selection of the inference parameters to use for the partial configuration requires a lot of discussion in RAN2 and, considering that the definition of the full inference configuration for the different types of prediction is not completed, we propose to finalize the definition of the full inference configuration before discussing the partial inference configuration.

RAN2 first discuss full inference configuration, before discussing partial inference configuration.

Before supporting partial inference configuration, RAN2 should discuss whether there is any meaningful difference between full and partial configurations.

Toggling between inference configuration and legacy RRM measurement configuration

Concerning the applicability reporting for the AI/ML for Mobility use case, AI for PHY agreements are taken as baseline. However, we think there is a substantial difference between AI for PHY and AI for Mobility. In AI for PHY the existing mechanism in activating and deactivating the CSI reports based on the measurement and/or prediction is very handy and helps in designing the solution merely based on applicability reporting. However, in AI for Mobility upon receiving an applicability reporting indicating that AI based functionality is not applicable, network should use RRC signal to reconfigure the UE with the legacy RRM measurements. This might become a bottleneck as the UE might bounce between applicability and non-applicability often, causing a signalling increase over air interface.

Unlike AI for Beam Management, where the gNB can (de)activate the inference via existing MAC CE, (de)activation of AI for Mobility requires an RRC reconfiguration which makes (de)activation more costly, in particular if the UE frequently switches between applicable and non-applicable states.

Instead, we think network should be able to pre-configure the UE with legacy RRM measurement configuration as well as inference configurations to avoid unnecessary signalling and unnecessary coordination upon change of the applicability status.

In this approach, the UE should be able to toggle between different configurations (legacy RRM measurement configuration and inference configuration) depending on the applicability status.

- If the inference configuration becomes inapplicable, the UE falls back to the (preconfigured) RRM measurement configuration and performs measurements

- If the inference configuration becomes applicable, the UE switches to the (pre-configured) inference configuration and performs predictions.

- Sending applicability indication when toggling between RRM measurements and/or prediction configurations should be optionally configured by the network via otherConfig.

Therefore, we propose the following:

RAN2 discuss UE’s toggling mechanism between preconfigured legacy RRM measurements configuration and inference configuration based on applicability status.

- If the inference configuration becomes inapplicable, the UE immediately applies the preconfigured measurement configuration and performs legacy measurements.

- If the inference configuration becomes applicable, the UE immediately applies the preconfigured inference configuration i.e., stop legacy measurements and start predictions.

## Associated ID

During the SI phase, the associated ID was discussed but it has been left to WI phase to decide whether and how to configure associated ID in AI/ML for mobility.

The intention of the associated ID is to ensure the consistency between the training and the inference with network side condition changes. In AI for PHY, associated ID indicates the properties of a DL Tx beam or beam set/list associated with set A and set B. However, there is a substantial difference between AI for PHY and AI for mobility: associated ID in AI for PHY is necessary to indicate the beam pattern for UE to perform predication and applicability reporting, whereas the discussion in AI for mobility is more about changes in network coverage for each cell/frequency or between different cells/frequencies. In an example, consistency issue (during training and inference) might stem from energy-saving policies in the network, where more cells might be active and providing coverage and capacity during model training, while some of the capacity cells may be turned off during inference which makes the neighbour cell’s prediction.

Unlike AI for PHY, where consistency issues (during training and inference) stem mainly from beam properties, in AI for Mobility, consistency issues are primarily associated with the changes in network coverage (during training and inference).

In an example, consistency issue stems from energy-saving policies in the network, where more cells might be active and providing coverage and capacity during model training, while some of the capacity cells may be turned off during inference which makes the neighbour cell’s prediction inconsistent.

However, we think the UE can detect such inconsistencies through proprietary mechanisms locally at the UE or with the help of over-the-top server. In such solutions the UE can perform measurements and scan the network environment (e.g., by comparing the cardinality of the detected cells and/or their measured radio quality during data collection and inference) to determine whether the network environment (number of cells and their coverage) have changed or not. We think a proprietary UE implementation can easily detect such network environment changes or differences between different cells/frequencies and determines whether the inference configuration is (in)applicable based on the network environment.

In light of the above, and given that the UE side can easily detect the consistency issues (across data collection for model training and inference phases), associated ID is not needed for AI for mobility use cases.

It is up to UE implementation to ensure the consistency between the training and the inference. No associated ID is needed for AI for Mobility use case.

## L3 filtering mechanism for combination of measurement and prediction

L3 filtering is a process used to smooth or average RRM measurements over time, helping to reduce the impact of short-term fluctuations and provide more stable parameters for HO decision-making.

In the case of temporal prediction Case B, the UE may substitute some of the actual measurements with predicted values (e.g. predicted RSRP). Consequently, L3 filtering mechanism needs to integrate and process both actual and predicted measurement data together to produce an accurate filtered output.

In case of temporal prediction Case B, the UE needs to perform L3 filtering on a combination of actual and predicted measurements samples.

RAN2 should evaluate if the existing L3 filtering process needs to be updated to effectively accommodate the dual data sources (actual and predicted measurements) when the UE uses temporal domain Case B.

RAN2 discuss L3 filtering process when temporal domain prediction Case B is used together with legacy RRM measurements.

# Conclusion

In the previous sections we made the following observations:

- Unlike AI for Beam Management, where the gNB can (de)activate the inference via existing MAC CE, (de)activation of AI for Mobility requires an RRC reconfiguration which makes (de)activation more costly, in particular if the UE frequently switches between applicable and non-applicable states.

Unlike AI for PHY, where consistency issues (during training and inference) stem mainly from beam properties, in AI for Mobility, consistency issues are primarily associated with the changes in network coverage (during training and inference).

In an example, consistency issue stems from energy-saving policies in the network, where more cells might be active and providing coverage and capacity during model training, while some of the capacity cells may be turned off during inference which makes the neighbour cell’s prediction inconsistent.

- Based on the discussion in the previous sections we propose the following:

- RAN2 first discuss full inference configuration, before discussing partial inference configuration.

Before supporting partial inference configuration, RAN2 should discuss whether there is any meaningful difference between full and partial configurations.

RAN2 discuss UE’s toggling mechanism between preconfigured legacy RRM measurements configuration and inference configuration based on applicability status.

- If the inference configuration becomes inapplicable, the UE immediately applies the preconfigured measurement configuration and performs legacy measurements.

- If the inference configuration becomes applicable, the UE immediately applies the preconfigured inference configuration i.e., stop legacy measurements and start predictions.

It is up to UE implementation to ensure the consistency between the training and the inference. No associated ID is needed for AI for Mobility use case.

RAN2 discuss L3 filtering process when temporal domain prediction Case B is used together with legacy RRM measurements.

# References

RP-252899 - Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, OPPO, Interdigital, RAN#109, September 2025